3GPP TSG RAN WG2 Meeting #132                          	   R2-2508754

Dallas, USA, 17th – 21st November 2025

Agenda item:	10.3.3.2

Source:	Qualcomm Incorporated

Title:	Discussion on 6G energy and power saving features

WID/SID:	FS_6G_Radio

Document for:	Discussion

# Introduction

RAN#108 approved new SI for 6G Radio (6GR) which was further revised at RAN#109 [1]. One of the objectives of the study is as follows:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

#### At RAN2 #131bis, network and UE energy efficiency had been discussed over the following aspects [2]:

- design target,

- alignment of power saving features,

- time domain, and

- frequency domain.

This contribution discusses energy and power saving features for 6G.

# Discussion

## UE DRX

Without requiring direct signaling from the network to a UE, DRX allows the UE to wake up or stay active at an active time (e.g., DRX on duration, or Inactive timer or HARQ retransmission timer is running) to monitor for DCI transmissions. During the non-active time, the UE may enter sleep or deep sleep state for power saving.

Observation 1. UE DRX effectively reduces UE power consumption by enabling autonomous transitions between active and sleep states.

A network node can improve system-level resource efficiency by aligning the UE’s DRX cycle with its traffic patterns. For example, based on the UE’s data transmission patterns, the network can schedule traffic during the DRX active time  which further optimizes radio resource usage.

Observation 2. UE DRX allows network nodes to allocate resources more effectively, enhancing overall system efficiency.

In RRC Idle or Inactive state, the UE's DRX cycle plays a critical role in determining when and how the network can reach the device. Specifically, it is used to allocate the appropriate paging frame and paging occasion(s), allowing the UE to periodically wake up and check for incoming paging messages. This mechanism ensures that the UE remains reachable by the network while spending most of its time in a low-power sleep mode, thereby significantly reducing energy consumption without compromising connectivity.

Observation 3. UE DRX serves as the underlying feature that enables the network to page a UE and allows the UE to monitor for paging messages.

In addition, network nodes can leverage UE DRX patterns to further enhance their own energy efficiency. By aligning the timing and patterns of UE activity within the DRX cycle, the network can make informed decisions about when to activate or deactivate transmissions, such as configuring cell-level DTX (i.e., cell-DTX) or cell-level DRX (i.e., cell-DRX). This approach allows the network to align its operational cycles with UE behavior, minimizing unnecessary transmissions and conserving energy across the system.

Observation 4. UE DRX can play a valuable role in enhancing the energy efficiency of network nodes.

Based on the above observations, we propose the following proposal.

Proposal 1. UE DRX is adopted as a key feature in 6G to support energy and power saving for both UEs and network nodes.

As 6G networks are expected to support a wide range of applications - from low-latency XR to sporadic IoT transmissions, DRX mechanisms must evolve to accommodate varying data volumes and arrival rates as well as service requirements such as latency or reliability.

In the case of extended reality (XR) applications, traffic patterns may be highly variable and bursty, requiring more flexible UE DRX configurations. For example, the use of non-uniform periodicity in DRX cycles may allow a UE to adapt its wake-up intervals based on the dynamic nature of XR data transmission. This approach can improve responsiveness while maintaining energy efficiency, especially in scenarios where latency and power constraints must be balanced.

With the integration of artificial intelligence and machine learning (AI/ML) into 6G communication system, UE DRX configurations may evolve to become more adaptive and intelligent. As an AI-native feature, DRX could enable UEs to dynamically manage their timers and determine when to exit active state based on learned traffic patterns, application requirements, or contextual awareness. This flexibility allows the UEs to optimize their wake-up intervals and sleep durations in real time, improving responsiveness while maintaining energy efficiency, especially in complex scenarios such as mobile AI generated traffic.

Observation 5. UE DRX mechanisms may be further enhanced in 6G to provide greater flexibility in adapting to increasingly diverse and dynamic traffic patterns.

In 6G, cell deployment is expected to become increasingly complex due to the integration of low-frequency bands with high-frequency bands such as FR2 and FR3. This multi-band architecture introduces challenges in maintaining consistent UE behavior across different frequency carriers. Optimizing UE DRX configurations across diverse carriers or carrier groups requires careful consideration of factors such as coverage disparity, propagation characteristics, and traffic distribution. For example, a UE connected to both a low-frequency anchor carrier and a high-frequency data carrier may need differentiated DRX cycles to balance responsiveness and power efficiency. Studying how DRX can be intelligently adapted in such multi-carrier environments is essential to ensure seamless connectivity and energy-efficient operation in 6G networks.

Observation 6. UE DRX mechanisms may be further enhanced in 6G to effectively support increasingly complex carrier deployment scenarios.

In light of the above observations, we recommend the following proposal.

Proposal 2. RAN2 studies on potential enhancements to UE DRX mechanisms in 6G, with the goal of improving both energy efficiency and communication performance.

## Downlink wake-up signal

At RAN1 #122 bis, the following agreements were made for downlink wake-up signal (DL WUS) [3].

Agreement Study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement, regarding at least the following aspects: - Coverage target for DL WUS (e.g., same as PDCCH, common sync signal, or other) - Measurements and/or synchronization. - System overhead and network energy consumption/UE energy saving for UE operation with the DL WUS. - RRC states - Other functionalities Agreement For evaluation purposes, study extending NR UE power consumption model for UE operation with DL WUS of OFDM-based sequence, regarding the following aspects: - Power state(s), sleep and non-sleep, and corresponding characteristics and power value(s) - Transition energy and time for each of sleep state(s) Companies to report the assumption(s) for achieving the proposed power value(s), e.g., time/frequency domain detection, noise figure assumption(s), synchronization assumption(s), BW/antenna assumption(s), etc.

With anticipated performance enhancements, such as coverage comparable to the PDCCH, synchronization capabilities similar to common synchronization signals, and other signals, the DL WUS, or Low-Power Wake-Up Signal (LP WUS), is expected to play a pivotal role in 6G systems. DL WUS can support a range of critical functions, including synchronization, radio measurements, and energy-efficient operations. For example, DL WUS can enable UEs to remain in low-power state while still being able to monitor for paging messages or other network signaling. This mechanism helps reduce unnecessary wakeups and signaling overhead, contributing to both UE and network energy savings. As 6G aims to support diverse use cases and device types, DL WUS may become a basic feature for maintaining connectivity with minimal power consumption (e.g., low power operation).

Observation 7. The low power wake-up signal (LP WUS) is expected to become a basic feature in 6G systems, enabling devices to maintain continuous network connectivity while operating in low-power state.

Based on RAN1’s agreement and the above observation, we propose the following proposal.

Proposal 3. To ensure alignment across working groups, RAN2 may start discussions on LP WUS at a high level until RAN1 has made sufficient progress in its ongoing evaluations.

## Cell-DTX and cell-DRX

In 5G NR, cell-DTX and cell-DRX are key mechanisms designed to improve energy efficiency at the network and UE.

Cell-DTX enables a gNB to enter a low power or energy saving operational mode by intentionally muting its downlink transmissions during specific time intervals when there is no user data or control signaling to be sent. This mechanism is particularly effective in scenarios with low traffic demand or during periods when connected UEs are in DRX sleep states and not actively receiving data. By suppressing unnecessary transmissions, such as PDCCH, PDSCH, or reference signals, the gNB can significantly reduce power consumption and minimize interference to neighboring cells.

Cell-DRX enables a gNB to enter a low-power or energy saving operational mode during cell-DRX “off” time. During this time, the gNB minimizes its activity while still maintaining essential network functionalities, such as monitoring random access channel (RACH) attempts from UEs. This approach enables the gNB to conserve energy without compromising its ability to respond to critical uplink events or maintain baseline connectivity with UEs.

In 6G networks, particularly in scenarios involving dense or heterogeneous deployments, there is considerable opportunity to further enhance the efficiency of cell-DTX and cell-DRX mechanisms to achieve greater energy and power savings on both the network and UE sides. As network topologies become more complex, with a mix of macro cells, small cells, and diverse frequency bands, the ability to intelligently manage transmission and reception activity becomes increasingly important. Optimizing cell-DTX allows network nodes to reduce unnecessary downlink transmissions during periods of low traffic, while enhanced cell-DRX strategies enable the network to align its operational cycles with UE activity patterns. Together, these mechanisms can minimize energy consumption, reduce interference, and support sustainable network operation, especially in environments with fluctuating traffic loads and diverse service requirements.

Observation 8. In 6G, cell-DTX and cell-DRX mechanisms can be further optimized to support more diverse network deployment scenarios.

As 6G networks are expected to support an increasingly variety of services, it becomes essential that energy-saving mechanisms like cell-DTX and cell-DRX should be carefully optimized so that the optimizations must not only reduce power consumption at both the gNB and UE sides but also ensure that the stringent performance requirements of each service type are fully met For example, XR or immersive communication may require non-uniform or non-integer DRX periodicities which cannot be well aligned with the current cell-DTX or cell-DRX. Intelligent coordination of transmission and reception cycles, potentially enhanced by AI/ML-based traffic prediction and context-aware scheduling, can allow the network to suppress unnecessary activity while still delivering the required levels of reliability, latency, and throughput. Achieving this balance is critical to enabling sustainable and scalable 6G deployments without compromising the quality of experience for end users or the performance guarantees expected by diverse applications.

Observation 9. In 6G, cell-DTX and cell-DRX mechanisms can be further optimized to intelligently suppress unnecessary transmission and reception activities while still delivering the required performance such as reliability, latency, and throughput.

Based on the observations, we propose the following:

Proposal 4. RAN2 starts the study on the applicability and potential enhancements of cell-DTX and cell-DRX in 6G, with a particular focus on improving energy and power saving without compromising system performance or user experience of diverse 6G services.

In 6G, cell-DTX and cell-DRX mechanisms may be further optimized in coordination with UE DRX configurations or based on UE-specific preferences and behavior patterns. As 6G networks are expected to support a wide range of device types and service requirements, enabling flexible and adaptive coordination between network-side and UE-side energy-saving features become increasingly important. For example, a network node may dynamically adjust its transmission and reception activity based on the DRX cycles of connected UEs, or even tailor its behavior to individual UE preferences, such as latency sensitivity, application type, or historical traffic patterns. This joint optimization can help maximize energy efficiency for both network and UEs while ensuring that the network continues to meet the performance expectations of diverse 6G services or 6G UE types.

Observation 10. In 6G, cell-DTX and cell-DRX mechanisms can be further optimized jointly with UE’s DRX or preferences to maximize energy efficiency for both network and UEs while ensuring performance expectations.

Based on the observation, we propose the following:

Proposal 5. RAN2 starts the study on the applicability of joint optimization with cell-DTX and DRX and UE DRX or UE preferences in 6G.

# Conclusion

In this contribution, we discussed the energy and power saving features for 6G and concluded with the following observations and proposals.

## UE DRX

Observation 1. UE DRX effectively reduces UE power consumption by enabling autonomous transitions between active and sleep states.

Observation 2. UE DRX allows network nodes to allocate resources more effectively, enhancing overall system efficiency.

Observation 3. UE DRX serves as the underlying feature that enables the network to page a UE and allows the UE to monitor for paging messages.

Observation 4. UE DRX can play a valuable role in enhancing the energy efficiency of network nodes.

Proposal 1. UE DRX is adopted as a key feature in 6G to support energy and power saving for both UEs and network nodes.

Observation 5. UE DRX mechanisms may be further enhanced in 6G to provide greater flexibility in adapting to increasingly diverse and dynamic traffic patterns.

Observation 6. UE DRX mechanisms may be further enhanced in 6G to effectively support increasingly complex carrier deployment scenarios.

Proposal 2. RAN2 studies on potential enhancements to UE DRX mechanisms in 6G, with the goal of improving both energy efficiency and communication performance.

## Downlink wake-up signal

Observation 7. The low power wake-up signal (LP WUS) is expected to become a basic feature in 6G systems, enabling devices to maintain continuous network connectivity while operating in low-power states.

Proposal 3. To ensure alignment across working groups, RAN2 may start discussions on LP WUS at a high level until RAN1 has made sufficient progress in its ongoing evaluations.

## Cell-DTX and cell-DRX

Observation 8. In 6G, cell-DTX and cell-DRX mechanisms can be further optimized to support more diverse network deployment scenarios.

Observation 9. In 6G, cell-DTX and cell-DRX mechanisms can be further optimized to intelligently suppress unnecessary transmission and reception activities while still delivering the required performance such as reliability, latency, and throughput.

Proposal 4. RAN2 starts the study on the applicability and potential enhancements of cell-DTX and cell-DRX in 6G, with a particular focus on improving energy and power saving without compromising system performance or user experience of diverse 6G services.

Observation 10. In 6G, cell-DTX and cell-DRX mechanisms can be further optimized jointly with UE’s DRX or preferences to maximize energy efficiency for both network and UEs while ensuring performance expectations.

Proposal 5. RAN2 starts the study on the applicability of joint optimization with cell-DTX and DRX and UE DRX or UE preferences in 6G.

# References

- RP-252912, Revised SID: Study on 6G Radio, RAN#109, Beijing, China, September 15th-18th, 2025.

- “R2_131bis_ChairNotes_25-10-17_13-00”, RAN2 #131bis, Prague, Czech Republic, Oct. 13th-17th, 2025.

- “Chair notes RAN1#122b eom”, RAN1 #122bis, Prague, Czech Republic, Oct. 13th-17th, 2025.