3GPP TSG-RAN WG2 Meeting #132	R2-2508766

Dallas, USA, Nov 17th- 21st, 2025

Agenda item:		10.3.3.1

Source:	Rakuten

Title:	Discussion on AI-ML, ISAC, SON/MDT, QoE data transfer

Document for:		Discussion and Decision

# Introduction

This contribution discusses aspects which impact both user plane and control plane.

The SID of Study on 6G Radio in RAN #108 includes the following objectives [1]

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

The highlighted text mentions about the data collection for AI-ML use-cases and sensing data from the UEs/devices and network support to transfer this data.

# Discussion

The below direction was provided in RAN2#131bis, wrt data collection

For next meeting companies can consider at least the following aspects:

- Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc);

  - For each type of data, study:

  - Applicable use case(s)

  - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM;

  - Data size in a single reporting and total data size

  - Frequency of data reporting.

  - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

In this contribution, we analyse the characteristics of the diverse types of data mentioned, i.e AI/ML related data, sensing data, QoE, SON/MDT, etc and present our views regarding transfer of data in 6GR

Note: We refer to the different types of data including AI-ML, sensing, QoE, SON/MDT etc as non-CP/UP data for ease of reference in this document.

## AI-ML, sensing, QoE, SON/MDT data in 6GR (non-CP/UP data)

### Key requirements of AI-ML, Sensing data, SON/MDT, QoE etc

In our understanding, some of the key requirements associated with data collection and data transfer of the different types of data over the air-interface are listed below.

AI-ML Specific

- UE data collection has to be performed for the training of UE models, network models or two-sided models.

- The information transfer involves huge data volume, as it encompasses multiple use-cases associated with thousands of UEs/devices, where the termination points associated with each use-case could be different eg, RAN, CN, O&M, or OTT server. Therefore, this data transfer requires configurable data termination point.

- Data management functions in 6G should enable flexible prioritization, monitoring, and visibility of data and model exchanges, allowing initiation, termination, and configuration of collection and transfer with support for multiple termination points and differentiation of AI/ML from CP/UP traffic.

- Data control functions may be implemented in the RAN, the core network, or both; however, they must ensure MNO visibility and support the initiation, termination, and configuration of data collection and transfer for use-case requirements of both CN and RAN.

- Data reuse is a key requirement to avoid duplicated data collection from UE/NF, as UEs keep moving from one NG-RAN node to another.

- Enhanced network controllability is required to enable efficient configuration and monitoring of data transfers, while minimizing the impact of AI/ML-related traffic on conventional control and user-plane services. In 6G, greater visibility into both data and model/parameter transfer will allow networks to manage resources intelligently and prevent AI/ML operations from affecting other services.

- AI/ML model training data generally has no strict latency or bit rate requirements. They are delay-tolerant traffic.

Sensing specific

- 6G sensing (e.g., radio-based environment mapping, object detection, motion tracking) produces large, high-resolution datasets.

- Sensing data are time-critical — they must be captured, processed, and fused within tight latency bounds to support real-time applications (e.g., autonomous driving, industrial robotics).

- It is classified as highly sensitive, requiring strong security, anonymization, and consent-based access control.

SON/MDT, QoE etc

- 5G data collection is constrained by fragmented mechanisms—such as SON/MDT, Trace, QoE, and OAM—along with inefficient transport over SRBs/DRBs and redundant measurements across use cases, leading to higher overhead and system complexity.

- SON/MDT can generate high volumes of measurement reports, especially in dense 5G deployments, but data flow is predictable, making it suitable for batch processing or aggregation at the OAM.

- SON/MDT data must be timely, accurate, and reliable to ensure correct automated actions.

Common to all

- The UE data collection for network use-cases should have minimal impact to the UE battery, CPU Processing and memory utilization.

- The UE data collection for network use-cases should have minimal impact to UE power saving features such as UE-DRX, CELL DTX/DRX, RRC inactive state, and other NES features.

- The system shall ensure enhanced security protection for data generated by AI/ML, sensing functions, SON/MDT, QoE, etc, recognizing the high sensitivity of such information.

- The system shall safeguard user privacy through consent-based data handling and anonymization mechanisms.

### Predicted size of non-CP/UP data in 6GR

### Estimating the size of AI/ML and sensing data in 6G based on several factors like ultra-dense networks, massive MIMO, THz bandwidths, ubiquitous sensing, and AI-driven operations. The figures are speculative and provide rough projections.

AI/ML Data Volume in 6G

- Sources:

  - Network KPIs, mobility traces, QoE/QoS metrics, traffic patterns, device context.

  - Edge-collected training data for distributed/federated learning

  - Models and model parameters

- Rough estimates

  - Per dense urban macro cell: ~10–50 GB/day of raw telemetry and KPI data

  - Model updates: ~100 MB–1 GB/day per edge node depending on model size

  - Network-wide AI data: petabyte-scale per day for nationwide coverage, assuming millions of UEs and edge nodes.

Sensing Data Volume in 6G

- Sources:

  - Integrated Sensing and Communication (ISAC) functions: RF sensing, environment mapping, object detection, gesture recognition, vehicular radar, digital twins.

- Rough estimates

  - Single UE/vehicle with radar/lidar-level sensing: ~1–10 GB/hour raw data.

  - Base station with environmental RF sensing (high-resolution beamforming): ~100 GB–1 TB/day per site.

  - City-scale deployment (dense ISAC network): hundreds of TB to several PB/day.

The existing data transfer framework (control and user plane data) is not suitable for the data of this magnitude.

### Non-CP/UP data transfer using existing framework

The Control Plane (SRBs) has clear bottleneck for carrying AI/ML data with limited segmentation, as such data may be too frequent/large. The UE is only required to support 45kB RRC buffer size, according to TS 38.306, while the dataset/parameters are in the order of hundreds of MBs. The maximum RRC message size is up to ~16 KB per message while larger RRC messages are segmented at PDCP/RLC. Practically, SRB throughput is limited because SRBs are prioritized and small, usually tens of kbps to a few Mbps.

The existing data transfer framework of control plane data is not suitable for the non-CP/UP data (AI-ML, Sensing data, SON/MDT, QoE etc).

The User Plane (DRB) is unsuitable for carrying AI/ML data, due to the difference in characteristics of data handled in UP and the need to process/consume the data at nodes within RAN (eg: at DU, CU-CP and CU-UP), apart from the other termination points. Moreover, the AI/ML data may have to be terminated at OAM, OTT Server, RAN or CN depending on the use-case.

If the termination point is an OTT server, the volume of data that is being transferred could be huge. Transfer of huge volumes of AI/ML and sensing data requires support for flexible routing, and conventional PDU sessions and/or DRBs may not be optimal.

In 5G, MAC layer, the maximum TBS (Transport Block Size) depends on numerology, MIMO layers, and slot length. Max TBS per slot can reach ~10–12 Mb (megabits) for high-order MIMO (8–16 layers) and 400 MHz bandwidth at mmWave frequencies. Despite this, the non-CP/UP data cannot be always transmitted with the above configuration. Considering the enormous amount of non-CP/UP data, it is infeasible to use the existing UP framework without causing a detrimental effect on user plane data.

The existing data transfer framework of user plane data is not suitable for the non-CP/UP data (AI-ML, Sensing data, SON/MDT, QoE etc).

The priority of AL/ML data can be treated differently from Control Plane data and User Plane data, i.e., if Control Plane data is missed, the network and UEs cannot exchange User Plane data effectively or even the UEs being disconnected from the network; some User Plane data becomes useless if the end-to-end delay is long, while the time to arrive at the destination for majority of AL/ML data can be with less stringent delay requirements. The priority of CP, UP and AI-ML data are inherently different and is very important information for scheduler, which is responsible to schedule transmissions with limited radio resources (bandwidth, power and number of antennas).

The flexibility and smart adjustment in QoS handling required for the efficient transfer of AI/ML and sensing data is not available in the current CP/UP framework in 5G.

The transfer of data for AI/ML and sensing operations has QoS requirements that are different from normal UP/CP traffic and the flexibility required for efficient transfer of AI/ML and sensing data is not available in the current CP/UP framework in 5G.

Therefore, the 6G data collection framework should adopt a unified architecture that integrates legacy capabilities (such as SON and MDT) with new service requirements (including AI/ML and ISAC). It must ensure secure, consent-based data acquisition, provide operator control and transparency, avoid redundancy through coordinated data reuse, and remain scalable to accommodate future 6G services.

### Data Plane DRBs

A new “Data plane Data Radio Bearer (DP-DRB)” (different from the control plane SRBs and user plane DRBs) could be a way-forward to address the above issues.

- Primarily, avoid detrimental impact on CP/UP data.

- A DP-DRB can be configured between any data producer and consumer entity in the system -- UE, RAN, CN, OAM, OTT server i.e a DP-DRB can be both originated and terminated either at the UE device or at different entities in the network based on the use-case and requirement.

- A DP-DRB can be configured for AI-ML data, sensing data, SON/MDT requirements, QoE etc.

- A DP-DRB could be configured flexibly to support the following

  - Provide different QoS based on the use-case requirements.

  - Provide different security as required.

  - Routed and terminated at different entities as per the requirement.

  - Be configured with latency, bitrate and other characteristics as required rather than being mixed with CP/UP data.

  - Provide enhanced security to all the different types of data listed above.

  - Transfer enormous data volumes associated with the different types of data without impacting the latency critical CP and UP traffic.

  - Perform data collection for multiple AI-ML use cases simultaneously, which results in improved efficiency. This way, data duplication can be avoided.

Figure 1: Exemplary diagram illustrating Data Plane DRB

Introduce a new integrated Data Plane framework including Data plane DRB(DP-DRB) for the purpose of data transfer of AI-ML, ISAC, SON/MDT, QoE etc data.

# Conclusion and Proposals

Based on the discussion above, we conclude this contribution with the following observations and proposals:

Observation 1		The existing data transfer framework (control and user plane data) is not suitable for the data of this magnitude.

Observation 2		The existing data transfer framework of control plane data is not suitable for the non-CP/UP data (AI-ML, Sensing data, SON/MDT, QoE etc).

Observation 3		The existing data transfer framework of user plane data is not suitable for the non-CP/UP data(AI-ML, Sensing data, SON/MDT, QoE etc).

Observation 4		The transfer of data for AI/ML and sensing operations has QoS requirements that are different from normal UP/CP traffic and the flexibility required for efficient transfer of AI/ML and sensing data is not available in the current CP/UP framework in 5G.

Proposal 1		Study the introduction of a new Data Plane DRB for the purpose of AI-ML data collection over the air interface and transfer of sensing data to/from sensing equipment.

# References

- RP-251881. (n.d.). New SID: Study on 6G Radio. NTT Docomo.