3GPP TSG-RAN WG2 #132	R2-2508767

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	10.2.1

Source:	InterDigital, Inc.

Title:	RRC State Model for 6GR

WID/SID:	FS_6G_Radio – Release 20

Document for:	Discussion

# Introduction

The study on 6G contains the following CP related objectives.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In the previous meeting, the following agreements were made

Agreements Continue study - Idle mode – - Connected mode – - Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW. Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements. We can consider the pros/cons of different modelling, after having some discussions on the functionality.

This contribution explores the potential functionality for each state, in the context of both of the models (3-state model using inactive state similar to NR, or 2-state model which uses connected sub-states). The overarching observation is that a 2‑state model with RRC sub‑states is functionally equivalent to the three‑state model; it does not materially simplify signaling and largely repackages the same behaviors.

# Discussion

In the previous meeting, it was agreed that we can consider the pros/cons of different modelling, after having some discussions on the functionality.

Figure 2.1-1: Comparison of 2-state and 3-state RRC model.

The distinction between the two-state and three-state RRC models is primarily one of terminology rather than substance. Both frameworks define the same underlying UE behaviors — a fully connected state, a lightweight semi-connected (or inactive) condition with stored context, and a fully idle mode with no RAN context. The so-called two-state model simply embeds the semi-connected behavior as a sub-state within CONNECTED, whereas the three-state model represents it as a separate INACTIVE state.

In functional terms — including signaling, context handling, and security continuity — the two are identical; only the placement of the boundary differs.

As shown, the 3-state diagram draws solid lines between IDLE↔INACTIVE↔CONNECTED, while the 2-state diagram draws the same lines but makes one of them (Inactive) dashed — indicating that the INACTIVE↔CONNECTED transition is viewed as an internal sub-state change rather than a formal state transition. Consequently, adopting a two-state representation does not remove, change, or simplify any procedure; it merely re-labels the same transitions and redraws one boundary as dashed instead of solid.

Observation 1: A two-state representation does not remove, change, or simplify any procedure compared to a three-state model; it merely re-labels the same transitions and redraws one boundary as dashed instead of solid.

However, the modelling chosen would impact the structure and complexity of the specification. In order to determine which model is most logical and convenient we must first understand the functionality itself. In the following sections, we summarize some of the proposed enhancements and functionality, and how those fit into the 2 and 3 state models. As a starting point, this has been separated into 3 enhancement/functionality themes – latency, UE power saving, NW power saving.

## 2.1 Latency

This theme targets fast transition between light and full connectivity by leveraging stored UE context. It covers mechanisms such as resume token validation, UE-specific DCI triggers, configured grant usage, and UL timing alignment. Both state models can realize this functionality using similar message flows.

| Aspect | Sub-Mechanisms / Options | Purpose | 3-State Model | 2-State Model |
| --- | --- | --- | --- | --- |
| Trigger methods for inactive to connected transition. | UE-specific DCI; MAC CE; RRC message; UE trigger (E.g. UL data, measurement event) | Immediate resume trigger | Yes | Yes |
| Context retention in inactive | Resume ID, RNTI, stored RRC configs | Skip full resume/reconfiguration when valid | Yes | Yes |
| Security context reuse | Continue KgNB/KeNB | Avoid NAS re-activation/refresh | Yes | Yes |
| Configured Grant / TA readiness | CG validity; early UL sync | Enable immediate UL transmission | Yes | Yes |
| Latency | — | e.g. <10 ms transition | Yes | Yes |

## 2.2. UE Power Saving

This theme focuses on mobility continuity and reachability with minimal UE energy consumption. The UE relies on reselection and conditional triggers instead of continuous connected-mode activity. Resource pattern adaptation and relaxed SSB/CSI-RS density may allow further savings.

| Aspect | Mechanisms | Purpose | 3-State Model | 2-State Model |
| --- | --- | --- | --- | --- |
| Mobility mechanism in inactive | Cell reselection | UE power saving, signalling reduction. | Yes | Yes |
| Measurements in inactive | Relaxed compared to connected | UE power saving | Yes | Yes |
| DRX / Wake cycles | Short/adaptive DRX windows compared to connected | UE power saving, network resource management | Yes | Yes |
| Resource configuration | E.g. SSB/CSI-RS density | UE power saving, network energy saving | Yes | Yes |

## 2.3. Network Power Saving, simplicity, flexibility

The network may manage UE reachability without CN paging, while minimising control overhead. The RAN can maintain a minimal per-UE context, allowing configuration and monitoring frequency to vary by device type (e.g. RedCap, sensors, XR).
This approach reduces CN involvement, supports differentiated configurations, and enhances network-level energy efficiency..

| Aspect | Mechanisms / Examples | Purpose | 3-State Model | 2-State Model |
| --- | --- | --- | --- | --- |
| RAN-managed reachability (no/minimal CN paging) | RAN keeps per-UE token/id; scheduling via DCI or area-RNTI | Eliminate or simplify CN paging; RAN paging by default if inactive is the default state. | Yes | Yes |
| Eliminate RAN paging | Paging in idle mode only, direct addressing in inactive using RNTI. | Faster UE trigger to transmit data | Yes | Yes |
| Device-type configuration | Device-type-specific inactive config (RedCap/mMTC/XR) | Tailor control density, DRX, and CG patterns | Yes | Yes |
| Lightweight broadcast/system info | Reduced SI; area-specific CG, on demand SIB1, on-demand SSB. | Limit broadcast overhead, keeping UEs in inactive limits idle mode usage. | Yes | Yes |

## 2.3. Key take-aways

The enhancements across Themes 1–3 are achievable under both the existing three-state model and a potential two-state model with sub-states. The number of UE behaviors, transitions, and signaling procedures remains effectively identical. The explicit INACTIVE boundary in the three-state model provides a convenient way to group parameters and behaviors, while the two-state model simply renames these boundaries. Hence, the simplification claimed for the two-state approach is mainly editorial rather than functional. It is therefore proposed that RAN2 focuses first on defining the common mechanisms, followed by evaluating which state representation offers the clearest specification.

Observation 2: Functionally, the behaviors achievable under a two‑state model (CONNECTED with sub‑states) and the existing three‑state model (IDLE/INACTIVE/CONNECTED) are equivalent.

Although functionally these models achieve the same thing, the choice will have implications on the specification structure and complexity. However, until we identify the required behaviour, it will be impossible to determine which model represents this most conveniently.

Proposal 1: Consider model agnostic functionality for idle, inactive, connected as well as state transition mechanisms, with the assumption that any discussed functionality can work using either 2 state or 3 state model and that “inactive” refers to the intermediate state in both models.

Proposal 2: Revisit the state model and decide whether inactive as a connected sub-state, or as a separate RRC state, best expresses the functionality once it is known (e.g. WI phase).

Based on previous discussions, it appears that the most important feature to consider when designing the RRC sate model and transitions it the mechanism for transitioning to connected. Therefore, we suggest as a priority to study this. First we need a design target/requirement in order to evaluate solutions.

Proposal 3: Define latency target for fast transition from inactive to connected, e.g. 10ms.

Proposal 4: Study and define the detailed mechanisms for performing a fast transition between inactive and connected, including context storage, UE identification, and signalling mechanism.

An important aspect to inherit from NR is the UE power saving by allowing UE based mobility, and it looks like nobody disagrees. However, the mobility functionality might differ slightly from NR. Although it is reasonable to assume a cell reselection as a baseline procedure, the mechanism by which RAN/CN tracks the location of the UE (e.g. how paging works) and performs context relocation upon mobility might differ depending on the agreed mechanisms.

Proposal 5: Support UE controlled mobility in the inactive state, with cell reselection as a baseline, including paging and context relocation.

In addition to UE controlled mobility, the inactive state should also provide better power saving for the UE. As a minimum 6GR should support longer DRX cycles in inactive state, but it is likely that this configuration might need to adapt based on e.g. UE capability and network energy saving.

Proposal 6: Support UE power saving configurations in inactive state such as longer and adaptive DRX.

Finally, it has already been agreed that NW energy saving must be taken into account. The primary savings are achieved when the network can switch off transmissions in the downlink temporarily, or adjust the pattern. Such savings typically manifest as idle mode features. For example, on demand SIB1, sparse SSBs, etc.

By limiting the scope of idle mode, and making inactive the default low power state, UEs will need to use idle mode far less frequently than in NR. For example, idle mode might be used for initial cell search only, then UE goes to inactive after that. In this case there is no need to transmit SSBs very frequently to ensure fast cell detection. In other words, since idle would be an infrequently used state we can accept that this can be made very simple – no need to optimize idle mode procedures for latency or UE, which allows idle mode to be optimized for NES.

Proposal 7: Support limiting the scope of idle mode to enable network energy efficiency.

# Conclusion

Fast state change using stored context, UE power saving through reselection-based mobility, and lightweight idle enabling network power saving represent the main enhancement themes for the RRC state model evolution. Both models can realize these mechanisms, and the optimal representation should be chosen once functionality is fully defined.

Observation 1: A two-state representation does not remove, change, or simplify any procedure compared to a three-state model; it merely re-labels the same transitions and redraws one boundary as dashed instead of solid.

Observation 2: Functionally, the behaviors achievable under a two‑state model (CONNECTED with sub‑states) and the existing three‑state model (IDLE/INACTIVE/CONNECTED) are equivalent.

Proposal 1: Consider model agnostic functionality for idle, inactive, connected as well as state transition mechanisms, with the assumption that any discussed functionality can work using either 2 state or 3 state model and that “inactive” refers to the intermediate state in both models.

Proposal 2: Revisit the state model and decide whether inactive as a connected sub-state, or as a separate RRC state, best expresses the functionality once it is known (e.g. WI phase).

Proposal 3: Define latency target for fast transition from inactive to connected, e.g. 10ms.

Proposal 4: Study and define the detailed mechanisms for performing a fast transition between inactive and connected, including context storage, UE identification, and signalling mechanism.

Proposal 5: Support UE controlled mobility in the inactive state, with cell reselection as a baseline, including paging and context relocation.

Proposal 6: Support UE power saving configurations in inactive state such as longer and adaptive DRX.

Proposal 7: Support limiting the scope of idle mode to enable network energy efficiency.

# References

- RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator), Prague, CR, June 9-13, 2025.