3GPP TSG RAN WG2 #132	R2-2508777

Dallas, USA, 17th – 21st November, 2025

Agenda item	: 10.2.2

Source	: Jio Platforms Limited

Title	: 6G NTN TN Harmonization requirement

Document for	: Discussion and Decision

# Introduction

According to 6G study item [1], 6GR aim is to harmonize 6G radio design for TN and NTN.

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

In the RAN2#131-bis meeting, the following has been noted in the chair notes:

For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

According to RAN#2 131bis meeting chair notes, RAN2 need to identify which functionalities can be addressed commonly for TN and NTN and which functionalities should be kept specific to NTN only.

In this document, we are sharing our thoughts on which functionalities can be made common in TN-NTN, and which functionalities will be NTN specific in 6GR.

# Discussion

## NTN specific system information

In NR Release 17 and onwards, SIB19 and SIB25 provide NTN-specific assistance information to support access, measurements, and mobility across satellite and terrestrial networks [1,2]. SIB19 contains satellite assistance information for NTN access. It includes parameters such as ntn-Config, which carries satellite ephemeris and timing data, referenceLocation, t-Service, and distanceThresh for location-based measurement initiation, as well as ntn-NeighCellConfigList describing neighboring satellite cells [4]. Later releases extend SIB19 with fields like movingReferenceLocation, ntn-CovEnh, and satSwitchWithReSync to support coverage enhancement and satellite switch procedures.

SIB25, on the other hand, provides TN coverage assistance information to UEs located in an NTN cell. It enables the UE to perform terrestrial neighbor cell measurements efficiently by broadcasting parameters that describe nearby TN cells accessible from an NTN coverage area. Together, SIB19 and SIB25 form a complementary framework allowing UEs in NTN or hybrid environments to acquire synchronization, timing, and measurement assistance for both satellite and terrestrial systems, supporting seamless TN-NTN integration.

Given the complementary roles of SIB19 and SIB25 in current NTN operation, it is observed that both messages collectively serve NTN-related purposes such as satellite access assistance and TN-NTN mobility support. Specifically, SIB19 provides NTN-related configuration and assistance information and may be broadcast by both TN and NTN cells, while SIB25 is limited to NTN cells and provides TN coverage information to assist UEs in determining whether TN frequency measurements are necessary within an NTN coverage area. This dual-message structure serve complementary NTN-related purposes, supporting satellite access assistance and TN–NTN mobility functions.

Both SIB19 and SIB25 serve complementary NTN-related purposes, supporting satellite access assistance and TN-NTN mobility functions.

Building on the complementary functions of SIB19 and SIB25, it may be beneficial for RAN2 to consider a unified approach to streamline system information design and reduce signaling complexity. In particular, the information elements currently defined in SIB25, used exclusively by NTN cells to indicate TN coverage availability and assist UEs in determining the need for TN frequency measurements, could be integrated into SIB19 as optional fields applicable when broadcast by an NTN cell. Such consolidation would enable a single, adaptable SIB structure capable of conveying both NTN-specific and TN-assistive information. This approach could simplify UE implementation, improve system information management, and maintain the existing power-saving advantages achieved through conditional TN frequency measurements.

RAN2 should study integrating the SIB25 information elements into SIB19 as optional fields when broadcast by an NTN cell, to support a unified system information design for TN-NTN operation.

## HARQ design for TN and NTN

In NTN deployment scenarios, a large round-trip time (RTT) is introduced due to the significant propagation distance between the user terminal and the satellite. The RTT varies based on the satellite constellation type (LEO, MEO, GEO) and the satellite payload architecture (transparent or regenerative), both of which influence the end-to-end signal path and overall latency.

Below table “NTN scenarios versus delay constraints” [2] provide detailed about RTT based on type of satellite constellation and satellite payload type.

| NTN scenarios | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Satellite altitude | 35786 km |  | 600 km |  |  |  |
| Relative speed of Satellite with respect to earth | negligible |  | 7.56 km per second |  |  |  |
| Min elevation for both feeder and service links | 10° for service link and 10° for feeder link |  |  |  |  |  |
| Typical Min / Max NTN beam foot print diameter (note 1) | 100 km / 3500 km |  | 50 km / 1000 km |  |  |  |
| Maximum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 541.46 ms (Worst case) | 270.73 ms | 25.77 ms |  | 12.89 ms |  |
| Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 477.48 ms | 238.74 ms | 8 ms |  | 4 ms |  |
| Maximum Round Trip Delay variation as seen by the UE (note 2) | Negligible |  | Up to +/- 93.0 µs/sec (Worst case) |  | Up to +/- 47.6 µs/sec |  |
| NOTE 1:The beam foot print diameter are indicative. The diameter depends on the orbit, earth latitude, antenna design, and radio resource management strategy in a given system. NOTE 2:The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time. The worst case is when the satellite moves towards/away from the UE at 10⁰ elevation angle, assuming the UE speed is 1200 km/h. It is expressed in µs/s and is negligible for GEO scenario. NOTE 3:Void NOTE 4:Speed of light used for delay calculation is 299792458 m/s. |  |  |  |  |  |  |

As mentioned in the above table, In NTN deployment RTT may vary between ~542 ms to ~13 ms depending on payload type and satellite constellation. Large RTT is having a significant impact on HARQ procedures.

Considering a subcarrier spacing (SCS) of 15 kHz, where each 10 ms radio frame consists of 10 slots, the first HARQ process initiated in slot 1 would receive its ACK/NACK feedback after approximately 542 ms in the case of a deployment of GEO satellite constellation with transparent payload. This duration corresponds to about 54 radio frames (or 540 slots) of delay. The gNB cannot reuse the same HARQ process ID until it receives the corresponding ACK/NACK feedback from the UE.

As a result of this stalling of HARQ, the transmitted data remains buffered for an extended duration, which adversely impacts overall user throughput and degrades end-to-end performance efficiency.

Till 3GPP release 16, a total 16 HARQ process were available so that GNB can schedule data parallelly with 16 HARQ processes. But these processes are not sufficient when it comes to long propagation delays.

In 3GPP Release 17, the specification increased the number of HARQ processes to 32, providing partial relief for LEO satellite deployments with RTT below approximately 30 ms, where the RTT is relatively lower compared to GEO satellite constellations. However, to effectively handle the long RTT scenarios observed in GEO and MEO systems, 3GPP introduced an option to disable downlink HARQ feedback for specific NTN configurations, allowing error recovery to rely on higher-layer retransmission mechanisms such as RLC Acknowledged Mode (AM).

In 5G NTN, the specification increases the number of downlink and uplink HARQ processes up to 32 and introduces an option to disable downlink HARQ feedback to prevent HARQ stalling under large RTT conditions.

During 6GR design, RAN2 shall study a unified HARQ design that can mitigate HARQ process stalling caused by large RTTs in NTN scenarios.

During 6GR design, RAN2 shall consider inclusion of the downlink HARQ feedback disable functionality from the initial design phase (“day 1”).

## NTN specific measurement events

In 5G NR-NTN, conventional terrestrial mobility procedures are insufficient due to the dynamic movement of satellites which results in frequent variation in cell or beam coverage. To address this challenge of fast-moving satellite and rapidly changing beam coverage, 3GPP Release 17 introduced new measurement events based on location (Event D1 and D2) and based on time (CondEvent T1).

In 5G NR-NTN, 3GPP introduced location-based measurement events (Event D1 and Event D2) and time-based conditional events (Conditional Event T1) to enhance mobility support under fast-moving satellite conditions.

In 6GR design, RAN 2 shall consider location-based measurement events (Event D1 and Event D2) and time-based conditional events (Conditional Event T1) as baseline NTN specific functionality for NTN mobility.

## Timing Advance

NTN introduce long propagation delays due to the significantly increased distance between the UE and the radio access network. To accommodate this, the timing advance (TA) mechanism must account for the extended propagation time compared to terrestrial deployments. However, there is no explicit modification to the timing advance procedure itself for NTN. Instead, the 3GPP 5G NR-NTN specification adopts an extended TA design that supports both TN and NTN operation, ensuring compatibility while allowing for extended TA values to compensate for large signal propagation delays inherent to satellite links.

In NTN network, the uplink frame transmission takes place (NTA + NTA.offset +NTA.adjcommon + NTA.adjUE)x Tc [3] before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell.

NTA and NTA.offset parameters remain common for TN and NTN. In NTN, 3GPP introduced two additional parameters NTA.adjcommon and NTA.adjUE to compensate for the extended propagation delay specific to satellite communication. In Terrestrial Network,  NTA.adjcommon and NTA.adjUE values shall be considered as 0.

NTA.adjcommon is Delaycommon(t) which is derived from the higher-layer parameters ta-Common, taCommonDrift, and ta-CommonDriftVariant configured in SIB 19, otherwise NTA.adjcommon = 0.

NTA.adjUE is computed by the UE based on UE position and serving satellite ephemeris related information provided by higher layer in SIB 19, otherwise NTA.adjUE = 0.

In 5G NR, NTN specification adopts a unified TA design that supports both TN and NTN operation, ensuring compatibility while allowing for extended TA values to compensate for large signal propagation delays inherent to satellite links.

In 6GR design, RAN2 shall explore unified design for TN and NTN for timing advance computation and also RAN 2 shall study on extension of NTA value limit.

# Conclusion

Observation 1:	 Both SIB19 and SIB25 serve complementary NTN-related purposes, supporting satellite access assistance and TN–NTN mobility functions.

Proposal 1:	 RAN2 should study integrating the SIB25 information elements into SIB19 as optional fields when broadcast by an NTN cell, to support a unified system information design for TN/NTN operation.

Observation 2:	 In 5G NTN, the specification increases the number of downlink and uplink HARQ processes up to 32 and introduces an option to disable downlink HARQ feedback to prevent HARQ stalling under large RTT conditions.

Proposal 2:	 During 6GR design, RAN2 shall study a unified HARQ design that can mitigate HARQ process stalling caused by large RTTs in NTN scenarios.

Proposal 3:	 During 6GR design, RAN2 shall consider inclusion of the downlink HARQ feedback disable functionality from the initial design phase (“day 1”).

Observation 3:	 In 5G NR-NTN, 3GPP introduced location-based measurement events (Event D1 and Event D2) and time-based conditional events (Conditional Event T1) to enhance mobility support under fast-moving satellite conditions.

Proposal 4:	 In 6GR design, RAN 2 shall consider location-based measurement events (Event D1 and Event D2) and time-based conditional events (Conditional Event T1) as baseline NTN specific functionality for NTN mobility.

Observation 4:	 In 5G NR, NTN specification adopts a unified TA design that supports both TN and NTN operation, ensuring compatibility while allowing for extended TA values to compensate for large signal propagation delays inherent to satellite links.

Proposal 5: In 6GR design, RAN2 shall explore unified design for TN and NTN for timing advance computation and also RAN 2 shall study on extension of NTA value limit.

# References

RP-252912: “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

3GPP TR 38.821: “Solutions for NR to support non-terrestrial networks (NTN)”

3GPP TR 38.811: “Study on New Radio (NR) to support non-terrestrial network”

3GPP TS 38.331: “NR; Radio Resource Control (RRC); Protocol specification”.

3GPP TS 38.321: “Medium Access Control (MAC) protocol specification”