3GPP TSG RAN WG2 #132	R2-2508778

Dallas, USA, 17th – 21st November 2025

Agenda item	: 10.4

Source	: Jio Platforms Limited

Title	: Necessity of DAPS Support in 6G Mobility

Document for	: Discussion and Decision

# Introduction

In RAN2#131bis meeting, the following has been agreed on 6G Mobility:

Agreements - Study mobility with the following requirements in 6G mobility design: - Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions. - Robustness of mobility procedures - Energy efficiency for both UE and NW - Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

As noted in the chair notes, the reduction of interruption time and the assurance of service continuity will be key study areas within RAN2 for 6G. In this contribution, we analyse existing 5G mobility frameworks, including L3 Handover, CHO, LTM, and DAPS, from the perspective of interruption time, robustness, and implementation complexity. The goal is to assess their suitability for 6G service requirements and to identify which mechanisms can effectively meet the stringent continuity and reliability expectations of next-generation networks. In particular, this contribution discusses the critical role and necessity of DAPS support in 6G mobility design, as it is currently the only mechanism that inherently provides make-before-break capability and true interruption-free handover.

# Discussion

3GPP has defined different services which are having different requirements in terms of interruption time and robustness. Below table summarizes these services:

| Category | Service Use case | Interruption Time Requirement | Robustness Requirement |
| --- | --- | --- | --- |
| Enhanced Mobile Broadband (eMBB) - Standard | Web Browsing, File Downloads, Social Media | Around 100 ms. Brief interruptions are often masked or tolerated by application buffers. | Moderate. Mobility failure is inconvenient (e.g., download restarts), but not critical. |
| Voice Over NR | Voice calls based on 5QI 1 | Around 50-100 ms. | High. Handover failures lead to call drop |
| Mission Critical Services | Mission Critical Push to Talk (MCPTT), Mission critical voice, Mission critical video. | 75 ms - 100 ms | These services are designed for public safety, emergency, and defense communication, requiring high reliability, low latency, and robust mobility support even in under extreme network conditions |
| Time/Delay Critical Services | Real-time Video Streaming (4K/8K), Augmented Reality (AR)/Virtual Reality (VR) | 50 ms. Short interruptions cause noticeable quality degradation (freezing, jitter). | High. Mobility failures severely degrade User Experience (QoE) and may cause application crashes. |
| Ultra-Reliable Low-Latency Communication (URLLC) | Remote Surgery | 1ms | Extremely High. Mobility procedures (handover) must almost never fail or result in a dropped connection. |
|  | Industrial Automation | 10 ms |  |
|  | Vehicle-to-Everything (V2X) communication | 5 ms |  |
| Massive Machine Type Communication (mMTC) | Smart Metering, Asset Tracking, Environmental Sensors | High Tolerance (Seconds to minutes). Data is often small and non-real-time; devices are primarily stationary or low-speed. | Low to Moderate. Robustness is more about maintaining long-term connectivity and low power consumption than seamless handover. |

The interruption time refers to the duration when a UE is unable to transmit or receive user data. As defined above services/applications can be categorized in low and high interruption time requirements, such as:

- Low Interruption Time: Services like VoNR, Mission Critical, URLLC, remote control or AR/VR are highly time critical. An interruption of even a few tens of milliseconds can lead to system instability, loss of control, or severe user experience degradation. These require almost loss free, fast handover procedures.

- High Interruption Time: Services like basic eMBB applications, Simple data transfers or sensor reporting can tolerate longer interruption times. Web traffic, for instance, uses buffering and retransmission mechanisms to recover from short network breaks, making the interruption less noticeable to the user.

Robustness refers to the network's ability to maintain the connection and prevent a failure (such as a Radio Link Failure or "call drop") during mobility. It is a measure of the reliability of the mobility procedure itself. In general, for a good user experience, all the handovers are desired to be robust in nature irrespective of service type. Network’s handover success rate should be close to 100 % but due to wireless media it is impossible to meet this requirement therefore services can be categorized in two different buckets in terms of high and low robustness requirements:

- High Robustness: Voice over NR, Mission-critical services and other real-time interactive applications demand extremely high robustness. Mobility failure means a complete loss of service, which is unacceptable in scenarios like remote health or automated guided vehicles (AGVs).

- Lower Robustness : While desirable, a mobility failure in these services is less catastrophic. For a standard mobile broadband user, it results in a dropped call or a failed download which requires manual or automatic re-establishment of the on going RRC connection, which is inconvenient but not critical.

In 5G, different services have varying mobility requirements depending on their tolerance to interruption time (how long a service can tolerate a break in connectivity during handover) and robustness (how resilient the connection must be to mobility-related disruptions).

In NR 3GPP has defined multiple mobility procedures from Release 15 to Release 19. Below is the summary table for standalone deployment mobility procedures:

| 5G Status | Feature | Details | Target Cell(s) Preparation | Pre-Execution | Execution | Impacted KPI |
| --- | --- | --- | --- | --- | --- | --- |
| Rel-15 | L3 Handover | Handover procedure triggered by RRC signalling. | One cell only, shortly before HO execution | No | Basic RRC signaling based HO, RACH procedure involved | Reference for other mobility frameworks’ KPIs |
| Rel-16 | DAPS | A handover procedure that maintains the source gNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target gNB. | One cell only, shortly before HO execution | No | Basic RRC signaling based HO, RACH procedure involved | Interruption time Reduction |
|  | CHO | A handover that is executed by the UE when one or more handover execution conditions are met. | Up to 8 cell long before HO execution | No | Evaluation of Layer 3 execution criteria and then RACH based execution via RRC signaling | Mobility robustness Improvement |
| Rel-18 | Intra-gNB LTM | gNB receives L1 or L3 measurement report(s) from a UE, and on their basis the gNB may change UE serving cell by a cell switch command signaled via a MAC CE. | Up to 8 cell long before HO execution | TCI state (re)activation Early TA acquisition Early CSI acquisition and reporting | MAC CE based cell switch command. It can be RACH-based or RACH less. | Interruption time Reduction |
| Rel-19 | Inter-gNB LTM | Support inter-gNB mobility by using LTM mechanism. | Up to 8 cell long before HO execution | TCI state (re)activation Early TA acquisition Early CSI acquisition and reporting | MAC CE based cell switch command. It can be RACH-based or RACH less. | Interruption time Reduction |
|  | C-LTM | LTM cell switch which executed by the UE when L1-based or L3-based LTM cell switch execution conditions are met. | Up to 8 cell long before HO execution | TCI state (re)activation Early TA acquisition Early CSI acquisition and reporting | Evaluation of Layer 3 execution criteria and then RACH based execution via MAC CE | Mobility Robustness Improvement and Interruption time Reduction |

The different 5G service categories, such as eMBB, VoNR, Mission Critical Services, Time/Delay Critical Communication, URLLC, and mMTC, each have unique interruption time and robustness requirements. For instance, eMBB applications like web browsing can tolerate moderate interruptions, while URLLC services such as remote surgery demand extremely low latency and near-zero interruption. To address these diverse needs, 5G has introduced several mobility enhancement mechanisms, including DAPS, CHO, and LTM, which improve either mobility robustness or reduce interruption time. However, these mechanisms are optimized for specific service types and network conditions. Therefore, no single mobility framework can effectively support all service types simultaneously. As we move toward 6G, it becomes evident that multiple, service-aware mobility frameworks will be required, each dynamically configured based on the service’s latency sensitivity, reliability requirements, and robustness expectations to ensure optimal performance across all communication scenarios.

A single mobility framework cannot meet the diverse interruption time and robustness requirements of all service types, highlighting the need for multiple, service-specific mobility frameworks in 6G.

In mobility performance evaluations, the interruption time is typically measured during the handover execution phase, that is, between the moment the UE detaches from the serving cell and the moment it resumes transmission with the target cell. However, in practice, assessing the interruption time of a mobility procedure solely within this interval does not provide a complete picture of its performance. For instance, in conditional handover mechanisms, i.e., CHO and C-LTM, the radio link between the UE and the serving cell may be lost before the execution phase begins, causing the UE to experience service interruption well in advance of the actual handover execution, depending on the configured failure timers and assuming that the handover is triggered before any link failure occurs.

Measuring interruption time only during the handover execution phase does not fully represent the overall interruption performance of a mobility procedure.

Mechanisms such as CHO and C-LTM primarily enhance mobility robustness rather than reducing interruption time, as service interruptions can still occur before the handover execution phase.

LTM aims to reduce handover latency through procedures such as early UL and DL synchronization. In early UL sync, the Timing Advance (TA) is obtained via a PDCCH-ordered RACH, where the UE transmits a PRACH preamble to the target cell without receiving the RAR, and the TA is later delivered via a MAC CE cell switch command prior to LTM execution. However, the interval between TA acquisition and LTM execution can be significant, potentially invalidating the TA value and forcing the UE to perform a new RACH procedure toward the target cell—thereby negating the intended UL interruption time reduction. Similarly, although early DL sync allows the UE to align with the target cell’s downlink, additional delays from RRC processing and UL synchronization (especially in RACH-based LTMs) still contribute to noticeable interruption. Consequently, while L1/L2 triggered mobility mechanisms theoretically aim to minimize interruption time, practical limitations prevent them from consistently achieving this goal.

It is a misconception to claim that LTM inherently provides low interruption time, as its performance heavily depends on timing validity and procedural delays that can negate its intended benefits.

As explained above, mobility mechanisms such as L3 Handover, CHO, LTM, and C-LTM do not always fulfill the goal of minimizing interruption time for services that demand seamless connectivity. In contrast, the DAPS mechanism introduces a make-before-break concept, where the UE maintains its connection with the source cell while simultaneously establishing a new connection with the target cell. During this phase, the UE continues to receive downlink traffic from the source cell until the target connection is fully established, after which traffic is seamlessly switched to the target cell without noticeable interruption. Moreover, DAPS has already been commercially deployed, demonstrating its feasibility and maturity in real network environments [1]. This capability directly enhances service continuity and perceived QoE, which are key metrics from an operator’s perspective. As network operators, ensuring uninterrupted connectivity is critical not only for meeting the technical requirements of ultra-reliable and latency-sensitive services but also for maintaining customer satisfaction and trust. Therefore, in the context of 6G, DAPS should be considered an essential mobility framework, as it uniquely enables true interruption-free mobility, a prerequisite for delivering superior end-user experience and sustaining competitive service quality.

L3 handover, CHO, LTM, and C-LTM mechanisms are not adequate for services that require guaranteed interruption time reduction, as their design primarily focuses on improving mobility robustness rather than ensuring seamless, interruption-free connectivity.

DAPS, with its make-before-break architecture, effectively minimizes service interruption by maintaining the source link while establishing the target connection, making it the only existing mobility framework capable of supporting services that demand guaranteed low interruption time.

DAPS should be considered in 6G as a fundamental mobility framework to ensure guaranteed low-interruption handovers and deliver seamless service continuity for latency-sensitive applications.

While DAPS offers significant advantages in minimizing downlink interruption through its make-before-break design, it also introduces certain limitations that must be considered. One of the primarily argued drawbacks is the increased UE implementation complexity, as DAPS requires the UE to operate and maintain two active protocol stacks simultaneously, one for the source cell and another for the target cell. This dual-stack operation increases processing demands, memory usage, and signalling coordination within the UE, potentially affecting power consumption and hardware resource allocation. Another important limitation of DAPS lies in its asymmetry: while it effectively minimizes downlink interruption time, it does not inherently provide simultaneous uplink connectivity to both cells. As a result, uplink transmissions may still experience a brief interruption during the handover completion phase, particularly when the UE transitions its uplink path from the source to the target cell.

However, these drawbacks should not be viewed as prohibitive barriers to commercial deployment. Most 5G UEs already support dual connectivity, which similarly requires maintaining two active protocol stacks to communicate with both a Master and Secondary Node. This demonstrates that the underlying hardware and software architectures in current devices are already capable of handling such complexity. Therefore, the implementation of DAPS should not be considered a showstopper from a UE perspective. Instead, the perceived complexity warrants further evaluation within RAN2, where a detailed analysis can be conducted to quantify and compare the actual overhead introduced by DAPS relative to dual connectivity. Moreover, the uplink limitation in DAPS presents an opportunity for enhancement rather than a fundamental flaw. By investigating early uplink and downlink synchronization procedures from a DAPS perspective, it may be possible to further reduce the overall handover completion time, thereby improving both uplink and downlink interruption performance. Such advancements would not only mitigate existing concerns but also solidify DAPS as a key enabler of seamless, low-latency mobility in 6G networks.

Although DAPS requires dual active protocol stacks, this complexity is not a showstopper because most 5G UEs already support dual connectivity with two stacks.

Enhancing DAPS with early UL/DL sync has strong potential to further reduce HO execution time and improve overall interruption performance.

RAN2 should investigate the implementation complexity and feasibility of DAPS in analogy with dual connectivity, and study early UL/DL synchronization enhancements to optimize DAPS performance for seamless mobility in 6G.

# Conclusion

Below is the list of the observations and the proposals:

Observation 1: In 5G, different services have varying mobility requirements depending on their tolerance to interruption time (how long a service can tolerate a break in connectivity during handover) and robustness (how resilient the connection must be to mobility-related disruptions).

Observation 2: A single mobility framework cannot meet the diverse interruption time and robustness requirements of all service types, highlighting the need for multiple, service-specific mobility frameworks in 6G.

Observation 3: Measuring interruption time only during the handover execution phase does not fully represent the overall interruption performance of a mobility procedure.

Observation 4: Mechanisms such as CHO and C-LTM primarily enhance mobility robustness rather than reducing interruption time, as service interruptions can still occur before the handover execution phase.

Observation 5: It is a misconception to claim that LTM inherently provides low interruption time, as its performance heavily depends on timing validity and procedural delays that can negate its intended benefits.

Observation 6: L3 handover, CHO, LTM, and C-LTM mechanisms are not adequate for services that require guaranteed interruption time reduction, as their design primarily focuses on improving mobility robustness rather than ensuring seamless, interruption-free connectivity.

Observation 7: DAPS, with its make-before-break architecture, effectively minimizes service interruption by maintaining the source link while establishing the target connection, making it the only existing mobility framework capable of supporting services that demand guaranteed low interruption time.

Proposal 1: DAPS should be considered in 6G as a fundamental mobility framework to ensure guaranteed low-interruption handovers and deliver seamless service continuity for latency-sensitive applications.

Observation 8: Although DAPS requires dual active protocol stacks, this complexity is not a showstopper because most 5G UEs already support dual connectivity with two stacks.

Observation 9:	 Enhancing DAPS with early UL/DL sync has strong potential to further reduce HO execution time and improve overall interruption performance.

Proposal 2: RAN2 should investigate the implementation complexity and feasibility of DAPS in analogy with dual connectivity, and study early UL/DL synchronization enhancements to optimize DAPS performance for seamless mobility in 6G.

# References

Anritsu Corporation, “MediaTek and Anritsu Achieve Industry First Dual Active Protocol Stack (DAPS) Handover for 5G New Radio (NR)” February 28, 2022. [Online]. Available: https://www.anritsu.com/en-us/test-measurement/news/news-releases/2022/2022-02-28-us03