3GPP TSG-RAN WG2 Meeting #132	R2-2508786

Dallas, Texas, USA, 17 - 21 November 2025

Agenda item:	8.20.1

Source:	Eutelsat Group, Thales, Hispasat, Airbus

Title:	Linear polarization orientation RRC signalling for NR NTN

WID/SID:	Release 19 / NR_NTN_Ku_bands

Document for:	Discussion and Decision

# 1	Introduction

This contribution provides some background on linear antenna polarisation orientation and proposes the introduction of related optional RRC signalling for NR NTN in the context of Rel-19.

# 2	Discussion

## 2.1	Antenna polarization background

Antenna polarization is a technique used in radio transmission, specifying the orientation of the electric field of a radio wave propagation. More specifically, polarization is a parameter defining how the electromagnetic wave is emitted or received by an antenna. Different polarization types may be selected depending on the expected properties and performances a telecommunication system.

Different polarization types have been specified for NR NTN [1] (TS 38.331) from Rel-17 onwards, namely:

- Circular polarization, with two possible orientations: Right Hand Circular Polarization (rhcp) or Left Hand Circular Polarization (lhcp),

- Linear polarization (linear).

Considering a typical use of circular polarization for initial NR NTN use cases, signalling those values by the satellite network to the UE has been considered sufficient to properly support NR NTN transmission in Rel-17 or Rel-18.

Observation 1:	Considering a typical use of circular polarization for initial NR NTN use cases, signalling one of rhcp, lhcp and linear values by the satellite network to the UE was deemed sufficient to properly support NR NTN transmission in Rel-17 or Rel-18.

## 2.2	Linear antenna polarisation orientation

However, the specification of Ku bands in Rel-19 [2] (Ku bands WID) is unlocking the support by 3GPP of new satellite communication scenarios that require the signalling of additional information for the case of linear polarization. More specifically, existing Ku band geostationary satellite systems use linear polarization that can take two orthogonal orientations (see [3] (Type Approval procedures - Eutelsat - section 12):

- Horizontal polarization, referred to as "X" (or "H") polarization,

- Vertical polarization, referred to as "Y" (or "V") polarization.

In such systems, the terrestrial VSAT User Equipment (UE) needs to be aware of - before accessing a NR NTN cell - which polarization orientation of the signals it is expected to receive from the satellite access node in the downlink on the service link, and which polarization orientation it shall use for the uplink transmission.

Note: current geostationary satellite systems use the two linear polarizations either:

to transmit two different signals on the same frequency, thus increasing the capacity over a region; or

to increase the isolation with respect to adjacent co-frequency beams (thus reducing interference).

In both cases it is essential that the UE uses the correct linear polarization in both downlink and uplink.

Observation 2:	A VSAT UE served by a linearly polarized satellite access node will need to distinguish between two orthogonal linear polarizations for both downlink reception and uplink transmission, independently.

This would align with the principles specified in Rel-17 for circular polarization, by which the satellite access node signals to the UE whether RHCP or LHCP polarization orientation is to be used in the downlink and in the uplink, information without which the system would not work appropriately.

Observation 3:	The introduction of new NR NTN operating bands - such as the Ku band in Rel-19 - is unlocking the support by 3GPP of new scenarios requiring the signalling of additional information for linear polarization, more specifically whether the polarization used at the satellite or at the UE is either horizontal or vertical.

## 2.3	Linear polarization orientation RRC signalling

As aforementioned, the signalling of linear polarization orientation is required in the same circumstances as the signalling of circular polarization orientation from Rel-17 onwards, that is:

In the NTN-Config information element (IE), which is further included in SIB19 (providing serving cell as well as neighbour cells configuration information to UEs), and in the ServingCellConfigCommon IE (typically used to provide cell information to a UE via dedicated signalling - e.g. in case of mobility scenarios)

In the CellsToAddModExt IE, which is further included in MeasObjectNR > cellsToAddModListExt IE (providing intra/inter-frequency measurements information to the UEs).

- It is therefore proposed to add an optional Rel-19 extension for conveying horizontal or vertical polarization orientation (e.g. xlp or ylp, respectively referring to X or Y polarization - see subclause 2.2) in the identified IEs (NTN-Config and CellsToAddModExt) when linear polarization is signalled as part of the existing Rel-17 parameters.

- NOTE: while both downlink and uplink polarizations are provisioned independently in the two listed Rel-17 IEs, only the downlink information is useful / used for measurement objects.

- A corresponding CR to TS 38.331 is provided in [5].

Proposal 1:	It is proposed to introduce Rel-19 optional extensions to signal, in the relevant IEs, one of the two horizontal / vertical polarization orientations when "linear" polarization is signalled as part of the Rel-17 signalling. The considered IEs are:

- NTN-Config IE (it shall be possible to signal DL and UL polarizations independently),

- CellsToAddModExt IE (only DL polarization orientation is required).

## 2.4	UE capabilities for linear polarization

In Rel-17, it has been decided that the "Support of polarization signalling in NR NTN" was an optional feature "without UE radio access capability parameters" (see [4] (TS 38.306 - clause 5).

It is therefore suggested that the same principles should apply for the Rel-19 extensions herein proposed.

A corresponding CR to TS 38.306 is provided in [6].

Proposal 2:	It is proposed that the support by the UE of the Rel-19 optional signalling extensions to provide the linear polarization orientation to the UE, as defined by Proposal 1, is an optional feature without UE radio access capability parameters.

# 3	Conclusion

This contribution made the following observations and proposals:

Observation 1:	Considering a typical use of circular polarization for initial NR NTN use cases, signalling one of rhcp, lhcp and linear values by the satellite network to the UE was deemed sufficient to properly support NR NTN transmission in Rel-17 or Rel-18.

Observation 2:	A VSAT UE served by a linearly polarized satellite access node will need to distinguish between two orthogonal linear polarizations for both downlink reception and uplink transmission, independently.

Observation 3:	The introduction of new NR NTN operating bands - such as the Ku band in Rel-19 - is unlocking the support by 3GPP of new scenarios requiring the signalling of additional information for linear polarization, more specifically whether the polarization used at the satellite or at the UE is either horizontal or vertical.

Proposal 1:	It is proposed to introduce Rel-19 optional extensions to signal, in the relevant IEs, one of the two horizontal / vertical polarization orientations when "linear" polarization is signalled as part of the Rel-17 signalling. The considered IEs are:

- NTN-Config IE (it shall be possible to signal DL and UL polarizations independently),

- CellsToAddModExt IE (only DL polarization orientation is required).

Proposal 2:	It is proposed that the support by the UE of the Rel-19 optional signalling extensions to provide the linear polarization orientation to the UE, as defined by Proposal 1, is an optional feature without UE radio access capability parameters.

# 4	References

[1]	TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

[2]	RP-250799: "Revised WID: Introduction of Ku Bands for NR NTN", RAN#109, Beijing (September 2025)

[3]	ESOG 120 Issue 8 Rev. 2: "Type Approval and characterization procedures" at https://www.eutelsat.com/sites/default/files/2025-08/DOC_Access-Space-Sgmnt_ESOG120_8_rev2_20240627.pdf - Eutelsat (June 2024)

[4]	TS 38.306: "NR; User Equipment (UE) radio access capabilities"

[5]	R2-2508788: Rel-19 CR to TS 38.331 Linear polarization orientation RRC signalling for NR NTN

[6]	R2-2508789: Rel-19 CR to TS 38.306 Linear polarization orientation RRC signalling for NR NTN