3GPP TSG-RAN WG2 Meeting #132	R2-2508788

# Dallas, Texas, USA, 17th - 21st November 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5603 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Linear polarization orientation RRC signalling for NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Eutelsat Group, Thales, Hispasat, Airbus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The introduction of new NR NTN operating bands - such as the Ku band in Rel-19 - is unlocking the support by 3GPP of new scenarios requiring the signalling of additional information for linear polarization, more specifically whether the polarization used at the satellite or at the UE is either horizontal (X-plane polarization) or vertical (Y-plane polarization). |  |  |  |  |  |  |  |  |
| Summary of change: |  | New information indicating which linear polarization orientation (xlp: X-plane linear polarization, ylp: Y-plane linear polarization) is used by the network for downlink transmission on the service link, or has to be used by the UE for uplink transmission, is added as Rel-19 optional fields to the following RRC IEs: - NTN-Config - MeasObjectNR (in which only DL information is meaningful in this release of the specification) The SIB19 field descriptions table is updated to state that NTN polarization information is provided by the ntn-Config IE. Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: NR NTN Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A VSAT UE served by a linearly polarized satellite access node will not be able to distinguish between the two X-Y linear polarization orientations for downlink reception or for uplink transmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.306 CR 1388 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 6.3.1	System information blocks

< Unmodified Text omitted >

#### –	SIB19

SIB19 contains satellite assistance information for NTN access.

SIB19 information element

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720         NTN-NeighCellConfigList-r17                     OPTIONAL        -- Need R

]],

[[

movingReferenceLocation-r18              ReferenceLocation-r17                           OPTIONAL,       -- Need R

ntn-CovEnh-r18                           NTN-CovEnh-r18                                  OPTIONAL,       -- Need R

satSwitchWithReSync-r18                  SatSwitchWithReSync-r18                         OPTIONAL        -- Need R

]]

}

NTN-NeighCellConfigList-r17 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfig-r17 ::=              SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-r17                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r17                           PhysCellId                                      OPTIONAL        -- Need R

}

NTN-CovEnh-r18 ::=                       SEQUENCE {

numberOfMsg4HARQ-ACK-Repetitions-r18     BIT STRING (SIZE(4)),

rsrp-ThresholdMsg4HARQ-ACK-r18           RSRP-Range                                      OPTIONAL        -- Need R

}

SatSwitchWithReSync-r18 ::=              SEQUENCE {

ntn-Config-r18                           NTN-Config-r17,

t-ServiceStart-r18                       INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

ssb-TimeOffset-r18                       INTEGER (0..159)                                OPTIONAL        -- Need R

}

-- TAG-SIB19-STOP

-- ASN1STOP

SIB19 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 50m. This field is only present in an NTN cell.

movingReferenceLocation Reference location of the serving cell of an NTN Earth-moving cell at a time reference. It is used in the evaluation of eventD2 and condEventD2 criteria for the serving cell in RRC_CONNECTED, and location-based measurement initiation in RRC_IDLE and RRC_INACTIVE when distanceThresh is also configured, as defined in TS 38.304 [20]. The time reference of this field is indicated by epochTime in ntn-Config of the serving cell. This field is excluded when determining changes in system information, i.e., changes to movingReferenceLocation should neither result in system information change notifications nor in a modification of valueTag in SIB1. This field is only present in an NTN cell.

ntn-Config Provides parameters needed for the UE to access NR via NTN access such as Ephemeris data, common TA parameters, k_offset, validity duration for UL sync information,epoch time and NTN polarization. In a TN cell, this field is only present in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt.

ntn-NeighCellConfigList, ntn-NeighCellConfigListExt Provides a list of NTN neighbour cells including their ntn-Config, carrier frequency and PhysCellId. This set includes all elements of ntn-NeighCellConfigList and all elements of ntn-NeighCellConfigListExt. If ntn-Config is absent for an entry in ntn-NeighCellConfigListExt, the ntn-Config provided in the entry at the same position in ntn-NeighCellConfigList applies. Network provides ntn-Config for the first entry of ntn-NeighCellConfigList. If the ntn-Config is absent for any other entry in ntn-NeighCellConfigList, the ntn-Config provided in the previous entry in ntn-NeighCellConfigList applies.

referenceLocation Reference location of the serving cell provided via NTN (quasi-)Earth fixed cell and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. This field is only present in an NTN cell.

satSwitchWithReSync Provides parameters for the target satellite required to perform satellite switch with resynchronization. This field is only present in an NTN cell and its presence indicates that satellite switch without PCI change is supported in the cell.

t-Service Indicates the time information on when a cell provided via NTN is going to stop serving the area it is currently covering. This field applies for both service link switches in NTN quasi-Earth fixed cell and feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). The exact stop time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field. The reference point for t-Service is the uplink time synchronization reference point of the cell. This field is only present in an NTN cell.

NTN-CovEnh field descriptions

numberOfMsg4HARQ-ACK-Repetitions The number of repetition slots for PUCCH transmission with HARQ-ACK information for Msg4, see clause 9.2.6 in TS 38.213 [13]. The first/leftmost bit corresponds to the repetition factor 1, the second bit corresponds to the repetition factor 2, the third bit corresponds to the repetition factor 4, and the last/rightmost bit corresponds to the repetition factor 8. The repetition factor 1 shall be indicated together with at least one other repetition factor.

rsrp-ThresholdMsg4HARQ-ACK This threshold is used by the UE for determining the configuration of the MAC entity for PUCCH repetition for Msg4 HARQ-ACK, as specified in clause 6.2.1 in TS 38.321 [3].

SatSwitchWithReSync field descriptions

ssb-TimeOffset Indicates the time offset of the SSB from target satellite at its uplink time synchronization reference point with respect to the SSB from source satellite at its uplink time synchronization reference point. The starting boundaries of SFN0 of the source satellite and the target satellite are aligned at the uplink time synchronization reference point. The value is given in number of subframes and can only be configured as 5 subframes or multiples of 5 subframes. This field is included only for soft satellite switch with re-synchronization.

t-ServiceStart Indicates the time information on when the target satellite is going to start serving the area currently covered by the serving satellite. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1st January 1900 (midnight between Sunday, December 31, 1899, and Monday, January 1, 1900). The exact start time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field. The reference point for t-ServiceStart is the uplink time synchronization reference point of the serving satellite.

< Unmodified Text omitted >

==============Next change==============

### 6.3.2	Radio resource control information elements

< Unmodified Text omitted >

#### –	MeasObjectNR

The IE MeasObjectNR specifies information applicable for SS/PBCH block(s) intra/inter-frequency measurements and/or CSI-RS intra/inter-frequency measurements.

MeasObjectNR information element

-- ASN1START

-- TAG-MEASOBJECTNR-START

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                        OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                   OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                          OPTIONAL,   -- Need R

nrofCSI-RS-ResourcesToAverage       INTEGER (2..maxNrofCSI-RS-ResourcesToAverage)                   OPTIONAL,   -- Need R

quantityConfigIndex                 INTEGER (1..maxNrofQuantityConfig),

offsetMO                            Q-OffsetRangeList,

cellsToRemoveList                   PCI-List                                                        OPTIONAL,   -- Need N

cellsToAddModList                   CellsToAddModList                                               OPTIONAL,   -- Need N

excludedCellsToRemoveList           PCI-RangeIndexList                                              OPTIONAL,   -- Need N

excludedCellsToAddModList           SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

allowedCellsToRemoveList            PCI-RangeIndexList                                              OPTIONAL,   -- Need N

allowedCellsToAddModList            SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

...,

[[

freqBandIndicatorNR                 FreqBandIndicatorNR                                             OPTIONAL,   -- Need R

measCycleSCell                      ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}  OPTIONAL    -- Need R

]],

[[

smtc3list-r16                       SSB-MTC3List-r16                                                OPTIONAL,   -- Need R

rmtc-Config-r16                     SetupRelease {RMTC-Config-r16}                                  OPTIONAL,   -- Need M

t312-r16                            SetupRelease { T312-r16 }                                       OPTIONAL    -- Need M

]],

[[

associatedMeasGapSSB-r17            MeasGapId-r17                                                   OPTIONAL,   -- Need R

associatedMeasGapCSIRS-r17          MeasGapId-r17                                                   OPTIONAL,   -- Need R

smtc4list-r17                       SSB-MTC4List-r17                                                OPTIONAL,   -- Need R

measCyclePSCell-r17                 ENUMERATED {ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, spare1}

OPTIONAL,   -- Cond SCG

cellsToAddModListExt-v1710          CellsToAddModListExt-v1710                                      OPTIONAL    -- Need N

]],

[[

associatedMeasGapSSB2-v1720         MeasGapId-r17                                               OPTIONAL, -- Cond AssociatedGapSSB

associatedMeasGapCSIRS2-v1720       MeasGapId-r17                                               OPTIONAL  -- Cond AssociatedGapCSIRS

]],

[[

measSequence-r18                    MeasSequence-r18                                                OPTIONAL,   -- Need R

cellsToAddModListExt-v1800          CellsToAddModListExt-v1800                                      OPTIONAL    -- Need N

]],

[[

smtc5list-r19                       SSB-MTC5List-r19                                                OPTIONAL,   -- Need R

smtc6list-r19                       SSB-MTC6List-r19                                                OPTIONAL,   -- Need R

smtc7-SSBAdapt-r19                  SSB-MTC-SSBAdapt-r19                                      OPTIONAL,   -- Cond IntraFreqConnected

neighSCellMeasSkipping-r19          ENUMERATED { enabled }                                          OPTIONAL    -- Cond SCellMO

cellsToAddModListExt-v19xx          CellsToAddModListExt-v19xx                                      OPTIONAL    -- Need N

]]

}

SSB-MTC3List-r16::=                 SEQUENCE (SIZE(1..4)) OF SSB-MTC3-r16

SSB-MTC4List-r17::=                 SEQUENCE (SIZE(1..3)) OF SSB-MTC4-r17

SSB-MTC5List-r19::=                 SEQUENCE (SIZE(1..3)) OF SSB-MTC5-r19

SSB-MTC6List-r19::=                 SEQUENCE (SIZE(1..6)) OF SSB-MTC

SSB-MTC-SSBAdapt-r19 ::=            SEQUENCE (SIZE(1..2)) OF SSB-MTC

T312-r16 ::=                        ENUMERATED { ms0, ms50, ms100, ms200, ms300, ms400, ms500, ms1000}

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

SSB-ConfigMobility::=               SEQUENCE {

ssb-ToMeasure                       SetupRelease { SSB-ToMeasure }                                  OPTIONAL,   -- Need M

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                             OPTIONAL,   -- Need M

...,

[[

ssb-PositionQCL-Common-r16              SSB-PositionQCL-Relation-r16                                OPTIONAL,   -- Cond SharedSpectrum

ssb-PositionQCL-CellsToAddModList-r16   SSB-PositionQCL-CellsToAddModList-r16                       OPTIONAL,   -- Need N

ssb-PositionQCL-CellsToRemoveList-r16   PCI-List                                                    OPTIONAL    -- Need N

]],

[[

deriveSSB-IndexFromCellInter-r17    ServCellIndex                                                   OPTIONAL,   -- Need R

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                                OPTIONAL,   -- Cond SharedSpectrum2

ssb-PositionQCL-Cells-r17           SetupRelease {SSB-PositionQCL-CellList-r17}                     OPTIONAL    -- Need M

]],

[[

cca-CellsToAddModList-r17           PCI-List                                                        OPTIONAL,   -- Need N

cca-CellsToRemoveList-r17           PCI-List                                                        OPTIONAL    -- Need N

]],

[[

ssb-ToMeasureAltitudeBasedList-r18  SetupRelease { SSB-ToMeasureAltitudeBasedList-r18 }             OPTIONAL    -- Need M

]]

}

Q-OffsetRangeList ::=               SEQUENCE {

rsrpOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrqOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

sinrOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrpOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

rsrqOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

sinrOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0

}

ThresholdNR ::=                     SEQUENCE{

thresholdRSRP                       RSRP-Range                                                      OPTIONAL,   -- Need R

thresholdRSRQ                       RSRQ-Range                                                      OPTIONAL,   -- Need R

thresholdSINR                       SINR-Range                                                      OPTIONAL    -- Need R

}

CellsToAddModList ::=               SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddMod

CellsToAddModListExt-v1710 ::=      SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddModExt-v1710

CellsToAddModListExt-v1800 ::=      SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddModExt-v1800

CellsToAddModListExt-v19xx ::=      SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddModExt-v19xx

CellsToAddMod ::=                   SEQUENCE {

physCellId                          PhysCellId,

cellIndividualOffset                Q-OffsetRangeList

}

CellsToAddModExt-v1710 ::=          SEQUENCE {

ntn-PolarizationDL-r17              ENUMERATED {rhcp,lhcp,linear}                                   OPTIONAL,  -- Need R

ntn-PolarizationUL-r17              ENUMERATED {rhcp,lhcp,linear}                                   OPTIONAL   -- Need S

}

CellsToAddModExt-v1800 ::=          SEQUENCE {

ntn-NeighbourCellInfo-r18           NTN-NeighbourCellInfo-r18                                       OPTIONAL   -- Cond NeighbourCell

}

CellsToAddModExt-v19xx ::=          SEQUENCE {

ntn-LinearPolarizationDL-r19              ENUMERATED {xlp,ylp}                                      OPTIONAL,  -- Need R

ntn-LinearPolarizationUL-r19              ENUMERATED {xlp,ylp}                                      OPTIONAL   -- Need S

}

RMTC-Config-r16 ::=                 SEQUENCE {

rmtc-Periodicity-r16                ENUMERATED {ms40, ms80, ms160, ms320, ms640},

rmtc-SubframeOffset-r16             INTEGER(0..639)                                                 OPTIONAL,   -- Need M

measDurationSymbols-r16             ENUMERATED {sym1, sym14or12, sym28or24, sym42or36, sym70or60},

rmtc-Frequency-r16                  ARFCN-ValueNR,

ref-SCS-CP-r16                      ENUMERATED {kHz15, kHz30, kHz60-NCP, kHz60-ECP},

...,

[[

rmtc-Bandwidth-r17                  ENUMERATED {mhz100, mhz400, mhz800, mhz1600, mhz2000}           OPTIONAL,   -- Need R

measDurationSymbols-v1700           ENUMERATED {sym140, sym560, sym1120}                            OPTIONAL,   -- Need R

ref-SCS-CP-v1700                    ENUMERATED {kHz120, kHz480, kHz960}                             OPTIONAL,   -- Need R

tci-StateInfo-r17               SEQUENCE {

tci-StateId-r17                  TCI-StateId,

ref-ServCellId-r17               ServCellIndex                                                  OPTIONAL   -- Need R

} OPTIONAL -- Need R

]],

[[

ref-BWPId-r17                   BWP-Id                                                              OPTIONAL   -- Need R

]]

}

SSB-PositionQCL-CellsToAddModList-r16 ::= SEQUENCE (SIZE (1..maxNrofCellMeas)) OF SSB-PositionQCL-CellsToAddMod-r16

SSB-PositionQCL-CellsToAddMod-r16 ::= SEQUENCE {

physCellId-r16                        PhysCellId,

ssb-PositionQCL-r16                   SSB-PositionQCL-Relation-r16

}

SSB-PositionQCL-CellList-r17 ::= SEQUENCE (SIZE (1..maxNrofCellMeas)) OF SSB-PositionQCL-Cell-r17

SSB-PositionQCL-Cell-r17         ::= SEQUENCE {

physCellId-r17                        PhysCellId,

ssb-PositionQCL-r17                   SSB-PositionQCL-Relation-r17

}

SSB-ToMeasureAltitudeBasedList-r18 ::= SEQUENCE (SIZE (1..maxNrofAltitudeRanges-r18)) OF SSB-ToMeasureAltitudeBased-r18

SSB-ToMeasureAltitudeBased-r18 ::=     SEQUENCE {

altitudeRange-r18                      SEQUENCE {

altitudeMin-r18                        Altitude-r18                                             OPTIONAL,  -- Need S

altitudeMax-r18                        Altitude-r18                                             OPTIONAL,  -- Need S

altitudeHyst-r18                       HysteresisAltitude-r18                                   OPTIONAL   -- Need R

},

ssb-ToMeasure-r18                      SSB-ToMeasure                                                OPTIONAL   -- Need S

}

NTN-NeighbourCellInfo-r18 ::=          SEQUENCE {

epochTime-r18                          EpochTime-r17,

ephemerisInfo-r18                      EphemerisInfo-r17,

referenceLocation-r18                  ReferenceLocation-r17                                        OPTIONAL   -- Need R

}

-- TAG-MEASOBJECTNR-STOP

-- ASN1STOP

CellsToAddMod field descriptions

cellIndividualOffset Cell individual offsets applicable to a specific cell.

ntn-LinearPolarizationDL If present, this parameter indicates linear polarization information used by the network for downlink transmission on the service link, i.e. X-plane (horizontal) linear polarization (xlp) or Y-plane (vertical) linear polarization (ylp).

ntn-LinearPolarizationUL In this version of the specification, the network does not include this field.

ntn-NeighbourCellInfo Includes satellite assistance information of an NTN neighbour cell.

ntn-PolarizationDL If present, this parameter indicates polarization information for downlink transmission on service link: including Right hand, Left hand circular polarizations (RHCP, LHCP) and Linear polarization.

ntn-PolarizationUL In this version of the specification, the network does not include this field.

physCellId Physical cell identity of a cell in the cell list.

referenceLocation Reference location of a neighbor NTN Earth-moving cell for the evaluation of the trigger criteria of an associated ReportConfig which contains EventD2 or condEventD2.

MeasObjectNR field descriptions

absThreshCSI-RS-Consolidation Absolute threshold for the consolidation of measurement results per CSI-RS resource(s) from L1 filter(s). The field is used for the derivation of cell measurement results as described in 5.5.3.3 and the reporting of beam measurement information per CSI-RS resource as described in 5.5.5.2.

absThreshSS-BlocksConsolidation Absolute threshold for the consolidation of measurement results per SS/PBCH block(s) from L1 filter(s). The field is used for the derivation of cell measurement results as described in 5.5.3.3 and the reporting of beam measurement information per SS/PBCH block index as described in 5.5.5.2.

allowedCellsToAddModList List of cells to add/modify in the allow-list of cells. It applies only to SSB resources.

allowedCellsToRemoveList List of cells to remove from the allow-list of cells.

associatedMeasGapSSB Indicates the associated measurement gap for SSB measuring identified by ssb-ConfigMobility in this measurement object. When multiple MeasObjectNR with the same SSB frequency are configured, the network configures the same measurement gap ID in this field for each MeasObjectNR. If this field is absent, the associated measurement gap is the gap configured via gapFR1, gapFR2, or gapUE.

associatedMeasGapSSB2 Indicates the associated additional measurement gap for SSB measuring identified by ssb-ConfigMobility in this measurement object for NTN deployments. When multiple MeasObjectNR with the same SSB frequency are configured, the network configures the same measurement gap ID in this field for each MeasObjectNR. If this field is absent, the associated measurement gap is the gap indicated by associatedMeasGapSSB.

associatedMeasGapCSIRS Indicates the associated measurement gap for CSI-RS measuring identified by csi-rs-ResourceConfigMobility in this measurement object. If this field is absent, the associated measurement gap is the gap configured via gapFR1, gapFR2, or gapUE.

associatedMeasGapCSIRS2 Indicates the associated additional measurement gap for CSI-RS measuring identified by csi-rs-ResourceConfigMobility in this measurement object for NTN deployments. If this field is absent, the associated measurement gap is the gap indicated by associatedMeasGapCSIRS. In this release of the specification, this field is not configured for NTN deployments.

cellsToAddModList List of cells to add/modify in the cell list. If the network includes cellsToAddModListExt-v1710 and/or cellsToAddModListExt-v1800 and/or cellsToAddModListExt-v19xx, it contains the same number of entries listed in the same order as in cellsToAddModList (i.e. without suffix).

cellsToRemoveList List of cells to remove from the cell list.

excludedCellsToAddModList List of cells to add/modify in the exclude-list of cells. It applies only to SSB resources.

excludedCellsToRemoveList List of cells to remove from the exclude-list of cells.

freqBandIndicatorNR The frequency band in which the SSB and/or CSI-RS indicated in this MeasObjectNR are located and according to which the UE shall perform the RRM measurements. This field is always provided when the network configures measurements with this MeasObjectNR.

measCyclePSCell The parameter is used only when the PSCell is configured on the frequency indicated by the measObjectNR and the SCG is deactivated, see TS 38.133 [14]. The field may also be configured when the PSCell is not configured on that frequency. The network always configures measCyclePSCell for the measObjectNR associated with the PSCell if bfd-and-RLM is set to true and the SCG is deactivated. Value ms160 corresponds to 160 ms, value ms256 corresponds to 256 ms and so on.

measCycleSCell The parameter is used only when an SCell is configured on the frequency indicated by the measObjectNR and is in deactivated state, see TS 38.133 [14]. gNB configures the parameter whenever an SCell is configured on the frequency indicated by the measObjectNR, but the field may also be signalled when an SCell is not configured. Value sf160 corresponds to 160 sub-frames, value sf256 corresponds to 256 sub-frames and so on.

measSequence Indicates the recommended sequence for intra/inter-RAT intra/inter-frequency measurement. Value 1 means the corresponding frequency is measured firstly. Value 2 means the corresponding frequency is measured secondly and so on. If more than one frequency is configured with the same value, it means no recommended sequence among these frequencies. If not provided, it means there is no recommended sequence for the corresponding frequency. This field is only configured for NR standalone or if the measObject is associated to the MCG.

neighSCellMeasSkipping If configured, the UE is not required to perform neighbour cell measurements on the frequency of this MeasObjectNR. This field is only applicable to UE supporting antennaArrayType-r18 and CA.

nrofCSI-RS-ResourcesToAverage Indicates the maximum number of measurement results per beam based on CSI-RS resources to be averaged. The same value applies for each detected cell associated with this MeasObjectNR.

nrofSS-BlocksToAverage Indicates the maximum number of measurement results per beam based on SS/PBCH blocks to be averaged. The same value applies for each detected cell associated with this MeasObject.

offsetMO Offset values applicable to all measured cells with reference signal(s) indicated in this MeasObjectNR.

quantityConfigIndex Indicates the n-th element of quantityConfigNR-List provided in MeasConfig.

referenceSignalConfig RS configuration for SS/PBCH block and CSI-RS.

refFreqCSI-RS Point A which is used for mapping of CSI-RS to physical resources according to TS 38.211 [16] clause 7.4.1.5.3.

smtc1 Primary measurement timing configuration. (see clause 5.5.2.10).

smtc2 Secondary measurement timing configuration for SS corresponding to this MeasObjectNR with PCI listed in pci-List. For these SS, the periodicity is indicated by periodicity in smtc2 and the timing offset is equal to the offset indicated in periodicityAndOffset modulo periodicity. periodicity in smtc2 can only be set to a value strictly shorter than the periodicity indicated by periodicityAndOffset in smtc1 (e.g. if periodicityAndOffset indicates sf10, periodicity can only be set of sf5, if periodicityAndOffset indicates sf5, smtc2 cannot be configured). This field is not configured together with smtc4list.

smtc3list Measurement timing configuration list for SS corresponding to IAB-MT. This is used for the IAB-node's discovery of other IAB-nodes and the IAB-Donor-DUs.

smtc4list, smtc5list Measurement timing configuration list for NTN deployments, see clause 5.5.2.10. smtc5list may include an additional SMTC periodicity to the one indicated in smtc1.

smtc6list Measurement timing configuration list for OD-SSB, see clause 5.5.2.10.

smtc7-SSBAdapt Measurement timing configuration for adapted SSB transmission, see clause 5.5.2.10.

ssbFrequency Indicates the frequency of the SS associated to this MeasObjectNR. For operation with shared spectrum channel access, this field is a k*30 kHz shift from the sync raster where k = 0,1,2, and so on if the reportType within the corresponding ReportConfigNR is set to reportCGI (see TS 38.211 [16], clause 7.4.3.1). Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15], or TS 38.101-5 [75]).

ssb-PositionQCL-Common Indicates the QCL relationship between SS/PBCH blocks for all measured cells as specified in TS 38.213 [13], clause 4.1.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

t312 The value of timer T312. Value ms0 represents 0 ms, ms50 represents 50 ms and so on.

ReferenceSignalConfig field descriptions

csi-rs-ResourceConfigMobility CSI-RS resources to be used for CSI-RS based RRM measurements.

ssb-ConfigMobility SSB configuration for mobility (nominal SSBs, timing configuration).

RMTC-Config field descriptions

measDurationSymbols Number of consecutive symbols for which the Physical Layer reports samples of RSSI (see TS 38.215 [9], clause 5.1.21). Value sym1 corresponds to one symbol, sym14or12 corresponds to 14 symbols of the reference numerology for NCP and 12 symbols for ECP, and so on. If measDurationSymbols-v1700 is signalled, the UE ignores measDurationSymbols-r16.

ref-BWPId Indicates the reference BWP for the TCI state indicated in tci-StateInfo. Network includes this field if tci-StateInfo is present. This field is only applicable for operation with shared spectrum channel access in FR2-2 and network does not configure this if the UE does not have any serving cells in FR2-2.

ref-SCS-CP Indicates a reference subcarrier spacing and cyclic prefix to be used for RSSI measurements (see TS 38.215 [9]). Value kHz15 corresponds to 15kHz, kHz30 corresponds to 30 kHz, value kHz60-NCP corresponds to 60 kHz using normal cyclic prefix (NCP), and kHz60-ECP corresponds to 60 kHz using extended cyclic prefix (ECP). If ref-SCS-CP-v1700 is signalled, the UE ignores ref-SCS-CP-r16.

ref-ServCellId Indicates the FR2-2 reference serving cell index for the TCI state. Network includes this field if tci-StateInfo is present. This field is only applicable for operation with shared spectrum channel access in FR2-2 and network does not configure this if the UE does not have any serving cells in FR2-2.

rmtc-Bandwidth Indicates the bandwidth for the RSSI measurement (see TS 38. 215 [9], clause 5.1.21).

rmtc-Frequency Indicates the center frequency of the measured bandwidth for a frequency which operates with shared spectrum channel access (see TS 38. 215 [9], clause 5.1.21).

rmtc-Periodicity Indicates the RSSI measurement timing configuration (RMTC) periodicity (see TS 38.215 [9], clause 5.1.21).

rmtc-SubframeOffset Indicates the RSSI measurement timing configuration (RMTC) subframe offset for this frequency (see TS 38.215 [9], clause 5.1.21). For inter-frequency measurements, this field is optional present and if it is not configured, the UE chooses a random value as rmtc-SubframeOffset for measDurationSymbols which shall be selected to be between 0 and the configured rmtc-Periodicity with equal probability.

tci-StateId Indicates the TCI state to be used for RSSI measurements. This field is only applicable for shared spectrum channel access in FR2-2. Network does not configure this if the UE does not have any serving cells in FR2-2 and in such a case, it is up to UE implementation how to determine the spatial domain filter for the inter-frequency RSSI measurement in FR2-2.

SSB-ConfigMobility field descriptions

cca-CellsToAddModList, cca-CellsToRemoveList Lists of cells to be added or removed from the list of neighbor cells that apply channel access mode procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2.

deriveSSB-IndexFromCell If this field is set to true, UE assumes SFN and frame boundary alignment across cells on the same frequency carrier as specified in TS 38.133 [14]. Hence, if the UE is configured with a serving cell for which (absoluteFrequencySSB, subcarrierSpacing) in ServingCellConfigCommon is equal to (ssbFrequency, ssbSubcarrierSpacing) in this MeasObjectNR, this field indicates whether the UE can utilize the timing of this serving cell to derive the index of SS block transmitted by neighbour cell. Otherwise, this field indicates whether the UE may use the timing of any detected cell on that target frequency to derive the SSB index of all neighbour cells on that frequency.

deriveSSB-IndexFromCellInter If this field is present, UE assumes SFN and frame boundary alignment between the reference serving cell indicated by ServCellIndex and all neighbour cells in this MeasObjectNR as specified in TS 38.133 [14]. This field also indicates that the UE can utilize the timing of the reference serving cell indicated by ServCellIndex to derive the index of SS block transmitted by all inter-frequency neighbour cells on the frequency indicated by the MeasObjectNR. When this field is included, the network should set deriveSSB-IndexFromCell to true.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not to be measured while value 1 indicates that the corresponding SS/PBCH block is to be measured (see TS 38.215 [9]). When the field is not configured the UE measures on all SS blocks. Regardless of the value of this field, SS/PBCH blocks outside of the applicable smtc are not to be measured. See TS 38.215 [9] clause 5.1.1.

ssb-ToMeasureAltitudeBasedList List of altitude-dependent ssb-ToMeasure. When the UE is within an altitude range indicated by altitudeRange, it ignores the ssb-ToMeasure (without suffix), and applies the corresponding ssb-ToMeasure-r18 if present, otherwise (i.e., the UE is within an altitude range indicated by altitudeRange and ssb-ToMeasure-r18 is absent) it measures on all SS-blocks. When the UE is outside all the altitude ranges indicated by altitudeRange (if any), ssb-ToMeasure (without suffix) applies. For each altitude range, altitudeMin indicates the minimum altitude in meters relative to sea level, altitudeMax indicates the maximum altitude in meters relative to sea level, and if included, altitudeHyst indicates hysteresis in meters for determination of the altitude range. I.e., when altitudeHyst is configured for an altitude range, the UE considers itself to have entered the range if altitudeMin ≤ UE altitude ≤ altitudeMax and after entering the range considers itself to be in the range while (altitudeMin – altitudeHyst) ≤ UE altitude ≤ (altitudeMax + altitudeHyst). For each altitudeRange, if altitudeMin is absent, value minAltitude-r18 is used and if altitudeMax is absent, value maxAltitude-r18 is used.

SSB-PositionQCL-CellsToAddMod field descriptions

physCellId Physical cell identity of a cell in the cell list.

ssb-PositionQCL Indicates the QCL relation between SS/PBCH blocks for a specific cell as specified in TS 38.213 [13], clause 4.1. If provided, the cell specific value overwrites the value signalled by ssb-PositionQCL-Common.

| Conditional Presence | Explanation |
| --- | --- |
| AssociatedGapCSIRS | This field is optionally present, Need R if associatedMeasGapCSIRS is configured, otherwise, it is absent. |
| AssociatedGapSSB | This field is optionally present, Need R if associatedMeasGapSSB is configured, otherwise, it is absent. |
| CSI-RS | This field is mandatory present if csi-rs-ResourceConfigMobility is configured, otherwise, it is absent. |
| IntraFreqConnected | This field is optionally present, Need R if the UE is configured with a serving cell for which (absoluteFrequencySSB, subcarrierSpacing) in ServingCellConfigCommon is equal to (ssbFrequency, ssbSubcarrierSpacing) in this MeasObjectNR, otherwise, it is absent. |
| SCellMO | This field is optionallly present, Need R, in the MeasObjectNR associated with an SCell. Otherwise the field is absent. |
| SCG | This field is optionallly present, Need R, in the measConfig associated with the SCG. It is absent in the measConfig associated with the MCG. |
| NeighbourCell | This field is mandatory present if this MeasObject is configured by the serving cell for a neighbour cell served by a NTN Earth-moving cell and is associated with a ReportConfig which contains EventD2 or condEventD2. Otherwise, it is optional, Need R. |
| SharedSpectrum | This field is mandatory present if this MeasObject is for a frequency which operates with shared spectrum channel access in FR1. Otherwise, it is absent, Need R. |
| SharedSpectrum2 | This field is optionally present if this MeasObject is for a frequency which operates with shared spectrum channel access in FR2-2, Need R. Otherwise, it is absent, Need R. |
| SSBorAssociatedSSB | This field is mandatory present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. Otherwise, it is absent, Need R. |

< Unmodified Text omitted >

#### –	NTN-Config

The IE NTN-Config provides parameters needed for the UE to access NR via NTN access.

NTN-Config information element

-- ASN1START

-- TAG-NTN-CONFIG-START

NTN-Config-r17 ::=             SEQUENCE {

epochTime-r17                  EpochTime-r17                                                            OPTIONAL,  -- Need R

ntn-UlSyncValidityDuration-r17 ENUMERATED{ s5, s10, s15, s20, s25, s30, s35,

s40, s45, s50, s55, s60, s120, s180, s240, s900}              OPTIONAL,  -- Cond SIB19

cellSpecificKoffset-r17        INTEGER(1..1023)                                                         OPTIONAL,  -- Need R

kmac-r17                       INTEGER(1..512)                                                          OPTIONAL,  -- Need R

ta-Info-r17                    TA-Info-r17                                                              OPTIONAL,  -- Need R

ntn-PolarizationDL-r17         ENUMERATED {rhcp,lhcp,linear}                                            OPTIONAL,  -- Need R

ntn-PolarizationUL-r17         ENUMERATED {rhcp,lhcp,linear}                                            OPTIONAL,  -- Need S

ephemerisInfo-r17              EphemerisInfo-r17                                                        OPTIONAL,  -- Need R

ta-Report-r17                  ENUMERATED {enabled}                                                     OPTIONAL,  -- Need R

...,

[[

ntn-LinearPolarizationDL-r19   ENUMERATED {xlp,ylp}                                                     OPTIONAL,  -- Need R

ntn-LinearPolarizationUL-r19   ENUMERATED {xlp,ylp}                                                     OPTIONAL,  -- Need S

]]

}

TA-Info-r17 ::=                 SEQUENCE  {

ta-Common-r17                  INTEGER(0..66485757),

ta-CommonDrift-r17             INTEGER(-257303..257303)                                                 OPTIONAL,  -- Need R

ta-CommonDriftVariant-r17      INTEGER(0..28949)                                                        OPTIONAL   -- Need R

}

-- TAG-NTN-CONFIG-STOP

-- ASN1STOP

NTN-Config field descriptions

ephemerisInfo This field provides satellite ephemeris either in format of position and velocity state vector or in format of orbital parameters. This field is excluded when determining changes in system information, i.e. changes to ephemerisInfo should neither result in system information change notifications nor in a modification of valueTag in SIB1.

epochTime If this field is absent for the NTN serving cell, the epoch time is the end of SI window where this SIB19 is scheduled. This field is mandatory present when ntn-Config is provided in dedicated configuration. If this field is absent in ntn-Config provided via NTN-NeighCellConfig or SatSwitchWithReSync in an NTN cell, the UE uses epoch time of the serving cell, otherwise the field is based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the serving cell. If this field is absent in ntn-Config provided via NTN-NeighCellConfig in a TN cell, the epoch time is the end of SI window where this SIB19 is scheduled. In case of satellite switch with resynchronization, this field is based on the timing of the cell served by the source satellite. This field is excluded when determining changes in system information, i.e. changes to epochTime should neither result in system information change notifications nor in a modification of valueTag in SIB1.

cellSpecificKoffset Scheduling offset used for the timing relationships that are modified for NTN (see TS 38.213 [13]). The unit of the field K_offset is number of slots for a given subcarrier spacing of 15 kHz. If the field is absent UE assumes value 0.

kmac Scheduling offset provided by network if downlink and uplink frame timing are not aligned at gNB. If the field is absent UE assumes value 0. The unit of kmac is number of slots for a given subcarrier spacing of 15 kHz.

ntn-LinearPolarizationDL If present, this parameter indicates linear polarization information used by the network for the downlink transmission on the service link, i.e. X-plane (horizontal) linear polarization (xlp) or Y-plane (vertical) linear polarization (ylp). If present, the UE shall ignore the information indicated by the field ntn-PolarizationDL if the latter is included.

ntn-LinearPolarizationUL If present, this parameter indicates linear polarization information to be used by the UE on the uplink service link. If not present and ntn-LinearPolarizationDL is present, UE assumes the same linear polarization information for UL and DL. If none of ntn-LinearPolarizationUL and ntn-LinearPolarizationDL is present, the UE releases any stored value. If either ntn-LinearPolarizationUL or ntn-LinearPolarizationDL is present, the UE shall ignore the information indicated by the fields ntn-PolarizationDL or ntn-PolarizationUL if any of these parameters is included.

ntn-PolarizationDL If present, this parameter indicates polarization information for downlink transmission on service link: including Right hand, Left hand circular polarizations (RHCP, LHCP) and Linear polarization.

ntn-PolarizationUL If present, this parameter indicates Polarization information for uplink service link. If not present and ntn-PolarizationDL is present, UE assumes the same polarization for UL and DL. If both ntn-PolarizationUL and ntn-PolarizationDL are not present, the UE releases any stored value.

ntn-UlSyncValidityDuration A validity duration configured by the network for assistance information (i.e. Serving and/or neighbour satellite ephemeris and Common TA parameters) which indicates the maximum time duration (from epochTime) during which the UE can apply assistance information without having acquired new assistance information. The unit of ntn-UlSyncValidityDuration is second. Value s5 corresponds to 5 s, value s10 indicate 10 s and so on. If this field is absent in ntn-Config provided via NTN-NeighCellConfig or SatSwitchWithReSync in an NTN cell, the UE uses validity duration from the serving cell assistance information. If this field is absent in ntn-Config provided via NTN-NeighCellConfig in a TN cell, how the UE sets the validity duration is left to UE implementation. This field is excluded when determining changes in system information, i.e. changes of ntn-UlSyncValidityDuration should neither result in system information change notifications nor in a modification of valueTag in SIB1. ntn-UlSyncValidityDuration is only updated when at least one of epochTime, ta-Info, ephemerisInfo is updated.

ta-Common Network-controlled common timing advanced value and it may include any timing offset considered necessary by the network. ta-Common with value of 0 is supported. The granularity of ta-Common is 4.072 × 10^(-3) μs. Values are given in unit of corresponding granularity. This field is excluded when determining changes in system information, i.e. changes of ta-Common should neither result in system information change notifications nor in a modification of valueTag in SIB1.

ta-CommonDrift Indicate drift rate of the common TA. The granularity of ta-CommonDrift is 0.2 × 10^(-3) μs⁄s. Values are given in unit of corresponding granularity. This field is excluded when determining changes in system information, i.e. changes of ta-CommonDrift should neither result in system information change notifications nor in a modification of valueTag in SIB1.

ta-CommonDriftVariant Indicate drift rate variation of the common TA. The granularity of ta-CommonDriftVariant is 0.2×10^(-4) μs⁄s^2. Values are given in unit of corresponding granularity. This field is excluded when determining changes in system information, i.e. changes of ta-CommonDriftVariant should neither result in system information change notifications nor in a modification of valueTag in SIB1.

ta-Report When this field is included in SIB19, it indicates reporting of timing advanced is enabled during Random Access due to RRC connection establishment or RRC connection resume, and during RRC connection reestablishment. When this field is included in ServingCellConfigCommon within dedicated signalling, it indicates TA reporting is enabled during reconfiguration with sync (see TS 38.321 [3], clause 5.4.8).

| Conditional Presence | Explanation |
| --- | --- |
| SIB19 | The field is mandatory present for the serving cell in SIB19. The field is optionally present, Need R, otherwise. |

< Unmodified Text omitted >

==============End of change==============