3GPP TSG-RAN WG2 Meeting #132	R2- 2508814

Dallas, USA, November 17-21, 2025

Agenda Item:	10.2.2

Source:	THALES, TNO, Airbus, ESA, Novamint, EchoStar

Title:	6G TN/NTN Integration

Document for:	Discussion

# 1	Introduction

The 6G Radio Study Item approved at the RAN#108 meeting and updated at RAN#109 [1] states The main objectives related to the radio protocol architecture and the target to have a harmonized 6G radio design for TN and NTN:

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] a) Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE ca i) Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. […] j) Aim at a harmonized 6G Radio design for TN and NTN, including their integration. …

The intent is therefore to achieve a 6G Radio design harmonized for the support of both TN and NTN deployment scenarios as well as deployment scenarios integrating NTN and TN components from Rel-21. Some aspects were discussed in [2]. In this contribution, we share our views on the support of NTN in the 6GR.

General aspects were discussed in [3] during the RAN2#131bis meeting, and we shared our views on the support of NTN in the 6GR.

In this contribution the focus will be on what are the minimal essential set of requirements for NTN, what are the essential features for day-1, what may be left for later. Identify the set of functionalities that should be considered in common design with TN and how to move forward with TN/NTN integration in the study phase. As per the following guidelines from the RAN2#131bis meeting:

For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

# 2	Discussion

## 2.1	Design implications of native NTN support in 6G

The study of 5G Non-Terrestrial Networks , including both NR-NTN and IoT-NTN, began in Release 16, with a wide range of NTN-specific adaptations [6] studied and specified across Releases 17 to 19. Significant effort was made to the discussion and evaluation of these features, even though large-scale field deployments are still ongoing. To prevent repetitive efforts and address already identified challenges, it is essential to take these NTN-specific features into account when advancing toward 6G NTN/TN harmonization.

While, in 5G, NTN support was introduced after the fundamental 5G NR access design principles were already established, it is strongly recommended for 6G that the basic enablers for NTN be defined and specified as integral components of the 6G radio access framework, starting from the initial 6G specification in Release 21. This approach will allow for:

- Achieving ubiquitous connectivity and availability of services in remote and sparsely populated areas.

- Improving network resiliency, especially in situations where disasters disrupt terrestrial networks or damage undersea communication cables.

By natively incorporating NTN, 6G can deliver unprecedented levels of performance, resilience, and global coverage, fulfilling the evolving demands of the future mobile ecosystem.

Achieving comprehensive integration requires a unified and adaptable radio interface that seamlessly supports both terrestrial and satellite connections. This requires that we should take into consideration the NTN aspects while designing control plane , user plane , and mobility aspects. In addition, the common design must be flexible / adaptable to support the integration of 6G-NTN-specific characteristics and scenarios, and it should consider the following points and detailed in the next section :

- Compatibility with diverse satellite orbits, such as Very LEO, LEO, MEO, and GSO.

- Robustness against challenges like Doppler shifts, significant and variable round-trip times (RTT), delay spreads, and low signal-to-noise ratios (SNRs).

- Mobility management and handover mechanisms to account for the high relative velocities and extended coverage areas of non-terrestrial networks.

## 2.2	6G NTN Specific requirements

### 2.2.1	NTN Payload types and scenarios

Figure 1 : Proposal for a 6G NTN deployment scenario

Below are some considerations for practical NTN scenarios for 6G. The different scenarios are illustrated on the figure above. The following orbits should be considered:

- VLEO (very low Earth orbit) based scenario at minimum altitude of 300 km: Satellites operating at this altitude benefit from reduced latency due to their proximity to the surface of Earth, which can enhance communication and observation capabilities. At this altitude, the Round Trip Delay becomes closer to the one of terrestrial extended cells. Further, this closer proximity to Earth can offer more accurate ground metrics(e.g SNR,BER), and potentially lower cost for certain types of satellite operations. However, deployment at a VLEO orbit would require a very high number of satellites to generate overlapping footprint and hence service continuity in the targeted coverage. This is now made possible with the ever decreased launch cost. Typically, a maximum satellite beam footprint diameter of [250] km or lower should be supported.

- LEO (Low Earth Orbit) based scenarios (respectively 600 and 1200 km altitude):While these scenarios enable lower latency and higher data throughput in the context of 5G, they also introduce challenges such as frequent handovers and increased Doppler effects due to rapid satellite movement.

- MEO (Medium Earth Orbit) scenarios (10000 km altitude):  offer a balance between coverage area and latency, providing broader coverage than LEO and lower latency than GSO, but they still face moderate signal delays and more complex constellation management.

- GSO (Geosynchronous Orbit, including GEO) based scenarios (35786 km altitude):  GSO differs from GEO by a freedom to orbit around the 0° latitude. This allows to extend the satellite lifetime by optimizing the necessary fuel for station keeping. Ensure continuous and stable coverage over large regions with simpler ground equipment, but their high altitude results in significant signal latency and reduced performance for delay-sensitive applications.

Different payload architecture need be studied, including the transparent, regenerative and semi-regenerative payloads as discussed in R3-257192 [4].Additionally, the 6G radio access should also support multi-orbit space segment using combining at least a GSO with an NGSO based scenario, illustrated in Figure 1 and described on  R3-257192 [4].

### 2.2.2	Types of cells supported on NTN

The different types of cells supported in Non-Terrestrial Networks (NTN) are:

1. Quasi-Earth Fixed Cells: Quasi-Earth fixed cells are radio coverage areas on the ground that are almost, but not completely, stationary relative to the Earth’s surface. These cells are typically associated with NGSO (such as those in LEO and MEO) whose orbits cause their coverage footprints to move slowly or only slightly shift on the ground.

2. Earth Moving Cells: Earth moving cells are coverage zones that move continuously across the Earth’s surface. These are commonly linked to satellites in orbits where the coverage area does not stay fixed to one location but sweeps across different regions as the satellite passes overhead.

3. Earth Fixed Cells: Earth fixed cells are coverage zones that remain fixed relative to a specific geographic location on the Earth’s surface. This is mainly possible with Geosynchronous satellites, which orbit at the same rotational speed as the Earth and thus "hover" over one position. The coverage area of an Earth fixed cell does not change over time.

Table 1 Types of cells

| Cell Type | Coverage Movement | Typical Satellite Orbit |
| --- | --- | --- |
| Quasi-Earth fixed | Slightly moves | LEO/MEO (with tracking) |
| Earth moving | Continuously changes place | LEO/MEO |
| Earth fixed | Stays over same location | GSO |

### 2.2.3	NTN Scenarios

The table below provides the list of reference practical scenarios (single orbit) to be supported by the 6G radio access with the related NTN parameters.

Table 2 NTN reference orbit scenarios and characteristics for 6G

| NTN Scenarios | Very LEO | LEO-600 | LEO-1200 | MEO | GSO |
| --- | --- | --- | --- | --- | --- |
| Orbit type | circular around the earth | circular around the earth | circular around the earth | circular around the earth | station keeping around a nominally fixed position in terms of elevation/azimuth with respect to a given earth point (NOTE 6) |
| Altitude | 300 km (NOTE 1) | 600 km | 1200 km | 2000 to ~35000 km | 35,786 km |
| Relative speed of satellite with respect to earth | 7,73 km/s | 7.56 km/s | 7.26 km/s | 6.9 to ~3.1 km/s | negligible |
| Earth-fixed beams | Yes (Satellite generated steerable beams, NOTE 2) |  |  |  |  |
| Max beam footprint size (edge to edge) regardless of the elevation angle (NOTE 3) | [<250] km | [<250] km | [<250] km | [<250] km | [<250] km(Note 7) |
| Min Elevation angle for both sat-gateway and User Equipment | 10° for service link and 10° for feeder link |  |  |  |  |
| Max distance between satellite and User Equipment at min elevation angle | 1160 km | 1 932 km | 3 131 km | TBD km | 40 581 km |
| Max Round Trip Delay (propagation delay only NOTE 4) | 15.48 ms | 25.77 ms | 41.77 ms | 180 ms | 541.46 ms |
| Max Round Trip Delay variation as seen by the UE (NOTE 4) | Up to +/- 95,07 μs/sec | Up to +/- 93,0 μs/sec | Up to +/- 89.24 μs/sec | Up to +/- 84,87 μs/sec (Worst case) | negligible |
| Max Doppler shift (earth fixed user equipment) (NOTE 5) | 24.23ppm | 22.68ppm | 20.04ppm | TBD ppm | 0.93ppm |
| Max Doppler shift variation (earth fixed user equipment) (NOTE 4) | TBD ppm/s | 0.27 ppm/s | 0.13 ppm/s | TBD ppm/s | 0.000 045 ppm/s |
| NOTE 1: VLEO is characterized by altitudes ranging from approximately 300 to 600 kilometers above Earth. However, the lower the altitude, the higher the number of satellites is required to ensure service continuity and the higher the atmospheric drag which would need constant propulsion and re-boosting and hence reduced lifetime. NOTE 2: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite. NOTE 3: This beam size refers to the Nadir pointing of the satellite. NOTE 4: Assuming transparent payload with service and feeder links and return NOTE 5: Max Doppler shift and Max Doppler shift variation in the absence of any device pre-compensation of satellite Doppler shift on the service link. NOTE 6: In case of GSO(Geosynchronous satellites), the inclination may be non-zero. This means the satellite’s ground track may trace a figure-eight pattern (analemma) as seen from Earth. NOTE 7 : in GSO case the beam size could be greater than 250km. |  |  |  |  |  |

Study 6G radio interface adaptations to support practical NTN deployment scenarios across all orbits (VLEO, LEO, MEO, GSO), and evaluate the impact of NTN characteristics (e.g., initial access timers, HARQ, mobility management, paging).

## 2.3		Common Control Plane design for 6G TN and NTN

### 2.3.1	Initial Access

Initial access in NTN presents several challenges from a RAN2 perspective. Firstly, a high density of user terminals attempting to access the network simultaneously increases the risk of collisions and contention on the Random Access Channel, especially on limited satellite bandwidth. Standard initial access procedures and timers, such as those for Random Access Response and contention resolution, may not be directly suitable for NTN scenarios because satellite links introduce longer round-trip times and propagation delays compared to terrestrial networks. In addition, the intermittent and limited visibility of LEO satellites restricts the available window for a UE to complete the access procedure, if the timers and protocol steps are not appropriately adapted, there is a significant risk that a UE may lose satellite coverage before completing initial access. These challenges can be solved with a grant-free access protocol that limits the number of RTTs for the initial access. It is also beneficial in terrestrial networks for latency-critical applications.

Observation 1:  Enabling grant-free initial access addresses terminal density and minimize access delays in LEO satellite scenarios, It is also beneficial also for some TN use cases.

### 2.3.2	Optimized and flexible control signalling

5G NR introduced an ultra-lean design for control signals compared to LTE, i.e. minimize the “always-on” transmissions. Ultra-lean design significantly improved network energy efficiency in terrestrial networks. Legacy satellite systems usually implement beam-hopping techniques to increase coverage of a given satellite as illustrated below.

Figure 2 - Beam Hopping in satellite networks

In 5G-Advanced Rel-19, downlink coverage enhancements have been introduced to support beam-hopping permitted by the extending SSB periodicity to 160 ms and the lean control signal design. In 6G, the effort shall be pursued to allow flexible transmission in time with a reduced overhead of control signaling for the satellite system efficiency.

Observation 2:  Reduced signaling supports efficient satellite transmission in 6G-NTN.

### 2.3.3	RRC energy efficient state

From the terminal standpoint, the connectivity to the satellite network can be energy consuming, which is inconsistent with the objective of energy efficiency for the 6G radio access network. Moving the terminal to an idle state is not desirable because of the latency and cost associated with reconnecting to the satellite when  a new transmission is available. So an “inactive” state should be supported for NTN, that was not the case in 5G specifications for regenerative payload, partly due to  security context inter-satellite propagation issue. 6G may offer the opportunity to design such energy efficient RRC with appropriate control plane procedures to support this state across different NTN architectures, i.e. transparent, regenerative and split architectures.

The 6G radio access shall study from Day-1:

- A unified grant-free initial access mechanism applicable to both NTN and TN.

- A control plane signaling adapted to support network energy savings and address NTN-specific constraints (e.g. beam hopping techniques, reduced overhead, etc.)

## 2.4	Common User Plane design for 6G TN and NTN

In 5G, the user plane design integrates these mechanisms between different protocols :

- SDAP is responsible for mapping QoS bearers to radio bearers according to their quality of service.

- PDCP performs IP header compression, ciphering and integrity protection, retransmissions and in-sequence delivery, and removal duplicates in case of mobility.

- RLC protocol is responsible for segmentation and retransmission handling.

- MAC handles multiplexing/demultiplexing, scheduling, and HARQ retransmissions.

In the satellite systems, the link budget is in general calculated to ensure reliable communications at the physical layer (to  10-5 error rate) to avoid retransmission mechanisms due to long RTT. The remaining errors are handled either at the point of presence (PoP, e.g. the gateway) or at the application endpoint by transport protocol retransmission mechanisms (e.g. QUIC or TCP). This tradeoff may increase latency in case of losses on the physical layer but reduces the need for multiple retransmission mechanisms at the different protocol layers. With a low error rate at the physical layer by design and FEC mechanisms, the HARQ may be deactivated; that is beneficial to reduce memory (i.e., buffer sizes) and computational footprint at the MAC layer.

Observation 3:  Deactivation of retransmission mechanisms at user plane is beneficial for 6G-NTN.

The 6G radio access shall from Day-1 allow to dynamically activate/deactivate the retransmission mechanisms (e.g., HARQ) at user plane level depending on the type of services and operating point.

## 2.5	Mobility aspects

### 2.5.1	NTN-TN mobility for service continuity

The purpose is to ensure global service continuity in both idle and connected modes. NTN is expected to provide in areas that overlap with TN, as well as in regions beyond TN coverage.

While mobility from TN to NTN is mostly triggered by the unavailability of the TN service, the mobility from NTN to TN may be determined by other criteria. In both cases, the UE shall be able to camp on a TN cell while being able to monitor NTN cells and, vice versa.

Latency and mobility management differ between TN and NTN. While in TN we have low latency and stable signal transmission, facilitating accurate handover timing and mobility for UEs within a fixed coverage area. On the other hand, NTNs experience high propagation delays and short time of service (particularly those using LEO satellites), resulting in higher rate of handovers and greater risks of service degradation.

When TNs and NTNs coexist, these effects compound, makes mobility management more complex, increasing handover frequency, and increasing the chance of signaling issues and failures.

Study a design for 6G procedures that support mobility from TN to NTN and from NTN to TN for UEs in both Idle and Connected modes.

### 2.5.2	NTN-NTN mobility

In satellite networks, frequent handovers are inevitable as satellites move. The handover time depends on the satellite speed, the cell diameter, and also on the UE speed.

The handover time can reach up to 6.92 s for cell sizes smaller than 50 km, and up to 138.38 s for cell sizes of 1000 km, depending on the user equipment (UE) speed, assuming a constant satellite velocity of 7.56 km/s as shown below from TR 38.821 [6]:

Table 3: Time to HO for min/max cell diameter and varying UE speed

| Cell Diameter Size (km) | UE Speed (km/hr) | Satellite Speed (km/s) | Time to HO (s) |
| --- | --- | --- | --- |
| 50 (lower bound) | +500 | 7.56 (NOTE 1) | 6.49 |
|  | -500 |  | 6.74 |
|  | +1200 |  | 6.33 |
|  | - 1200 |  | 6.92 |
|  | Neglected |  | 6.61 |
| 1000 (upper bound) | +500 |  | 129.89 |
|  | -500 |  | 134.75 |
|  | +1200 |  | 126.69 |
|  | - 1200 |  | 138.38 |
|  | Neglected |  | 132.28 |

A satellite cell can correspond to a single beam or multiple beams. The “service link” handovers occur as inter-beam (between satellite beams), inter-cell (between ground cells under satellite coverage). They can interrupt connectivity, especially if switching occurs before the new connection is ready so it’s essential to make these transitions as rare and efficient as possible. By refining decision criteria, improving beam management, and carefully selecting target stations, service disruptions and processing demands can be reduced.

On the ground, “feeder link” handovers also reshape the network, influencing how traffic is routed. To maintain performance and avoid data loss, networks usually prepare new connections before breaking the old ones. Group handovers are another tool to avoid network overload when many users are affected at once.

Both the user device and the network benefit from smart selection among different non terrestrial networks: devices should choose networks and satellite types suited to their needs, while the network can optimize handover decisions based on quality requirements. Together, these strategies help maintain smooth communication in the ever-shifting landscape of satellite connectivity.

Additionally, the 6G radio access should also support multi-orbit mobility between at least a GSO with an NGSO based scenario, where NGSO may cover VLEO, medium/high LEO and/or MEO.

Study mobility mechanisms for 6G satellite networks (NTN to NTN), for both idle and connected modes. The study should address:

- Inter-beam mobility

- Inter-cell mobility

- Inter-satellite mobility

In view of NTN characteristics, it is interesting to consider the following handover and mobility mechanisms for 6G study:

Time-based CHO: designed for quasi-Earth-fixed cells, such as NGSO satellite beams. Handover events can be scheduled in advance because the satellite footprint remains stationary over the area, providing predictable periods for mobility control.

Location-based CHO : best suited for Earth-moving cells, such as those created by LEO or MEO satellite constellations. Handover decisions are triggered when the UE approaches a specified geographic threshold, leveraging knowledge of satellite trajectory and the user's position for more accurate and timely mobility.

RACH-less Handover: removes the need for the UE to perform the random access procedure during handover. This approach significantly reduces access latency and avoids extra delay, which is crucial in NTN where long propagation times would otherwise lengthen handover interruption.

Cell Switching (MAC-triggered, e.g. LTM): particularly effective in network architectures that use a Central Unit/Distributed Unit (CU/DU) split. The MAC layer directly triggers the cell or beam switch, allowing very fast changes between satellites or beams without involving higher-layer protocols, cutting down signaling and improving responsiveness.

Group Handover: optimized for quasi-Earth-fixed cell scenarios where a large number of UEs are served within the same satellite beam. Supports simultaneous handover of groups of UEs, reducing signaling storms and resource contention when many devices move at once (e.g., passengers on a moving train or aircraft).

To study the following mobility mechanisms, in order of priority:

- Time-based and Location-based Conditional Handover (CHO)

- RACH-less handover

- Cell switch mechanism triggered by MAC (e.g. LTM)

- Group handover

## 2.6	6G services using NTN

### 2.6.1	Positioning, Navigation and Timing

The provision of robust, high-accuracy positioning is driven by regulatory requirements, demand for ultra-precise location services, need for GNSS independent solutions, and the expansion of service areas as described on the PNT use case of TR 22.870 [7].

NTN is a key enabler of ubiquity, resilience, and wide area coverage representing a significant opportunity to enhance not only global connectivity but also positioning services especially in remote, underserved, and maritime areas where traditional terrestrial networks are not feasible or cost-effective. Complementing TN with NTN in 6G is crucial not only for connectivity but also for positioning services to maximize synergies between the two.

### 2.6.2	Broadcast services

Satellite communication's extensive coverage enables the delivery of broadcast services over wide and often transnational areas, supporting applications such as public information dissemination, emergency alerts, and multimedia content delivery on a large scale. However, in Non-Terrestrial Networks (NTN), the radio cells of satellites frequently span across country borders, while broadcast services may be subject to national regulations that restrict content to specific territories. This unique challenge highlights the importance of implementing geo-fencing mechanisms within the 6G system to ensure broadcast transmissions are confined to legally permissible areas.

To comply with these requirements, the 6G Radio with satellite access shall be equipped with capabilities that allow precise definition and enforcement of geographical boundaries for broadcast services. This includes mechanisms for user equipment to dynamically acquire service configuration, resource allocation, and control information, only when located within the targeted geographical area. Such geo-fencing ensures that broadcast content is only received by users within permitted regions, supporting both regulatory compliance and efficient use of satellite resources.

### 2.6.3	Multicast Services

Satellite networks are equally well-suited to support multicast services, delivering the same content to multiple users who are grouped based on service requirements. Multicast capabilities are essential for group communications in contexts such as corporate communications, public safety, and content distribution networks. A technical challenge arises when multicast groups span multiple beams or satellites, especially in non-GSO constellations, where frequent inter-satellite handovers occur and user equipment may move across jurisdictional boundaries.

To address this, the 6G system must enable efficient grouping, management, and routing for multicast sessions, while also implementing geo-fencing to limit service delivery to authorized geographic areas. This requires dynamic session management, beam-level routing, and continuous synchronization with national regulatory boundaries. As a result, multicast streams can be delivered to all group members located within the defined area, regardless of their movement between cells or satellites.

### 2.6.4	Regulatory services

Public Warning Systems (PWS):

Contemporary Public Warning Systems (PWS) uses a multicast or broadcast services to provide instantaneous, region-specific alerts during emergencies, offering far greater effectiveness than legacy approaches. Core components include an intuitive alert management portal and secure gateway, empowering authorities to swiftly craft and disseminate messages tailored to affected populations. Through the use of robust cell broadcast mechanisms, these systems deliver high-priority notifications to millions of mobile devices simultaneously, resilient to network congestion and independent of user opt-in.

Non-Terrestrial Networks (NTN) play a key role by extending these PWS capabilities to underserved or remote areas, ensuring no region is left unprotected during critical events.

Lawful Intercept (LI):

Lawful Intercept (LI) is a crucial legal and technical framework enabling authorized agencies to access and monitor communications in the interest of national security, crime prevention, and public safety. Telecommunications infrastructure must be equipped with mechanisms to selectively capture, store, and transfer intercepted data to law enforcement in accordance with national and international regulatory standards, such as those stipulated by the European Council.

NTN enhances LI by enabling interception capabilities across hybrid terrestrial-satellite environments, guaranteeing lawful compliance and intelligence accessibility even in regions lacking traditional network coverage.

NTN with the help of mechanisms like Network-verified UE location allows that the network itself determines and confirms the UE’s position, rather than relying only on the UE’s self-reported data. This typically uses measurements from multiple cells or satellites, which is crucial for Lawful Intercept.

Emergency Calling:

Reliable access to emergency call services (such as 112 or 911) is a fundamental human safety need (i.e: eCall, which uses 112 in Eruope, is a mandatory feature for all new cars sold in Europe since 2018), requiring robust telecommunications infrastructure capable of handling distress traffic under any circumstance. Modern emergency call solutions support accurate geolocation, rapid call setup, and prioritization within the network to ensure that callers are connected to emergency responders without delay.

By integrating NTN communication in 6G, individuals in rural, maritime, or otherwise isolated zones gain dependable access to voice services ensuring emergency calls can be placed and received without geographic constraints, significantly improving the overall efficacy of public safety initiatives.

Emergency Messaging:

Emergency messaging systems are engineered to disseminate critical information to the public or specific groups rapidly, using channels such as SMS, cell broadcast, and application-based alerts. These systems are designed to function even in network congestion scenarios, ensuring delivery reliability and message visibility. They also enable message delivery tracking, subscriber location analysis, and real-time updates during dynamic situations.

The inclusion of NTN communication infrastructure strengthens emergency messaging coverage, guaranteeing the continuity of information distribution in disaster-hit or remote areas where terrestrial networks may fail or be temporarily unavailable, thus supporting situational awareness for both citizens and first responders.

Observation 4 :    The following services are expected to be supported by NTN in 6G:

- Positioning, Navigation and Timing.

- Broadcast services over an intended area.

- Multicast services to a group of user equipment distributed over an intended area.

- Regulatory services including lawful intercept, emergency calling/messaging, and PWS.

Study the following essential NTN capabilities for supporting NTN services in 6G:

- NTN based positioning system for Positioning, Navigation, and Timing services.

- Network verified UE location for regulatory services

- Geo-fencing (e.g. intended service area) for Broadcast, multicast and PWS

# 3	Conclusion

- Proposal 1	Study 6G radio interface adaptations to support practical NTN deployment scenarios across all orbits (VLEO, LEO, MEO, GSO), and evaluate the impact of NTN characteristics (e.g., initial access timers, HARQ, mobility management, paging).

Observation 1:  Enabling grant-free initial access addresses terminal density and minimize access delays in LEO satellite scenarios, It is also beneficial also for some TN use cases.

- Observation 2:  Reduced signaling supports efficient satellite transmission in 6G-NTN

- Proposal 2	The 6G radio access shall study from Day-1:

- A unified grant-free initial access mechanism applicable to both NTN and TN.

- A control plane signaling adapted to support network energy savings and address NTN-specific constraints (e.g. beam hopping techniques, reduced overhead, etc.)

Observation 3:  deactivation of retransmission mechanisms at user plane is beneficial for 6G-NTN.

- Proposal 3	The 6G radio access shall from Day-1 allow to dynamically activate/deactivate the retransmission mechanisms (e.g., HARQ) at user plane level depending on the type of services and operating point

- Proposal 4	Study a design for 6G procedures that support mobility from TN to NTN and from NTN to TN for UEs in both Idle and Connected modes.

- Proposal 5	Study mobility mechanisms for 6G satellite networks (NTN to NTN), for both idle and connected modes. The study should address:

- Inter-beam mobility

- Inter-cell mobility

- Inter-satellite mobility

- Proposal 6	RAN2 to consider the following mobility mechanisms in 6GR study, in order of priority:

- Time-based and Location-based Conditional Handover (CHO)

- RACH-less handover

- Cell switch mechanism triggered by MAC (e.g. LTM)

- Group handover

Observation 4 : The following services are expected to be supported by NTN in 6G:

- Positioning, Navigation and Timing.

- Broadcast services over an intended area.

- Multicast services to a group of user equipment distributed over an intended area.

- Regulatory services including lawful intercept, emergency calling/messaging, and PWS.

- Proposal 7     Study the following essential NTN capabilities for supporting NTN services in 6G:

- NTN based positioning mechanisms for Positioning, Navigation, and Timing services.

- Network verified UE location for regulatory services

- Geo-fencing (e.g. intended service area) for Broadcast, multicast and PWS.

# References

RP-252912, “Revised SID: Study on 6G Radio”, NTT Docomo, CMCC, AT&T, Vodafone, Beijing, CN, September 2025.

6GWS-250040, “Vision and priorities for 6G RAT to support NTN”, Thales, Incheon, KR, March 2025.

R2-2507644 “6G RAT General Aspects for NTN” THALES, Prague, CZ, October, 2025.

R3-257192,” 6G RAN overall architecture for NTN” THALES, Prague, CZ, October, 2025.

3GPP TR 38.914, “Study on 6G Scenarios and requirements”, v0.1.1, Beijing, CN, September 2025.

3GPP TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN)”  v16.2.0

3GPP TR 22.870, “Study on 6G Use Cases and Service Requirements” , October 2025.