3GPP TSG-RAN WG2 Meeting #132	R2-2508823

Dallas, TX, USA, Nov 17th – 21st, 2025

Agenda item:		10.3.1.1

Source:		Futurewei

Title:		Discussions on 6G User Plane Functionalities and Requirements

Document for:		Discussion and Decision

# Introduction

In RAN2#131-bis, the following agreements were reached [1]:

Agreements on UP:

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

To achieve these objectives, it is important for RAN2 to study the key UP functionalities and requirements for 6GR, drawing lessons learned from 5G NR. In this paper, we share some of our views on this important aspect and on potential opportunity to improve beyond NR limitations.

# Discussions

## Key functionalities for 6GR user plane

Key functions performed at the SDAP layer in 5G NR include QoS flow to DRB mapping for DL and UL, QFI marking for DL and UL when more than one QoS flows are mapping onto the same DRB, and reflective QoS for UL. However, reflective QoS significantly increases the UE processing load while the benefit is very limited and hence should be excluded from 6G.

Proposal 1. Reflective QoS is not to be considered for 6GR.

Key functions performed at the PDCP layer in 5G NR include:

- functions related to header compression (e.g., RoHC, UDC, and EHC),

- functions related to security (e.g., cyphering and deciphering, integrity protection and verification, and SN maintenance),

- functions related to QoS handling (e.g., SDU and PDU Set discarding, PDCP duplication),

- functions related to mobility or dual connectivity (e.g., PDCP retransmission, bearer split and routing),

- functions related to SDU delivery (e.g., in-order delivery and reordering timer and buffering, out-of-order delivery, duplicate discarding).

Except that PDCP duplication and bearer split are pending on RAN plenary’s decision on whether DC is supported in 6GR, we think that the other key functions performed at the PDCP layer in 5G NR are also required in 6GR and hence should be supported.

Proposal 2. The following key functions performed at the PDCP layer in 5G NR are to be supported in 6GR:

- functions related to header compression (e.g., RoHC, UDC, and EHC),

- functions related to security (e.g., cyphering and deciphering, integrity protection and verification, and SN maintenance),

- functions related to QoS handling (e.g., SDU and PDU Set discarding),

- functions related to mobility (e.g., PDCP retransmission),

- functions related to SDU delivery (e.g., in-order delivery and reordering timer and buffering, out-of-order delivery, SDU discarding indication, duplicate discarding).

Key functions performed at the RLC layer in 5G NR include:

- functions related to ARQ operation (e.g., ARQ polling, ARQ status report, NACK-based RLC retransmission, timer-based autonomous RLC retransmission),

- functions related to segmentation (e.g., segmentation, re-segmentation, SN and Offset marking, reassembly based on SN and Offset marking),

- functions related to transmission buffering (e.g., transmission buffering, data volume calculation).

We think that all the above key functions performed at the RLC layer in 5G NR are also required in 6GR and hence should be supported.

Proposal 3. The following key functions performed at the RLC layer in 5G NR are to be supported in 6GR:

- functions related to ARQ operation (e.g., ARQ polling, ARQ status report, NACK-based RLC retransmission, timer-based autonomous RLC retransmission),

- functions related to segmentation (e.g., segmentation, re-segmentation, SN and Offset marking, reassembly based on SN and Offset marking),

- functions related to transmission buffering (e.g., transmission buffering, data volume calculation).

Key functions performed at the MAC layer in 5G NR include:

- functions related to random access (e.g., CBRA and CFRA),

- functions related to UL scheduling (e.g., SR, BSR, DSR, LCP, HARQ, CG, and PHR),

- functions related to bandwidth management (e.g., BWP and CA),

- functions related to energy savings (e.g., UE DRX and cell DTX/DRX, LP-WUS, UL WUS).

We think that all the above key functions performed at the MAC layer in 5G NR can be studied for 6GR with possibility for further optimization. For example, BSR and DSR may be merged into a unified mechanism for buffer reporting.

Proposal 4. The following key functions performed at the MAC layer in 5G NR can be studied for 6GR with possibility for further optimization:

- functions related to random access (e.g., CBRA and CFRA),

- functions related to UL scheduling (e.g., SR, BSR, DSR, LCP, HARQ, CG, and PHR),

- functions related to bandwidth management (e.g., BWP and CA),

- functions related to energy savings (e.g., UE DRX and cell DTX/DRX, LP-WUS, UL WUS).

## On opportunities to improve beyond NR limitations

For the 6GR UP design to meet the goal of being hardware-processing friendly while keeping memory requirements low and minimizing data movements across protocol layer, one important aspect to consider is if and how to consolidate the above key functions into a smaller number of protocol sublayers within Layer 2 protocol stack.

For example, whether the remaining two functions of SDAP layer, i.e., QoS flow to DRB mapping and QFI marking, can be performed at the PDCP layer will determine whether the SDAP layer can be avoided. Hence, we propose that RAN2 study whether QoS flow to DRB mapping and QFI marking can be performed at the PDCP layer so that the SDAP layer can be avoided in 6GR.

Proposal 5. RAN2 to study whether QoS flow to DRB mapping and QFI marking can be performed at the PDCP layer so that the SDAP layer can be avoided in 6GR.

For another example, due to the simplicity of functions related to ARQ operation, segmentation, and transmission buffering, and due to the wide support of tight coordination between HARQ and ARQ, whether there is a need to keep the RLC layer as a separate sublayer of Layer 2 or whether these functions can be moved into the MAC sublayer can be further studied.

Proposal 6. RAN2 to study whether there is a need to keep the RLC layer as a separate sublayer of Layer 2 or whether functions related to ARQ operation, segmentation, and transmission buffering can be moved into the MAC layer.

As mentioned before, there is a possibility that some of the key functions performed at the MAC layer can be further optimized, e.g., BSR and DSR may be merged into a unified mechanism for buffer reporting.

Proposal 7. RAN2 to study potential optimization of the key functions performed at the MAC layer, with a unified design of BSR/DSR reporting as a starting point.

# Conclusions

Proposal 1. Reflective QoS is not to be considered for 6GR.

Proposal 2. The following key functions performed at the PDCP layer in 5G NR are to be supported in 6GR:

- functions related to header compression (e.g., RoHC, UDC, and EHC),

- functions related to security (e.g., cyphering and deciphering, integrity protection and verification, and SN maintenance),

- functions related to QoS handling (e.g., SDU and PDU Set discarding),

- functions related to mobility (e.g., PDCP retransmission),

- functions related to SDU delivery (e.g., in-order delivery and reordering timer and buffering, out-of-order delivery, SDU discarding indication, duplicate discarding).

Proposal 3. The following key functions performed at the RLC layer in 5G NR are to be supported in 6GR:

- functions related to ARQ operations (e.g., ARQ polling, ARQ status report, NACK-based RLC retransmission, timer-based autonomous RLC retransmission),

- functions related to segmentation (e.g., segmentation, re-segmentation, SN and Offset marking, reassembly based on SN and Offset marking),

- functions related to transmission buffering (e.g., transmission buffering, data volume calculation).

Proposal 4. The following key functions performed at the MAC layer in 5G NR can be studied for 6GR with possibility for further optimization:

- functions related to random access (e.g., CBRA and CFRA),

- functions related to UL scheduling (e.g., SR, BSR, DSR, LCP, HARQ, CG, and PHR),

- functions related to bandwidth management (e.g., BWP and CA),

- functions related to energy savings (e.g., UE DRX and cell DTX/DRX).

Proposal 5. RAN2 to study whether QoS flow to DRB mapping and QFI marking can be performed at the PDCP layer so that the SDAP layer can be avoided in 6GR.

Proposal 6. RAN2 to study whether there is a need to keep the RLC layer as a separate sublayer of Layer 2 or whether functions related to ARQ operation, segmentation, and transmission buffering can be moved into the MAC layer.

Proposal 7. RAN2 to study potential optimization of the key functions performed at the MAC layer, with a unified design of BSR/DSR reporting as a starting point.

# References

R2_131bis_ChairNotes_25-10-17_17-00_final.