3GPP TSG-RAN WG2 Meeting #132	R2-2508824

Dallas, TX, USA, Nov 17th – 21st, 2025

Agenda item:		10.3.1.2

Source:		Futurewei

Title:		Discussions on 6G QoS, QoE, and Service-awareness

Document for:		Discussion and Decision

# Introduction

RAN2 is the leading WG to study User Plane architecture and protocol design for 6GR [1]:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

a)    User Plane architecture and protocol design [RAN2]

Considerations of QoS and QoE design and service-awareness are some of the important aspects of UP architecture and protocol design for 6GR. In this paper, we share some of our views on these important aspects.

# Discussions

## On the traffic types to study

Given the variety of data traffic that 6GR is expected to support, we suggest that RAN2 focus only on those that push the KPI envelope and those that require new functionalities or new ways of handling traffic in 6GR. Following are some example types of traffic to consider, as a starting point:

Immersive communication

In immersive communication, the data traffic is aimed to support immersive applications such as extended reality (XR), remote driving, holographic telepresence, etc. Such data traffic requires not only ultra-high data rates but also highly synchronized transmission of video, audio, and pose or ambient data, with some requiring support for ultra-low latency and ultra-high reliability to ensure seamless interaction with real and virtual environments. It also pushes the envelope for spectral efficiency and increased system capacity to handle high density of connected devices. Overall, this shift represents a wider evolution from the conventional eMBB services in the 5G era towards smart and immersive communication services in the 6G.

Hyper Reliable and Low-Latency Communication (HRLLC)

As an evolution of the Ultra Reliable and Low-Latency Communication usage scenario (URLLC) in 5G, HRLLC in 6G is aimed to meet even more stringent requirements for reliability and latency, which are critical for time synchronized operations where communication failure can lead to severe consequences such as in industrial automation, autonomous systems, and healthcare. Use cases include real-time control of robotic systems in manufacturing, remote medical procedures such as telesurgery, and low-latency coordination among connected autonomous vehicles. HRLLC can also support use cases that demand high connection density and high precise positioning, such as smart grid control, hazard or environmental monitoring, and smart agriculture.

AI/ML for networking

The integration of artificial intelligence (AI) into communication systems represents a major shift toward building intelligent, adaptive, and secure 6G wireless networks. It is expected that AI will play a key role in enabling service-awareness, real-time adaptation and optimization, autonomous decision-making, and predictive resource management across all network layers. Typical applications include autonomous driving, real-time collaboration among a team of robots, environmental monitoring, and digital twin.

AI tokens

Token communication service has been proposed as a new 6G use case in SA1 [2], where tokens are used as fundamental units of data that the AI model process to understand and generate content. For example, they serve as the smallest meaningful building blocks that represent portions of the input data in a way that the AI algorithms can efficiently analyze. For such applications and AI models, the consumption of tokens has significantly increased in the past year, and such trend is forecast to continue in the coming years. For 3GPP, token has the potential to become a new way of how such AI applications send/consume traffic. Hence, 6GR should consider the special characteristics related to such data traffic to provide communications to the AI applications efficiently, while providing good user experience.

Proposal 1: RAN2 to study the following types of traffic for services to be supported in 6GR:

- Immersive communication

- Hyper Reliable and Low-Latency Communication (HRLLC)

- AI/ML for networking

- AI tokens

## On QoE-related considerations

Quality of experience (QoE), as a subjective performance metric, can reflect the satisfaction level of users on networking services. Different users usually have differentiated QoE model structures and parameters. However, in 5G, QoE models are learned offline and do not account for the impact of real-time user behaviors and environmental factors. Also, in 5G, the QoE performance metric is measured largely based on the assumption that all packets within the same QoS flow have homogenous QoS requirements and they all impact on the QoE uniformly. Also, in 5G, no special handling was considered for synchronized transmissions among multi-modality services. However, for certain immersive applications, the QoE may be significantly affected by how well the transmissions among the multi-modality services, e.g. between the video and the haptic feedback, can be synchronized.

In 6G, sensing and AI/ML services can be used by the network to better understand the context and user behaviors in real-time. Service-awareness, if supported, can provide the UE and the gNB information about the nature or certain characteristics of the data packets on a per-packet basis, and hence the QoE performance metric can be specified, measured, and hence controlled with a finer granularity based on the classification of data packets, even if they are within the same QoS flow/DRB. To better evaluate the impact on QoE for multi-modality services that require synchronized transmissions, new performance metric and measurements (such as transmission synchrony) may need to be defined. Therefore, it is worthwhile for RAN2 to study how to effectively and efficiently construct and update comprehensive QoE models for individual users in 6G, e.g., based on service-awareness, sensing, AI/ML, and new performance metric and measurements such as transmission synchrony among multi-modality services.

Proposal 2. RAN2 to study how to efficiently construct and update comprehensive QoE models for individual users in 6G, e.g., based on service-awareness, sensing, AI/ML, and new performance metric and measurements such as transmission synchrony among multi-modality services.

## On over-all approach for RAN-level QoS in 6G

The 5G QoS framework is largely based on static or semi-static configuration with the assumption that all packets within the same QoS flow have homogenous QoS requirements, and hence the RAN-handling of packets within the same DRB is mostly the same, except that some differentiated handling (such as importance-based SDU discarding) based on XR-awareness was introduced in the late stage of 5G for XR services.

In 6G, we think service-awareness, which can be seen as an extension of XR-awareness, and awareness-based differentiated RAN-handling on a per-PDU or per-PDU-set basis should be supported from day one for certain traffic, such as for immersive communication and AI tokens, where the QoS/QoE of packets within the same flow are no longer homogenous.

Proposal 3. Service-awareness and awareness-based differentiated handling in 6GR should be supported from day one in 6G.

## Mechanisms to integrate/standardize application/service-awareness

Mechanisms to enable service-awareness in 6GR heavily depend on the work of other WGs (such as SA2/SA4/CT1) to identify and define service-awareness information that can be used for classifying packets, and for DL, the work of RAN3 to specify mechanisms for transferring such classification information from 6G CN to 6GR. Therefore, RAN2 needs to work closely with these WGs, e.g., proposing requirements regarding what kind of classification information may be needed and describing how the classification information can be used in 6GR.

Proposal 4. RAN2 to work closely with SA2, SA4, CT1, and RAN3 to identify and define service-awareness information that can be used for classifying packets and to introduce the related packet-classification information in 6GR.

Regarding how to utilize the service-awareness information for differentiated RAN-handling in 6GR, we propose that the following can be studied as a starting point:

- SDU and PDU Set discarding;

- Adaptive ARQ mode;

- Adaptive maximal retransmission number for HARQ and ARQ;

- Adaptive LCP based on remaining time.

Proposal 5. RAN2 to study mechanisms for differentiated RAN-handling based on awareness in 6GR. The following can be considered as a starting point:

- SDU and PDU Set discarding;

- Adaptive ARQ mode;

- Adaptive maximal retransmission number for HARQ and ARQ;

- Adaptive LCP based on remaining time.

# Conclusions

Proposal 1: RAN2 to study the following types of traffic for services to be supported in 6GR:

- Immersive communication

- Hyper Reliable and Low-Latency Communication (HRLLC)

- AI/ML for networking

- AI tokens

Proposal 2. RAN2 to study how to efficiently construct and update comprehensive QoE models for individual users in 6G, e.g., based on service-awareness, sensing, AI/ML, and new performance metric and measurements such as transmission synchrony among multi-modality services.

Proposal 3. Service-awareness and awareness-based differentiated handling in 6GR should be supported from day one in 6G.

Proposal 4. RAN2 to work closely with SA2, SA4, CT1, and RAN3 to identify and define service-awareness information that can be used for classifying packets and to introduce the related packet-classification information in 6GR.

Proposal 5. RAN2 to study mechanisms for differentiated RAN-handling based on awareness in 6GR. The following can be considered as a starting point:

- SDU and PDU Set discarding;

- Adaptive ARQ mode;

- Adaptive maximal retransmission number for HARQ and ARQ;

- Adaptive LCP based on remaining time.

# References

RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, et al., RAN#109.

S1-254040, New use case on 6G system providing Token Communication service, Huawei, et al., SA1#112.