3GPP TSG-RAN WG2 #132	R2-2508830

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	8.11.2

Source:	InterDigital, Inc

Title:	Open issues on SBFD

WID/SID:	NR_duplex_evo-Core – Release 19

Document for:	Discussion

# Discussion

RAN2 needs to discuss further the following issues from the outcome of the open issue discussion [1].

- MAC-1: Whether/how to address the issue in case RO type switching and Msg1 repetition number fallback are triggered simultaneously.

(MAC-1)

RAN2 has not reached a consensus on how to handle the case when RO type switching and MSG1 repetition number fallback are triggered simultaneously [1]. The issue arises when two conditions are satisfied, a UE may select higher MSG1 repetition number after RO type is switched. To avoid this issue, network can configure different threshold numbers for RO type switching and MSG1 reception fallback because the RO type switching and MSG1 repetition fallback can be considered independent procedures with different objectives. The RO type switching procedure is focused on switching SBFD-RO (or legacy ROs) after a number of failures, while the MSG1 repetition fallback is focused on UL coverage extension based on the increased number of MSG1 transmissions.

When two conditions are satisfied at the same time (as a corner case), related UE actions are described in different sections with respective parameter values according to the MAC spec. Hence, the UE performs RO type switching to the same set of Msg1 repetition number, then the UE may determine to fallback to a higher Msg1 repetition number. Also, if the UE performs RO type switching to a set of (next) higher MSG1 repetition number, then UE should not further increase the MSG1 repetition number since a higher number has already been selected.

Proposal 1: [MAC-1] Do not specify in MAC spec when conditions for RO type switching and MSG1 repetition number fallback are satisfied simultaneously.

# Conclusion

Proposal 1: [MAC-1] Do not specify in MAC spec when conditions for RO type switching and MSG1 repetition number fallback are satisfied simultaneously.

# References

[1] R2-2508163, Summary of Rel-19 SBFD MAC open issues for maintenance.