3GPP TSG-RAN2 Meeting #132	R2-2508835

Dallas, USA, November 17 - November 21, 2025

Agenda item:	10.3.2.3

Title:	Discussion on 6G initial access and spectrum aggregation

Source:	CMCC

Document for:	Discussion

Introduction

In RAN2#131bis meeting, some agreements are made for spectrum aggregation, system information and paging [1]:

Agreements on Spectrum aggregation 1 Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed. 2 Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc). Understand the deployment scenarios and problems to address Agreements on System Information - System information design should consider energy efficiency and low complexity for both network and UE. - System information design should ensure that the public warning requirements are met. - Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs) - Study how to improve flexibility and extensibility of scheduling SI design - Study mechanisms to improve SI update mechanism - Study aspects related to SIB1 size. Understand the issues and desired content/size from RAN2 point of view. - Study areas specific SIB design. Understand 5G pain points and what improvements can be considered. NOTE: this is a starting list Agreements on paging - Paging design should consider energy efficiency and simplicity for both UE and Network.

In this contribution, we would like to provide our further views on initial access, system information, paging and spectrum aggregation aspects.

Discussion

Spectrum aggregation

In RAN2#131bis meeting, it has been agreed that [1]:

Agreements on Spectrum aggregation 1 Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed. 2 Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc). Understand the deployment scenarios and problems to address

In this section, we will analyse the lessons learned from 5G to understand which issuse(s) need to be addressed in 6G and provide our views on 6G spectrum aggregation framework.

Lessons learned from 5G

In 5G, spectrum aggregation, which includes carrier aggregation (CA) and supplementary uplink (SUL), are supported. For SUL, it is mainly used to extend the uplink coverage by configuring one low-frequency carrier as SUL carrier, and only one SUL carrier is supported within one 5G cell. NR UE can select either SUL carrier or NUL carrier based on its measured signal quality during random access procedure or the indication from network. In terms of CA, two or more carriers can be aggregated and used by one UE to increase UE’s throughput. However, CA is only applicable to UE in RRC_CONNECTED, which also limits the benefits it can bring. Based on current SUL and CA scheme, the drawbacks of NR spectrum aggregation framework includes:

- Inefficient usage of fragmented spectrum

In NR, certain solutions (e.g., low-low band CA, UL Tx switching, etc.) are employed to maximize spectrum utilization. However, conventional carrier aggregation still requires separate handling of different carriers, which increases the signalling overhead and limits the effectiveness of spectrum utilization optimization to a large extent.

- Lack of efficient offloading when UEs transiting from RRC_IDLE/INACTIVE to RRC_CONNECTED

In NR, when UEs are about to transit from RRC_IDLE/INACTIVE to RRC_CONNECTED, the legacy scheme cannot help to solve congestion during initial access procedure. In other words, for UEs in RRC_IDLE/RRC_INACTIVE, flexible carrier selection for offloading cannot be achieved in NR.

- Inflexible UL and DL carrier association

In NR, one cell can be with one TDD carrier, or one TDD carrier and one SUL carrier, or one FDD carrier, or one FDD carrier and one SUL carrier. Therefore, it is hard to achieve dynamic pairing of UL and DL to adapt to the uplink and downlink traffic situation of the system.

Observation 1: In NR, the drawbacks of multi-carrier framework include inefficient usage of fragmented spectrum, lack of efficient offloading scheme for UEs in RRC_IDLE/RRC_INACTIVE, inflexible UL and DL carrier association, etc.

6G spectrum aggregation framework - hyper cell

In 6G, frequencies may be more scattered. For example, there may be some carriers re-farmed from 2G or 4G networks, and their bandwidths may vary, ranging from less than 20 MHz to 50 MHz, or even larger. If each carrier with small bandwidth is managed separately, it will bring significant overhead (e.g. handover signalling overhead, common signalling overhead, etc.). Therefore, an unified, efficient and simple spectrum aggregation framework is demanding. From our point of view, at least the following requirements should be considered:

- Energy saving for network and/or UEs;

- Efficient spectrum aggregation with low signalling overhead;

- Efficient offloading for UEs in all RRC states;

- Flexibly adapt to the traffic load of network and/or UEs.

Fig. 1. An example of access procedure in 6G hyper cell.

Based on above analysis, we think that more than one scattered spectrums of same/different bandwidths can be aggregated into one hyper cell, and one example is given in Fig. 1. Within one hyper cell, multiple carriers can be aggregated. Some features of 6G hyper cell can be listed below:

- The common signalling can mainly be transmitted in anchor carrier to reduce the signalling overhead and achieve network energy saving. What’s more, if only the anchor carrier is on the sync raster, the number of frequencies that the UE attempts to search will be significantly reduced, thereby enabling energy saving for the UE as well.

- With multi-carriers available, the capacity of one hyper cell becomes scalable and can be adjusted based on the latest system overhead. To be specific, when hyper cell serves very few users, only the anchor carrier is activated. With the increase in the number of UEs served by this hyper cell, more carriers can be activated to ensure service performance for UEs. And the dynamic carrier activation/deactivation can be achieved with simple signalling, e.g. MAC CE.

- For non-anchor carrier(s), the network can either offload the RACH procedures of some UEs in RRC_IDLE to it/them to improve capacity of RACH, or offload the data transmission of UEs in RRC_CONNECTED to it/them to ensure the QoS/QoE of UEs.

- Flexible UL/DL coupling can be achieved to adapt to the service characteristics of UEs. As shown in Fig. 2, for different UL/DL traffic load of one hyper cell, different UL and DL carrier association scheme is adopted. To be specific, when DL traffic load is heavy at moment 1, more carriers can be configured as DL carriers. After a period of time (i.e. moment 2), the UL traffic load becomes heavier than DL, more carriers can be configured as UL carriers.

Fig. 2. Flexible UL/DL ccoupling for 6G hyper cell.

Proposal 1: A unified multi-carrier framework, where scattered spectrums of same/different bandwidths can be aggregated into one hyper cell, is supported from Day 1. At least the following requirements/issues should be considered: energy saving for network and/or UEs; efficient spectrum aggregation with low signalling overhead; efficient offloading for UEs in all RRC states; flexibly adapt to the traffic load of network and/or UEs.

Initial access

In 5G, before the UE enters RRC_CONNECTED state for data transmission/reception, it performs several procedures, which is known as initial access. Generally, initial access includes the following phases: cell search and synchronization, necessary system information acquisition, random access and registration, as depicted in Fig. 3.

Fig. 3. Overview of initial access procedure

In this section, we provide our views on the first two steps of the initial access procedure: Cell search and DL synchronization, and system information acquisition.

Cell search and DL synchronization

In 5G, the UE scans all RF channels in the NR bands according to its capability to find a suitable cell and on the one it camps, it can acquire SIB1 and monitor paging. From our perspective, in 6G there could be single cell associated with one carrier (i.e. normal cell) or multi-carrier (i.e. hyper cell) cases. For the case of normal cell, the 5G cell search procedure can be reused. For the case of hyper cell as analyzed in section 2.1.2, there could be no CD-SSB (no SSB or only NCD SSB) broadcasted in non-anchor carrier(s), thus, such non-anchor carrier(s) may not be selected or camped by the UE. That’s to say, the 5G procedure can also be applied to both single cell associated with multi-carrier cases.

Observation 2: 5G cell search procedure can be inherited in 6G, applied to both single cell associated with one carrier (i.e. normal cell) and multi-carrier (i.e. hyper cell) cases.

In 5G, the UE assumes that SSB is transmitted with the periodicity of 20ms. By monitoring SSB, the UE calculates the PCI of a cell and achieve DL synchronization with the cell. To assure UE’s access performance, the network needs to broadcast SSB per 20ms, but the “always-on” SSB transmission leads to heavy energy consumption in the network side. Based on 5G’s lessons, energy saving should be considered as early as cell search phase. In 5G NES WI, multiple network energy saving approaches were specified, i.e., on-demand SIB1, on-demand SSB for SCell and SSB/PRACH adaptation. We think the thoughts of these NES techniques can be inherited for cell search and DL synchronization phase for sake of network energy saving.

Observation 3: Only supporting short period always-on SSB is not energy efficient for network.

Since the most important signal in this phase is SSB, therefore, 6G initial access should consider an energy saved SSB transmission. To achieve this, two solutions can be considered:

- Solution 1: Always-on long periodicity SSB + UE triggered on-demand short periodicity SSB

- Solution 2: Cell DTX/DRX for common signaling

For solution 1, SSB transmission can be considered to be adapted to two-stage way. In the first stage, there’s a long periodicity, which can be used for cell detection, basic sync and provide necessary information (e.g. UL-WUS configuration). With such design, the 6G BS can have more chance to turning off hardware components in low load or multi-carrier scenario, so as to save more energy for the network. While in the second stage, the UE may trigger a short periodicity SSB (and other common channels/signals if needed) that can be used for finer synchronization or fast measurement, and to reduce the impact to UE’s access due to long-period “always-on” SSB.

Spontaneously, the SSB transmission design can be applied to the scenario that both always-on SSB and short periodicity transmitted SSB are on the same cell/carrier, and it can also be applied to the scenario that always-on SSB and short periodicity transmitted SSB are on different cells/carriers (e.g. 6G hyper cell). To be specific, as shown in Fig.1, the always-on SSB can be mainly transmitted on one anchor carrier, and the non-anchor carrier can be either SSB-less or with sparse synchronization signal (SS). Besides, we are also open to discuss whether on-demand SSB/SS is supported on anchor carrier and/or non-anchor carrier(s). For example, with the long-period SSB on anchor, whether UE can trigger a short-period SS on non-anchor carrier(s) before it initiates random access procedure in that carrier(s).

What’s more, considering the benefits of LP-WUS identified in 5G, it can be considered to support UL LP-WUS as UE request to further reduce network energy consumption, this can be further studied by RAN2.

For solution 2, Cell DTX/DRX concept is extended from 5G RRC_CONNECTED data transmission to RRC_IDLE state common signaling transmission. That's to say, the network only transmit/receive common signalling like SSB, system information, RACH and paging in the Cell DTX/DRX active period in one carrier (i.e. the unique carrier in normal cell or the anchor carrier in 6G hyper cell), while in the non-active period, the BS can tuns off the hardware components to achieve network energy saving. And this does not affect the SSB/SI transmission configuration.

Proposal 2: The SSB transmission design in 6G should consider the energy efficiency for both network and UE side, the following solutions can be studied: 1)SSB periodicity adaptation, e.g., always-on long periodicity SSB and UE triggered on demand short periodicity SSB; 2) Cell DTX/DRX for common signalling, i.e., SSB, system information and paging.

System information

In RAN2#131bis meeting, the following agreements are made for system information design [1]:

Agreements on System Information - System information design should consider energy efficiency and low complexity for both network and UE. - System information design should ensure that the public warning requirements are met. - Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs) - Study how to improve flexibility and extensibility of scheduling SI design - Study mechanisms to improve SI update mechanism - Study aspects related to SIB1 size. Understand the issues and desired content/size from RAN2 point of view. - Study areas specific SIB design. Understand 5G pain points and what improvements can be considered. NOTE: this is a starting list

Based on the guidance, we analyze the following aspects related to system information:

- System information content and size

- System information delivery

- System information updating

- System information content and size

Based on TS 38.300 [2], system Information (SI) consists of a MIB and a number of SIBs, which are divided into Minimum SI and Other SI:

-	Minimum SI comprises basic information required for initial access and information for acquiring any other SI. Minimum SI consists of:

-	MIB contains cell barred status information and essential physical layer information of the cell required to receive further system information, e.g. CORESET#0 configuration. MIB is periodically broadcasted on BCH.

-	SIB1 defines the scheduling of other system information blocks and contains information required for initial access.

-	Other SI encompasses all SIBs not broadcast in the Minimum SI, e.g. information for intra-frequency/inter-frequency/inter-RAT cell reselection, information for slicing/NTN/MBS, etc.

In our view, the 5G system information framework can be reused in 6G, that minimum SI(MIB, SIB1) and other SIs comprise 6G system information.

Proposal 3: 5G system information framework consisting of Minimum SI (MIB, SIB1) and other SI can be inherited in 6G.

The following table depicts the information included in Minimum SI, and the related size is also indicated in it, but it should be noticed that, we only include the most important IEs, which are also needed in 6G, like cell selection and access, SI scheduling related IEs into account.

Table 1. Minimum SI contents

| System information | Content | Description | Size or size range (bit) |
| --- | --- | --- | --- |
| MIB | systemFrameNumber | The 6 most significant bits (MSB) of the 10-bit System Frame Number (SFN) | 6 |
|  | subCarrierSpacingCommon | Subcarrier spacing for SIB1, Msg.2/4 and MsgB for initial access, paging and broadcast SI-messages. | 1 |
|  | ssb-SubcarrierOffset | Parameter related to kSSB | 4 |
|  | dmrs-TypeA-Position | Position of (first) DM-RS for downlink and uplink | 1 |
|  | pdcch-ConfigSIB1 | Determines a common ControlResourceSet (CORESET), a common search space and necessary PDCCH parameters | 8 |
|  | cellBarred | Indicate whether the cell is barred | 1 |
|  | intraFreqReselection | Controls cell selection/reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE | 1 |
|  | Spare |  | 1 |
| SIB1 | cellSelectionInfo | Parameters for cell selection related to the serving cell | ~ 20 |
|  | cellAccessRelatedInfo | To indicates cell access related information for this cell | [70,4000] Note: the size is related to the number of PLMNs |
|  | connEstFailureControl | To configure parameters for connection establishment failure control | ~10 |
|  | si-SchedulingInfo | Contains information needed for acquisition of SI messages | [100,10000] Note: the size is related to the number of SIs |
|  | servingCellConfigCommon | To configure cell specific parameters of a UE's serving cell | [300,1500] Note: the size is related to the number of rate matching pattern |
|  | ue-TimersAndConstants | Timer and constant values to be used by the UE | ~20 |
|  | uac-BarringInfo | Provides a list of access control parameter sets | [100,10000] Note: the size is related to the number of UAC categories and the number of PLMNs |

Currently, MIB content only costs 23 bits, and it is associated with SS. As to SIB1, the size range varies greatly, the minimum size of SIB1 is less than 1000 bits, and the maximum size may exceed 20000 bits, while the maximum TB size for SI delivery is 2976 bits in 5G, which is related to the RNTI, MCS table, etc.

Observation 4: The size of SIB1 varies from 600 bits to more than 20000 bits based on different network configuration.

And it should be noticed that, the calculated result is for single cell associated with single carrier case (i.e. normal cell), while for multi-carrier case (i.e. hyper cell), such IEs, like RACH configuration for different carriers may need to be contained in SIB1 which may enlarge the SIB1 size. To avoid too much singalling overhead, RAN2 is asked to identify the common configuration of multi carriers.

What’s more, during 4G and 5G era, NB-IoT was deployed as a separate network, with different system bandwidth and different system information, comparing with LTE and NR network. However, the Total Cost of Ownership (TCO) is quite high by running separate networks. According to 6G SID, 6GR will support diverse device types, including eMBB devices and LPWA devices, and target for a scalable and forward compatible design. To achieve a unified network, the design of system information is one of the most essential parts.

The most straight forward way is to design a unified system information for different types of devices, from spectrum efficiency point of view. However, it should be noticed that the capability of different type devices may exhibit significant variations, for example the baseband of LPWA devices and eMBB devices may be different. Note that this issue is related with the device type discussion in RAN plenary and RAN1. From RAN2 point of view, RAN2 should aim to design the 6GR system information that support diverse types of devices at this stage. Whether any type of devices require separate system information should wait for further RAN plenary and RAN1 conclusions.

Besides, 6G also aims at a harmonized 6G Radio design for TN and NTN, while in 5G part of the necessary information for NTN access is conveyed in SIB19 due to the size limitation of SIB1, thus, minimum SI design should also consider NTN UEs’ initial access.

Proposal 4: The following principles should be considered in 6G minimum SI design:

- Including information required for initial access, including SFN, cell selection and access related information, serving cell configuration and cell barring/access control related information, as well as scheduling information of other SI. What’s more, how to divide the information into MIB and SIB1 may depends on RAN1’s design;

- With carefully designed on minimum SI size, especially SIB1 size to avoid too much signalling overhead or physical layer design impact;

- Supporting diverse device types, like eMBB, LPWA and NTN;

- Supporting both normal cell (a cell associated with one carrier) and hyper cell (a cell associated with multiple carriers) deployment.

- System information delivery

From 5G, other SI can be delivered in a cell not only in periodical broadcasting way but also in an on-demand way. And after several releases’ discussion, both SIB1 and SSB with on-demand delivery are supported in 5G, considering their advantages in network energy saving. However, in the real 5G, on-demand (other) SI, which is supported from the first release of 5G, is not configured due to its limited energy saving gain, as compared to the energy consumption caused by periodical SSB transmission, the saved energy via reduced SI transmission is negligible. Hence, RAN2 should consider how to achieve on-demand SIB1/SIB with reduced SSB transmission, i.e., SI transmission is associated with always-on SSB but not on-demand SSB, or SIB1/SI transmission is only during the activation time of Cell DTX/DRX.

Observation 5: To further reduce the network energy consumption, on-demand SIB1/SI transmission should consider how to work with the reduced SSB, e.g. SI transmission is associated with always-on SSB but not on-demand SSB, or SIB1/SI transmission is only during the activation time of Cell DTX/DRX.

What’s more, once SIB1/other SI is requested by UE, it’s network implementation to deliver SIB1/other SI to UE by broadcast signalling or dedicated signalling.

Besides, area specific SI is supported in 5G, which helps to UE’s power saving, as the UE can avoid SI request if the same system area ID is indicated in two cells. Also, combined with on-demand SI delivery, it can also help to network energy saving, therefore, this mechanism can also be reused in 6G.

Note that above analysis is focused on normal cell case. For 6G hyper cell, the system information (e.g. SIB1, other SI, etc.) is mainly broadcasted in anchor carrier. Within the anchor carrier, all relevant mechanisms related to the aforementioned system information (e.g. on-demand SI, area specific SI, etc.) can be considered for adoption as well.

Proposal 5: On-demand SIB1/other SI and area specific SI can be supported for network/UE energy saving.

- System information updating

Once the content of system information is updated, the network may first indicate to the UEs in one SI modification period and then the UE acquires the updated SI in the next SI modification period, if needed.

In 5G, paging short message is used to indicate the SI change, but it cannot directly indicate which SI is updated, and this requires UE to check the value tag in SIB1, which results in extra UE complexity. In 6G, RAN2 can study whether a finer granularity of SI can be indicated directly to UE to reduce UE’s complexity. Besides, considering the potential on-demand mechanism in 6G, like on-demand SSB or on-demand SIB1, the finer granularity indication is also beneficial to network energy saving, because it can reduce the opportunity of UE waking up network.

As to the updating step, we think the 5G way of updating SI in the next SI modification period can be reused for single carrier case, since SI is broadcasted for multiple UEs, and a smart network may configure SI modification properly to ensure all ( or most) UEs are paged to ensure UEs camping in the cell to acquire the updated system information, in other words, it can avoid the SI content mismatch in the UE side and network side. As to multi-carrier case (i.e. hyper cell introduced in section 2.1.2, since we prefer to transmit SIB1 and other SI only on the anchor-carrier, the 5G way can also work well.

Proposal 6: The following way can be considered for 6G SI updating:

- A finer granularity SI change indication can be used to reduce UE’s false alarm;

- 5G SI modification period mechanism can be reused in both normal cell and hyper cell scenario.

Paging

In RAN2#131bis meeting, the following agreement has been made for paging [1]:

Agreements on paging - Paging design should consider energy efficiency and simplicity for both UE and Network.

To design an energy-efficient and simple paging scheme for 6G, we think that the well-established paging schemes adopted in 5G NR can serve as a baseline, and further enhancements can be considered to address the following issues in 6G.

- Paging power wasting: The lack of awareness of the network-side energy-saving operations on the UE side may result in unnecessary paging monitoring by the UE.

It has been added to objectives of studying 6G single technology framework to consider energy efficiency/energy saving for both network and device as approved in RAN#108 meeting [3]. And the following solutions have been discussed and can be supported in 6G:

­On-demand common signalling, including SSB, SIB1 and other SI;

­Common signalling adaption;

­Cell DTX/DRX applied to any RRC states;

­(S)Cell/carrier activation/deactivation, especially for multi-carrier case.

In NR, paging occasions (POs) are uniformly distributed in a periodic manner, with relatively fixed positions. In deep-sleep NES case suspending paging messages transmission, if the UE’s behavior of periodically monitoring the paging does not changed, unnecessary paging monitoring will waste UE’s power consumption. Thus, adaptive paging design is required for achieving joint energy saving in both network and UE side.

- Excessive paging overhead: In case that the tracking area (TA) of a UE crosses different frequency bands, sending the same paging message across all carriers within a Tracking Area (TA) incurs excessive overhead.

If the tracking area (TA) of a UE crosses different frequency bands, each carrier within the corresponding TA may send the same paging messages, even in the case of multi-carrier case (i.e. 6G hyper cell), where the UE only needs to monitor its resident frequency band. In condition that a single carrier can meet the paging capacity requirements, the redundant transmission of paging messages brings excessive signaling overhead and waste of resources.

Observation 6: In 6G, the enhancements of paging mechanism are essential for addressing the following issues:

- Paging power wasting: The lack of awareness of the network-side energy-saving operations on the UE side may result in unnecessary paging monitoring by the UE, so adaptive paging design is required for achieving joint energy saving in both network and UE side.

- Excessive paging overhead: In multi-carrier scenario, sending the same paging message across all carriers within a Tracking Area (TA) incurs excessive overhead.

Based on the discussion above, we propose the following design principle for 6G paging:

- Paging power saving: In energy saving scenarios, paging adaptation in 5G NR can be served as a baseline, and LP-WUS for indicating UE whether to enter the wake-up state to monitor paging information can also be considered, either as a replacement for PEI or in conjunction with PEI. In addition, it can avoid unnecessary paging monitoring by UE and be beneficial for saving UE power consumption that network informs UE the NES operation suspending paging messages transmission.

- Paging overhead reduction: In multi-carrier scenarios, to reduce the complexity of UE monitoring and system overhead, paging messages can always be transmitted on one carrier which can be the anchor-carrier, or the carrier with the best coverage as shown in Fig. 1.

Proposal 7: 6G paging should consider the following enhancements:

- To achieve joint energy saving in both network and UE side, RAN2 consider that network informs UE the NES operation suspending paging messages transmission.

- To reduce the complexity of UE monitoring and system overhead, RAN2 considers that paging messages can always be transmitted on one carrier, e.g., the anchor-carrier, or the carrier with the best coverage.

Conclusions

Observation 1: In NR, the drawbacks of multi-carrier framework include inefficient usage of fragmented spectrum, lack of efficient offloading scheme for UEs in RRC_IDLE/RRC_INACTIVE, inflexible UL and DL carrier association, etc.

Observation 2: 5G cell search procedure can be inherited in 6G, applied to both single cell associated with one carrier (i.e. normal cell) and multi-carrier (i.e. hyper cell) cases.

Observation 3: Only supporting short period always-on SSB is not energy efficient for network.

Observation 4: The size of SIB1 varies from 600 bits to more than 20000 bits based on different network configuration.

Observation 5: To further reduce the network energy consumption, on-demand SIB1/SI transmission should consider how to work with the reduced SSB, e.g. SI transmission is associated with always-on SSB but not on-demand SSB, or SIB1/SI transmission is only during the activation time of Cell DTX/DRX.

Observation 6: In 6G, the enhancements of paging mechanism are essential for addressing the following issues:

- Paging power wasting: The lack of awareness of the network-side energy-saving operations on the UE side may result in unnecessary paging monitoring by the UE, so adaptive paging design is required for achieving joint energy saving in both network and UE side.

- Excessive paging overhead: In multi-carrier scenario, sending the same paging message across all carriers within a Tracking Area (TA) incurs excessive overhead.

Proposal 1: A unified multi-carrier framework, where scattered spectrums of same/different bandwidths can be aggregated into one hyper cell, is supported from Day 1. At least the following requirements/issues should be considered: energy saving for network and/or UEs; efficient spectrum aggregation with low signalling overhead; efficient offloading for UEs in all RRC states; flexibly adapt to the traffic load of network and/or UEs.

Proposal 2: The SSB transmission design in 6G should consider the energy efficiency for both network and UE side, the following solutions can be studied: 1)SSB periodicity adaptation, e.g., always-on long periodicity SSB and UE triggered on demand short periodicity SSB; 2) Cell DTX/DRX for common signalling, i.e., SSB, system information and paging.

Proposal 3: 5G system information framework consisting of Minimum SI (MIB, SIB1) and other SI can be inherited in 6G.

Proposal 4: The following principles should be considered in 6G minimum SI design:

- Including information required for initial access, including SFN, cell selection and access related information, serving cell configuration and cell barring/access control related information, as well as scheduling information of other SI. What’s more, how to divide the information into MIB and SIB1 may depends on RAN1’s design;

- With carefully designed on minimum SI size, especially SIB1 size to avoid too much signalling overhead or physical layer design impact;

- Supporting diverse device types, like eMBB, LPWA and NTN;

- Supporting both normal cell (a cell associated with one carrier) and hyper cell (a cell associated with multiple carriers) deployment.

Proposal 5: On-demand SIB1/other SI and area specific SI can be supported for network/UE energy saving.

Proposal 6: The following way can be considered for 6G SI updating:

- A finer granularity SI change indication can be used to reduce UE’s false alarm;

- 5G SI modification period mechanism can be reused in both normal cell and hyper cell scenario.

Proposal 7: 6G paging should consider the following enhancements:

- To achieve joint energy saving in both network and UE side, RAN2 consider that network informs UE the NES operation suspending paging messages transmission.

- To reduce the complexity of UE monitoring and system overhead, RAN2 considers that paging messages can always be transmitted on one carrier, e.g., the anchor-carrier, or the carrier with the best coverage.

References

- Chair’s notes, RAN2 chair(InterDigital), RAN2#131bis.

- 3GPP TS 38.300 V19.0.0 (2025-09).

- RP-251881, “New SID: Study on 6G Radio”.