# 3GPP TSG-WG2 Meeting #132                 	   R2-2508836

# Dallas, Texas, USA, November 17th – November 21st  , 2025

# Source: 			CMCC

# Title:  Discussion on Store and Forward remaining issues

# Agenda item:	8.9.3

# Document for: 	Discussion

Introduction

In the last RAN2#131bis meeting, the following agreements [1] were approved as below:

Agreements:

1.	An AS UE radio capability signaling is introduced to indicate whether UE supports S&F mode operation (to be captured in a CR for next meeting).

2.	The UE RRC indicates to upper layers that the cell is operating in normal mode, upon acquiring a SIB1 that does not include a sf-OperationMode.

3.	S&F Architecture option is not indicated via any RRC signalling including SIB indication (Can come back to if we receive an indication from SA2 to cover the scenario of mixed (split MME and Full CN) deployments)

4.	We clarify in RRC that a change of the S&F mode indication and the S&F mode transition time of neighbour cells in SIB33 does not trigger SI update procedure.

5.	RAN2 understands that in idle mode the UE may re-acquire SIB33 upon transition time expiry for new S&F mode indication and transition time (if available) (check if anything needs to be added to an existing note for this)

6.	RAN2 understands that in case the operation mode transition time is not aligned to the modification period, legacy mechanisms shall apply (e.g. paging for UEs in idle) (no new enhancements are introduced)

7.	UE based paging monitoring relaxation based on NAS configured satellite list is not supported

Come back:

Come back in the next meeting to check whether we can have an indication (likely in SIB31) without mandating any UE behaviour

Based on the discussions and conclusions in the previous meetings, we continue discussing remaining issues on S&F topic in this paper.

Discussion

System Information Update and Cell Reselection

In the last meeting, the case of not enough time for updating SI message after satellite mode switching was discussed and the following agreement was achieved: RAN2 understands that in case the operation mode transition time is not aligned to the modification period, legacy mechanisms shall apply (e.g. paging for UEs in idle) (no new enhancements are introduced). Based on this, we would like to discuss UE behaviour without impacting legacy mechanisms and legacy UEs when facing this issue. When satellite mode switching happens, SI updating procedure will be triggered and network will send paging message to notify UE that the system information (SIB1 and SIB31) is changed and needs to be re-obtained in the next modification period boundary. Following legacy mechanism, all UE will wake up at the next modification period boundary to check if the stored SI message need to be updated. However, if the left t-service time is smaller than the modification period boundary after mode switching, UE will not be able to update the stored system information (e.g. the satellite operation mode) which may have impacts on UE mode configuration.

Observation 1：Following legacy mechanisms, all UE will not be able to update the stored system information before t-service expired if the satellite service time is smaller than the next modification period boundary after satellite mode switching.

For S&F capable UE，it can deduce the mode switching time even without SI update based on the operation mode, the time information and the paging message for SI updating received from the network.

Observation 2：S&F capable UE knows the current operation mode, the time information before satellite mode switching and receives the paging message for SI updating after satellite mode switching.

Thus, without conflicting with the current agreement on system information acquire mechanism, we think S&F UE can trigger cell reselection measurement for this case. That means when S&F UE deduces the mode switching occurred based on existing information like mode operation, time information and paging message for SI update and realizes the left t-service is smaller than the next modification boundary, it can trigger cell reselection measurement to find a better TN or NTN cell. If one suitable cell can be found, then S&F capable UE can reselect to that cell and transmit data normally. If no suitable cell can be found, then S&F capable UE can still follow the legacy mechanisms.

Proposal 1: S&F UE can trigger cell reselection measurement if UE receives paging message after mode switching and knows the current satellite service time is smaller than the next modification period boundary.

Paging Monitoring Relaxation for S&F mode

- In the last meeting, paging monitoring relaxation has been discussed and an LS to SA2 has been sent. Some companies have concerns that paging monitoring skipping will impact legacy system information update procedure. We have a look at the previous agreements to find what changes will lead to SI update in S&F scenario and find two agreements as below:

RAN#129 Agreements [2] :

- A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

RAN#131bis Agreements [1] :

- We clarify in RRC that a change of the S&F mode indication and the S&F mode transition time of neighbour cells in SIB33 does not trigger SI update procedure

- From the above agreements, it is clear that we preclude the system information update due to t-ModeSwitching time information changes and neighbour cell mode indication and time information changes. Thus, system information update due to S&F reasons can only be the current serving satellite S&F operation mode changes. SI will not be changed frequently.

- Observation 3: In S&F operation, SI will not be updated frequently. Most of updates are due to satellite mode change.

- On the other hand, the base station knows whether there is CN-initiated paging coming from the core network and knows when SI message needs to be updated. Thus, RAN can decide when to send paging (e.g. to add indication in SIB31 to indicate UE skipping PO).

- Observation 4: RAN is aware of when CN-initiated paging may occur and knows the appropriate timing for triggering SI updates. RAN can autonomously determine the duration of paging monitoring skipping.

- Based on the above analysis, if SA2 replies with “no CN-initiated paging”, we think it is useful to support paging monitoring skipping for UE power saving reason when there is no stored DL date in satellite S&F operation mode. The indication can be broadcasted in SIB31 message. Furthermore, considering scenarios involving SI updates, the introduction of the paging monitoring skipping indication should not impact the legacy SI update procedure. Since the network can clearly determine when paging needs to be sent and when SI messages require updating, it is recommended that the paging monitoring skipping duration time is also broadcasted in SIB31 with skipping indication together. This approach ensures no impacts on the existing SI update procedure. The configuration of the paging skip duration time can be based on network implementation, but its maximum value can be determined based on the presence of satellite mode switching, e.g. if satellite mode switching exists during t-service (e.g., S&F to normal), the maximum value of the paging skip indication can be set equal to t-modeSwitching; and if there is no satellite mode switching, the maximum value can be set equal to t-service.

- Proposal 2: If SA2 replies there is no CN-initiated paging, an indication is introduced in SIB message to indicate that the UE could skip monitoring paging.

- Proposal 3: If Proposal 2 is agreed, a time information (paging skipping duration) is introduced in SIB message to indicate that how long the UE could skip monitoring paging.

Conclusions

- System Information Update and Cell Reselection

Observation 1：Following legacy mechanisms, all UE will not be able to update the stored system information before t-service expired if the satellite service time is smaller than the next modification period boundary after satellite mode switching.

Observation 2：S&F capable UE knows the current operation mode, the time information before satellite mode switching and receives the paging message for SI updating after satellite mode switching.

Proposal 1: S&F UE can trigger cell reselection measurement if UE receives paging message after mode switching and knows the current satellite service time is smaller than the next modification period boundary.

- Paging Monitoring Relaxation for S&F mode

- Observation 3: In S&F operation, SI will not be updated frequently. Most of updates are due to satellite mode change.

- Observation 4: RAN is aware of when CN-initiated paging may occur and knows the appropriate timing for triggering SI updates. RAN can autonomously determine the duration of paging monitoring skipping.

- Proposal 2: If SA2 replies there is no CN-initiated paging, an indication is introduced in SIB message to indicate that the UE could skip monitoring paging.

- Proposal 3: If Proposal 2 is agreed, a time information (paging skipping duration) is introduced in SIB message to indicate that how long the UE could skip monitoring paging.

References

- 3GPP Chair_Notes, R2-131bis NR-NTN-IoT-NTN (Sergio) EOM

- 3GPP Chair_Notes, R2-129 NR-NTN-IoT-NTN (Sergio) EOM