3GPP TSG-RAN WG2 Meeting #132     	                       R2-2508843

Dallas, US, 17th-21th Nov, 2025

Agenda item:	10.3.3.1

Source:	CMCC

Title:	Discussion on data collection and transfer framework

WID/SID:	FS_6G_Radio

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 discussed requirements on AI/ML data collection/management/transfer, framework, and achieved the following agreements:

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions Agreements on LCM Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

This contribution will discuss transfer of diverse types of data (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc) and understanding of its services/use case scenarios, requirements, end point pairs, size and frequency of reporting, etc.

- Discussion

- 2.1 AI/ML

### 2.1.1 Data type and termination points

During R18/R19 study on AI/ML PHY and AI Mob, RAN2 discussed the data collection for UE-sided model, NW-sided model (including gNB-sided model and LMF-sided model), and two-sided model for different use cases.

Table 1. AI/ML model type and applicable use cases in 5G-A

| Model type | Applicable use cases |
| --- | --- |
| UE-sided model | - Beam management - CSI prediction - Positioning (case 1, 2a) - RRM measurement prediction - Measurement event prediction |
| gNB-sided model | - Beam management - Positioning (case 3a) - RRM measurement prediction |
| LMF-sided model | - Positioning (case 2b, 3b) |
| two-sided model | - CSI compression |

The sub-use cases of Positioning are listed here for information.

-	Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

-	Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

-	Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

-	Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

-	Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

For UE-sided model, the model training resides at UE-side OTT server, the following data collection/transfer solutions were studied and captured in TR 38.843 [2].

1a.	UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO).

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server.

3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

Based on RAN1 LS [3] and above analysis, the typical data content, data size, latency requirement, and data producer, consumer, transfer path for UE-sided model are listed as following table, where beam management is taken as an example.

Table 2. Typical data type for UE-sided model (e.g. beam management)

| LCM procedure | Typical data content | Typical data size (per data sample) | Typical latency requirement | Producer | Consumer | Transfer path |
| --- | --- | --- | --- | --- | --- | --- |
| Training | L1-RSRPs and/or beam-IDs | ~ tens to hundreds bits | Relaxed | UE | UE-side OTT server | 1a.UE->UE-side OTT server 2.UE->CN->UE-side OTT server 3.UE->OAM->UE-side OTT server |
| Inference | Beam prediction results | Small (10s of bits) | Time-critical | UE | gNB | UE->gNB |
| Monitoring | Event occurrence and/or calculated performance metrics, L1-RSRP(s) and/or beam-ID(s) | ~ tens to hundreds bits | Near-real-time | UE | gNB | UE->gNB |

Common Note for all tables, the following descriptions are used:

- - Relaxed (e.g., minutes, hours, days, or no latency requirement)

- - Near-real-time (e.g., several tens of msecs to a few seconds)

- - Time-critical (e.g., a few msecs)

Observation 1: For use cases with UE-sided model (e.g. beam management), the data can be produced by the UE, and consumed by UE-side OTT server for model training or by gNB for inference reporting and monitoring. The typical data content includes L1 measurement results.

For gNB-sided model, the model can be trained at gNB or OAM, and data collection can be gNB-centric and OAM-centric. For OAM-centric solution, the immediate MDT is taken as baseline with enhancements.

Similarly, the typical data content, data size, latency requirement, and data producer, consumer, transfer path for gNB-sided model are listed as following table based on RAN1 LS [3], where beam management is taken as an example.

Table 3. Typical data type for gNB-sided model (e.g. beam management)

| LCM procedure | Typical data content | Typical data size (per data sample) | Typical latency requirement | Producer | Consumer | Transfer path |
| --- | --- | --- | --- | --- | --- | --- |
| Training | L1-RSRPs and/or beam-IDs | ~ tens to hundreds bits | Relaxed | UE | gNB, OAM | UE->gNB, UE->gNB->OAM |
| Inference | L1-RSRPs, and Beam-IDs if needed, for Set B | ~ tens to hundreds bits | Time-critical | UE | gNB | UE->gNB |
| Monitoring | L1-RSRP(s) and/or beam-ID(s) | ~ tens to hundreds bits | Near-real-time | UE | gNB | UE->gNB |

Observation 2: For use cases with gNB-sided model (e.g. beam management), the data can be produced by the UE, and consumed by gNB/OAM for model training, or by gNB for inference and monitoring. The typical data content includes L1 measurement results.

For LMF-sided model, the model can be trained at LMF, and data collection is initiated by LMF. The typical data content, data size, latency requirement, and data producer, consumer, transfer path for LMF-sided model are listed as following table based on RAN1 LS [3], where positioning case 2b and case 3b are taken as examples.

Table 4. Typical data type for LMF-sided model (e.g. Positioning case 2b, 3b)

| LCM procedure | Typical data content | Typical data size (per data sample) | Typical latency requirement | Producer | Consumer | Transfer path |
| --- | --- | --- | --- | --- | --- | --- |
| Training | Measurements: timing, power, and/or phase info Label for Case 2b: Location coordinates | Measurement: ~100 bits to 1000s bits per PRS/SRS resource Label: 56 to 144 bits | Relaxed | UE | LMF | UE->LMF |
|  | Measurements: timing, power, and/or phase info Label for Case 3b: Location coordinates | Measurement: ~100 bits to 1000s bits per PRS/SRS resource Label: 56 to 144 bits | Relaxed | UE (location), gNB (measurement) | LMF | UE->LMF, gNB->LMF |
| Inference | Case 2b: Measurements: Timing, power, and/or phase info | ~100 bits to 1000s bits per PRS/SRS resource | Time-critical | UE | LMF | UE->LMF |
|  | Case 3b: Measurements: Timing, power, and/or phase info | ~100 bits to 1000s bits per PRS/SRS resource | Time-critical | gNB | LMF | gNB->LMF |

Observation 3: For use cases with LMF-sided model (e.g. positioning), the data can be produced by the UE or gNB, and consumed by LMF for model training, inference and monitoring. The typical data content includes positioning measurement results and UE location information.

For two-sided model, the model training collaborations can include the following types.

-	Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided.

-	Type 2: Joint training of the two-sided model at network side and UE side, respectively.

-	Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the NW-side CSI reconstruction part are trained by UE side and network side, respectively.

Furthermore, RAN1/RAN2 discussed that NW-side shares model parameters and/or dataset to the UE or UE-side via standardized signaling, where the model parameters and/or dataset received at the UE or UE-side go through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., model (re-)training. The following solutions are captured in TR 38.843 [2].

-	Non-Over-The-Air (non-OTA) approach: gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)

-	Over-The-Air (OTA) approach: gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)

Based on RAN1 LS [3,4], the typical data content, data size, latency requirement, and data producer, consumer, transfer path for two-sided model are listed as following table, where CSI compression is taken as an example.

Table 5. Typical data type for two-sided model (e.g. CSI compression)

| LCM procedure | Typical data content | Typical data size | Typical latency requirement | Producer | Consumer | Transfer path |
| --- | --- | --- | --- | --- | --- | --- |
| Training, model parametersand/or dataset sharing | Target CSI | See Note 1 | Relaxed | UE | gNB | UE->gNB |
|  | Option 4-1: sharing {target CSI, CSI feedback} dataset | 225 MB (eType-II-like format) | Relaxed | gNB | UE training entity | Non-OTA: gNB->[CN/OAM]->UE training entity OTA: gNB->UE->UE training entity |
|  | Option 3a-1 without target CSI: sharing encoder parameter | 36KB~52M median size: 11.6 MB | Relaxed | gNB | UE training entity |  |
|  | Option 3a-1 with target CSI: sharing encoder parameters, along with {target CSI} dataset | N2 * (2000 bits) / (8bits/Byte) + 11.6 MB Note 2 | Relaxed | gNB | UE training entity |  |
| Inference | CSI Feedback | Per data sample: up to ~ 1000 bits | Time-critical | UE | gNB | UE->gNB |
| Monitoring | Reconstructed CSI from NW to UE, Calculated performance metrics, Target CSI | See Note 1 | Near-real-time | UE | gNB | UE->gNB |
| Note 1: Data size for target CSI depends on the format. The example size per data sample is as: eType-II format (up to ~1000 bits), eType-II-like format (~ a few 1000 bits), and float32 format (up to ~ 150K bits). Note 2: N2 is the number of samples, same or smaller than 40K-1200K. |  |  |  |  |  |  |

Observation 4: For use cases with two-sided model (e.g. CSI compression), the data can be produced by UE and consumed by gNB for model training, inference and monitoring, the dataset and/or model parameters can be produced by gNB and consumed by UE-side OTT server for model training. The typical data content includes target CSI.

The above use cases are studied and specified in RAN1/RAN2 for AI/ML PHY. RAN2 also studied AI Mob including RRM measurement prediction and measurement event prediction. For AI Mob, L1 beam measurement results, L3 beam level and L3 cell level measurement results, measurement event occurrence time information are collected for UE-sided and gNB-sided model training, inference and monitoring, where the producer, consumer and transfer path are similar to beam management.

Based on the above analysis and observations, it can be observed that the AI/ML data collection termination points (i.e. data producer and consumer) can be various for different use cases and LCM procedures, e.g. UE<->UE-side OTT server, UE<->gNB, UE<->OAM, UE<->LMF, gNB<->LMF, gNB<->UE-side OTT server.

Observation 5: AI/ML data collection termination points (i.e. data producer and consumer) can be various for different use cases and LCM procedures, e.g. UE<->UE-side OTT server, UE<->gNB, UE<->OAM, UE<->LMF, gNB<->LMF, gNB<->UE-side OTT server. The typical data content includes L1/L3 measurement results, CSI information, positioning information, etc..

For data terminated between UE and UE-side OTT server, the potential solutions are still under discussion in 5G-A. Therefore, the discussion on UE side data collection in 6G can wait the progress in 5G-A. For other termination points, such as UE<->LMF, gNB<->LMF, gNB<->UE-side OTT server, it can be foreseen that there are impacts on other WGs, e.g. RAN3, SA2. From the perspective of RAN2, we can first study the use cases and potential mechanisms for data terminated in RAN, i.e. study which use cases and data types will be procuded and terminated within RAN side, e.g. L1/L3 measurement results, CSI information.

Proposal 1: RAN2 study the use cases and potential mechanisms for data terminated in RAN.

### 2.1.2 Potential solutions

Based on the analysis in section 2.1.1, it can be seen that the data volume for AI/ML model training is very large, e.g. up to hundreds of MB. On one hand, the data volume is much larger than the legacy RRC signaling size (the maximum size of UL/DL RRC signaling is 144kbytes/45kbytes). On the other hand, for some data terminated in gNB, the existing DRB is not applicable since it cannot be setup between gNB and UE directly, which is always associated with PDU session terminated in UPF. So, the large volumes of data collected by the UE cannot be transmitted to the gNB directly via DRB.

In 6G, we think large volume of AI data transmission and model transfer/delivery should be supported in Day1. The following options can be taken as a starting point and further studied:

- Option 1: Extend RRC message size, including UL and DL

- Option 2: Introduce a new radio bearer which is terminated between UE and base station for data/model transmission

For option1, it can be considered to extend the size of single RRC message, or extend the number of RRC segmentation. Furthermore, the priority of RRC message for data transfer should be discussed. In 5G-A, a new SRB (i.e. SRB6) is introduced for NW-side data collection which is lower than SRB2 since the latency requirement on data transfer for model training is relaxed. For 6G, the similar principle and lower priority SRB can be reused if option 1 is adopted. Another issue is that the SRB transmission will be restarted upon mobility in the existing framework, thus how to support service continuity for SRB segmentation in 6G should be studied.

For option 2, the new radio bearer (e.g. AI bearer) is dedicated for AI data or model transmission, which is separate from legacy DRB and SRB. Similarly, the priority of new radio bearer and service continuity upon mobility should be also discussed and studied.

The potential solutions and details can be further investigated.

Proposal 2: It is proposed to study the potential solutions for AI/ML data and model transfer.

- Option 1: Extend RRC message size, including UL and DL

- Option 2: Introduce a new radio bearer which is terminated between UE and base station

In addition to AI/ML data collection and transfer, data storage is another critical issue which should be discussed and studied.

Based on the above analysis, L1/L3 measurement results, CSI information, positioning measurement results, and UE location information will be collected for model training, inference or monitoring for different use cases, among which L1 measurement results and CSI information are purely generated and used within RAN side. And taking beam management as example, the data for UE-sided model and gNB-sided model training is the same (i.e. L1-RSRPs and/or beam-IDs), which is collected by the UE and consumed by different entities (i.e. RAN and OTT server). If the data collection is triggered for UE-sided model training and gNB-sided model training separately, the duplicated data collection within UE and duplicated reporting via air interface would occur.

Observation 6: For the given use case, the same data produced by the UE can be used for both UE-sided model training and NW-sided model training, e.g. L1-RSRPs and/or beam-IDs for beam management, consumed by RAN and OTT server respectively.

For the data consumed by different entities, a centralized storage can be considered since it is beneficial for efficient data management and sharing. In our understanding, the uses cases such as beam management and CSI compression are RAN-centric, and the data (e.g. the measurement results via air interface) is collected within RAN side. Therefore, it is reasonable to manage and store the data in RAN domain. That means it is unnecessary to transfer and store the data generated and used within RAN to other domain (e.g. CN).

That is, 6G RAN and CN can maintain their own data repositories, wherein the data collected and used within RAN side can be managed and stored in a RAN data repository. Of course, 6G RAN can also share its data to other data consumers if needed, such as CN or OAM. For example, we observe that L3 measurements and cell information can be used for both RAN and CN for mobility optimization. Of course, the data stored in RAN can be also shared to the 3rd entity, e.g. UE-side OTT server for UE-sided model training as we mentioned above, and the details for data sharing can be further discussed and studied, such as via CN or OAM to the 3rd party.

Furthermore, the RAN data repository can also coordinate data collection request from multiple data consumers to minimize the duplicated data collection and reporting. From the perspective of data sharing, we believe that the RAN data repository owned by MNO is reasonable, since it is secured and can support interoperability across different vendors.

Figure 1. An example for RAN data repository

- Proposal 3: 6G RAN should maintain its own data repository, where data collected and used within RAN side can be stored and managed. Furthermore, the data can be shared to different data consumers (e.g. OAM, CN, UE-side OTT server) if needed.

- 2.2 Non-AI services

- SON/MDT

5G NR SON/MDT has been evolved from R16 to R20 to support network deployment and performance optimization for better user experience. Several use cases have been enhanced, including inter-RAT handover, RACH enhancement, SHR, NPN, NR-U, LTM, CHO with candidate SCG, MHI, SDT, etc. SOn/MDT supports the collection of UE-specific measurements using control plane architecture, as well as the configuration and collection are controlled by OAM.

To support MDT mechanism, two modes have been discussed, i.e. logged MDT and immediate MDT. For logged MDT, measurements are logged by UE in IDLE and INACTIVE state, and UE reports the logging information when it enters CONNECTED state. For immediate MDT, the measurements are performed by the UE in CONNECTED state, and reported to gNB when the event is satisfied.

For SON/MDT, the typical data content, data size, latency requirement, and data producer, consumer, transfer path are listed in the following table.

Table 6: Typical data type for SON/MDT

| Use case | Typical data content | Typical data size | Typical latency requirement, or reporting period | Producer | Consumer | Transfer path |
| --- | --- | --- | --- | --- | --- | --- |
| Logged MDT | UE location, L1/L3 measurement report, time stamp, etc | Maximum size per report: 9KB Minimum buffer: 64 KB | Relaxed Note 1 | UE | OAM | UE->gNB->OAM |
| Immediate MDT | UE location, L1/L3 measurement report, etc | Maximum size per report: 9KB | Near-real-time to relaxed Note 2 |  |  |  |
| SON report | RLF report, SHR, RACH report, etc. | Maximum size per report: 9KB | Relaxed Note 3 | UE | gNB | UE->gNB |
| Note 1: UE->gNB: Upon gNB request after entering connected state. gNB->OAM: ~15 minutes periodicity in CMCC’s commercial network. Note 2: Event triggered or period reporting with interval 120ms~30min, depends on NW configuration. Note 3: Upon gNB request. |  |  |  |  |  |  |

Observation 7: For SON/MDT, data can be produced by the UE and consumed by gNB or OAM. The typical data includes UE location, L1/L3 measurements, RLF report, RACH report, etc..

- QoE

5G NR QoE supports the Application Layer Measurement Collection function, which enables the collection of application layer measurements from the UE. The configuration of QoE measurement is provided by OAM, and QoE measurement triggering and stopping can be realized using time-based and/or threshold-based criteria, which also configured by the OAM. RAN is not allowed to intervene, i.e. pause, activate or deactivate an ongoing QoE measurement collection unless instructed otherwise by the OAM. For QoE, different service types are supported, including streaming services, MTSI services, VR, MBMS, XR, etc. In addition to the legacy QoE, RAN-visible QoE is introduced to enable RAN make use of the QoE metrics for such as optimization of RAN resource scheduling. Figure 2 [5] shows the message flow for RAN-visible QoE information reporting.

Figure 2: RAN-visible QoE information reporting

For QoE measurement, the typical data content, data size, latency requirement, and data producer, consumer, transfer path for different service types or use cases are listed in the following table.

Table 7: Typical data type for QoE measurement

| Use case | Typical data content | Typical data Size | Typical latency requirement, or reporting period | Producer | Consumer | Transfer path |
| --- | --- | --- | --- | --- | --- | --- |
| MTSI | Corruption duration metric, successive loss of RTP packets, frame rate, jitter duration, round-trip time, etc. | Maximum size per report: 9KB Minimum buffer: 128 KB | Relaxed | UE | QoE server | UE->[OAM]->QoE server |
| Streaming service | Average throughput, initial playout delay, buffer level, device information, etc. |  |  |  |  |  |
| VR | Average throughput, buffer level, VR device resolution, VR device refresh rate, etc. |  |  |  |  |  |
| RAN-visible QoE | Buffer level, playout delay for media start-up | Maximum size per report: 9KB | Reporting period is 120ms~1024ms | UE | OAM | UE->gNB->OAM |

For QoE measurement, the legacy QoE metrics are transparent for RAN, except for buffer level and playout delay for media start-up. OAM assembles all QoE measurements and forwards to QoE server for application optimization.

Observation 8: For QoE, the data is produced by UE and consumed by QoE server or OAM. The data content includes application layer related information for different services, where buffer level and playout delay for media start-up are visible for RAN.

- Sensing

For sensing scenario, 6G is expected to support all sensing modes as illustrated in the following figure, including both mono-static and bi-static with both TRP and UE involved to explore more use cases for improvement of sensing performance.

Figure 3. Sensing mode

In our understanding, the following data types can be considered for sensing service:

- Sensing raw data (e.g., IQ data): It contains all the information obtained by an ISAC system without signal processing, including time-domain samples or sub-carrier-domain samples of original received signals from each antenna port. [Data size: GB level]

- Delay/angular/Doppler spectrum: Spectrum is the first-stage processing output derived from sensing raw data. It could retain most essential information while significantly reducing data volume compared to the raw data. Delay spectrum, angular spectrum and Doppler spectrum are fundamental for most of the sensing algorithms. [Data size: hundreds of MB]

- Cloud of points: It is extracted by implementing detection algorithms on delay, angular and Doppler spectrum. This stage typically discards most of the information, resulting in a relatively low data volume while preserving critical parameters. [Data size: tens of KB to tens of MB]

- Sensing result: It is the final output of sensing signal processing. A sensing result is described by target’s attributes, including location, velocity and type, which has the lowest data volume. [Data size: several KB]

However, the details of data type, data content, producer and consumer need more RAN1’s inputs for further discussion. Even though, it can be also observed that the volume of sensing data is substantial.

Observation 9: RAN1’s inputs are needed for the details of data type, data content, producer, consumer of sensing. For now, it can be observed that the data volume of sensing is large.

Based on the above observations, for SON/MDT and QoE, the data is produced by the UE and consumed by gNB, OAM, or QoE server. It can be observed that for SON report and RAN-visible QoE, the data is produced and consumed within RAN side, which is similar to some AI/ML use cases as we analyzed in section 2.1. Therefore, we think that the similar mechanism for AI/ML can be also considered to reuse for RAN-visible QoE and SON report.

Proposal 4: For RAN-visible QoE and SON report, the similar mechanism to AI/ML can be considered since the data is also produced and consumed within RAN side.

On the other hand, it can be seen that L1/L3 measurement results are collected for logged MDT and immediate MDT and consumed by OAM, while they can be also used for AI/ML use cases, such as beam management and mobility. For this type of data, we think that they can be also stored within RAN side, and shared to OAM for network optimization.

Proposal 5：For the data such as L1/L3 measurement results which can be used for both MDT and AI/ML, it can be stored within RAN side and shared to OAM for network optimization.

- 2.3 Data collection requirements

Based on the above analysis, the collected data can include measurement results (e.g. L1-RSRP, CSI feedback) in air interface, UE trajectory information, UE location information, etc.. In 5G-A, user consent is required for SON/MDT since the UE location information is collected.

However, SA3 agreed that the user consent requirements can be configured by OAM in R18. It means the operator can configure which data requires user consent and which data does not, thereby ensuring 3GPP system is in compliance with local laws and regulations. The details are specified in section 4.9 of TS 32.422 [6] and highlighted as follows.

4.9 User consent handling in MDT for NG-RAN 4.9.1 Signalling based MDT In case of signalling based MDT, getting user consent before activating the MDT functionality is required because of privacy and legal obligations. It is the Operator’s responsibility to collect user consent before initiating an MDT for a specific IMSI, IMEI number or SUPI. Collecting the user consent shall be done via customer care process. The user consent information availability should be considered as part of the subscription data and as such this shall be provisioned in the UDM database. The following figure summarizes the functionality. Figure 4.9.1.1: Example for delivering user consent information in signalling based MDT When the IMSI/IMEI(SV)/SUPI based MDT is activated, it is targeted the UDM. The UDM shall check the MDT user consent requirements configured by OAM. If one or more MDT measurement name(s) is/are specified in MDT activation, the UDM shall: - For user-consent-required MDT measurement name(s), start a Trace Session for the given subscriber if the user consent is available. - For user-consent-required MDT measurement name(s), not start a Trace Session for the given subscriber if the user consent is not available. - For user-consent-not-required MDT measurement name(s), start a Trace Session for the given subscriber without considering user consent. <omitted>

Observation 10: In 5G-A, MDT user consent can be configured by OAM.

Given the large amount of data collected in 6G, requiring user consent for all collected data will significantly prolong the time for network optimization, AI service provision and 6G commercial deployment. Therefore, it is expected to discuss which kind of data really requires user consent. In 6G, the same principle in 5G-A should be followed that user consent can be configured by OAM.

Proposal 6：6G follows 5G-A mechanism that MDT user consent can be configured by OAM.

In our understanding, the user consent should be respected for UE privacy information (e.g. UE location), but it is unnecessary for legacy L1/L3 measurement results in air interface since they can be obtained within the existing RAN framework without user consent actually.

Proposal 7: For UE location information, the user consent should be respected. However, for the measurement results in air interface, it is unnecessary to require user consent.

# 3	Conclusion

Here are the observations and proposals for AI/ML framework and data collection in 6G.

Observation 1: For use cases with UE-sided model (e.g. beam management), the data can be produced by the UE, and consumed by UE-side OTT server for model training, or by gNB for inference reporting and monitoring. The typical data content includes L1 measurement results.

Observation 2: For use cases with gNB-sided model (e.g. beam management), the data can be produced by the UE, and consumed by gNB/OAM for model training, or by gNB for inference and monitoring. The typical data content includes L1 measurement results.

Observation 3: For use cases with LMF-sided model (e.g. positioning), the data can be produced by the UE or gNB, and consumed by LMF for model training, inference and monitoring. The typical data content includes positioning measurement results and UE location information.

Observation 4: For use cases with two-sided model (e.g. CSI compression), the data can be produced by UE and consumed by gNB for model training, inference and monitoring, the dataset and/or model parameters can be produced by gNB and consumed by UE-side OTT server for model training. The typical data content includes target CSI.

Observation 5: AI/ML data collection termination points (i.e. data producer and consumer) can be various for different use cases and LCM procedures, e.g. UE<->UE-side OTT server, UE<->gNB, UE<->OAM, UE<->LMF, gNB<->LMF, gNB<->UE-side OTT server. The typical data content includes L1/L3 measurement results, CSI information, positioning information, etc..

Proposal 1: RAN2 study the use cases and potential mechanisms for data terminated in RAN.

Proposal 2: It is proposed to study the potential solutions for AI/ML data and model transfer.

- Option 1: Extend RRC message size, including UL and DL

- Option 2: Introduce a new radio bearer which is terminated between UE and base station

Observation 6: For the given use case, the same data produced by the UE can be used for both UE-sided model training and NW-sided model training, e.g. L1-RSRPs and/or beam-IDs for beam management, consumed by RAN and OTT server respectively.

- Proposal 3: 6G RAN should maintain its own data repository, where data collected and used within RAN side can be stored and managed. Furthermore, the data can be shared to different data consumers (e.g. OAM, CN, UE-side OTT server) if needed.

Observation 7: For SON/MDT, data can be produced by the UE and consumed by gNB or OAM. The typical data includes UE location, L1/L3 measurements, RLF report, RACH report, etc..

Observation 8: For QoE, the data is produced by UE and consumed by QoE server or OAM. The data content includes application layer related information for different services, where buffer level and playout delay for media start-up are visible for RAN.

- Observation 9: RAN1’s inputs are needed for the details of data type, data content, producer, consumer of sensing. For now, it can be observed that the data volume of sensing is large.

Proposal 4: For RAN-visible QoE and SON report, the similar mechanism to AI/ML can be considered since the data is also produced and consumed within RAN side.

Proposal 5：For the data such as L1/L3 measurement results which can be used for both MDT and AI/ML, it can be stored within RAN side and shared to OAM for network optimization.

Observation 10: In 5G-A, MDT user consent can be configured by OAM.

Proposal 6：6G follows 5G-A mechanism that MDT user consent can be configured by OAM.

Proposal 7: For UE location information, the user consent should be respected. However, for the measurement results in air interface, it is unnecessary to require user consent.

# 4	Reference

- R2_131bis_ChairNotes

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R1-2310681 Reply LS on Data Collection Requirements and Assumptions

- R1-2410922 LS on signalling feasibility of dataset and parameter sharing

- TR 38.890 Study on NR QoE (Quality of Experience) management and optimizations for diverse services

- TS 32.422 Telecommunication management; Subscriber and equipment trace; Trace control and configuration management