3GPP TSG-RAN WG2 Meeting #132	R2-2508852

Dallas, USA, Nov. 17th – 21st, 2025

# Agenda item:	10.3.2.2

# Source:	ETRI

# Title:	Discussion on RRC Structure and Configuration in 6G

# Document for:	Discussion, Decision

- Introduction

In RAN#131-bis meeting, agreements on RRC signaling design and reconfigurations have been made as the followings [1]:

Agreements - ASN.1 is used for encoding of RRC signalling for 6G air interface (same as NR). - Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signaling - Study how to efficiently, reliably and unambiguously configure UEs while keeping signaling size small (e.g. improvements to delta signaling or no delta signaling) - RAN2 will study modular design of RRC for 6G. Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

This contribution provides a comparative analysis of different modularization axes for the modular RRC design and proposes a direction for further study within RAN2.

- Discussion

In the current 5G RRC structure, all configuration parameters are defined within a single hierarchical ASN.1 framework. While this approach provides flexibility and backward compatibility, it has led to excessive message sizes, redundant signaling, and high implementation complexity. In the last RAN2 meeting, there was a consensus that a modularized RRC architecture should be introduced, allowing independent management and signaling of RRC components. However, it remains to be determined how to define the modularization axis—whether it should be based on features, functions, or verticals (e.g., service or device categories). To support future study, a detailed comparison of these three approaches is presented in this paper.

A feature-based modular RRC design groups configuration elements according to individual radio features such as carrier aggregation, sidelink, integrated access and backhaul (IAB), and multi-connectivity. This approach enables intuitive activation and deactivation of specific capabilities, thereby improving operational efficiency. It also provides a direct mapping between a network feature and its corresponding RRC configuration module. However, this design tends to introduce redundancy since many parameters—such as measurement, timing, or beam management—are reused across multiple features. As a result, the RRC structure may contain duplicated definitions, which complicates version management and increases the risk of inconsistency when updates are made across working groups. Furthermore, UE implementation becomes complex due to the large number of possible feature combinations that must be handled in parallel.

Observation 1: A feature-based modular design simplifies operation and management but causes redundancy and inter-feature dependency, which complicate maintenance and evolution.

A function-based modular RRC design organizes modules according to protocol functions or roles, such as measurement, mobility control, carrier and bandwidth management, QoS and bearer control, CSI and beam management, and power saving. This approach allows each function to evolve independently, minimizing cross-dependencies between modules. It also enables reuse of shared parameters, improving the consistency of the RRC structure and reducing duplication. From a maintenance perspective, RAN1 updates affect only the relevant function module, which simplifies inter-WG coordination. Additionally, a function-based structure provides a natural fit for reference-based delta signaling, since each function module can define its own reference configuration and transmit only differential updates. This can significantly reduce signaling overhead and processing time during reconfiguration procedures, including handovers.

Some coordination is required to define functions and to set clear boundaries between them, but this approach provides long-term benefits in terms of modular independence and implementation simplicity. It also allows higher layers to build service- or feature-specific configurations as a profile composed of multiple function modules without adding complexity to the RRC structure.

Observation 2: A function-based modular design provides the balance between efficiency, maintainability, and scalability.

A vertical-based modular RRC design groups configuration modules by device or service category, such as eMBB, URLLC, RedCap, or industrial IoT. This approach simplifies deployment and operation since a predefined configuration profile can be directly applied to devices of a given type. While this provides high operational convenience, it also creates duplication since similar parameters must be repeated in multiple vertical modules. As new features and releases are added, keeping consistency across vertical-specific configurations becomes more difficult. Moreover, when a UE supports multiple verticals, the overlap of configuration sets increases memory use and processing complexity. Inter-WG coordination is also more difficult because updates to core parameters require changes in all vertical modules.

Observation 3: A vertical-based modular design simplifies deployment and operation but increases redundancy and long-term maintenance cost, making it unsuitable for sustainable RRC evolution.

Based on the above analysis, the function-based modular approach offers the most balanced trade-off among efficiency, maintainability, and extensibility. It minimizes redundancy, isolates inter-working group impacts, and supports modular evolution without affecting unrelated functions. The feature-based approach improves clarity but increases duplication and dependency, while the vertical-based approach simplifies deployment and operation but sacrifices structural consistency. Therefore, further study should focus on refining the function-based modular design and evaluating its effectiveness in reducing signaling size, complexity, and cross-release maintenance cost.

To achieve these objectives, it is proposed that RAN2 continue the study of function-based modular RRC design, where each protocol function (e.g., measurement, mobility control, BWP and carrier management, QoS and bearer control, CSI and beam management, and power saving) is defined as an independent RRC module. Each module should have a clearly defined role and scope so that its parameters are not duplicated or ambiguously shared with other modules. The relationships among modules should also be explicitly specified, including which modules provide information to others and which modules depend on that information. This structured definition will make each module self-contained, easier to maintain, and interoperable with other modules, while minimizing unnecessary dependencies. Vertical- or feature-specific configurations should be expressed as higher-layer profiles composed of multiple function modules, rather than being directly embedded in the ASN.1 structure. This separation enables flexible profile management without increasing the complexity of the RRC structure.

Furthermore, to improve signaling efficiency, it is proposed that each RRC module support reference-based delta signaling. In this framework, each module may define a reference configuration that represents its baseline state, and subsequent configuration or reconfiguration messages would only carry the parameters that differ from the reference. In addition, the network may predefine and manage a set of module-specific reference configurations, allowing the UE to reference these predefined baselines and perform delta signaling relative to them. This per-module delta signaling mechanism enables efficient partial updates, reduces overall message size, and enhances robustness through version tracking and integrity protection at the module level. Together, the function-based modular architecture and reference-based delta signaling are expected to provide a scalable, efficient, and evolvable RRC configuration framework suitable for diverse device types and future 6G services.

Proposal 1: RAN2 studies a function-based modular RRC structure, where each function is defined as an independent ASN.1 module. Vertical/feature-specific configurations can be realized as high-layer profiles composed of multiple function modules.

Proposal 2: RAN2 studies reference-based delta signaling at the module granularity. (i.e., each module defines a reference configuration and supports delta updates based on the reference configuration)

- Conclusions

This paper presents some of our initial views on the RRC configuration framework. We have the following observations and proposals:

Observation 1: A feature-based modular design simplifies operation and management but causes redundancy and inter-feature dependency, which complicate maintenance and evolution.

Observation 2: A function-based modular design provides the balance between efficiency, maintainability, and scalability.

Observation 3: A vertical-based modular design simplifies deployment and operation but increases redundancy and long-term maintenance cost, making it unsuitable for sustainable RRC evolution.

Proposal 1: RAN2 studies a function-based modular RRC structure, where each function is defined as an independent ASN.1 module. Vertical/feature-specific configurations can be realized as high-layer profiles composed of multiple function modules.

Proposal 2: RAN2 studies reference-based delta signaling at the module granularity. (i.e., each module defines a reference configuration and supports delta updates based on the reference configuration)

- References

- Chair notes RAN2#131-bis

- RP-251395, Revised SID: Study on 6G Scenarios and requirements

- RP-251881, New SID: Study on 6G Radio

- TR 38.914, Study on 6G Scenarios and requirements