3GPP TSG-RAN WG2 Meeting #132	R2-2508856

Dallas, USA, 17 – 21 November, 2025

Source: 	Huawei, HiSilicon

Title: 	Remaining issues on LTE TN to NR NTN mobility

Agenda Item:	8.8.4

Document for:	Discussion and decision

# 1	Introduction

In RAN2#129bis meeting[1], the following agreements were made.

Agreements: 1. RAN2 confirms that redirection from LTE TN to NR NTN is not supported before Rel-19. 2. Extend the capability signaling for E-UTRA TN to NR NTN redirection to also indicate the support of IRAT E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives this capability common to E-UTRA TN to NR NTN redirection and cell reselection (based on NW implementation). 3. The redirection to NR NTN information element is enclosed in an NTN specific CarrierInfoNR, which is introduced in the choice structure as nr-19.

In this paper, we further discuss the remaining issue on LTE TN to NR NTN mobility postponed in the last meeting.

# 2	Discussion

In RAN2#131bis meeting[2], the remaining issue on LTE TN to NR NTN mobility were left open for discussion:

R2-2507493 Remaining issues on LTE TN to NR NTN mobility Huawei, HiSilicon discussion Rel-19 NR_NTN_Ph3-Core Proposal 1: RAN 2 agrees that there is no need UE should adjust the offset by itself. Smtc The SSB periodicity/offset/duration configuration of the redirected target NR frequency. It is based on the timing reference of EUTRAN PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If smtc-r19 is configured, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the UE's propagation delay difference between serving cell and neighbour cells equals to 0 ms, and UE can adjust the offset based on the actual propagation delay. Come back in the next meeting ([V230] is still ToDo) Proposal 2: RAN 2 agrees that it is up to the network implementation whether to include the smtc in the CarrierInfoNR-r19 or not. Agreed

Before addressing the specific open issues, it should be noted, in general, no further enhancements to the RRCRelease message should be discussed, as these kind of enhancements were not considered even for NTN–NTN mobility during the Rel-17 or Rel-18 discussions.

With respect to the specific issues raised in the previous meeting that remain open, our views are as follows

Whether the UE should adjust the offset according to the actual propagation delay for the case when smtc-r19 is not present in the CarrierInfoNR-r19. In our view, there is no need for UE to adjust the offset. It should be noted that the RRCRelease message is a dedicated UE message, and given the fact that the LTE TN coverage is quite small, the gNB can configure appropriated SMTC for the UE in RRCRelease message. Hence we propose

Proposal 1: RAN 2 agrees that there is no need UE should adjust the offset by itself.

# 3	Conclusion

In this contribution the following proposals are made for LTE to NR NTN mobility:

Proposal 1: RAN 2 agrees that there is no need UE should adjust the offset by itself .

# 4	References

- R2-2502983, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 129bis, Wuhan, China, Apr 7th – 11th, 2025

- R2-2507703, Report from Break-out session on NES, NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 131bis, Prague, Czech Republic, Oct 13th – 17th , 2025