3GPP TSG-RAN WG2 Meeting #132	R2-2508857

Dallas, USA, 17 – 21 November, 2025

Agenda Item:	8.9.3

Source:	Huawei, HiSilicon

Title:	Impact on discontinuous coverage for Store and Forward

Document for: Discussion and Decision

# 1	Introduction

In the last meeting [1], the impact of the S&F mode on AS was postponed to this meeting:

- Come back in the next meeting

This contribution focuses on addressing one remaining issue for Store and Forward satellite operation, namely UE behaviour related to the Satellite ID list.

# 2	Discussion

During RAN2#129 meeting, RAN2 received the reply LS on satellite IDs from SA2 as follows [2]:

- RAN2 understanding: The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

[SA2 response]: SA2 has discussed the scenario and clarified this in the TS 23.401 clause 4.13.9.

Question: If a satellite is included in the satellite list signalled by the MME to a UE, does it mean one of the below options, or possibly other option(s):

- the satellite has the UE context but does not necessarily support store-and-forward operation; or

- the satellite has the UE context and must support store-and-forward operation (but may be operating in normal IoT NTN mode or in store-and-forward mode when the satellite serves the area where the UE is located)?

[SA2 Answer]: SA2 confirms that option 2 is the correct understanding.

According to the LS content and the updated TS 23.401, the following RAN2 understanding is confirmed by SA2: “the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.” This is mainly to avoid any restriction on the UE camping on a normal cell which is not in the satellite list. However, according to SA2’s answer to the second question, it means NW may not put a satellite in the satellite list if there is no UE context on that satellite. Because UE may fail upon accessing that satellite.

In this case, when a S&F UE determines whether there is NTN coverage, it should not only refer to the legacy SIB32 (for normal cells) but also the NAS wait timer and the NAS satellite list (for S&F cells with UE context).

Proposal 1: If the S&F Monitoring List and the S&F Wait Timer have been received from the upper layers, an S&F UE needs also to consider this NAS information when determining whether it is out of NTN coverage, besides SIB32.

# 3	Conclusion

This contribution discusses the issue related to defining UE behaviour for handling the Satellite ID list for store and forward operation for IoT NTN and we have the following proposals:

Proposal 1: If the S&F Monitoring List and the S&F Wait Timer have been received from the upper layers, an S&F UE needs also to consider this NAS information when determining whether it is out of NTN coverage, besides SIB32.

The text proposal for TS 36.304 is provided in the Annex.

# 4	References

- R2-2507703, Report from Break-out session on NES, NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 131bis.

- S2-2502450, Reply LS on Satellite IDs for store-and-forward operation, SA2.

# 4	Annex – Text Proposal for TS 36.304

## 4.1	Overview

The idle mode tasks can be subdivided into four processes:

-	PLMN selection;

-	Cell selection and reselection;

-	Location registration;

-	Support for manual CSG selection.

The relationship between these processes is illustrated in Figure 4.1-1.

Figure 4.1-1: Overall Idle Mode process

When a UE is switched on, a public land mobile network (PLMN) is selected by NAS. For the selected PLMN, associated RAT(s) may be set TS 23.122 [5]. The NAS shall provide a list of equivalent PLMNs, if available, that the AS shall use for cell selection and cell reselection.

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

For E-UTRA a cell may be associated with more than one CN type (EPC and/or 5GC) and hence the selected cell can be suitable for more than one CN type. The CN type(s) for which the selected cell is suitable are reported to NAS which selects a CN type to be used for camping and for the NAS registration procedure (see below). Note that CN type selection is only applicabe for UE supporting E-UTRA connected to 5GC.

For E-UTRA a cell may be associated with more than one tracking area. The UE reports all the broadcasted tracking area codes in the selected cell to NAS for registration procedure.

The UE shall, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN TS 23.122 [5].

If the UE finds a more suitable cell, according to the cell reselection criteria, it reselects onto that cell and camps on it. Similar to cell selection procedure, if the reselected cell is an E-UTRA cell and the UE supports E-UTRA connected to 5GC, the CN type(s) for which the cell is suitable are reported to NAS which selects one of them. If the new cell does not belong to at least one tracking area to which the UE is registered, location registration is performed. In RRC_INACTIVE state, if the new cell does not belong to the configured RNA, a RNA update procedure is performed.

If necessary, the UE shall search for higher priority PLMNs at regular time intervals as described in TS 22.011 [4] and search for a suitable cell if another PLMN has been selected by NAS.

Search of available CSGs may be triggered by NAS to support manual CSG selection.

If the UE loses coverage of the registered PLMN, either a new PLMN is selected automatically (automatic mode), or an indication of which PLMNs are available is given to the user, so that a manual selection can be made (manual mode).

Registration is not performed by UEs only capable of services that need no registration.

The UE may perform sidelink communication or V2X sidelink communication or sidelink discovery or NR sidelink communication while in-coverage or out-of-coverage for sidelink, as specified in clause 11.

The purpose of camping on a cell in idle mode is fivefold:

a)	It enables the UE to receive system information from the PLMN.

b)	When registered and if the UE wishes to establish an RRC connection, it can do this by initially accessing the network on the control channel of the cell on which it is camped.

c)	If the PLMN receives a call for the registered UE, it knows (in most cases) the set of tracking areas (in RRC_IDLE state) or RNAs (in RRC_INACTIVE state) in which the UE is camped. It can then send a "paging" message for the UE on the control channels of all the cells in this set of tracking areas. The UE will then receive the paging message because it is tuned to the control channel of a cell in one of the registered tracking areas and the UE can respond on that control channel.

d)	It enables the UE to receive ETWS and CMAS notifications.

e)	It enables the UE to receive MBMS services.

If the UE is unable to find a suitable cell to camp on or if the location registration failed (except for LR rejected with cause #12, cause #14, cause #15 or cause #25, see TS 23.122 [5] and TS 24.301 [16]), it attempts to camp on a cell irrespective of the PLMN identity, and enters a "limited service" state.

When NAS indicates that PSM starts, the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, all running timers continue to run but the UE need not perform any idle mode tasks. If a timer expires while the UE is in PSM it is up to UE implementation whether it performs the corresponding action immediately or the latest when PSM ends. When NAS indicates that PSM ends, the UE shall perform all idle mode tasks.

If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks related to NTN. In addition, if the S&F Monitoring List and the S&F Wait Timer have been received from the upper layers, an S&F UE needs also to consider this NAS information when determining whether it is out of NTN coverage. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in NTN coverage the UE shall perform all idle mode tasks related to NTN. If SystemInformationBlockType32 includes carrierFreqList the UE may store and use this information for the cell selection process when UE resumes the idle mode tasks related to NTN once in NTN coverage.