3GPP TSG-RAN WG2 Meeting #132	R2-2508859

Dallas, USA, Nov. 17th – 21st, 2025

# Agenda item:	10.4

# Source:	ETRI

# Title:	Discussion on 6GR mobility

# Document for:	Discussion, Decision

- Introduction

In RAN#131-bis meeting, agreements on 6G mobility have been made as the followings [1]:

Agreements - Study mobility with the following requirements in 6G mobility design: - Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions. - Robustness of mobility procedures - Energy efficiency for both UE and NW - Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

This paper discusses how to design a unified mobility framework for 6G mobility.

- Discussion

During the previous RAN2 meeting, the concept of a unified mobility framework for 6G mobility design was introduced, but detailed discussions could not be completed. As a next step, this paper focuses on the handover (HO) signaling aspect of the unified mobility framework, specifically addressing pre-configuration strategy, HO command message type, and configuration optimization.

In 5G, different HO types (L3 HO, CHO, LTM) use different signaling steps, even though they all do the same basic things: prepare a candidate/target cell, send a cell switch command, and confirm the cell switch is done. This difference complicates UE and network implementations, and it creates more work in standardization. A unified signaling framework with common steps can solve these problems. This would also allow reuse of existing messages and reduce unnecessary diversity across all scenarios.

Observation 1: Handover signaling in 5G specifications differs across scenarios, although the basic phases are the same. Unified HO signaling with common steps can simplify implementation and standardization.

To build a unified HO signaling, we need to answer two key questions:

- Should we always use pre-configuration?

- Should the unified HO command be sent as an RRC message or a MAC CE?

From the UE’s point of view, mobility without pre-configuration — like legacy L3 HO — delivers both the target cell configuration and the cell switch command in one RRC message called RRCReconfiguration with reconfigurationWithSync. The UE does not know anything about the target cell before this message. In contrast, mobility with pre-configuration — like CHO and LTM — separates these steps. The network first sends the candidate cell configurations in an RRC message when it decides to prepare. Later, when the right condition is met, it sends a separate HO command, for example a MAC CE in LTM.

This difference affects latency, robustness, and signaling overhead in important ways. For latency, the biggest difference comes from when the UE can finish processing the target cell configuration. In the without pre-configuration case (legacy L3 HO), the UE receives the full target cell configuration and the HO command at the same time inside one big RRCReconfiguration message. The UE can only start processing the new configuration after it has fully received and decoded this message. This means the UE needs extra time to process a large RRC message before it can switch to the target cell. As a result, the actual cell switch is delayed. In the with pre-configuration case (CHO, LTM), the UE receives and processes all candidate cell configurations much earlier, during the preparation phase. By the time the HO command arrives (for example, as a small MAC CE), the UE has already finished processing the target cell configuration. So, the UE can switch to the new cell immediately when it receives the command. This pre-processing makes the cell switch much faster. Therefore, from the pure latency point of view, with pre-configuration always has lower interruption time because the UE does not waste time processing a big RRC message during the critical handover execution moment.

For robustness, without pre-configuration has two important concerns that can increase the chance of handover failure. The first concern is that the network has to make the HO decision earlier to leave enough time for preparation and to send the large RRCReconfiguration message. If the radio condition suddenly becomes much worse between the moment the network decides to trigger HO and the moment the UE actually receives the HO command, the UE may not receive the command at all. In the worst case, the link is lost and the UE declares radio link failure (RLF). The second concern is that there is only one target cell and no backup. Even if the UE successfully receives the command, random access to that single cell can still fail when the channel is poor. With pre-configuration, the network does not need to decide early. It prepares candidate cells in advance and continuously monitors the radio condition. As soon as the channel starts to degrade, the network can immediately send a tiny HO command (MAC CE) while the link is still good enough. The UE receives it quickly and switches to a prepared cell right away. Also, if the chosen cell fails, the UE can instantly try another prepared cell, as already supported in CHO or through fast recovery in LTM. This fast reaction and multiple backups make handover much more reliable.

For signaling and resource overhead, the without-pre-configuration approach requires less signaling since RRCReconfiguration message includes a configuration only for a single target cell. In contrast, the pre-configuration approach involves sending configurations for multiple candidate cells in advance, even though not all of them will be used. It increases signaling and storage overhead at the UE. However, in practical deployments, the number of pre-configured candidate cells is typically limited, and can be further optimized using AI/ML-based prediction of potential target cells. Such techniques can significantly reduce unnecessary configurations by pre-selecting only the most probable candidates. Additionally, unused configurations can be released after HO completion or upon timer expiry, and delta signaling can be used to minimize configuration updates. Although pre-configuration incurs a certain amount of overhead, its advantages in latency reduction and mobility robustness generally outweigh this cost.

Supporting both approaches in a unified framework would preserve backward compatibility but introduce dual execution paths, inconsistent configuration management, and increased UE/network complexity, which directly contradict the goal of unification. Since CHO and LTM which rely on pre-configuration have already been proved effective for achieving high reliability and low latency, the unified framework should adopt mobility with pre-configuration as the sole baseline.

Observation 2: From the unified framework perspective, it is preferable to adopt a single approach between the with pre-configuration and without pre-configuration options. Although the with pre-configuration approach requires relatively higher signaling and storage overhead, it is preferred due to its advantages in latency, robustness, and procedural consistency.

The second design issue is the message type and the layer for delivering the HO command. An RRC-based HO command, as in legacy L3 HO, is carried within RRCReconfiguration. When the HO trigger comes from L1/L2 measurements (for example, a beam-level event in LTM), the trigger must travel from PHY to MAC to RRC. Then RRC builds the message, PDCP adds security, and the message goes back down to MAC for scheduling and transmission. This round-trip through the RRC layer adds several milliseconds of delay due to RRC processing, PDCP security operations, and scheduling latency.

In contrast, a MAC CE-based HO command, as used in LTM cell switch, is generated and transmitted directly within the MAC layer. When an L1 measurement event is triggered, the MAC layer can immediately construct and send the command as a MAC CE. This achieves near-instant execution with minimal latency, which is essential for L1/L2-centric mobility where responsiveness is critical.

The primary concern with MAC CE is the current lack of integrity protection. However, RAN2 has already sent an LS to SA3 requesting study on security for critical MAC CEs, including mobility commands. Given the tight 6G development timeline, RAN2 should not delay framework progress while waiting for SA3 feedback. The MAC CE has clear advantages in speed, so we propose to take MAC CE-based HO command as the baseline now. The final decision can be confirmed later based on the SA3 outcome.

Observation 3: A MAC CE HO command has advantages in terms of latency because it avoids the RRC round-trip delay. The security aspect of the MAC CE can be further discussed based on the feedback from SA3.

Figure 1 illustrates the proposed unified HO signaling, which consists of three clearly defined steps. In the Preparation step, the network sends one or more candidate cell configurations via RRCReconfiguration. In the Command step, the network sends a small HO Command MAC CE when execution is needed. In the Completion step, the UE sends RRCReconfigurationComplete to the target cell. The key feature of this design is the clear separation between preparation and command phase. Most signaling messages can be reused from existing 5G procedures. The HO command MAC CE can be derived with minimal changes from the current LTM cell switch command. This unified framework reduces signaling diversity, improves flexibility, and enables fast and reliable mobility across different HO scenarios.

Figure 1 An example of unified handover signaling

Proposal 1: Study a unified HO signaling framework based on a common three-phase structure (Preparation, Command, Completion)

Proposal 2: Study “mobility with pre-configuration” as a baseline design for 6G HO framework.

Proposal 3: Study the use of MAC CE format as a unified HO command. This can be revisited depending on the inputs from SA3.

Configuration management at the RRC level is another area where unnecessary complexity arises. Today, the same candidate cell may be configured multiple times depending on whether it is used for CHO, LTM-based HO, or conventional L3 HO. This redundancy increases signaling load, wastes UE resources, and may result in inconsistent configurations between procedures. For example, one UE may store multiple RRC configurations for the same target cell when it appears in different HO contexts. In 6G, a unified principle should be adopted: each candidate cell is associated with a single, scenario-independent RRC configuration. Different HO schemes that reference the same candidate cell should reuse that configuration. Specific HO features can be indicated via sub-IEs or parameters. This ensures consistency, reduces redundancy, and simplifies both UE and network operations.

Observation 4: The same candidate cell can be configured multiple times under different HO scenarios. A single common RRC configuration per candidate cell can avoid duplication and inconsistency.

Proposal 4: Study use of one common RRC configuration for the same candidate cell across all handover scenarios.

- Conclusions

In this paper, we discuss a unified mobility framework. Followings are observations and proposals made in this paper:

Observation 1: Handover signaling in 5G specifications differs across scenarios, although the basic phases are the same. Unified HO signaling with common steps can simplify implementation and standardization.

Observation 2: From the unified framework perspective, it is preferable to adopt a single approach between the with pre-configuration and without pre-configuration options. Although the with pre-configuration approach requires relatively higher signaling and storage overhead, it is preferred due to its advantages in latency, robustness, and procedural consistency.

Observation 3: A MAC CE HO command has advantages in terms of latency because it avoids the RRC round-trip delay. The security aspect of the MAC CE can be further discussed based on the feedback from SA3.

Observation 4: The same candidate cell can be configured multiple times under different HO scenarios. A single common RRC configuration per candidate cell can avoid duplication and inconsistency.

Proposal 1: Study a unified HO signaling framework based on a common three-phase structure (Preparation, Command, Completion)

Proposal 2: Study “mobility with pre-configuration” as a baseline design for 6G HO framework.

Proposal 3: Study the use of MAC CE format as a unified HO command. This can be revisited depending on the inputs from SA3.

Proposal 4: Study use of one common RRC configuration for the same candidate cell across all handover scenarios.

- References

- Chair notes RAN2#131-bis

- RP-251395, Revised SID: Study on 6G Scenarios and requirements

- RP-251881, New SID: Study on 6G Radio

- TR 38.914, Study on 6G Scenarios and requirements