3GPP TSG-RAN WG2 Meeting #132	R2-2508874

Dallas, Texas, USA, 17-21 November 2025

Agenda item:	10.3.2.2

Source:	Samsung

Title:	RRC Restructuring and modular aspects for 6G

Document for:	Discussion & Decision

Introduction

In RAN2#131bis, RAN2 initiated a new study item focused on developing a new radio access technology for 6G. Key agreements were made in the control plane to study the RRC structure, signaling design, and modular design of RRC for 6G.

Agreements

  - ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

  - Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling

  - Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g. improvements to delta signaling or no delta signaling).

  - RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

In this contribution we share our initial views on RRC structure and Modular Design aspects for 6G.

Discussion

The 5G control plane, specifically the Radio Resource Control (RRC) layer, is designed to handle a wide range of device types, enabling diverse services and features to support multiple use cases. The 5G ecosystem accommodates a broad spectrum of devices, from simple wearables and IoT devices to advanced smartphones and Extended Reality (XR) devices. Each device type has distinct requirements, such as power efficiency, bandwidth, and latency, as shown in Figure 1.

Figure 1: Diverse Multi-Device Ecosystem

5G was inherently designed to first meet the higher end Enhanced Mobile Broadband (eMBB) targets of throughput and latency before considering the other design requirements like that of Ultra-Reliable Low-Latency Communication (uRLLC) and Massive Machine-Type Communication (mMTC). The subsequent customization of standards for lower end spectrum of device types like RedCap, IoT, etc. was based on the philosophy of ‘chopping’ the not needed functionality from a full blown standards. While this approach facilitated the addition of new device types, it resulted in complex standards due to hierarchically nested message and data structures. Additionally, designing features specifically for certain device types led to extensive changes across multiple specifications, increasing development and maintenance efforts. This complexity hinders the 'one protocol stack for all' model, making service or device adoption more challenging. The RRC specification has been growing rapidly with each release, primarily due to the ad-hoc addition of new capabilities and verticals or services.

Observation 1: 5G RRC design couldn't achieve the 'one protocol stack for all' model due to its complexity and ad-hoc customizations for diverse devices.

As we transition to 6G, the diverse use cases will require an equally diverse range of device types to deliver optimal experiences for end consumers. The 6G era will encompass a seamless spectrum of devices, from zero-power ambient IoT devices to miniaturized wearables and high-performance immersive XR devices. The primary objective will be to support these diverse devices with reduced development and maintenance efforts while ensuring faster time-to-market.

Modular Design for RRC

The RAN Control Plane protocol comprises of the essential functionality for the working of the cellular system from finding a suitable cell to camp on, to establishing and managing a radio connection, handing over to another cell during mobility and combating wireless channel fluctuations besides many other functionalities. This part of the RAN has been built on the preceding generations of cellular systems like 3G and 4G due to which it has become nothing short of a large monolith that comprises of highly intertwined and dependent functional modules as depicted in Figure 2. Introducing new functionalities or services in such design is complex and it further drifts the system towards an even more complex monolith eventually cascading to higher maintenance and development efforts. Additionally, defining subsets or supersets of functionalities for various device types from a single framework poses significant challenges, further escalating development efforts for different device categories

Figure 2: 5G RRC - Monolithic Control Plane

A critical issue with this architecture is the lack of modularity, which results in high interdependencies between modules. Consequently, adding new features or services often requires extensive modifications across multiple modules for example, when Multicast and RedCap feature have been introduced in RRC, it has impacted multiple modules and changes have been introduced in various messages in an ad-hoc manner as shown in Figure 3. This design approach has increased the complexity of the specification, further complicating the design and implementation process. Furthermore, the RRC specification continues to expand rapidly with each release, primarily due to the ad-hoc addition of new capabilities and verticals.

Observation 2: The monolithic design of RRC has led to the following observations:

  - Multiple modules are impacted when adding new features.

  - High interdependencies between RRC modules increase specification efforts and complexity.

  - The RRC specification has grown rapidly due to ad-hoc additions of capabilities and verticals.

Figure 3: Example of IEs introduced for Redcap and MBS feature

6G Control Plane can be redesigned to overcome the intertwined and highly interdependent functionality, leading to a cleaner design that aligns with the objective of a single base stack for multiple device types while lowering maintenance and development efforts. This modularity can be achieved through the Essential and Minimal Protocol Stack (EMPS), which serves as the foundation of the modular design. The EMPS is envisioned as a minimalist version within RRC (Radio Resource Control), where functionalities are reduced to support only the essential messages required for basic operation. It provides a common framework for all device types, including core functionalities such as system information, idle mode measurements, connection management, etc. making it applicable to various device types from 6G Day 1. The EMPS components are designed with reduced functionalities, focusing on essential operations, such as connection management may only support RRCSetup to establish SRB or low-rate DRB. The functionality of these core components can be extended based on specific requirements. However, these extended functionalities or services for different device types, such as CA, MBS, etc. are not inherently part of the EMPS components but can be added in a plug-and-play manner, as shown in Figure 4. To further enhance modularity, the control plane protocol design can adopt an easily extendable Radio Resource Control (RRC) ASN.1 structure. This will allow for easier integration of new functionalities and services without significant changes to the existing protocol stack. By combining the EMPS with an extendable RRC ASN.1 structure, 6G can ensure a balanced design that meets the performance, power, and memory constraints required for each device type, while maintaining scalability and maintainability.

Figure 4: Concept of RRC Modular design based on EMPS

Proposal 1: RAN2 should discuss and define the components of essential protocol stack functionality within the RRC applicable to all device types or services.

RRC ASN.1 structure

6G aims to address multiple device types and services from the outset, unlike 5G. During the RAN2# 131bis meeting, it was agreed that ASN.1 is used for encoding of RRC signalling for 6G air interface. The current 5G Radio Resource Control (RRC) is characterized by a deeply nested and highly complex ASN.1 structure, primarily driven by the need to accommodate diverse use cases and device types. This ASN.1 structure was not designed with a modular approach, leading to the ad-hoc addition of Information Elements (IEs) across multiple or same messages whenever new use cases, features, or device types are introduced as shown in figure 3 .Table 1 shows examples of some IEs where feature-related IEs are spread across multiple or the same message.

Table 1: New Vertical/Service Introduction – Idle Mode

The deeply hierarchical and nested nature of the RRC ASN.1 structure complicates the introduction of new services or device types, as it requires extensive modifications, development, and integration efforts. Additionally, the ad-hoc addition of parameters across different or same messages further exacerbates the issue, making the specifications difficult to understand and maintained. Furthermore, IEs related to specific verticals or device types are spread across multiple components in RRC, making it difficult to manage and organize this information effectively. Currently, UEs have to filter out relevant parameters based on their supported device type or services, which consumes high memory and increases processing time.

Observation 3: Multiple parameters across same or different messages in an ad-hoc manner have been introduced for new service or device types.

Observation 4: The deeply nested and hierarchical nature of the RRC ASN.1 structure complicates the introduction of new services or device types, requiring extensive modifications, development, and integration efforts.

To address these challenges, a more structured and modular approach to RRC design is essential. By considering multiple device types and services from the outset, 6G can ensure that the specifications are scalable, flexible, and efficient. Establishing clear guidelines for designing the RRC ASN.1 structure will reduce development and testing efforts while enhancing the overall maintainability of the specifications. Below are some possible guidelines or principles for designing the RRC structure that can help derive the solution:

- It should support multiple device types and services, ensuring compatibility across diverse use cases

- Device type should be considered a key factor in RRC design from a high-level perspective.

- Addition of IEs in RRC message should be under a structured containers which could be specific to device types or verticals, to enable easier access to information.

- Define IEs/fields for common purposes to ensure high signaling efficiency across all devices, services, and features.

- Ensure structured containers in RRC messages are well-defined and allow easy addition or removal for future updates

- The final ASN.1 structure should be decided considering synergy of RRC modularity and other new RRC enhancements

There are multiple ways to achieve these guidelines and address the issue. One possible approach is to enhance the existing 5G RRC structure by organizing information into containers based on verticals, features, services, or devices. Another option is to design a new RRC framework where parameters are categorized into common and feature/device/service-specific containers, or structured around device types, with features and services included in these device-specific containers. The final ASN.1 structure should balance RRC modularity with other new enhancements for a cohesive design.

Proposal 2: RAN2 should discuss and establish the guidelines or principles for designing the RRC structure to improve readability of ASN.1 for RRC signalling.

In addition to the challenges discussed, many complex RRC parameters such as Bandwidth Part (BWP), Cell-Specific Configuration, MIMO, antenna configuration, etc. are associated with Layer 1. This strong association highlights the importance of sharing clear guidelines for introducing new features and device types to ensure consistency and compatibility across the RAN Working Groups (WGs). Collaborative discussions with RAN1 are essential to integrate Layer 1 configurations into the RRC specification with a new modular approach, avoiding potential conflicts or inefficiencies. The same approach will apply to Layer 2 parameters depending on need and requirements.

Observation 5: Most of RRC parameters are RAN1 specific, so the clear guidelines should be sent to the RAN1 (i.e. how to define the RRC parameters and cross check between WIs).

Proposal 3: RAN2 should share agreed guidelines with RAN1 to maintain the consistency among various groups.

Handling of RRC Reconfiguration Failure

In 5G, when the UE receives an RRC reconfiguration message, it either successfully applies the configuration and sends an "RRC Reconfiguration Complete" message to the network or, initiates the RRC Connection Re-establishment procedure if it unable to comply with the configuration, effectively ending the reconfiguration process as shown in Figure 5a and 5b.

This RRC connection re-establishment procedure is resource-intensive for the UE and negatively impacts service continuity. Additionally, after completing the re-establishment procedure, the network may resend the same configuration to the UE. This occurs because the UE does not report the specific cause or the part of the configuration that triggered this procedure such behaviour is particularly problematic for latency-sensitive 6G use cases like XR/AR and AI, where seamless service continuity is critical. Therefore, there is a need to improve this procedure in 6G.

Observation 6: The 5G RRC reconfiguration process is resource-intensive and disrupts service continuity when the UE cannot comply with the network's configuration.

Observation 7: After RRC re-establishment, the network may resend the same configuration because it is unaware of the failure cause.

To address these challenges, the RRC reconfiguration procedure must be redesigned in 6G to handle failure scenarios more efficiently, minimizing interruptions to ongoing procedures. One potential solution is for the UE to communicate the exact cause of the failure to the network. This would enable the network to correct the configuration and resend it to the UE. The UE could report the failure cause via a new message, such as an "RRC Reconfiguration Failure" message or an existing message. However, a key challenge lies in determining the UE's behaviour in different cases, as it cannot ignore the trigger to initiate the re-establishment procedure during every failure instance. For example, certain configurations, such as mobility-related, may require a trigger of re-establishment procedure as a mandatory response. So, it is also important to study and discuss scenarios where the UE can revert to its previous configuration instead of initiating re-establishment.

Proposal 4: RAN2 should study how the UE can report the RRC Reconfiguration Failure message to the network in cases where the UE (partially) cannot comply with the configuration.

Delta Configuration for RRC

In the RAN2#131bis meeting, challenges associated with delta signalling were discussed, particularly its ambiguous nature, which often leads to implementation and error issues. The delta configuration is currently ambiguous due to the existing RRC ASN.1 structure and design. One approach to address this ambiguity is the introduction of a modular RRC structure in 6G, which could provide a more structured and efficient framework. Delta configuration could be effectively applied using this modular RRC structure proposed for 6G. RAN2 must first determine the modular RRC design and the restructuring of ASN.1. This is important because the delta signalling mechanism may be significantly impacted by the modular design and restructuring of the ASN.1. Once these foundational design decisions are made, RAN2 can then further explore and evaluate different approaches for delta configuration, such as common and dedicated configurations, parameter-level, etc.

Observation 8: The delta configuration is currently ambiguous due to the existing RRC ASN.1 structure.

Proposal 5: RAN2 need to consider how delta configuration is applied in 6G RRC based on new RRC structuring and design.

Conclusion

Based on the discussion, we made the following observations:

Observation 1: 5G RRC design couldn't achieve the 'one protocol stack for all' model due to its complexity and ad-hoc customizations for diverse devices.

Observation 2: The monolithic design of RRC has led to the following observations:

  - Multiple modules are impacted when adding new features.

  - High interdependencies between RRC modules increase specification efforts and complexity.

  - The RRC specification has grown rapidly due to ad-hoc additions of capabilities and verticals.

Observation 3: Multiple parameters across same or different messages in an ad-hoc manner have been introduced for new service or device types.

Observation 4: The deeply nested and hierarchical nature of the RRC ASN.1 structure complicates the introduction of new services or device types, requiring extensive modifications, development, and integration efforts.

Observation 5: Most of RRC parameters are RAN1 specific, so the clear guidelines should be sent to the RAN1 (i.e., how to define the RRC parameters and cross check between WIs).

Observation 6: The 5G RRC reconfiguration process is resource-intensive and disrupts service continuity when the UE cannot comply with the network's configuration.

Observation 7: After RRC re-establishment, the network may resend the same configuration because it is unaware of the failure cause.

Observation 8: The delta configuration is currently ambiguous due to the existing RRC ASN structure.

Based on the observations, RAN2 is requested to discuss and agree the following proposals:

Proposal 1: RAN2 should discuss and define the components of essential protocol stack functionality within the RRC applicable to all device types or services.

Proposal 2: RAN2 should discuss and establish the guidelines or principles for designing the RRC structure to improve readability of ASN.1 for RRC signalling.

Proposal 3: RAN2 should share agreed guidelines with RAN1 to maintain the consistency among various groups.

Proposal 4: RAN2 should study how the UE can report the RRC Reconfiguration Failure message to the network in cases where the UE (partially) cannot comply with the configuration.

Proposal 5: RAN2 need to consider how delta configuration is applied in 6G RRC based on new RRC structuring and design.