3GPP TSG-RAN2 #132	R2-2508878

# Dallas, USA, Nov. 17th - 21st, 2025

Agenda item:	10.3.1.1 (FS_6G_Radio)

Source:	LG Electronics

Title: 	Discussion on support for L4S in 6G radio protocol

Document for:	Discussion and decision

1.	Introduction

This document states our views on support for L4S in 6G radio protocol.

2.	Support for L4S in 6G radio protocol

In recent years, demand for communication services with high bandwidth and low latency has increased. Web conferencing, videoconferencing, and video telephony applications have been widely used. Furthermore, services providing users with an immersive experience, exemplified by extended reality (XR) services, cloud gaming, immersive real-time communication services, and digital twin system, have attracted much attention. In addition to the aforementioned applications, various applications which require high bandwidth and low latency such as V2X communication, edge computing, real-time multimedia streaming, remote medical service, etc. are also attracting growing interest. In 5G, enhancements for the support for XR and media services have been considered in NG-RAN and 5G systems.

For end-to-end traffic delivery and flow control, TCP is a dominantly used protocol, and it adopts a loss-based congestion control to control traffic flows. The loss-based congestion control adopted by the TCP has proven to work well for typical data transfer (web browsing, file transfer) and traffic with weak real-time requirements. However, it is not easy for a traditional TCP to provide reliable transmission with high bandwidth while maintaining low latency persistently. It is well-known that the TCP is not suitable to deliver traffic with hard real-time constraints (e.g., delay-sensitive traffic), such as traffic generated from videoconferencing applications.

The loss-based congestion control needs to induce a reasonably large queue in order to fully utilize data link capacity. For sizing buffers, the bandwidth-delay product is used as a rule of thumb in conjunction with loss-based congestion control. The bandwidth-delay product is the product of a data link's capacity and its round-trip time (RTT). Since a large queue may incur a long delay, it is difficult to provide high bandwidth sustainably for a communication service without considerable delay. In order words, it was believed that very low delay was only possible by limiting throughput.

To support communication services requiring high bandwidth and low latency like XR services, a new congestion control scheme, called as L4S (Low Latency, Low Loss, Scalable throughput), has been introduced. L4S aims to provide reliable real-time transmissions by minimizing queueing delay through the scalable transmission rate.

The L4S (Low Latency, Low Loss, Scalable throughput) enables real-time applications to always enjoy a low latency, no matter how busy the network is. It is expected that the L4S is widely used to serve time critical services such as XR services. For a radio network targeting to time critical services, it seems important to support L4S.

The L4S is intended to keep queuing delay consistently low while fully utilizing available capacity. To achieve the goal, the L4S sender reduces TCP congestion window proportionally to the level of congestion. The level of congestion is estimated from the fraction of bytes which encountered congestion. Explicit Congestion Notification (ECN) is used to indicate packets that encountered congestion. Each network node on the transmission path detects congestion and sets the congestion encountered (CE) codepoint in the IP header. The receiver echoes the congestion information back to the sender, using the ECN-Echo (ECE) flag in the TCP header. In the classic ECN, an ECN-marked packet was to be interpreted as a packet loss and used simply to detect that some congestion has occurred. The receiver reports one ECN mark per RTT if some congestion has happened. According to RFC3168, the receiver sets the ECE flag if any of the packets being acknowledged had the CE codepoint set. Then, the receiver continues to set the ECE flag until it receives a packet with the congestion window reduced (CWR) flag set in order to provide robust indication of the one mark against the possibility of a dropped ACK packet carrying an ECE flag. On the other hand, L4S extends the ECN processing to estimate congestion level. One possible solution is that the receiver sets the ECE flag in the ACK packet if and only if the CE codepoint was set in the packet being acknowledged. The packet associated with the ECE flag is considered as the packet that encountered congestion. The level of congestion is estimated from the fraction of bytes which encountered congestion. Legacy TCP behaviour and L4S behaviour are illustrated conceptually in Fig. 1.

Fig. 1 Conventional TCP congestion control vs. L4S congestion control

As is well known, with conventional TCP congestion control, the TCP congestion window is reduced sharply upon detecting congestion, while it is increased slowly during no congestion. Therefore, a legacy TCP flow shows a big saw-teeth pattern, and the average duration of a saw tooth is long. Each network node needs large buffer to absorb the arrival rate fluctuations due to the big saw-teeth pattern of legacy TCP flows. To keep high utilization, large buffer or high threshold for ECN marking may be required. The latency is then much increased due to the long queue and the queueing delay. To keep queue short and to provide low latency, the threshold for ECN marking needs to be set as a lower value. Then, the data link is under-utilized.

On the other hand, L4S congestion control estimates the extent of congestion and reduce the TCP congestion window accordingly. In other words, it reduces TCP congestion window proportionally to the level of congestion. For example, in case of a mild congestion, L4S is able to reduce the congestion window mildly according to its extent, while a conventional TCP congestion control will reduce the window too aggressively. Therefore, the saw-teeth pattern of an L4S flow is kept as tiny, and queues at network nodes are kept short. L4S can adapt transmission rate to fully utilizes available capacity while keeping queuing delay consistently low.

In addition, each node needs a separate buffer for L4S traffic to distinguish an L4S packet from a normal packet and to protect L4S traffic from being delayed due to normal traffic (e.g., file download). Packets with codepoint of Not-ECT and ECT(0) are treated as normal traffic, where Not-ECT and ECT(0) mean ‘Not ECN-capable transport’ and ‘Not L4S-capable transport’, respectively. Packets with codepoints ECT(1) and CE are treated as L4S traffic, where ECT(1) and CE mean ‘L4S-capable transport’ and ‘Congestion Experienced’, respectively.

Some L4S aspects are already included in 3GPP network, e.g. ECN marking, but there has been no consideration in radio protocol. The congestion and queuing delay at the radio interface between a gNB and a UE is affected not only by data traffic but also by radio environment. Therefore, it is likely that the congestion situation varies more rapidly on a radio interface than on a network interface in the network.

In 6G, communication services requiring high bandwidth and low latency like XR services are expected to contribute to more traffic in 6G networks. Therefore, it is important to handle TCP traffic efficiently to make L4S work well and provide high data rate with low latency. Then, it is worth to consider efficient mechanism to support L4S even in radio protocol.

Consider support of L4S in radio protocol.

For example, the congestion information is echoed back to the sender, using the ECN-Echo (ECE) flag in the TCP ACK packet, by the receiver. If the ECE flag is delayed, the rate control in the sender is delayed, which may lead to packet loss, and it will degrade overall TCP throughput. Thus, it needs to prioritize TCP ACK packets including ECE flag as shown in Fig. 2.

Fig. 2. TCP ACK prioritization for L4S

Prioritize L4S feedback transmission (TCP ACK prioritization).

In addition, it also needs to be considered to have a separate L2 queue for L4S traffic and non-L4S traffic. The dual-Q is essential element for L4S, because, if they are mixed together into a single queue, the L4S operates as classic TCP, and the L4S gain disappears. Thus, RFC9332 suggests to have a separate queue as shown in Fig. 3.

In the network side, e.g. in UPF-gNB interface, the separation of L4S traffic and non-L4S traffic is possible by allocating them into different QoS flows. However, in radio interface, even if they are mapped to different DRBs, they can be multiplexed into a single MAC PDU by LCP procedure, and separation of two flows cannot be achieved, which may lead to decrease of L4S gain. Therefore, it should also be considered that L4S traffic and non-L4S traffic are separated in whole L2 protocols, e.g. separate logical channel, separate MAC PDU, and/or separate HARQ process.

Fig. 3 Dual-Q AQM for L4S (RFC9332)

Study how to separate L4S traffic from non-L4S traffic in L2 protocol layers.

Lastly, Active Queue Management (AQM) may also be considered in L2 protocol. Up to 5G, L2 buffer does not have a special mechanism to handle buffer overflow other than “tail drop”. The tail drop mechanism is nothing but discarding a newly received packet if the buffer is full. Although PDCP has another mechanism to discard PDCP SDU by discard timer, it does not smartly control the L2 queue, and the “tail drop” situation would occur if large amount of data is received at the same time.

There are well-known problems in “tail drop” mechanism, e.g. bufferbloat, TCP global synchronization, and frequent RTO (Retransmission Time Out) in TCP application. To overcome such problems, sophisticated queue management mechanisms are developed, e.g. RED (Random Early Detection), CoDel (Controlled Delay) in IETF.

To efficiently support L4S in 6G, a new mechanism similar to RED or CoDel can be considered in L2 buffer. In addition, ECN marking may also be considered in L2 header to correctly reflect the radio interface situation.

Consider support of active queue management (AQM) for L2 buffer.

3. Conclusion

Based on the above discussions, we present the following proposals:

- Consider support of L4S in radio protocol.

- Prioritize L4S feedback transmission (TCP ACK prioritization).

- Study how to separate L4S traffic from non-L4S traffic in L2 protocol layers.

- Consider support of active queue management (AQM) for L2 buffer.