3GPP TSG-RAN2 #132	   						        R2-250884

# Dallas, USA, 17th - 21st November 2025

Agenda Item:	10.3.3.1

Source:  	LG Uplus

Title: 	Considerations on AI/ML framework

Document for:	Discussion

# Introduction

In 6GR SID [1], there are the following objectives related to AI/ML framework:

(8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g. performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g. MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this contribution, we discuss the considerations on AI/ML framework in 6G.

# Discussion

6G should be designed to extend the existing 5G AI/ML framework while effectively supporting various AI/ML use cases. This contribution discusses the considerations for supporting AI/ML in 6G, focusing on efficient resource utilization and effective management of AI/ML.

In 5G Rel-19, a dedicated AI/ML processing unit (APU) was introduced to handle AI/ML-related features. Currently, the APU is utilized primarily for real-time inference in CSI-related features. However, 6G is expected to accommodate more diverse and advanced AI/ML use cases, leading to an increased demand for APU resources to support these applications. As the number of use cases expands, it becomes critical to discuss efficient methods for AI/ML processing.

One possible approach is to operate APUs separately for each use case, simplifying network management by separating APU resources for each case. However, as the number of supported use cases rises, this strategy may result in resource constraints and inefficiencies in utilizing APU capacity. Alternatively, a shared APU resource pool across multiple use cases enables efficient resource utilization while supporting diversified AI/ML applications. However, sharing resources introduces additional management complexities, such as handling priorities and resource allocation across use cases.

To maximize resource efficiency and address management overhead, we propose that discussions on these aspects begin in the early design phase of the 6G AI/ML framework. This will enable optimal solutions for UE-level AI/ML processing to emerge as the framework evolves.

Proposal 1: In 6G AI/ML framework, efficient utilization of limited APU resources for UE AI/ML processing should be discussed.

In addition, the specific AI/ML functionalities and use cases in each situation are likely to vary depending on network conditions and UE states. Flexibility in deploying AI/ML functionalities is thus needed to address diverse requirements across scenarios.

In 5G Rel-19, the Applicable Report enables UEs to report whether they are in an applicable or inapplicable state for AI/ML functionality. For 6G, however, the diversity of AI/ML use cases combined with limited APU resources calls for an extension of this functionality. An enhanced reporting mechanism should include additional information such as the status of APU resources, priority among multiple use cases, and so on. This enhanced report will allow the network to better manage UE AI/ML operations according to varying situations and use case requirements.

Proposal 2: Improved reporting mechanisms are needed to support various AI/ML use cases effectively, including extensions to the Applicable Report.

# Conclusion

In this contribution, we provide our views on 6G AI/ML framework with following observations and proposal.

Proposal 1: In 6G AI/ML framework, efficient utilization of limited APU resources for UE AI/ML processing should be discussed.

Proposal 2: Improved reporting mechanisms are needed to support various AI/ML use cases effectively, including extensions to the Applicable Report.

# References

- RP-251881 New SID: Study on 6G Radio