3GPP TSG-RAN WG2 Meeting #132		R2-2508890

Dallas, USA, 17th - 21st Nov. 2025

Source:	TCL

Title:	Discussion on UP Functionality for 6G RAN

Agenda Item:	10.3.1.1

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis meeting, the consensus on the user plane functionalities was limited. The related agreements are listed in what follows.

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

In the next chapter, the detailed analyses and views are provided.

- Discussion

In the 5G user plane process, there are indeed some redundant processes. For example,

- Re-transmission process: involving the HARQ process at the MAC layer, the ARQ process at the RLC layer, and the retransmission mechanism at the PDCP layer.

- Re-ordering process: involving the reassembly process at the RLC layer, and the re-ordering process at the PDCP layer on the receiving side.

- Feedback process: involving HARQ feedback and ARQ reception status report feedback.

These processes are essentially doing similar things, but because they reside at different protocol layers, they are indispensable in the current 5G architecture.

Observation 1: In the 5G user plane process, some processes have overlapping functionalities, but because they are in different protocol layers, they remain indispensable in the current 5G architecture.

Although the above processes are indispensable in 5G, those with overlapping functionalities do introduce additional processing overhead and latency to the entire user plane. Therefore, in the 6G user plane design, it is necessary to first identify all processes that have overlapping functionalities, and then analyze which processes can be combined, thereby further improving the overall processing efficiency of the user plane.

Proposal 1: In the 6G user plane design, all processes with overlapping functionalities should first be identified, and based on this analysis determine which processes can be merged, in order to further improve the overall processing efficiency of the user plane.

From the existing consensus, it can be seen that in future 6G services, AI services will undoubtedly become the most promising and important new services for 6G. Among them, AI services can be divided into two categories: one category is primarily used to enhance network performance (e.g., network optimization, signal processing enhancement, mobility prediction, etc.), and the other is mainly to provide services for the wide access of AI entities in the future (e.g., large-scale interactions among AI entities, token communication, etc.). Because the scope of AI services is very broad, AI services in different scenarios have introduced many new transmission requirements for 6G, which are not limited to 5G’s eMBB, URLLC and mMTC scenarios.

Observation 2: AI services in different scenarios pose many new transmission requirements for 6G, which are not limited to 5G’s eMBB, URLLC and mMTC scenarios.

In future 6G networks, there will be UEs with a variety of capabilities (e.g., UEs supporting high-bandwidth transmission, UEs with low power consumption requirements, fixed wireless access nodes, etc.). Therefore, considering the different service types in 6G and the need to support different UE types, the user plane design needs to be more targeted compared with 5G. For example, it can provide differentiated user plane processing frameworks for specific service requirements or specific UE types, thus supporting different service needs and UE capability requirements.

Proposal 2: The user plane design needs to be more targeted compared with 5G; for example, it can provide differentiated user plane processing frameworks for specific service requirements or specific UE types, thereby supporting different service needs and UE capabilities.

Although the consensus reached in the last meeting was limited, from the views in the documents of different companies, some similar views can be sorted in the following manner.

- Most of user plane functionalities in 5G can be reused in 6G user plane.

- The user plane functionalities may be required to be re-allocated.

- Some redundant functionalities need to be removed or combined.

- PDCP Concatenation is required to reduce the overhead of small data.

We also agree with the above four viewpoints.

Proposal 3: We support the following viewpoints:

1)	Most of user plane functionalities in 5G can be reused in 6G user plane.

2)	The user plane functionalities may be required to be re-allocated.

3)	Some redundant functionalities need to be removed or combined.

4)	PDCP Concatenation is required to reduce the overhead of small data.

At the last meeting, some companies suggested that in the 6G user plane design, the SDAP layer should be removed or its functionality merged into the PDCP layer. This idea is good and can further simplify the user plane process. However, the following issue needs to be resolved:

- In the 5G user plane, the SDAP and PDCP entities have a one-to-mlutiple mapping relationship. If the SDAP layer’s functionality is integrated into the PDCP layer, how can the one-to-mlutiple QoS flow mapping issue be resolved?

Proposal 4: Regarding the suggestion to remove the SDAP layer or integrate its functionality into the PDCP layer, the one-to-mlutiple QoS flow mapping issue needs to be resolved.

- Conclusion

In this contribution, we discuss the user plane functionality for 6G radio access technology. Based on the discussion, we have the following observations and proposals.

Observation 1: In the 5G user plane process, some processes have overlapping functionalities, but because they are in different protocol layers, they remain indispensable in the current 5G architecture.

Proposal 1: In the 6G user plane design, all processes with overlapping functionalities should first be identified, and based on this analysis determine which processes can be merged, in order to further improve the overall processing efficiency of the user plane.

Observation 2: AI services in different scenarios pose many new transmission requirements for 6G, which are not limited to 5G’s eMBB, URLLC and mMTC scenarios.

Proposal 2: The user plane design needs to be more targeted compared with 5G; for example, it can provide differentiated user plane processing frameworks for specific service requirements or specific UE types, thereby supporting different service needs and UE capabilities.

Proposal 3: We support the following viewpoints:

1)	Most of user plane functionalities in 5G can be reused in 6G user plane.

2)	The user plane functionalities may be required to be re-allocated.

3)	Some redundant functionalities need to be removed or combined.

4)	PDCP Concatenation is required to reduce the overhead of small data.

Proposal 4: Regarding the suggestion to remove the SDAP layer or integrate its functionality into the PDCP layer, the one-to-mlutiple QoS flow mapping issue needs to be resolved.