3GPP RAN WG2 Meeting #131bis	R2-2508902

Dallas, United States, November 17th – 21st 2025

Agenda Item:	8.1.2

Source:	InterDigital, LG, OPPO, Apple, Nokia, Xiaomi, Samsung, vivo

Title:		Handling of inapplicable periodic CSI inference configuration

Document for:	Discussion, Decision

# Introduction

UE behaviour is currently ambiguous in several aspects related to inapplicable periodic CSI inference configurations. This contribution discusses these remaining issues.

# Handling of periodic CSI inference configuration

In legacy operation, UE submits periodic CSI report configuration to lower layers upon reception by RRC layer. In RAN2#131, it was noted that if the configuration is for inference and is inapplicable, any corresponding results reporting by the UE are essentially ‘garbage’. An LS was sent to RAN1 in [1] to confirm the issue and to determine which of the following is the preferred UE behaviour:

- Option 1: Upon reception of RRC Reconfiguration message, UE’s RRC layer immediately submits inference configuration of periodic CSI to lower layer, regardless of whether the configuration is applicable/inapplicable.

- Option 2: Upon reception of RRC Reconfiguration message, UE’s RRC layer submits inference configuration of periodic CSI to lower layer only if it is reported as applicable in RRCReconfigurationComplete.

RAN1 has subsequently indicated in [2] that Option 2 resolves the issue without further impact to RAN1 specification. As there is no critical issue from RAN2 side it is suggested that Option 2 be confirmed as the intended UE behaviour if a periodic CSI inference configuration is inapplicable.

Proposal 1:	RAN2 confirms upon reception of RRC Reconfiguration message, UE’s RRC layer submits inference configuration of periodic CSI to lower layer only if it is reported as applicable in RRCReconfigurationComplete.

However, if the periodic CSI inference configuration becomes applicable before being released by the network, there are two options:

- Option A: UE reports change of applicability via UAI and autonomously ‘activates’ the periodic CSI inference configuration by submitting the stored configuration to lower layers.

- Option B: UE reports change of applicability via UAI, and periodic CSI inference configuration is ‘activated’ by receiving a subsequent reconfiguration by the network (i.e., no autonomous activation)

Option B is the preferred UE behaviour for the following reasons:

- It is aligned with legacy which does not support UE automous activation of periodic CSI configurations

- There is no need to optimize for a corner case. In most cases it is expected that the network will deconfigure the UE upon being notified the configuration is inapplicable.

- There is already existing mechanisms such as semi-persistent CSI configurations which can largely duplicate this functionality.

It is therefore proposed that RAN2 agree to Option B:

Proposal 2:	If a periodic CSI inference configuration previously reported as ‘inapplicable’ changes to ‘applicable’, is not autonomously activated (i.e., the stored configuration is not submitted to lower layers). UE reports the change of applicability at least via UAI and network may reconfigure the UE to ‘activate’ the periodic CSI inference configuration.

However, the following agreement was made in RAN2#131 related to reporting applicability change of periodic CSI inference configuration from 'inapplicable' to applicable' in RRC ReconfigurationComplete:

Agreements on applicability/inapplicability 1 RRCReconfigurationComplete shall include applicability/inapplicability status for: a) All inference configurations included in the immediately preceding RRCReconfiguration message, and b) Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report. [CB on how the UE handles previously configured periodic CSI config that becomes applicable]

Considering RAN1 input (and if Proposal 1 is agreed), a UE should not submit an inapplicable periodic CSI inference configuration to lower layers. Combining the RAN1 understanding with the above RAN2 agreement yields the following UE behaviour:

- Option 1: UE can report applicability change via RRCReconfigurationComplete, but UE can’t autonomously activate periodic CSI report configuration received in previous RRCReconfiguration (i.e., UE does not submit the configuration to lower layers).

Which results in the following UE behaviour:

- For Initial  RRCReconfiguration,  UE will activate periodic CSI resource immediately for applicable full inference configuration while not activate periodic CSI resource by holding the configuration in RRC layer for inapplicable full inference configuration.

- For subsequent RRCReconfiguration,  UE will activate periodic CSI resource immediately for newly added applicable full inference configuration while not activate periodic CSI resource by holding the configuration in RRC layer for newly added inapplicable full inference configuration. More addition, UE will not activate periodic CSI resource by holding the configuration in RRC layer for previously stored full inference configuration which status is from inapplicable to applicable.

Some may interpret the above as complex UE behaviour. Other alternatives to enable a more simplified UE behaviour could be the following:

- Option 2: RRC ReconfigurationComplete can include the applicability status for any previously configured inference configurations except previously configured periodic CSI config for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report

  - Pros:

    - This eliminates the issue of confusing UE behaviour

    - Does not reverse previous agreement and allows UE to use RRCReconfigurationComplete to report changes in applicability for all other types of configurations.

    - UE can still use UAI to provide network updates on configuration

  - Cons:

    - more complicated UE behaviour than simply releasing the configuration

- Option 3: When a UE receives a periodic CSI configuration, and it is inapplicable, the periodic configuration is not reported to the lower layers (compliant with RAN1’s reply) and the UE releases it (compliant with the legacy principle that a periodic CSI report configuration cannot be “inactive”). The NW finds out about it by receiving the status “inapplicable” in the applicability report.

  - Pros:

    - Very simple UE behaviour

    - Eliminates the problem use case for using RRCReconfigurationComplete to report changes in applicability

  - Cons:

    - UE can't store periodic CSI config for later if not applicable.

Finally, there is always the option that the previous agreement is reverted, and RRCReconfigurationComplete does not include any applicability status for any previously configured inference configurations, and any change is only reported via UAI:

- Option 4: RRCReconfigurationComplete does not include the applicability status for any previously configured inference configurations, i.e., changes of applicability for all CSI inference configuration(s) are only transmitted via UAI

It is therefore proposed that RAN2 discuss the following options related to reporting applicability change of periodic CSI inference configuration from 'inapplicable' to applicable' in RRC ReconfigurationComplete:

Proposal 3:	For reporting applicability change of CSI inference configuration from 'inapplicable' to applicable' in RRCReconfigurationComplete, consider at least the following options:

- Option 1: UE can report applicability change via RRCReconfigurationComplete, but UE can’t autonomously activate periodic CSI report configuration received in previous RRCReconfiguration (i.e., UE does not submit the configuration to lower layers).

- Option 2: RRCReconfigurationComplete can include the applicability status for any previously configured inference configurations, except previously configured periodic CSI config, for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report  (i.e., changes of applicability for periodic CSI inference configuration are only transmitted via UAI).

- Option 3: When a UE receives a periodic CSI configuration, and it is inapplicable, the periodic configuration is not reported to the lower layers and the UE autonomously releases it. NW is notified of release via 'inapplicable' status in applicability report.

- Option 4: RRCReconfigurationComplete does not include the applicability status for any previously configured inference configurations, i.e., changes of applicability for all CSI inference configuration(s) are only transmitted via UAI

# Conclusion

In this contribution the following proposals are made regarding handling of inapplicable periodic CSI inference configuration:

Proposal 1:	RAN2 confirms upon reception of RRC Reconfiguration message, UE’s RRC layer submits inference configuration of periodic CSI to lower layer only if it is reported as applicable in RRCReconfigurationComplete.

Proposal 2:	If a periodic CSI inference configuration previously reported as ‘inapplicable’ changes to ‘applicable’, is not autonomously activated (i.e., the stored configuration is not submitted to lower layers). UE reports the change of applicability at least via UAI and network may reconfigure the UE to ‘activate’ the periodic CSI inference configuration.

Proposal 3:	For reporting applicability change of CSI inference configuration from 'inapplicable' to applicable' in RRCReconfigurationComplete, consider at least the following options:

- Option 1: UE can report applicability change via RRCReconfigurationComplete, but UE can’t autonomously activate periodic CSI report configuration received in previous RRCReconfiguration (i.e., UE does not submit the configuration to lower layers).

- Option 2: RRCReconfigurationComplete can include the applicability status for any previously configured inference configurations, except previously configured periodic CSI config, for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report  (i.e., changes of applicability for periodic CSI inference configuration are only transmitted via UAI).

- Option 3: When a UE receives a periodic CSI configuration, and it is inapplicable, the periodic configuration is not reported to the lower layers and the UE autonomously releases it. NW is notified of release via 'inapplicable' status in applicability report.

- Option 4:RRCReconfigurationComplete does not include the applicability status for any previously configured inference configurations, i.e., changes of applicability for all CSI inference configuration(s) are only transmitted via UAI

# References

R2-2506519 – LS on when RRC layer submits periodic CSI inference configuration to lower layer (Apple)

R1-2508122 – LS reply on RAN2 LS on periodic CSI inference configuration to lower layer (Samsung)