3GPP TSG-RAN WG2 Meeting #132		R2-2508904

Dallas, USA, 17th - 21st Nov. 2025

Source:	TCL

Title:	Discussion on Re-transmissions for 6G RAN

Agenda Item:	10.3.1.3

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis meeting, a lot of views related to the enhancement of re-transmissions, i.e., HARQ, ARQ, PDCP re-transmission, have been proposed by many companies. To be specific, the related views can be mainly divided into the following parts:

- Reducing the latency of retransmission, especially for ARQ process in RLC layer.

- Re-designing the re-transmission process for the unified design of RLC and PDCP process.

- Re-allocating or customizing the UP functionalities for 6G RAN.

- Combining the HARQ and ARQ into one layer.

- Pre-submitting partial decoded data to the upper layer for the purpose of higher efficiency.

In the next chapter, the detailed analyses and views are provided.

- Discussion

In 5G, reliable user-plane transmission is mainly ensured by the HARQ mechanism at the MAC layer and the ARQ mechanism at the RLC layer, with the PDCP layer also providing a certain degree of reliability assurance. Specifically, the MAC HARQ mechanism provides fast reliability through immediate feedback and retransmissions. However, considering that HARQ feedback can be misdetected (e.g., an ACK misdetected as a NACK, or vice versa) and that HARQ may still fail to succeed after a number of consecutive retransmissions when the channel quality remains low, the reliability provided by the HARQ process is limited.

To ensure high transmission reliability, the user plane employs the RLC ARQ mechanism as a fallback to cover potential HARQ transmission errors, thereby greatly improving overall reliability. The trade-off is that the ARQ mechanism incurs relatively large feedback and retransmission delays, primarily because ARQ feedback and retransmissions are triggered by higher-layer timers or signaling. This means that while 5G can readily satisfy data transmissions requiring either ultra-low latency or ultra-high reliability, when faced with demands for both simultaneously, the existing retransmission mechanisms become a bottleneck.

Observation 1: Existing 5G retransmission mechanisms can ensure reliability, but still cannot simultaneously meet the requirements of ultra-low latency and ultra-high reliability.

Considering that the current HARQ and ARQ mechanisms operate independently (i.e., there is no interaction between HARQ and ARQ in 5G), one approach to reduce overall retransmission delay is to enhance the coordination between HARQ and ARQ. For example, in 6G an HARQ NACK could proactively trigger an ARQ retransmission at the sender side, avoiding the need for the sender to wait for a receiver’s status report before retransmitting. However, since this coordination does not involve the air interface and is an internal device optimization, it can be left to implementation without requiring additional standardization.

Proposal 1: In 6G, the transmitter can use HARQ NACK feedback to proactively trigger ARQ retransmissions, thereby reducing overall retransmission delay. Since this method does not involve over-the-air transmission (being an internal device optimization), it can be left to device implementation without additional standardization work.

In the 5G user‑plane processing, there are overlaps in functionality across different protocol layers. For example, MAC‑layer HARQ and RLC‑layer ARQ both provide retransmission; the RLC reassembly procedure and the PDCP reordering procedure both perform ordering; and both MAC HARQ and RLC ARQ require the receiver to send success/failure (ACK/NACK) feedback to the transmitter. Although these functions complement each other in 5G, their similarity also introduces redundancy into user‑plane processing, increasing processing overhead and reducing processing efficiency.

Observation 2: In the 5G user‑plane processing, there are overlapping functions among different protocol layers. The similarity of these functions leads to redundancy in the user‑plane processing, increasing processing overhead and reducing processing efficiency.

Considering that some functions at PDCP layer and RLC layer are similar, merging the functionalities of the two layers into a single layer may be a promising direction, for example, the RLC and PDCP use the same sequence number. This would accelerate processing efficiency and reduce over-the-air header overhead. However, if the ARQ functionality is also merged into the PDCP layer under this design, it is necessary to consider whether the RLC ARQ functionality and the PDCP retransmission mechanism should be merged as well. If they are merged, how would they be merged? If they are not merged, how should they operate together?

Proposal 2: If the RLC and PDCP layer functions are merged, consider whether the RLC ARQ functionality and the PDCP retransmission mechanism should also be merged. If merging is pursued, the method of merging needs to be defined; if not, the two should be coordinated in their operation.

In the 5G retransmission architecture, the MAC-layer HARQ mechanism provides reliability at the transport block level, whereas the RLC-layer ARQ mechanism provides reliability at the logical channel or radio bearer level. As a result, the RLC ARQ mechanism can directly cater to the transmission requirements of different radio bearers, while the HARQ mechanism operates at a coarser granularity and cannot flexibly adapt to the transmission needs of different data flows.

Observation 3: The RLC ARQ mechanism can directly address the transmission needs of different radio bearers, whereas the HARQ mechanism’s granularity is too coarse to flexibly meet the requirements of different data flows.

In a 5G enhancement, a HARQ disabling mechanism was introduced, which to some extent gave HARQ a finer-granularity service awareness; however, its dynamism and flexibility were still limited. To further improve user-plane retransmission efficiency, one consideration for 6G is to incorporate awareness of different radio bearers, and even the transmission requirements of different flows, into the HARQ process. This could further improve retransmission efficiency and reduce the waiting time for feedback.

Proposal 3: In 6G, it is proposed to study how to incorporate per-bearer and even per-flow transmission requirement awareness into the HARQ process, in order to further improve retransmission efficiency and reduce feedback waiting time.

- Conclusion

In this contribution, we discuss the re-transmission mechanism for 6G radio access technology. Based on the discussion, we have the following observations and proposals.

Observation 1: Existing 5G retransmission mechanisms can ensure reliability, but still cannot simultaneously meet the requirements of ultra-low latency and ultra-high reliability.

Proposal 1: In 6G, the transmitter can use HARQ NACK feedback to proactively trigger ARQ retransmissions, thereby reducing overall retransmission delay. Since this method does not involve over-the-air transmission (being an internal device optimization), it can be left to device implementation without additional standardization work.

Observation 2: In the 5G user‑plane processing, there are overlapping functions among different protocol layers. The similarity of these functions leads to redundancy in the user‑plane processing, increasing processing overhead and reducing processing efficiency.

Proposal 2: If the RLC and PDCP layer functions are merged, consider whether the RLC ARQ functionality and the PDCP retransmission mechanism should also be merged. If merging is pursued, the method of merging needs to be defined; if not, the two should be coordinated in their operation.

Observation 3: The RLC ARQ mechanism can directly address the transmission needs of different radio bearers, whereas the HARQ mechanism’s granularity is too coarse to flexibly meet the requirements of different data flows.

Proposal 3: In 6G, it is proposed to study how to incorporate per-bearer and even per-flow transmission requirement awareness into the HARQ process, in order to further improve retransmission efficiency and reduce feedback waiting time.