3GPP TSG-RAN WG2 Meeting #132	R2-2508940

Dallas, USA,  Nov. 17th – 21st, 2025

Agenda item:	10.3.3.2

Source:	Lenovo

Title:	Discussion on energy efficiency for network and UE

WID/SID:	Study on 6G Radio - Release 20

Document for:	Discussion and Decision

# 1	Introduction

During the discussion on 6GR in RAN2#131bis, Network and UE Energy Efficiency was widely discussed. And the design goal was agreed [1].

Design goal

1. 	Strive to develop solutions that consider network energy efficiency and UE power saving

Also, the direction to be discussed was achieved [1].

For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose.   Focus on solutions that have RAN2 impacts.

In this document, we discuss the related issues.

# 2	Discussion

In NR, both NES (Network side energy saving) and UE power optimizations were focused on different feature designs. NES features focus on reducing gNB energy consumption during low traffic by enabling mechanisms like on-demand SSB/SIB1 transmission, Cell Discontinuous Transmission and Reception (Cell DTRX), common channel adaptation, SSB-less transmission at the gNB etc. For UEs, power saving is achieved through mechanisms such as extended and connected-mode DRX, DRX/ eDRX for RRC Idle, bandwidth part configuration, wake-up signals, including Low Power WUS, SCell (de-)activation, PDCCH-switching (for CONNECTED) and reduced/relaxed mobility monitoring.

However, some of these features disadvantage the ‘other’ side. For instance, Cell DTRX is complex for UE implementation, on-demand SSB/SIB1 transmission increases UE power consumption, latency etc. On the other hand, “WUS” based features cannot be really zero cost for the network. And lack of backward compatibility of these features prevents mass adoption and economy of scale benefits.

Below we list candidates for the Network and UE power saving features and suggest a 6G shortlisting. A list could be:

UE side: LP-WUS, C-DRX, SCell (de-)activation and PDCCH-switching (for CONNECTED) AND DRX/ eDRX for RRC Idle

Network side:  Cell DTRX, on-demand SI, common channel adaptation, SSB-less transmission.

From the UE side, we assume LP-WUS and PEI based power reduction optimization overlap partially for paging false alarm related optimization. Of course, LP-WUS has been extended to cover also RRC Connected and measurement optimization so, we selected LP-WUS over PEI subgrouping.

From the network side features, Cell DTRX needs further consideration as this can likely be beneficial only when there are really a few RRC Connected UEs in the cell and it might instead be possible to switch off the cell and the UEs moved over to the neighbours – business as usual for network/ operator specific solution. If the number of RRC Connected UEs is moderate/ high or some UEs really have latency sensitive QoS, the Cell DTRX may not allow much sleep opportunity for the network. Also, this is one of the complex energy saving features.

So, based on the above first assessment, we propose:

Observation 1a: LP-WUS and PEI based power reduction optimization overlap partially for paging false alarm related optimization. LP-WUS has been extended to cover also RRC Connected and measurement optimization.

Observation 1b: Cell DTRX may not allow much sleep opportunity for the network if many RRC Connected UEs are in the cell. When the number of RRC Connected UEs is low, it might instead be possible to switch off the cell without requiring any specification.

Proposal 1: Following features are shortlisted for further study/ shortlisting towards 6G Day1 adoption:

- UE side: LP-WUS, C-DRX, SCell (de-)activation and PDCCH-switching (for CONNECTED) AND DRX/ eDRX for RRC Idle

- Network side:  On-demand SI, common channel adaptation, SSB-less transmission

Let’s go in detail on a couple of these technologies:

## LP-WUS

From UE side, among other factors like measurement/ reporting, the power consumption depends on the configured length of wake-up periods of UE, reducing the UE wake-up time to the extent of meeting QoS is the main orientation for UE power saving. In 5G, multiple features have been introduced to address the issue from PHY to MAC layer, for example, PEI has been introduced in Rel-17, while LP-WUS has been studied in Rel-18 and further specified in Rel-19, both the features aim to reducing the paging monitoring for power saving. According to the discussion of last RAN2 meeting [1], RAN2 should identify what power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purposes.

Among those features, LP-WUS has shown significant power saving benefits, which contributed to UE power saving for both RRC states. As indicated by NR LP-WUS objective, for RRC_IDLE/INACTIVE states, it’s observed that significant UE power saving gain (about 90%) is obtained by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation; For RRC_CONNECTED state, it’s observed, that moderate UE power saving gain (about 10%) is obtained with marginal impacts to cell capacity by using LP-WUS/WUR to trigger UE MR PDCCH monitoring compared with existing UE power saving techniques. However, since LP-WUS is defined in later releases of NR, and most of the power saving gain has been limited due to the late deployment and backward compatibility issue. Learning from the 5G, to maximize the UE power saving benefit for 6G, it is suggested RAN2 to study LP-WUS/WUR as one of UE power saving techniques for all RRC states in the first release of 6GR study, Specifically, following aspects in details can be considered to support LP-WUS in 6GR, and RAN2 to coordinate with RAN1 closely to avoid the overlapping discussion.

- In NR design, the LP-WUS/WUR is used for paging reception in RRC _INACTIVE/IDLE state，which greatly contributed to the UE power saving. 6GR is suggested to support LP-WUS/WUR for paging reception as day1 feature, at least taking 5G NR design as baseline.

- In NR design, power saving in RRC_INACTIVE/RRC_IDLE state can be achieved by allowing UE supporting LP-WUS to relax serving cell measurements on NR, further relax neighbouring cell measurement on MR and/or offload serving cell measurements from MR to LR. Since RAN1 is also considering enhancing the coverage of DL-WUS in 6GR design, then further offloading from MR to LR, e.g., neighbouring cell measurement, etc. may be studied from RAN2 perspective.

- For UEs in RRC_CONNECTED state, currently UE MR ultra-deep sleep is not considered in 5G NR LP-WUS, UE RRM/RLM/BFD/CSI measurements are performed by MR, the connected state optimizations are lower prioritized than idle/inactive state optimizations. To align with the DL WUS signal design from RAN1 for 6GR EE improvement, and with close coordination with RAN4, RAN2 may study using LR for e.g., RRM/RLM/BFD measurements in RRC_CONNECTED mode, especially on the criterion design for the LR measurements in RRC_CONNECTED mode.

Proposal 2: RAN2 to study LP-WUS/WUR as one of candidate UE power saving techniques for all RRC states in 6GR, including:

- To support LP-WUS/WUR for paging reception as a candidate for day1 feature, taking 5G NR design as baseline.

- To study further RRM measurement relaxation/offloading on LR, e.g., for neighboring cell measurement in RRC_INACTVIVE/IDLE state, etc.

- To study using LR for e.g., RRM/RLM/BFD measurements in RRC_CONNECTED mode.

## C-DRX

As mentioned above several UE power saving schemes have been introduced over the course of the different releases in 5G NR. However, all these mechanisms have a rather static behaviour based on NW configuration once provided to the UE. DRX in NR operates based on several timers, e.g. drx-onDurationTimer, drx-InactivityTimer, drx-HARQ_RTT_timer, and drx-HARQ Retransmission timer. Such timer values are as mentioned fixed once they are configured. In 5G NR specifications allow the NW to configure only one DRX configuration for a serving cell and, therefore, this single DRX/timer configuration cannot really match certain traffic characteristics or even multiple flows/services which are running in parallel, e.g. immersive services with multi-modalities. Thus, it cannot closely follow the potential short sleep opportunities and result in either low power saving gains, or in an increased traffic delay.

In Rel-18/19 there were proposals to study solutions with multiple active DRX configurations, e.g. each of them aligned with a different traffic flow/service. The motivation is that a single DRX configuration is not efficient from power saving perspective since traffic characteristics are too different and can’t be “tracked/covered” by a single DRX configuration.

When there are multiple data flows, especially when at least integer relationship does not exist among the periodicities of the different data flows, there might be the risk that packet arrival of some data flows arrival occurs in the DRX-Off duration, e.g., UE not monitoring PDCCH which would in turn lead to some additional delay. Having multiple simultaneous DRX configurations could help schedule transmissions corresponding to different traffic characteristics in a timely manner, e.g., by providing on-durations on several occasions according to expected traffic arrivals.

Another option would be to make the DRX scheme more adaptive and flexible compared to 5G NR, e.g. enabling some more dynamic adjustment of the DRX related timer values. DCI-based switching between DRX configurations was one solution discussed already for Rel-18/19. It refers to the case that NW dynamically activates/re-activates/deactivates DRX configurations, which allows for a dynamic adaptation of already activated DRX configurations or activation of additional DRX configuration. It could be also studied whether UE may also determine some DRX-related timer values.

In our views, RAN2 should discuss how C-DRX can be enhanced in 6G to realize UE power saving gain in a more intelligent manner.

Proposal 3: RAN2 to study enhancements to the C-DRX scheme in 6G, e.g. support of multiple active DRX configurations or adaptive DRX operation, to improve UE power efficiency.

## Energy efficiency as a KPI for 6G

Energy efficiency is specified as a KPI for 6G. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving [in relation to the traffic capacity provided]. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving [in relation to the traffic characteristics] [3]. This allows traffic to be delivered with diverse energy efficiency and energy consumption policies.

In RAN side, it may be reflected as the resource management for at least the UE in connected state. Further, to refine the energy efficiency, the resource management may be considered per traffic and UE basis. For example, the same traffic may be allocated different resources and achieve different level of energy efficiency even for the same UE in different scenarios. However, the different level of energy efficiency shall fulfil at least the minimum requirements for network energy efficiency and device energy efficiency.

Then, for DL transmission, it can be the network to control the resource allocation for the purpose of energy efficiency considering the procedure of resource management. For UL transmission, it may be both NW and UE to control the resource allocation for the purpose of energy efficiency e.g. some restriction information for UE power saving may be configured to UE.

Proposal 4: To achieve energy efficiency, also to align NES and UE power saving, the resource management may be considered per traffic and UE basis.

- For DL transmission, NW controls the energy efficiency.

- For UL transmission, NW and UE controls the energy efficiency e.g. some restriction information for UE power saving may be configured to UE.

# 3	Conclusion

In summary, we have following observations and proposals:

Observation 1a: LP-WUS and PEI based power reduction optimization overlap partially for paging false alarm related optimization. LP-WUS has been extended to cover also RRC Connected and measurement optimization.

Observation 1b: Cell DTRX may not allow much sleep opportunity for the network if many RRC Connected UEs are in the cell. When the number of RRC Connected UEs is low, it might instead be possible to switch off the cell without requiring any specification.

Proposal 1: Following features are shortlisted for further study/ shortlisting towards 6G Day1 adoption:

- UE side: LP-WUS, C-DRX, SCell (de-)activation and PDCCH-switching (for CONNECTED) AND DRX/ eDRX for RRC Idle

- Network side:  On-demand SI, common channel adaptation, SSB-less transmission

Proposal 2: RAN2 to study LP-WUS/WUR as one of candidate UE power saving techniques for all RRC states in 6GR, including:

- To support LP-WUS/WUR for paging reception as a candidate for day1 feature, taking 5G NR design as baseline.

- To study further RRM measurement relaxation/offloading on LR, e.g., for neighboring cell measurement in RRC_INACTVIVE/IDLE state, etc.

- To study using LR for e.g., RRM/RLM/BFD measurements in RRC_CONNECTED mode.

Proposal 3: RAN2 to study enhancements to the C-DRX scheme in 6G, e.g. support of multiple active DRX configurations or adaptive DRX operation, to improve UE power efficiency.

Proposal 4: To achieve energy efficiency, also to align NES and UE power saving, the resource management may be considered per traffic and UE basis.

- For DL transmission, NW controls the energy efficiency.

- For UL transmission, NW and UE controls the energy efficiency e.g. some restriction information for UE power saving may be configured to UE.

# 4	References

[1] RAN2#131bis chairman notes.

[2] R2-2507176	Discussion on general aspects on RAN2 study for 6GR, Lenovo

[3] 3GPP TR 38.914 v0.2.0