3GPP TSG-RAN WG2 Meeting #132		R2-2508942

Dallas, USA,  Nov. 17th - 21st

Source:	Qualcomm Incorporated

Title:		Discussion on 6G Initial Access

Agenda Item:		10.3.2.3

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses initial access related issues for 6G.

# Discussion

## 2.1	System Information

#### 2.1.0	General

In last RAN2 meeting, 6G system information was discussed briefly, and achieved the following general agreement. This section discusses the potential study directions on system information.

Agreements on System Information

- -	System information design should consider energy efficiency and low complexity for both network and UE.

-	System information design should ensure that the public warning requirements are met.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study how to improve flexibility and extensibility of scheduling SI design

-	Study mechanisms to improve SI update mechanism

-	Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

-	Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

#### 2.1.1	On-demand SSB

For on-demand SSB, RAN1 is discussing two separate cases: 1) there is already sparse SSB transmission in a cell, UE requests dense SSB transmission; 2) There is no SSB transmission in a cell, UE requests SSB transmission. RAN2 can study the upper layer procedure for both cases on aspects of on-demand SSB procedure e.g. which message is used to request the on-demand SSB, and the conditions under which UE requests SSB transmission.

The scenarios for on-demand SSB transmission include serving cell SSB transmission request and neighboring cell SSB transmission request. For the former, there is only sparse SSB transmission in the serving cell, then UE can request dense SSB transmission based on some configuration. For the latter, when UE moves close to the serving cell edge and does not detect neighboring cell SSB, UE can request SSB transmission. These two scenarios are also related to cell selection and cell reselection.

Since RAN1 has no progress which case should be supported, and consider this is RAN1-led topic, RAN2 should suspend on-demand SSB discussion until there is clear RAN1 progress.

RAN1 is discussing two cases in which there is not any SSB transmission and there is sparse SSB transmission for on-demand SSB, but no conclusion on which case should be supported.

- RAN2 should suspend on-demand SSB discussion until there is clear RAN1 progress.

#### 2.1.2	SIB1 content

In NR, SIB1 is broadcast with the periodicity of 160ms to provide necessary information for UE camping and access on the cell. SIB1 mainly contains the following information (from higher layer) shown in Table 1.

Table 1

| Main parameters in 5G SIB1 | Purpose |
| --- | --- |
| cellSelectionInfo | Cell selection |
| cellAccessRelatedInfo | Cell access |
| 1connEstFailureControl | Connection control |
| si-SchedulingInfo | SIB Schedule |
| UplinkConfigCommonSIB | Initial access |
| DownlinkConfigCommonSIB | BCCH and paging |
| uac-BarringInfo | Initial access |
| Others (cell capabilities etc.) | Network capabilities |

As shown in the above table, currently SIB1 mainly contains the information for cell selection, network discovery, SIB scheduling, some initial access related information and network status related information. Among the information, network discovery, cell selection, SIB schedule and network status information are necessary because UE needs the information to find a suitable cell and to camp on that cell. Initial access related information is also necessary and UE can obtain the configuration in advance to reduce the access latency.

- RAN2 considers 6G SIB1 includes at least information related to cell selection, cell access, network capabilities, SIB scheduling, paging, initial access related information (e.g. uplink common configuration, UAC parameters).

#### 2.1.3	System information modification

For system information modification, currently, when the network changes system information, it first notifies the UEs about this change (using Short Message transmitted with P-RNTI over DCI). Upon receiving a change notification, the UE needs to wake up to obtain SIB1 and check valueTag and area scope for other SIB(s) and acquire the updated SIB(s). The current mechanism is wasteful as it adds UE efforts to obtain SIB1 even if the UE is ultimately not interested in the updated SIB(s).

In last RAN2 meeting, RAN2 agreed to study mechanisms to improve SI update mechanism as follows:

- Study mechanisms to improve SI update mechanism

It is assumed the existing system information modification framework can be reused in 6G, i.e. using Short Message to notify UE system information change, and the UE acquires the latest system information at the modification boundary except PWS related SIs. In order to reduce UE power consumption, the modification notification should indicate more information than the existing information in Short Message. The notification information could indicate which SI or SI set are changed, and then UE only wakes up to read SI if the interested SI(s) are changed.

Existing NR system information change notification wastes UE power if the UE is not interested in the updated SIB(s).

- System modification notification should be enhanced to save UE power, e.g., indicate which SI or SI set are changed. UE only wakes up to read SI if the interested SI(s) are changed.

Furthermore, in the current RAN sharing scenario, all the SIBs are shared by multiple PLMN(s), except for UAC parameters. This does not provide differentiation in network management for different operators.

In NR RAN sharing scenario, all the SIBs are shared by multiple PLMN(s), except for UAC parameters. This does not provide differentiation in network management for different operators.

Operators sharing the RAN network can achieve different usage rights based on the agreements between the sharing operators. RAN2 should study how to provide more SIB content differentiation for different PLMNs e.g. in case of RAN sharing.

- Study mechanisms to provide SIB content differentiation for different PLMNs (e.g., in case of RAN sharing).

## 2.2	Paging

In last RAN2 meeting, 6G paging was discussed briefly, and achieved the following general agreement. This section discusses the potential ways to simplify the paging procedure.

Agreements

Paging design should consider energy efficiency and simplicity for both UE and Network.

In NR, CN paging is used to page UE in IDLE state. UE monitors paging in IDLE at paging occasions calculated using paging related parameters, e.g. (extended) DRX cycle, UE ID (S-TMSI), paging frame number in a paging cycle, paging occasion number in a paging frame, etc. These parameters are negotiated by UE and network or configured by network.

For 6G, in order to save UE and network power saving, paging DRX should be still used, and a similar paging framework with NR can be reused, i.e. UE negotiates with CN the paging parameters (e.g. DRX cycle) and UE ID for paging. UE determines paging occasions using the negotiated DRX cycle and UE ID and configured radio parameters (e.g. paging frame number in a paging cycle, paging occasion number in a paging frame).

- For 6G, similar framework as CN-initiated paging in NR should be reused for paging, i.e. UE negotiates with CN about the paging parameters (e.g. DRX cycle) and UE ID for paging. UE determines the paging occasions using the negotiated DRX cycle and UE ID and configured radio parameters (e.g. paging frame number in a paging cycle, paging occasion number in a paging frame).

In NR, both PEI and LP-WUS were introduced to save UE power for paging monitoring. From NR experience, LP-WUS has more power saving than PEI.

In NR, PEI and LP-WUS were introduced in different releases, so separate PEI and LP-WUS parameters and procedure were introduced.

Actually, the motivation and functionalities of the two features are generally same with the only difference of using MR or LR to monitor paging. The following table lists several parameters and procedures separately for PEI and LP-WUS.

Table 2.

|  | PEI | LP-WUS | Comment |
| --- | --- | --- | --- |
| Subgroup related | CN assigns subgrouping ID via NAS message | CN assigns subgrouping ID via NAS message | Separate NAS procedures and separate subgroup IDs assigned for PEI and LP-WUS |
|  | UE derives subgrouping ID based on configured parameters for PEI: subgroupsNumForUEID, subgroupsNumPerPO | UE derives subgrouping ID based on configured parameters for LP-WUS: lp-SubgroupsNumForUEID, lp-SubgroupsNumForUEID | Separate subgroup numbers are configured for PEI and LP-WUS. |
| Monitoring occasion related | The time location of PEI-O for UE's PO is determined by a reference point and an offset: - The reference point is the start of a reference frame determined by a frame-level offset from the start of the first PF of the PF(s) associated with the PEI-O, provided by pei-FrameOffset in SIB1; - The offset is a symbol-level offset from the reference point to the start of the first PDCCH MO of this PEI-O, provided by firstPDCCH-MonitoringOccasionOfPEI-O in SIB1. | The time location of an LO for UE’s PO is determined by a reference PF/PO and the configured frame-level offset: - The reference PF/PO is the start of the PF, or the first PF of the PF or PFs associated with the LO. The frame-level offset between the LO and the reference PF/PO is provided by lpwus-LoFrameOffsetList in SIB1. - The symbol leve offset for the starting time location of the first LP-WUS MO in an LO w.r.t. the reference point is provided by lpwus-OffsetFirstMoWithinLo in SIB1. | Highlighted parameters provide similar operation (needs to be determined by RAN1) |

It can be seen from the above table,

- For CN assigned subgroup ID, separate subgroup IDs are assigned by AMF for PEI and LP-WUS independently, and the procedures/signalling are duplicated for the two subgroup ID assignment.

- For UE derived subgroup ID, separate parameters and formulas are used to derive subgroup IDs for PEI and LP-WUS independently.

- For PEI-O and LO determination, separate parameters and procedures were introduced to determine the PEI occasions and LP-WUS occasions.

Separate procedures and parameters for PEI and LP-WUS make the specification complex in both AS and NAS layer, also bring additional implementation complexity to the UE and the network, and UE and network have to implement two separate procedures.

In NR, separate procedures and parameters for PEI and LP-WUS make the specification complex in both AS and NAS layer, also bring additional implementation complexity to the UE and the network.

For 6G, LP-WUS is being discussed in RAN1, so it is assumed LP-WUS will be supported in 6G from Day-1. Whether supporting PEI is still unclear. If both PEI and LP-WUS paging monitoring are supported, then a common procedure and common set of parameters could be considered for both PEI and LP-WUS.

In addition, in existing NR, LP-WUS monitoring coverage and legacy paging monitoring coverage are not equivalent. LP-WUS monitoring entry/exit condition is configured in SIB for UE to determine whether to apply LP-WUS monitoring. When the entry condition is met, UE will start LP-WUS monitoring; when the exit condition is met, UE will fall back to legacy paging monitoring (with or without PEI). Therefore, the UE has to switch among different paging monitoring mechanisms in a cell.

Besides, since gNB does not know whether UE is inside or outside of LP-WUS coverage, gNB has to send both LP-WUS signalling and PEI signalling to the UE if both are enabled in the UE, which will waste system signaling and network power. RAN2 can study the case that only one paging monitoring mechanism (PEI or LP-WUS) is applied in one cell or one area if UE and network support both.

In NR, LP-WUS monitoring coverage and legacy paging monitoring coverage are not equivalent, UE has to switch among different paging monitoring mechanisms in a cell.

In NR, gNB does not know whether UE is inside or outside of LP-WUS coverage, gNB has to send both LP-WUS signalling and PEI signalling to the UE if both are enabled in the UE, which will waste system signaling and network power.

From the above observation 4, 5, 6, it is worthy to study in 6G some enhancements to simplify paging procedure triggered by LP-WUS and PEI monitoring.

- RAN2 assumes LP-WUS triggered paging monitoring will be supported from Day-1. RAN2 considers the following potential directions to simplify paging procedure

  - Direction 1: Strive to achieve an upper layer common procedure and common set of parameters if both PEI and LP-WUS paging monitoring are supported.

  - Direction 2: Study the case that only one paging monitoring mechanism (PEI or LP-WUS) is used in one cell or one area if UE and network support both, i.e. UE does not need to switch between LP-WUS and PEI monitoring in a cell.

## 2.3	Spectrum aggregation aspects

RAN2#131bis briefly discussed spectrum aggregation aspects and made the following initial agreements:

Agreements on Spectrum aggregation

- 1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

- 2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

During the discussion, some companies indicated it may be beneficial to allow paging on multiple carriers, however several other companies commented that paging capacity may not be a real bottleneck in the actual 5G deployments.

It is unclear whether paging capacity is a real bottleneck in actual 5G deployments.

If it is concluded that paging capacity is a real issue and paging on multiple carriers is to be supported, we need to make sure that a UE is not required to monitor paging on multiple carriers, because that would be detrimental to the UE power consumption.

Monitoring paging on multiple carriers would increase UE power consumption.

In addition, multiple paging carriers may have different coverage, and UE may or may not have good coverage on secondary carriers. In such case, gNB and UE cannot be aligned on which carrier the paging should be transmitted and/or monitored.

UE should not be required to monitor paging on more than one carrier, i.e., the design should be such that from the UE point of view, it always knows which carrier it needs to monitor for the paging.

In general, since RAN1/4 have to make further progress on the spectrum aggregation aspects, detailed agreements will need to wait for RAN1/4 progress.

RAN2 should wait for RAN1/4 progress before having detailed discussions and agreements on the spectrum aggregation aspects.

- Capture the following challenges of distributing paging on multiple carriers in a cell into the TR.

  - Monitoring multiple paging carriers would increase UE power consumption.

  - Multiple carriers in a cell may have different coverage, UE may or may not have good coverage on secondary carriers, then NW and UE may not be aligned on which carrier the paging should be transmitted and/or monitored on.

  - UE should not be required to monitor paging on more than one carrier, i.e., from the UE point of view, it should always be clear which specific carrier it needs to monitor for the paging.

- Conclusion

System Information

- RAN1 is discussing two cases in which there is not any SSB transmission and there is sparse SSB transmission for on-demand SSB, but no conclusion on which case should be supported.

- RAN2 should suspend on-demand SSB discussion until there is clear RAN1 progress.

- RAN2 considers 6G SIB1 includes at least information related to cell selection, cell access, network capabilities, SIB scheduling, paging, initial access related information (e.g. uplink common configuration, UAC parameters).

NR existing system information change notification wastes UE power if the UE is not interested in the updated SIB(s).

- System modification notification should be enhanced to save UE power, e.g., indicate which SI or SI set are changed. UE only wakes up to read SI if the interested SI(s) are changed.

In NR RAN sharing scenario, all the SIBs are shared by multiple PLMN(s), except for UAC parameters. This does not provide differentiation in network management for different operators.

- Study mechanisms to provide SIB content differentiation for different PLMNs (e.g., in case of RAN sharing).

Paging

- For 6G, similar framework as CN-initiated paging in NR should be reused for paging, i.e. UE negotiates with CN about the paging parameters (e.g. DRX cycle) and UE ID for paging. UE determines the paging occasions using the negotiated DRX cycle and UE ID and configured radio parameters (e.g. paging frame number in a paging cycle, paging occasion number in a paging frame).

In NR, separate procedures and parameters for PEI and LP-WUS make the specification complex in both AS and NAS layer, also bring additional implementation complexity to the UE and the network.

In NR, LP-WUS monitoring coverage and legacy paging monitoring coverage are not equivalent, UE has to switch among different paging monitoring mechanisms in a cell.

In NR, gNB does not know whether UE is inside or outside of LP-WUS coverage, gNB has to send both LP-WUS signalling and PEI signalling to the UE if both are enabled in the UE, which will waste system signaling and network power.

- RAN2 assumes LP-WUS triggered paging monitoring will be supported from Day-1. RAN2 considers the following potential two enhancements to simplify paging procedure

  - Enhancement 1: Strive to achieve an upper layer common procedure and common set of parameters if both PEI and LP-WUS paging monitoring are supported.

  - Enhancement 2: Study the case that only one paging monitoring mechanism (PEI or LP-WUS) is used in one cell or one area if UE and network support both, i.e. UE does not need to switch between LP-WUS and PEI monitoring in a cell.

Spectrum aggregation aspects

It is unclear whether paging capacity is a real bottleneck in actual 5G deployments.

Monitoring paging on multiple carriers would increase UE power consumption.

UE should not be required to monitor paging on more than one carrier, i.e., the design should be such that from the UE point of view, it always knows which carrier it needs to monitor for the paging.

RAN2 should wait for RAN1/4 progress before having detailed discussions and agreements on the spectrum aggregation aspects.

- Capture the following challenges of distributing paging on multiple carriers in a cell into the TR.

  - Monitoring multiple paging carriers would increase UE power consumption.

  - Multiple carriers in a cell may have different coverage, UE may or may not have good coverage on secondary carriers, then NW and UE may not be aligned on which carrier the paging should be transmitted and/or monitored on.

  - UE should not be required to monitor paging on more than one carrier, i.e., from the UE point of view, it should always be clear which carrier it needs to monitor for the paging.

# References

[1] RP-252912, Revised SID: Study on 6G Radio, RAN#109, Sep 2025

[2] R2-2508002	RAN2#131bis Meeting Report