3GPP TSG-RAN WG2#132	R2-2508946

Dallas, USA, Nov 17th – 21st 2025

Source: 	Fujitsu

Agenda item:	10.3.2.2

Title:	Discussion on RRC Structure and (re)configuration in 6G

Document for:	Discussion and decision

# Introduction

The SID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

At RAN2#131bis meeting in Prague, the following agreements were achieved w.r.t RRC [2]:

Agreements

  - ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

  - Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling

  - Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g. improvements to delta signaling or no delta signaling).

  - RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

Further, regarding reconfiguration failure following guidance was provided:

For next meeting, can study the reasons why these failures happen and understand the root cause of the problem.

In this contribution, we would like to provide our views on RRC functions for 6G, RRC Structure and (re)configuration aspects.

# Discussion

According to [1], a “Single technology framework based on a stand-alone architecture” is envisaged for next generation 6G systems targeting the IMT-2030 requirements. This means the initial deployment of 6G system will be primarily standalone system unlike the initial 5G NR system deployment which were based on the Non-standalone (NSA) architecture relying on tight interworking between LTE RAT as anchor/master and NR as secondary RAT. The 5G NR control plane architecture is flexible and modular supporting inter node RRC messages, encapsulation of RRC messages generated by one RAT to be transported in RRC container messages of another RAT in case of tight inter-RAT interworking and embedding NAS container in RRC messages. To support new services and additional deployment architecture (apart from the standalone) in future the 6GR control plane architecture needs to be flexible, modular and forward compatible right from the beginning so that backward incompatibility can be avoided in future 6GR releases.

## 6G Control plane

Further, existing LTE and 5G NR deployment (both standalone and non-standalone) in many countries leverage on the distributed split RAN architecture with the standardized interfaces between CU-CP and DU (i.e. F1 interface) and CU-CP and CU-UP (i.e. E1 interface). The 6G RAN architecture as per [1], RAN3 will study “RAN internal functional split, interfaces, protocol stacks and procedures”. The 5G NR control plane architecture does not restrict the deployment option decided by the mobile operator whether it is centralized (i.e. non-split) or distributed (i.e. Split) and whether it is virtualized using cloud-native principles or monolithic software using customized hardware. It is therefore desirable that 6GR control plane design is not restricted to a particular approach of deploying the 6G system. At RAN3#129bis, the split RAN architecture was discussed [3], and the following guidance was provided “Capture pain points and benefits (and potentially derived requirements) of 5G HLS”

Control plane signalling robustness is an important requirement for some use cases and services requiring ultra-high reliability communication links. In 5G NR robustness of control plane signalling is supported in both standalone architecture for carrier aggregation scenario and in dual connectivity scenario. Therefore, given that existing 5G services and new services with ultra-high reliability requirement need to be supported by 6G, the 6GR control plane architecture design shall support robustness for the control plane signalling.

Additionally, according to [1] it states “6GR and RAN design shall ensure that the 6G System can also operate without AI/ML” so this means AI/ML may not necessarily put special requirement for the 6GR control plane architecture. The generic control plane framework shall be extendable (if needed) to support signalling required to configure the UE for AI/ML aspects. The framework that may be required for data collection from UE for AI/ML purposes is independent from the control plane design.

In summary we propose the 6GR control plane architecture needs to consider the following:

Proposal#1

- 6GR control plane design shall be flexible, modular and forward compatible to support existing/new services and additional architecture deployment (apart from standalone) in future.

- 6GR control plane design shall support both centralized (i.e. Non-split) and distributed (i.e. Split) RAN architecture deployments.

- 6GR control plane design shall support robustness for the control plane signalling.

- 6GR control plane baseline design shall be generic and shall be extendable to introduce AI/ML specific requirements.

### 6G RRC Structure and (Re)Configuration

At RAN2#131bis meeting in Prague, it was agreed to study the modular structure of RRC. However, modular RRC can be interpreted in several different ways. As mentioned earlier in the context of RAN3 discussions at Prague meeting the CU/DU split RAN architecture is considered for further study [3]. Considering the modular RRC design, from our perspective it may be possible that for some RRC functions the initial configuration and release is performed by CU-CP based on the L3 input while the subsequent re-configuration/modification of the functionality is performed by CU-CP or DU depending on whether the trigger condition or feedback is available at L2 or L3 as articulated in [4]. This basically means splitting the RRC function between CU-CP and DU as shown in Figure 1 below. It has been pointed by several company contributions submitted to RAN2#131bis, the issues related to the monolithic RRC structure in 5G NR [5-9] and hence the proposal for modular RRC but with different approaches.

Figure 1 - 6GR RRC Architecture

One approach for the modular RRC is to split the RRC such that L3 (re)configurations and L1/L2 (re)configurations can be generated by the CU-CP and DU respectively. The anchor or master RRC resides in the CU-CP and takes care of all initial configurations and L3 re-configurations. The auxiliary or assisting RRC resides in the DU and handles some re-configurations depending on the L2 trigger or feedback. Such split RRC architecture avoids frequent coordination between DU and CU-CP to provide the L2 trigger or feedback at CU-CP, reduces the control plane latency towards UE for receiving the configuration and subsequently reduces the processing load on the CU-CP, if the CU-CP takes action to prepare the final RRC message. Since co-ordination between CU-CP and DU is reduced the requirements on the F1-C can also be relaxed.

Observation#1: Split RRC design offers the following benefits

- avoids frequent coordination between DU and CU-CP on F1-C for RRC signalling

- reduces the control plane latency towards UE

- reduces the processing load on the CU-CP as the RRC signalling can be generated by both CU-CP and DU

- requirements on the F1-C can be relaxed

The signalling for the split RRC design can be done in modular way following the container approach. This means the RRC configuration generated by the Anchor RRC in CU-CP is encapsulated as a RRC Container in the RRC signalling of the DU and sent to UE. Further, the RRC configuration generated by the DU can sent be sent directly to the UE as a RRC message. Such containerized approach is followed in 5G EN-DC RRC signalling. However, with such approach the UE side RRC handling becomes complex, the specification effort is high, may introduce impacts to security handling and the UE knows which configuration is generated by DU and which one by CU. We do not prefer such a signalling approach for the split RRC design. Alternative approach is the split RRC function is transparent to the UE with minimum impact to security handling, less specification effort. Such an approach is described in section 2.1.2.  The proposed modular RRC design based on split RRC function as shown in Figure 1, can accommodate other modular RRC proposals as mentioned in [6]; wherein the modularity is based on separation of the RRC configuration on feature-specific level on top of the baseline RRC configuration. This means if a feature configured to the UE comprises of only L3 configuration then the CU-CP takes care of such (re)configuration, elseif the feature comprises both the L3 and L1/L2 configurations then the CU-CP and DU respectively perform the reconfiguration of the feature, else if the feature is solely a L1/L2 feature then DU performs the reconfigurations towards the UE. In our opinion the split RRC design complements the feature-specific modular RRC modelling. Such a RRC design is not only modular but flexible and forward compatible when new features get introduced in future 6G releases.

Observation#2: Split RRC design complements the feature-specific modular RRC modelling meeting the requirement on modularity, flexibility and forward compatibility.

### Impact of Split RRC design on UE

From the UE perspective, the split RRC function at the 6GR gNB is transparent to the UE. This means the UE is unaware the RRC signalling is generated by CU-CP or DU. RRC signalling generated by the CU-CP, the security function for encryption and integrity protection is managed by the CU-CP. RRC signalling generated by the DU, the security function for encryption and integrity protection is managed by the DU. This may come for free if the DU needs to also provide the security protection for some of the MAC-CE. The security function for decryption and integrity check of the received RRC messages at the UE side is handled at the PDCP layer. Security related details of the split RRC design are further explained in our companion contribution [10]. As per the arguments articulated in [10], Split RRC design does not impose complex security requirements. The same Krrc_Int/Krrc_Enc used for PDCP-C residing in CU-CP can be configured securely for the duplicate PDCP-C in the DU. In our view there are no security vulnerabilities due to such split RRC design. Further, the handling of the PDCP COUNT which is used as an input parameter in the security function is clarified in [10] such that the PDCP-C in the CU-CP and in the DU are duplicates of each other. Therefore, there is no impact at the UE side due to Split RRC design.

Observation#3: Split RRC is transparent to the UE.

We therefore propose the following:

Proposal#2: Split RRC design is further studied for 6GR.

### RRC Message handling in Split RRC

In 5G NR, the RRC messages sent by the gNB to the UE on DL DCCH are RRCReconfiguration, RRCRelease, RRCResume, RRCRestablishment, SecurityModeCommand, DlInformationTransfer, UECapabilityEnquiry, CounterCheck, MobilityfromNRCommand. In study items the RRC procedures involving these messages will be discussed in RAN2. Some of the messages will be adopted in 6G and some procedures like Resume and Reestablishment may get harmonized. Nevertheless, after from the RRCReconfiguration message none of the other messages is expected to have trigger point in L2. Therefore, for the Split RRC design, the DU is expected to generate RRCReconfiguration message corresponding to L1/L2 configuration update. Any other RRC message to be generated by the DU is FFS.

Observation#4: RRCReconfiguration message is potential candidate for the DU Auxilary RRC to generate corresponding to L1/L2 configuration update.

Therefore, we propose:

Proposal#3: Auxiliary RRC hosted by the DU can generate RRCReconfiguration message. Other RRC message to be generated by DU RRC is FFS.

### RRC (Re)Configuration Failures

In LTE/5G, if the UE cannot comply with the RRC reconfiguration then the UE triggers RRC re-establishment (RRE). This is quite common in commercial deployments at least during IoDT, initial roll-out of the network, upgrades to the network or during the swap of the vendor in the operator network. As pointed out in [6], the UE triggering RRE due to not able to comprehend with some IEs or fields in the RRC reconfiguration is not desirable from performance point of view. RRE due reconfiguration failures in commercial deployments is a reality, and one cannot expect that the network always provides the correct configuration. However, we also acknowledge arguments on the other hand that the UE taking autonomous actions may be harmful from network perspective as this can jeopardize the stability of the network. Nevertheless, always triggering RRE due to reconfiguration failure is also not a very good choice. It would be desirable that the network is in control of the UE behaviour with respect to reconfiguration failure handling. The network based on implementation specific logic and UE related information (e.g. UE capabilities and subscribed services) can decide for which UEs in the cell RRE should be configured and for which UEs RRE should not be configured upon encountering reconfiguration failure as shown in Figure 2. We believe this is a balanced approach that can be further studied in 6GR.

Figure 2 Network Configured Reconfiguration Failure Handling

We therefore propose the following:

Proposal#4: RAN2 shall further study efficient reconfiguration failure handling such that network is allowed to configure UE behaviour with respect to reconfiguration failure handling.

# Conclusion

In this paper we presented some views on RRC Structure and (re)configuration aspects and have come up with the following observation and proposals to be discussed and agreed:

Proposal#1 On 6G Control Plane

- 6GR control plane design shall be flexible, modular and forward compatible to support existing/new services and additional architecture deployment (apart from standalone) in future.

- 6GR control plane design shall support both centralized (i.e. Non-split) and distributed (i.e. Split) RAN architecture deployments.

- 6GR control plane design shall support robustness for the control plane signalling.

- 6GR control plane baseline design shall be generic and shall be extendable to introduce AI/ML specific requirements.

Observation#1: Split RRC design offers the following benefits

- avoids frequent coordination between DU and CU-CP on F1-C for RRC signalling

- reduces the control plane latency towards UE

- reduces the processing load on the CU-CP as the RRC signalling can be generated by both CU-CP and DU

- requirements on the F1-C can be relaxed

Observation#2: Split RRC design complements the feature-specific modular RRC modelling meeting the requirement on modularity, flexibility and forward compatibility.

Observation#3: Split RRC is transparent to the UE.

Proposal#2: Split RRC design is further studied for 6GR.

Observation#4: RRCReconfiguration message is potential candidate for the DU Auxilary RRC to generate corresponding to L1/L2 configuration update.

Proposal#3: Auxiliary RRC hosted by the DU can generate RRCReconfiguration message. Other RRC message to be generated by DU RRC is FFS.

Proposal#4: RAN2 shall further study efficient reconfiguration failure handling such that network is allowed to configure UE behaviour with respect to reconfiguration failure handling.

# References

- RP-252912, Revised SID: Study on 6G Radio.

- R2-25xxxx, Chairman Notes, RAN2#131Bis meeting, 13-17 Oct, 2025, Prague.

- R3-25xxxx, Chairman Notes, RAN3#129Bis meeting, 13-17 Oct, 2025, Prague.

- R2-2507131,	Initial consideration on Control plane aspects in 6G,	Fujitsu.

- R2-2507146,	On 6G RRC design, Nokia.

- R2-2507172,	Views on Control Plane for 6G, Qualcomm Incorporated.

- R2-2507433,	Control Plane for 6GR, InterDigital, Inc.

- R2-2507111,	Views on Directions of 6G Control Plane Enhancements, Apple.

- R2-2507332,	Discussion on Radio Protocol Architecture – Control Plane,	Rakuten Mobile, Inc

- R2-2508948, Discussion on Access stratum Security	Fujitsu