3GPP TSG-RAN WG2 Meeting #132	R2-2508955

Dallas, USA, Nov 17th – 21th, 2025 	Revision of R2-2506033

Agenda Item:	8.9.3

Source:			ASUSTeK

Title:	Discussion on usage of time information for S&F

Document for: 	Discussion and Decision

- Introduction

Following agreements are made in previous meetings for Store & Forward operation. In RAN2 #130 meeting:

Agreements:

1.	When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

3.	Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”.

In RAN2 #131 meeting:

Agreements:

1.	The UE forwards the S&F mode indication (e.g. sf-OperationMode) to upper layers upon acquiring it from SIB1(-NB). RAN2 to capture this UE behavior in 3GPP TS 36.331, sub-clause 5.2.2.7.

2.	The S&F mode indication (i.e., sf-OperationMode) and the S&F mode transition time (i.e., t-ModeSwitching) of the neighbor satellite are signaled in SIB33 per neighbor satellite.

3.	A Rel-19 UE may deprioritize the neighbor cells operating in the S&F mode (FFS on the detailed specification impact, i.e. whether a note is sufficient or some normative text. To be considered in the post meeting CR review)

4.	We clarify the stage-2 description in 36.300 to state that the satellite identifier values used at the AS-level (e.g., satellite identifiers included in SIB messages) and the NAS-level (e.g. satellite identifiers included in the S&F Monitoring list) are set consistently, i.e. with the same value, when they refer to the same satellite (FFS on the actual wording)

5.	We don’t increase the size of satellite ID and or define any new satellite ID as part of this Rel-19 WI

In RAN2 #131bis meeting:

Agreements:

1.	[V211] The UE RRC indicates to upper layers that the cell is operating in normal mode, upon acquiring a SIB1 that does not include a sf-OperationMode

3.	S&F Architecture option is not indicated via any RRC signalling including SIB indication (Can come back to if we receive an indication from SA2 to cover the scenario of mixed (split MME and Full CN) deployments)

4.	We clarify in RRC that a change of the S&F mode indication and the S&F mode transition time of neighbour cells in SIB33 does not trigger SI update procedure.

5.	RAN2 understands that in idle mode the UE may re-acquire SIB33 upon transition time expiry for new S&F mode indication and transition time (if available) (check if anything needs to be added to an existing note for this)

6.	RAN2 understands that in case the operation mode transition time is not aligned to the modification period legacy mechanisms shall apply (e.g. paging for UEs in idle) (no new enhancements are introduced)

7.	The S&F mode indication for neighbour satellite signalled in SIB33 contains no barring information.

8.	If the S&F mode indication for a neighbour satellite is absent from SIB33, UE assume the corresponding neighbour satellite is operating in normal mode.

And in RAN2 #131bis meeting, the Impact of the S&F mode on AS was noted as come back in the next meeting. In this contribution, we would like to discuss the usage of the time information for S&F mode.

- Discussion

The Store and Forward (S&F) satellite operation in IoT NTN is intended to provide some level of communication service for UEs (e.g., for delay-tolerant communication service) under satellite coverage with intermittent/temporary satellite connectivity, e.g., when feeder link in unavailable. Based on the current agreements and RRC CR [1], the UE could receive sf-OperationMode via SIB1 and t-ModeSwitching via SIB31 for serving cell, and receive sf-OperationModeNeigh and t-ModeSwitchingNeigh in SIB33 for neighbor cells.

If the S&F mode indication (i.e., sf-OperationMode for the serving cell, sf-OperationModeNeigh for a neighbor cell) is present, the transition time for S&F mode (i.e., t-ModeSwitching for the serving cell, t-ModeSwitchingNeigh for the neighbor cell) indicates the time information on when the NTN cell is going to switch from the S&F satellite operation mode to the normal mode. Otherwise, the transition time indicates the time information on when the NTN cell is going to switch from the normal mode to the S&F satellite operation mode. The transition time for S&F mode (i.e., t-ModeSwitching and t-ModeSwitchingNeigh) allows the UEs to know when the NTN cells including the serving cell and the neighbor cells switch their operation mode in advance.

Observation 1: With the S&F mode indications and transition time provided in SIB1, SIB31 and SIB33, the UE could know when the NTN cells are in which operation mode.

The UE could camp on or connect to a NTN cell supporting S&F while the NTN cell is operating in the normal mode. The UE would receive sf-OperationMode in SIB1 and receive t-ModeSwitching in SIB31 indicating when the cell is going to switch to the S&F satellite operation mode. However, for the UEs not supporting S&F, they should camp on cells in normal mode as long as possible. When a UE not supporting S&F is camping on a cell in normal mode and indicated that the S&F satellite operation mode is about to start, it would be beneficial if the UE could attempt to reselect to another cell which is in normal mode. And the UE could utilize the transition time (i.e., t-ModeSwitching) to perform the measurements of neighbouring cells.

Proposal 1: When t-ModeSwitching indicates the transition time on normal mode to S&F mode, for the UE not supporting S&F, the UE performs measurements of neighbouring cells before the transition time.

Then the UE could reselect to a cell which is operating in normal mode after the serving cell switches to S&F satellite operation mode. The UE could consider a neighbor cell based on the neighbor cell’s S&F mode status deriving from sf-OperationModeNeigh and t-ModeSwitchingNeigh in the neighbor cell list from SIB33 without other NW signallings. It would be beneficial for radio link handling.

Proposal 2: When performing measurements and reselection of neighbouring cells, the UE considers a cell based on the S&F mode indication (i.e., sf-OperationModeNeigh) and transition time (i.e., t-ModeSwitchingNeigh) of the cell.

- Conclusion

We have the following observation and proposals for the S&F transition time information:

Observation 1: With the S&F mode indications and transition time provided in SIB1, SIB31 and SIB33, the UE could know when the NTN cells are in which operation mode.

Proposal 1: When t-ModeSwitching indicates the transition time on normal mode to S&F mode, for the UE not supporting S&F, the UE performs measurements of neighbouring cells before the transition time.

Proposal 2: When performing measurements and reselection of neighbouring cells, the UE considers a cell based on the S&F mode indication (i.e., sf-OperationModeNeigh) and transition time (i.e., t-ModeSwitchingNeigh) of the cell.

- Reference

- R2-2507787, “Rapporteur correction on IoT NTN Ph3.”