3GPP TSG-RAN WG2 Meeting #132	R2-2508956

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	8.12.2

Source: 	ASUSTeK

Title:  	Discussion on MIMO MAC open issue-4

Document for:	Discussion and Decision

- Introduction

In report[1] for MIMO MAC open issues from the post meeting email discussion [Post131bis][215][MIMOevo], a few open issues were identified and discussed with proposals suggested by rapporteur. In this contribution, we’d like to discuss one open issue raised by other companies in more detail.

- Issue-4

2-1.  Discussion on PUCCH transmission regarding SCell deactivation

In the email discussion, an issue was raised regarding UE behaviour for PUCCH transmission when the SCell on which the CSI-ReportConfig is deactivated:

[Issue description]: If a mode-A or mode-B UE-initiated report is configured in an SCell that is deactivated, but has PUCCH resources for UEIRI on another cell (PCell or PUCCH SCell) that are valid (TA not expired), what should the UE behaviour be?

For mode-B UEI report, an agreement was made in the previous meeting so that the UE does not transmit the UEIRI on PUCCH if the SCell is deactivated:

If the BWP/SCell carrying the Mode-B CG PUSCH is deactivated, the UE shall not transmit the UEI Report Indication on PUCCH.

corresponding change has been captured in the rapporteur CR [2]:

1> if the SCell is deactivated: … 2> if the configured grant Type 1 included in a CSI-ReportConfig for mode-B UE-initiated CSI reporting is configured on the SCell: 3> not transmit UE Initiated Report Indication on PUCCH for this CSI-ReportConfig.

Observation 1: For mode-B UEI report, it was already agreed that the UE does not transmit UEIRI on PUCCH if SCell on which the CSI-ReportConfig is configured is deactivated. No further re-discussion is needed.

For mode-A, on the other hand, it was not discussed whether the UE should transmit the UEIRI if the SCell on which the CSI-ReportConfig is deactivated. On first glance, it would seem that the NW can send the DCI scheduling CSI report on other activated Cells. However, the UE can only receive the DCI on the cell in which the CSI-ReportConfig is included:

(38.331) #### – CSI-ReportConfig The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). The IE CSI-ReportConfig is also used to configure UE initiated CSI reporting. See TS 38.214 [19], clause 5.2.1.

Therefore, once the SCell with the CSI-ReportConfig is deactivated, the UE is no longer able to receive the DCI for mode-A CSI report transmission, and no CSI report can be transmitted.

Observation 2: For mode-A UEI report, the DCI scheduling the PUSCH for CSI report shall be received on the cell in which the CSI-ReportConfig is included, and the UE will not be able to receive DCI scheduling mode-A CSI report for a CSI-ReportConfig if the corresponding SCell has been deactivated.

Therefore, with the similar consideration of reducing UCI collision and UE power consumption on transmitting unnecessary UEIRI on PUCCH, we propose to apply a unified behaviour for mode-A and mode-B UEI report regarding SCell deactivation, and the UE should not transmit the corresponding UEIRI on PUCCH if the SCell is deactivated.

Proposal:  (MAC-Issue-4) If the SCell on which the mode-A or mode-B CSI-ReportConfig is configured is deactivated, the UE shall not transmit the UEI Report Indication on PUCCH for the CSI-ReportConfig.

2-2.  Text proposal

Text proposal based on the latest rapporteur CR [2] is as follows:

(Based on R2-2508128) 1> if the SCell is deactivated: … 2> if a CSI-ReportConfig for mode-A or mode-B UE-initiated CSI reporting is configured on the SCell: 3> not transmit UE Initiated Report Indication on PUCCH for this CSI-ReportConfig.

- Conclusion

We have the following observations and proposals for UEI report:

Observation 1: For mode-B UEI report, it was already agreed that the UE does not transmit UEIRI on PUCCH if SCell on which the CSI-ReportConfig is configured is deactivated. No further re-discussion is needed.

Observation 2: For mode-A UEI report, the DCI scheduling the PUSCH for CSI report shall be received on the cell in which the CSI-ReportConfig is included, and the UE will not be able to receive DCI scheduling mode-A CSI report for a CSI-ReportConfig if the corresponding SCell has been deactivated.

Proposal:  (MAC-Issue-4) If the SCell on which the mode-A or mode-B CSI-ReportConfig is configured is deactivated, the UE shall not transmit the UEI Report Indication on PUCCH for the CSI-ReportConfig.

- Reference

[1] R2-2508129 Report of Rel-19 MIMO MAC open issues, Samsung

[2] R2-2508128 Miscellaneous Corrections for MIMO, Samsung