3GPP TSG-RAN WG2 Meeting #132		       R2-2508959

Dallas, USA, Nov. 17th-21st

Agenda Item:	9.3.3

Source:	ASUSTeK

Title:	Discussion on measurement event prediction

Document for:	Discussion and Decision

- Introduction

In WI for AI mobility, one objective is to specify the applicability report and inference configuration for measurement event prediction. In this contribution, we’d like to discuss this procedure.

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. … Note 5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible.

- Discussion

## 2.1 Applicability reporting and inference configuration

In RAN2 #129bis, RAN2 made the following agreements, confirming that the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework:

Agreements - The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework - For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message. - FFS whether association ID is required and should provide motivation on per use case bases - As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework. - Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline. Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20. - Model transfer will not be discussed in this study item 7 For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring. FFS metrics per use case

In RAN2 #130, the following parameters are identified for the inference of different use cases:

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

In the RRM measurement framework, MeasId, MeasObject, and ReportConfig are used to configure the UE to perform RRM measurement. The UE is configured to perform measurement and reporting for a MeasObject when the MeasObject is included in a MeasId or is a ServingCellMO and there is at least one ReportConfig associated with a MeasId. The MeasObject indicates the target to measure (e.g., SSB, cell, frequency), and the ReportConfig indicates the condition and content to report (e.g., report quantity, trigger quantity).

Whether the UE is applicable of measurement event prediction may depend on several aspects. For example, whether the UE is applicable of RRM prediction for the applicable cells (e.g., for indirect prediction), or whether the UE has an AI model for the configured frequency (e.g., for direct prediction). Since the applicability for a measurement event prediction can be based on multiple frequencies, and the determination of applicable cells is associated with ServingCellMO and/or MeasObject associated with a ReportConfig, we think MeasId is suitable for the inference configuration for measurement event prediction.

Proposal 1: The inference configuration for measurement event prediction could be identified by a MeasId.

The steps for applicability reporting used in AI/ML PHY may be adopted as follows, serving as a baseline for discussion:

Step 1 and 2: Capability exchange.

Step 3a: The UE receives one or more MeasId for inference. FFS on the parameters and container/where to put the parameters (e.g., report/trigger quantity, PW, skipping pattern, frequency).

Step 3b: The UE receives one or more sets of inference related parameters. FFS on the parameters and container (e.g., applicability set).

Step 4: The UE reports applicability based on the received one or more MeasId for inference and sets of inference related parameters. FFS on how to activate/deactivate the inference configuration.

Step 5: The NW may provide further reconfiguration based on the received applicability report.

Proposal 2: Use the following steps for applicability reporting for measurement event prediction as a starting point:

Step 1 and 2: Capability exchange.

Step 3a (for option A): The UE receives one or more MeasId for inference. FFS on the parameters and container/where to put the parameters (e.g., report/trigger quantity, PW, skipping pattern, frequency).

Step 3b (for option B): The UE receives one or more sets of inference related parameters. FFS on the parameters and container (e.g., applicability set).

Step 4: The UE reports applicability based on the received one or more MeasId for inference and sets of inference related parameters. FFS on how to activate/deactivate the inference configuration.

Step 5: The NW may provide further reconfiguration based on the received applicability report.

After the inference configuration is activated (i.e., the UE is configured to perform inference), one issue is regarding which measurement results should the UE report. For example, when performing temporal prediction case A or inter-frequency prediction, the UE may have actually measured results and predicted results available. But when performing temporal prediction case B, the actually measured results for a time instance may be aging and may not be useful for the NW. In our opinion, since different use cases may have different characteristics, and whether the information is useful depends on the NW implementation, one way forward is for the NW to configure the UE which type or types of measurement result to report/trigger an event.

Proposal 3: The NW may configure the UE which type or types of measurement result to report measurement results and/or trigger an event.

## 2.2 Reporting of predicted measurement event

During the Rel-19 AI/ML Mobility study, RAN2 performed the system level simulation to study the system level impacts of using direct and indirect event prediction. In the TR 38.744, for temporal domain case A, 2 options are used for modelling the handover determination are captured, with majority of the simulation results for HO option 2 showing improvement over option 1 and baseline approach:

If measurement event is predicted based on intra-frequency temporal domain case A, there are two options w.r.t. how to decide on the time point to transmit handover command: Option 1: Relying on legacy measurement event Option 2: Relying on predicted measurement event Figure 5.5.1-1: Handover model option 1 Option 1 is illustrated in Figure 5.5.1-1. At current time i.e. t0 measurement event e.g. A3 event is predicted at some point of time in future. Network will not transmit handover command until a real measurement event is reported for the same neighbouring cell. In this way, the main benefit of this option is to save handover preparation time. Figure 5.5.1-2: Handover model option 2 Option 2 is illustrated in Figure 5.5.1-2. At current time, i.e. t0, measurement event e.g. A3 event is predicted @ future time t1. Network transmits handover command when entry condition of the predicted measurement event is met based on actual measurement result @ t2 unless t2 is earlier than handover preparation phase. In later case, network transmits handover command immediately after handover preparation phase. In this way, not only handover preparation could be saved but also handover can be executed earlier. … Compared to the existing L3 handover mechanism: - AI algorithm (with indirect measurement event prediction) following handover model option 1 and option 2 performs better than baseline in terms of HOF rate and total number of HOF per UE per second; - Majority companies show that AI algorithm (with indirect measurement event prediction) following handover model option 2 outperforms handover model option 1 when RRM prediction accuracy is good enough. A few companies show opposite observation due to the risk of too early handover in handover model option 2; Few companies shows that AI algorithm with direct measurement event prediction methodology can reduce the total number of HOF per UE per second in SLS based on FR2 intra-frequency temporal domain case A; - AI algorithm following handover model option 3 as illustrated in Figure 5.5.1-3 with MRRT=50% has a minor or even no degradation in terms of HOF rate and total number of handover attempts.

Based on the simulations, we can observe that relying on both prediction and actual triggers can bring improvement in handover performance. HO option 1 has less gain but guarantees to be not worse than the baseline. HO option 2 has the potential to gain more performance but some companies shows concerns on too early handover. It is up to NW implementation of which option to support. Therefore, we believe a way forward is to support mechanisms that enable both options to operate efficiently.

Proposal 4: The NW may configure the UE to predict an event and report the prediction to the NW, and configure the UE to report when actual measurements trigger the event and/or fulfill the entering condition of the event (e.g., support HO option 1/2).

- Conclusion

Proposal 1: The inference configuration for measurement event prediction could be identified by a MeasId.

Proposal 2: Use the following steps for applicability reporting for measurement event prediction as a starting point:

Step 1 and 2: Capability exchange.

Step 3a (for option A): The UE receives one or more MeasId for inference. FFS on the parameters and container/where to put the parameters (e.g., report/trigger quantity, PW, skipping pattern, frequency).

Step 3b (for option B): The UE receives one or more sets of inference related parameters. FFS on the parameters and container (e.g., applicability set).

Step 4: The UE reports applicability based on the received one or more MeasId for inference and sets of inference related parameters. FFS on how to activate/deactivate the inference configuration.

Step 5: The NW may provide further reconfiguration based on the received applicability report.

Proposal 3: The NW may configure the UE which type or types of measurement result to report measurement results and/or trigger an event.

Proposal 4: The NW may configure the UE to predict an event and report the prediction to the NW, and configure the UE to report when actual measurements trigger the event and/or fulfill the entering condition of the event (e.g., support HO option 1/2).

References

[1] RP-252899

[2] Meeting Report of RAN2 #129bis

[3] Meeting Report of RAN2 #130

[4] TR 38.744

Annex