3GPP TSG-RAN WG2 Meeting #132		       R2-2508960

Dallas, USA, Nov. 17th-21st

Agenda Item:	10.3.3.1

Source:	ASUSTeK

Title:	Discussion on 6G AIML framework

Document for:	Discussion and Decision

- Introduction

In the 6G SID [1], it has been agreed to study AI/ML for 6GR, leveraging 5G AI/ML framework:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this contribution, we would like to provide our views on the objective and discuss about the way forward.

- Discussion

In RAN2 #131bis, RAN2 agreed to study the termination points for AI/ML data collection:

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

During the Rel-18 and Rel-19 study, multiple data collection scenarios are identified for AI/ML, including UE data collection, NW data collection, and dataset/model transfer. Different scenarios may include different data transfer path and end point pairs (i.e., producer and consumer).

One scenario for UE data collection may be for the UE vendor (or chip vendor) to collect data from a variety of UEs. In this case, the producer of the data may be the UE, and the consumer of the data may be a UE side server located inside or outside of the OAM (e.g., cloud training).

Although on device training may seem too early consider the current development of hardware. But it can bring further customization for individual UEs, and it may be beneficial to take it into consideration for a future proof specification. When considering on device training, another scenario for UE data collection may be for the UE to collect data to finetune its model (e.g., on device training). In this case, the producer of the data may be the UE, and the consumer of the data may be a UE device (e.g., on device training).

Proposal 1: For UE data collection, consider the producer to be UE devices, and the consumer to include UE servers and/or UE devices.

The target of NW side data collection may be to optimize (or train) an AI model for a number of UEs, by providing information (e.g., reference signals) and collecting feedbacks from the UE. In this case, the producer of the data may include UE device and/or the NW device (e.g., gNB), and the consumer of the data may be a NW side server located inside or outside of the OAM. FFS whether the NW side server can be at the gNB.

Proposal 2: For NW data collection, consider the producer to include UE devices and/or NW devices (e.g., gNB), and the consumer to include a NW server. FFS whether the NW server can be at the gNB.

The current study for dataset/model transfer is based on two-sided models. In one scenario, the NW first creates a dataset (e.g., via synthetic data and/or NW data collection) used for training two-sided model at the NW. The dataset, along with some model/parameters, may then be transferred to the UE (e.g., UE server and/or UE device), to perform further training/finetuning of the two-sided model. In this case, the producer of the data may be the NW device (e.g., gNB) and/or NW server, and the consumer may be the UE server and/or UE device.

Proposal 3: For dataset/model transfer, consider the producer to be a NW server, and the consumer to include UE servers and/or UE devices. FFS whether the NW server can be at the gNB.

The procedures for UE data collection, NW data collection, dataset/model transfer, and normal measurements (i.e., not for data collection) may be independent. However, in some scenarios, these procedures may complement each other. For example, a UE may perform UE data collection, while the NW also wants to perform NW data collection on the same resources. For another example, the NW may configure a UE to perform normal measurements, however, this normal measurement may also be used for creating a dataset.

Proposal 4: It is suggested to study a unified framework to enable the collaboration between normal measurements, UE data collection, NW data collection, and dataset/model transfer to prevent transferring duplicate data.

- Conclusion

Proposal 1: For UE data collection, consider the producer to be UE devices, and the consumer to include UE servers and/or UE devices.

Proposal 2: For NW data collection, consider the producer to include UE devices and/or NW devices (e.g., gNB), and the consumer to include a NW server. FFS whether the NW server can be at the gNB.

Proposal 3: For dataset/model transfer, consider the producer to be a NW server, and the consumer to include UE servers and/or UE devices. FFS whether the NW server can be at the gNB.

Proposal 4: It is suggested to study a unified framework to enable the collaboration between normal measurements, UE data collection, NW data collection, and dataset/model transfer to prevent transferring duplicate data.

References

[1] RP-251881

[2] Meeting Report of RAN2 #131bis

Annex