3GPP TSG-RAN WG2 Meeting #132	                 	      R2-2508966

Dallas, USA, 17 – 21 November 2025

Agenda item:	10.2.3

Source:	China Unicom

Title:	Considerations on 6G Services and Requirements

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion and Decision

- Introduction

At RAN2#132 meeting, general aspects of 6GR have been discussed[1]. However the new services of 6G still need to be further discussed before we look into the detailed RAN2 protocol design. This contribution provides our views on new services and requirements for 6GR.

- Discussion

2.1 Requirements for AI and sensing

AI and sensing are typical and highly interested new services for 6G. Furthermore, they are also fundamental functions of 6G.

In recent years, especially recent 1~2 years, operators are investing heavily in distributed compute infrastructure (e.g. IDC, MEC) and seek to leverage these assets to enable new services, e.g. AI-driven services, sensing service. The current 5G system lacks native RAN capabilities to efficiently support AI and sensing services and these deployed compute infra have not been efficiently utilized. Thus operators are looking forward to revenue streams brought by new services within the 6G era. A tight integration between RAN and compute resources for 6G is necessary to minimize latency for AI feedback loops, optimize network based on AI/ML predictions, and enable 6G RAN more efficiently integrating sensing and communication.

Figure 1.  Diverse computing resources locations in 6G network

In operator’s opinion, when RAN2 study how to support the new service (AI and sensing), it should be considered that reusing the current compute infrastructure. As shown in figure 1, the computing resources could have different options for deployment. For example, for 6G RAN, there can be two options:

- Option 1, computing resources located at BBU level: in 5G-A, BS vendors provide computing board at BBU to bring AI capability. Considering 5G migration to 6G is potential scenario, this option should be considered.

- Option 2, computing resources located at CU level: even CU/DU separation has not been commercial deployed in 5G NW, CU option could still be considered since RAN virtualization turns to be more attractive in 6G. E.g. using servers and GPU to deploy CU and computing resource can be naturally supported.

- Option 3, new location: compared to option 2, there could be different for the relationship of CU and computing resources. CU/DU solution needs to introduce extra port similar as F1 and related RAN functions should be designed considering F1 framework and signaling. In option 3, computing resources can be deployed by general servers and GPU without CU/DU separation. And the its serving granularity could be tens of BBUs.

Observation 1: There are three different options for computing resources deployment in 6G RAN. These three options should be further studied.

For computing resources in 6G CN, it should consider reusing operators’ current computing infra. For example, computing resources centralized locate IDC centers.

Proposal 1: During RAN2 study on support AI and sensing services, it should be considered different computing resources deployment options, both considering located at RAN and CN.

2.2.2 Requirements for digital twin

Digital twin is emerging as a critical 6G use case for operators, enabling the creation of virtual, real-time digital representations of physical entities (e.g., factories, cities, or even the network itself). Digital twin can be considered as a comprehensive use case based on AI/ML and sensing. This requires the RAN to serve as a high-fidelity, massive-scale data acquisition and actuation sensing network. Current 5G systems are primarily designed for user data transport and are not optimized for the persistent, bidirectional, and highly synchronized flow  between massive numbers of physical objects and their digital counterparts. Operators need to leverage the RAN not just for communication, but as an integral part of the sensing-actuation loop for digital twin ecosystems, which is essential for new service offerings in vertical industries and for enhanced network autonomy.
RAN2 should design protocols capable of supporting the massive, concurrent, and low-latency collection of massive data (e.g., sensor readings, status updates) from a vast population of twinned entities, alongside the reliable delivery of control commands from the digital twin back to the physical world. This may impact the RAN2 protocol stack, e.g. new QoS mechanisms, RRC procedures for group management of UEs related to a digital twin service.

Proposal 2: Digital twin service should be further studied (e.g. traffic model, potential specification impacts) by RAN2.

2.2.3 Requirements for vertical services

2B and vertical markets are also important scenario for operators. Even current 5G network provides some attractive functions for 2B customers, e.g. network slicing, 5G LAN, the gap between vertical services requirements and network capability truly exists. 2B scenarios often require stricter performance isolation, more deterministic latency/jitter guarantees, and simpler network protocols stack than what is available from 5G. For example, compared to alternative wireless technologies (e.g., WiFi), current 5G private networks necessitate the development of a full protocol stack, which results in high costs on 5G network deployment in vertical scenario. Operators need 6GR to be more flexible and robust enabler for these customized 2B services.

Observation 2: Operators need 6GR to be more flexible and robust enabler for these customized 2B services.
RAN2 needs to study how to support 6G vertical services. For example, study enhancement based on 5G slicing framework or even new scheme to support stricter resource isolation and more deterministic performance guarantees for various 2B services. And investigate simplified protocol stack solutions to address convergence needs for 2B scenarios (e.g., public-private network convergence, wireline-wireless convergence).

Proposal 3: RAN2 needs to further study how to support 6G vertical services, at least protocol flexibility, suitability and simplification.

- Conclusion

The proposals of this contribution are as follows.

Observation: There are three different options for computing resources deployment in 6G RAN. These three options should be further studied.

Proposal 1: During RAN2 study on support AI and sensing services, it should be considered different computing resources deployment options, both considering located at RAN and CN.

Proposal 2: Digital twin service should be further studied (e.g. traffic model, potential specification impacts) by RAN2.

Proposal 3: RAN2 needs to further study how to support 6G vertical services, at least protocol flexibility, suitability and simplification.

- References

- Draft Report of 3GPP TSG RAN WG2 meeting #131bis, RAN2 chair