3GPP TSG RAN WG2 #132	R2-2508981

Dallas , US, October 17th – 21st, 2025

Source:		KDDI Corporation

Title:		Discussion on RRM measurement prediction

Agenda Item:	9.3.2	RRM measurement prediction

Document for:	Discussion and Decision

Introduction

At the RAN #108 meeting, a new Work Item entitled “Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR,” with the following objective was endorsed [1] .

RP-252899, “Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR” [1] . The objectives of this work item focused on AI/ML-assisted mobility in RRC_CONNECTED mode, wherein RRM measurement prediction and measurement event prediction could be applied. Only functionality-based LCM procedures are supported. In general, the RRM measurement configuration and reporting frameworks in RRC layer are taken as baseline.

This WID is related to the preceding study item and its technical report (TR 38.744 [2] ). In this contribution, we provide our views on the normative phase of RRM measurement prediction.

Discussion

Inference report for prediction

In any use case of RRM measurement prediction for UE-sided model, the SI assumed that a predicted but not actually measured result will be reported from UE to NW. TR 38.744 states the following:

TR 38.744 [2] ##### 6.1.2.1.2 Inference configuration and report Existing RRM measurement configuration and reporting framework in RRC layer is baseline for inference configuration and report. When a full inference configuration is received, UE shall maintain it until it is released by network explicitly. --- unnecessary parts are omitted --- UE can be configured with periodic or event triggered reporting of predicted and/or actual measurement result(s). For each reported cell in the measurementReport message: - For intra-frequency temporal domain case A, one or more instances of predicted measurement result(s) in PW can be reported; - For intra-frequency temporal domain case B, the latest actual or predicted measurement result can be reported; - For inter-frequency prediction, the latest predicted measurement result is reported.

In intra-frequency temporal domain case A, predicted future measurement results will be reported in advance for purposes such as an early handover. In intra-frequency temporal domain case B, measurement instances skipped for measurement reduction purposes can be reported as predicted values.

While the above functionality can possibly be realized through a combination of current measurement specifications and UE implementation, from an operational perspective, it is preferable that the report explicitly indicates that it is an inference result. Since the inference report is considered to have a significant impact on LCM and applicability, we should first start the discussion on the need for its specification.

- RAN2 should discuss necessity of measurement report extension that explicitly indicates inference for each scenario in UE-sided model.

As for the specification change, in the current specification, measured objects, measurement results, and reports have been linked by related IDs (measId, measObjectId, and reportConfigId). It is unclear whether instances for which measurement configuration is not applied can be reported as an inference result. We think it is necessary to either relax the linking of measId, measObjectId, and reportConfigId, or introduce a new measurement report configuration for inference results.

- RAN2 should discuss on either relaxing the linking of measId, measObjectId, and reportConfigId, or introduce a new measurement configuration for inference scenario.

Measurement skip (temporal domain case B)

In temporal domain case B, UE is expected to partially skip measurements. While the UE may autonomously skips measurements target by UE implementation, if the NW explicitly manages a measurement skip, it may conflict with the constraints of existing measurement period (periodicityAndOffset) or measurement gap period (MGRP) configurations. Therefore, we propose

- RAN2 should confirm whether current specifications related to measurement period (e.g., periodicityAndOffset and MGRP) imposes restrictions on the measurement skipping.

Conclusion

In this contribution, we showed our views on initial discussion of the WID “Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.” Based on above discussions, we have the following proposals:

Proposal 1: RAN2 should discuss necessity of measurement report extension that explicitly indicates inference for each scenario in UE-sided model.

Proposal 2: RAN2 should discuss on either relaxing the linking of measId, measObjectId, and reportConfigId, or introduce a new measurement configuration for inference scenario.

Proposal 3: RAN2 should confirm whether current specifications related to measurement period (e.g., periodicityAndOffset and MGRP) imposes restrictions on the measurement skipping.

- Reference

- RP-252899, “Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”, RAN Plenary #108.

- 3GPP TS 38.211 v19.0.0