3GPP TSG-RAN WG2 Meeting #132		R2-2508983

Dallas, USA, Nov. 17th - 21st , 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5521 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NCD-SSB Configuration for Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the RAN1#113 agreement, a UE in RRC_INACTIVE state can be configured with either a CD-SSB as the pathloss reference signal, or—at least for RedCap UEs—with either a CD-SSB or NCD-SSB as the pathloss reference signal for positioning configuration across multiple cells. The fields PhysCellId and ssb-IndexNcell in the IE SSB-InfoNCell are updated in Release 18 to clarify NCD-SSB can be configured. However, the current full SSB configuration in SSB-InfoNcell only includes CD-SSB information. To support UEs with restricted bandwidth capability (such as RedCap devices) in performing NCD-SSB measurements when moving to a new cell in RRC_INACTIVE state, the time-domain information (indicating the time offset between CD-SSB of the serving cell and NCD-SSB) and frequency-domain information specified in the nonCellDefiningSSB IE are needed. Therefore, it is necessary to add the missing time and frequency information for NCD-SSB to fully support the positioning SRS configured for UE in RRC_INACTIVE state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A new field ncd-SSB is added Impact Analysis Impacted 5G architecture options: NR SA and MR-DC Impacted functionality: Neigbor cell SSB measurements for Positioning Inter-operability: If UE implements this CR and NW does not Or If NW implements this CR and UE does not - Redcap UE may not be know where NCD SSB are available and thus may not be able to perform pathloss SSB measurements of the camped cell in RRC Inactive |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Positioning based on NCD-SSB can not be fully supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

#### –	SRS-Config

The IE SRS-Config is used to configure sounding reference signal transmissions. The configuration defines a list of SRS-Resources, a list of SRS-PosResources, a list of SRS-PosResourceSets and a list of SRS-ResourceSets. Each resource set defines a set of SRS-Resources or SRS-PosResources. The network triggers the transmission of the set of SRS-Resources or SRS-PosResources using a configured aperiodicSRS-ResourceTrigger (L1 DCI). The network does not configure SRS specific power control parameters alpha (without suffix) or pathlossReferenceRS if unifiedTCI-StateType is configured for the serving cell.

SRS-Config information element

-- ASN1START

-- TAG-SRS-CONFIG-START

SRS-Config ::=                          SEQUENCE {

srs-ResourceSetToReleaseList            SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId        OPTIONAL,   -- Need N

srs-ResourceSetToAddModList             SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet          OPTIONAL,   -- Need N

srs-ResourceToReleaseList               SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId              OPTIONAL,   -- Need N

srs-ResourceToAddModList                SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource                OPTIONAL,   -- Need N

tpc-Accumulation                        ENUMERATED {disabled}                                                   OPTIONAL,   -- Need S

...,

[[

srs-RequestDCI-1-2-r16                  INTEGER (1..2)                                                          OPTIONAL, -- Need S

srs-RequestDCI-0-2-r16                  INTEGER (1..2)                                                          OPTIONAL, -- Need S

srs-ResourceSetToAddModListDCI-0-2-r16  SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet          OPTIONAL, -- Need N

srs-ResourceSetToReleaseListDCI-0-2-r16 SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId        OPTIONAL, -- Need N

srs-PosResourceSetToReleaseList-r16     SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSetId-r16

OPTIONAL, -- Need N

srs-PosResourceSetToAddModList-r16      SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16        OPTIONAL,-- Need N

srs-PosResourceToReleaseList-r16        SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16            OPTIONAL,-- Need N

srs-PosResourceToAddModList-r16         SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16  OPTIONAL  -- Need N

]],

[[

dci-TriggeringPosResourceSetLink-r18    ENUMERATED { enabled }                                                  OPTIONAL  -- Need R

]]

}

SRS-ResourceSet ::=                     SEQUENCE {

srs-ResourceSetId                       SRS-ResourceSetId,

srs-ResourceIdList                      SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId    OPTIONAL, -- Cond Setup

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

aperiodicSRS-ResourceTrigger            INTEGER (1..maxNrofSRS-TriggerStates-1),

csi-RS                                  NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

slotOffset                              INTEGER (1..32)                                        OPTIONAL, -- Need S

...,

[[

aperiodicSRS-ResourceTriggerList            SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL  -- Need M

]]

},

semi-persistent                         SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

},

periodic                                SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

}

},

usage                                   ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching},

alpha                                   Alpha                                                          OPTIONAL, -- Need S

p0                                      INTEGER (-202..24)                                             OPTIONAL, -- Cond Setup

pathlossReferenceRS                     PathlossReferenceRS-Config                                     OPTIONAL, -- Need M

srs-PowerControlAdjustmentStates        ENUMERATED { sameAsFci2, separateClosedLoop}                   OPTIONAL, -- Need S

...,

[[

pathlossReferenceRSList-r16             SetupRelease { PathlossReferenceRSList-r16}                    OPTIONAL  -- Need M

]],

[[

usagePDC-r17                            ENUMERATED {true}                                              OPTIONAL, -- Need R

availableSlotOffsetList-r17             SEQUENCE (SIZE(1..4)) OF AvailableSlotOffset-r17               OPTIONAL, -- Need R

followUnifiedTCI-StateSRS-r17           ENUMERATED {enabled}                                           OPTIONAL  -- Need R

]],

[[

applyIndicatedTCI-State-r18             ENUMERATED {first, second}                                     OPTIONAL  -- Cond FollowUTCI

]]

}

AvailableSlotOffset-r17 ::=   INTEGER (0..7)

PathlossReferenceRS-Config ::=              CHOICE {

ssb-Index                                   SSB-Index,

csi-RS-Index                                NZP-CSI-RS-ResourceId

}

PathlossReferenceRSList-r16 ::=             SEQUENCE (SIZE (1..maxNrofSRS-PathlossReferenceRS-r16)) OF PathlossReferenceRS-r16

PathlossReferenceRS-r16 ::=                 SEQUENCE {

srs-PathlossReferenceRS-Id-r16              SRS-PathlossReferenceRS-Id-r16,

pathlossReferenceRS-r16                     PathlossReferenceRS-Config

}

SRS-PathlossReferenceRS-Id-r16 ::=          INTEGER (0..maxNrofSRS-PathlossReferenceRS-1-r16)

SRS-PosResourceSet-r16 ::=                  SEQUENCE {

srs-PosResourceSetId-r16                    SRS-PosResourceSetId-r16,

srs-PosResourceIdList-r16                   SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-PosResourceId-r16

OPTIONAL, -- Cond Setup

resourceType-r16                            CHOICE {

aperiodic-r16                               SEQUENCE {

aperiodicSRS-ResourceTriggerList-r16        SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-1))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL, -- Need M

...

},

semi-persistent-r16                         SEQUENCE {

...

},

periodic-r16                                SEQUENCE {

...

}

},

alpha-r16                                   Alpha                                                      OPTIONAL, -- Need S

p0-r16                                      INTEGER (-202..24)                                         OPTIONAL, -- Cond Setup

pathlossReferenceRS-Pos-r16                 CHOICE {

ssb-IndexServing-r16                        SSB-Index,

ssb-Ncell-r16                               SSB-InfoNcell-r16,

dl-PRS-r16                                  DL-PRS-Info-r16

}                                                                                                      OPTIONAL, -- Need M

...,

[[

ncd-SSB-r18                                  NonCellDefiningSSB-r17                                   OPTIONAL  -- Cond NCD-SSB

]]

}

SRS-ResourceSetId ::=                   INTEGER (0..maxNrofSRS-ResourceSets-1)

SRS-PosResourceSetId-r16 ::=            INTEGER (0..maxNrofSRS-PosResourceSets-1-r16)

SRS-Resource ::=                        SEQUENCE {

srs-ResourceId                          SRS-ResourceId,

nrofSRS-Ports                           ENUMERATED {port1, ports2, ports4},

ptrs-PortIndex                          ENUMERATED {n0, n1 }                                           OPTIONAL,   -- Need R

transmissionComb                        CHOICE {

n2                                      SEQUENCE {

combOffset-n2                           INTEGER (0..1),

cyclicShift-n2                          INTEGER (0..7)

},

n4                                      SEQUENCE {

combOffset-n4                           INTEGER (0..3),

cyclicShift-n4                          INTEGER (0..11)

}

},

resourceMapping                         SEQUENCE {

startPosition                           INTEGER (0..5),

nrofSymbols                             ENUMERATED {n1, n2, n4},

repetitionFactor                        ENUMERATED {n1, n2, n4}

},

freqDomainPosition                      INTEGER (0..67),

freqDomainShift                         INTEGER (0..268),

freqHopping                             SEQUENCE {

c-SRS                                   INTEGER (0..63),

b-SRS                                   INTEGER (0..3),

b-hop                                   INTEGER (0..3)

},

groupOrSequenceHopping                  ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

...

},

semi-persistent                         SEQUENCE {

periodicityAndOffset-sp                     SRS-PeriodicityAndOffset,

...

},

periodic                                SEQUENCE {

periodicityAndOffset-p                      SRS-PeriodicityAndOffset,

...

}

},

sequenceId                              INTEGER (0..1023),

spatialRelationInfo                     SRS-SpatialRelationInfo                                        OPTIONAL,   -- Need R

...,

[[

resourceMapping-r16                     SEQUENCE {

startPosition-r16                       INTEGER (0..13),

nrofSymbols-r16                         ENUMERATED {n1, n2, n4},

repetitionFactor-r16                    ENUMERATED {n1, n2, n4}

}                                                                                                      OPTIONAL    -- Need R

]],

[[

spatialRelationInfo-PDC-r17             SetupRelease { SpatialRelationInfo-PDC-r17 }                   OPTIONAL,   -- Need M

resourceMapping-r17                     SEQUENCE {

startPosition-r17                       INTEGER (0..13),

nrofSymbols-r17                         ENUMERATED {n1, n2, n4, n8, n10, n12, n14},

repetitionFactor-r17                    ENUMERATED {n1, n2, n4, n5, n6, n7, n8, n10, n12, n14}

}                                                                                                      OPTIONAL,   -- Need R

partialFreqSounding-r17                 SEQUENCE {

startRBIndexFScaling-r17                CHOICE{

startRBIndexAndFreqScalingFactor2-r17   INTEGER (0..1),

startRBIndexAndFreqScalingFactor4-r17   INTEGER (0..3)

},

enableStartRBHopping-r17                ENUMERATED {enable}                                        OPTIONAL    -- Need R

}                                                                                                      OPTIONAL,   -- Need R

transmissionComb-n8-r17                 SEQUENCE {

combOffset-n8-r17                       INTEGER (0..7),

cyclicShift-n8-r17                      INTEGER (0..5)

}                                                                                                      OPTIONAL,   -- Need R

srs-TCI-State-r17                       CHOICE {

srs-UL-TCI-State                        TCI-UL-StateId-r17,

srs-DLorJointTCI-State                  TCI-StateId

}                                                                                                      OPTIONAL    -- Need R

]],

[[

repetitionFactor-v1730                  ENUMERATED {n3}                                                OPTIONAL,   -- Need R

srs-DLorJointTCI-State-v1730            SEQUENCE {

cellAndBWP-r17                          ServingCellAndBWP-Id-r17

}                                                                                                 OPTIONAL -- Cond DLorJointTCI-SRS

]],

[[

nrofSRS-Ports-n8-r18                    ENUMERATED {ports8, ports8tdm}                                 OPTIONAL,   -- Need R

combOffsetHopping-r18                   SEQUENCE {

hoppingId-r18                           INTEGER (0..1023)                                          OPTIONAL,   -- Need R

hoppingSubset-r18                       CHOICE {

transmissionComb-n4                       BIT STRING (SIZE (4)),

transmissionComb-n8                       BIT STRING (SIZE (8))

}                                                                                                  OPTIONAL,   -- Need R

hoppingWithRepetition-r18               ENUMERATED {symbol, repetition}                            OPTIONAL    -- Need R

}                                                                                                      OPTIONAL,   -- Need R

cyclicShiftHopping-r18                  SEQUENCE {

hoppingId-r18                           INTEGER (0..1023)                                          OPTIONAL,   -- Need R

hoppingSubset-r18                       CHOICE {

transmissionComb-n2                       BIT STRING (SIZE (8)),

transmissionComb-n4                       BIT STRING (SIZE (12)),

transmissionComb-n8                       BIT STRING (SIZE (6))

}                                                                                                  OPTIONAL,   -- Need R

hoppingFinerGranularity-r18             ENUMERATED {enable}                                        OPTIONAL    -- Need R

}                                                                                                      OPTIONAL    -- Need R

]]

}

SRS-PosResource-r16::=                  SEQUENCE {

srs-PosResourceId-r16                   SRS-PosResourceId-r16,

transmissionComb-r16                    CHOICE {

n2-r16                                  SEQUENCE {

combOffset-n2-r16                       INTEGER (0..1),

cyclicShift-n2-r16                      INTEGER (0..7)

},

n4-r16                                  SEQUENCE {

combOffset-n4-r16                        INTEGER (0..3),

cyclicShift-n4-r16                      INTEGER (0..11)

},

n8-r16                                  SEQUENCE {

combOffset-n8-r16                       INTEGER (0..7),

cyclicShift-n8-r16                      INTEGER (0..5)

},

...

},

resourceMapping-r16                       SEQUENCE {

startPosition-r16                           INTEGER (0..13),

nrofSymbols-r16                             ENUMERATED {n1, n2, n4, n8, n12}

},

freqDomainShift-r16                       INTEGER (0..268),

freqHopping-r16                           SEQUENCE {

c-SRS-r16                                 INTEGER (0..63),

...

},

groupOrSequenceHopping-r16                ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType-r16                          CHOICE {

aperiodic-r16                             SEQUENCE {

slotOffset-r16                            INTEGER (1..32)                                      OPTIONAL,   -- Need S

...

},

semi-persistent-r16                       SEQUENCE {

periodicityAndOffset-sp-r16               SRS-PeriodicityAndOffset-r16,

...,

[[

periodicityAndOffset-sp-Ext-r16           SRS-PeriodicityAndOffsetExt-r16                      OPTIONAL    -- Need R

]],

[[

srs-PosPeriodicConfigHyperSFN-Index-r18   ENUMERATED {even0, odd1}                             OPTIONAL     --Need R

]]

},

periodic-r16                              SEQUENCE {

periodicityAndOffset-p-r16                SRS-PeriodicityAndOffset-r16,

...,

[[

periodicityAndOffset-p-Ext-r16            SRS-PeriodicityAndOffsetExt-r16                      OPTIONAL    -- Need R

]],

[[

srs-PosPeriodicConfigHyperSFN-Index-r18   ENUMERATED {even0, odd1}                             OPTIONAL     --Need R

]]

}

},

sequenceId-r16                            INTEGER (0..65535),

spatialRelationInfoPos-r16                SRS-SpatialRelationInfoPos-r16                               OPTIONAL,   -- Need R

...,

[[

txHoppingConfig-r18                       TxHoppingConfig-r18                                          OPTIONAL    --Need R

]],

[[

ncd-SSB-r18                               NonCellDefiningSSB-r17                                       OPTIONAL   -- Cond NCD-SSB

]]

}

SRS-SpatialRelationInfo ::=     SEQUENCE {

servingCellId                       ServCellIndex                                                      OPTIONAL,   -- Need S

referenceSignal                     CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId,

srs                                 SEQUENCE {

resourceId                          SRS-ResourceId,

uplinkBWP                           BWP-Id

}

}

}

SRS-SpatialRelationInfoPos-r16 ::=      CHOICE {

servingRS-r16                           SEQUENCE {

servingCellId                           ServCellIndex                                              OPTIONAL,   -- Need S

referenceSignal-r16                     CHOICE {

ssb-IndexServing-r16                    SSB-Index,

csi-RS-IndexServing-r16                 NZP-CSI-RS-ResourceId,

srs-SpatialRelation-r16                 SEQUENCE {

resourceSelection-r16                   CHOICE {

srs-ResourceId-r16                      SRS-ResourceId,

srs-PosResourceId-r16                   SRS-PosResourceId-r16

},

uplinkBWP-r16                           BWP-Id

}

}

},

ssb-Ncell-r16                           SSB-InfoNcell-r16,

dl-PRS-r16                              DL-PRS-Info-r16

}

SSB-Configuration-r16  ::=          SEQUENCE {

ssb-Freq-r16                     ARFCN-ValueNR,

halfFrameIndex-r16                  ENUMERATED {zero, one},

ssbSubcarrierSpacing-r16            SubcarrierSpacing,

ssb-Periodicity-r16                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2,spare1 }   OPTIONAL, -- Need S

sfn0-Offset-r16                     SEQUENCE {

sfn-Offset-r16                      INTEGER (0..1023),

integerSubframeOffset-r16           INTEGER (0..9)                                                 OPTIONAL  -- Need R

}                                                                                                      OPTIONAL, -- Need R

sfn-SSB-Offset-r16                  INTEGER (0..15),

ss-PBCH-BlockPower-r16              INTEGER (-60..50)                                                  OPTIONAL  -- Cond Pathloss

}

SSB-InfoNcell-r16  ::=              SEQUENCE {

physicalCellId-r16                  PhysCellId,

ssb-IndexNcell-r16                  SSB-Index                                                          OPTIONAL, -- Need S

ssb-Configuration-r16               SSB-Configuration-r16                                              OPTIONAL  -- Need S

}

DL-PRS-Info-r16  ::=                SEQUENCE {

dl-PRS-ID-r16                      INTEGER (0..255),

dl-PRS-ResourceSetId-r16           INTEGER (0..7),

dl-PRS-ResourceId-r16              INTEGER (0..63)                                                     OPTIONAL  -- Need S

}

SRS-ResourceId ::=                      INTEGER (0..maxNrofSRS-Resources-1)

SRS-PosResourceId-r16 ::=               INTEGER (0..maxNrofSRS-PosResources-1-r16)

SRS-PeriodicityAndOffset ::=            CHOICE {

sl1                                     NULL,

sl2                                     INTEGER(0..1),

sl4                                     INTEGER(0..3),

sl5                                     INTEGER(0..4),

sl8                                     INTEGER(0..7),

sl10                                    INTEGER(0..9),

sl16                                    INTEGER(0..15),

sl20                                    INTEGER(0..19),

sl32                                    INTEGER(0..31),

sl40                                    INTEGER(0..39),

sl64                                    INTEGER(0..63),

sl80                                    INTEGER(0..79),

sl160                                   INTEGER(0..159),

sl320                                   INTEGER(0..319),

sl640                                   INTEGER(0..639),

sl1280                                  INTEGER(0..1279),

sl2560                                  INTEGER(0..2559)

}

SRS-PeriodicityAndOffset-r16 ::=        CHOICE {

sl1                                     NULL,

sl2                                     INTEGER(0..1),

sl4                                     INTEGER(0..3),

sl5                                     INTEGER(0..4),

sl8                                     INTEGER(0..7),

sl10                                    INTEGER(0..9),

sl16                                    INTEGER(0..15),

sl20                                    INTEGER(0..19),

sl32                                    INTEGER(0..31),

sl40                                    INTEGER(0..39),

sl64                                    INTEGER(0..63),

sl80                                    INTEGER(0..79),

sl160                                   INTEGER(0..159),

sl320                                   INTEGER(0..319),

sl640                                   INTEGER(0..639),

sl1280                                  INTEGER(0..1279),

sl2560                                  INTEGER(0..2559),

sl5120                                  INTEGER(0..5119),

sl10240                                 INTEGER(0..10239),

sl40960                                 INTEGER(0..40959),

sl81920                                 INTEGER(0..81919),

...

}

SRS-PeriodicityAndOffsetExt-r16 ::=     CHOICE {

sl128                                   INTEGER(0..127),

sl256                                   INTEGER(0..255),

sl512                                   INTEGER(0..511),

sl20480                                 INTEGER(0..20479)

}

SpatialRelationInfo-PDC-r17 ::=   SEQUENCE {

referenceSignal                   CHOICE {

ssb-Index                         SSB-Index,

csi-RS-Index                      NZP-CSI-RS-ResourceId,

dl-PRS-PDC                        NR-DL-PRS-ResourceID-r17,

srs                               SEQUENCE {

resourceId                        SRS-ResourceId,

uplinkBWP                         BWP-Id

},

...

},

...

}

TxHoppingConfig-r18 ::=             SEQUENCE {

overlapValue-r18                    ENUMERATED {zeroRB, oneRB, twoRB, fourRB},

numberOfHops-r18                    INTEGER(1..6),

slotOffsetForRemainingHopsList-r18  SEQUENCE (SIZE (1..maxNrofHops-1-r18) ) OF SlotOffsetForRemainingHops-r18,

...

}

SlotOffsetForRemainingHops-r18 ::=  SEQUENCE {

slotOffsetRemainingHops-r18         CHOICE {

aperiodic-r18                       SEQUENCE {

slotOffset-r18                      INTEGER (1..32)                                            OPTIONAL,   -- Need S

startPosition-r18                   INTEGER (0..13)                                            OPTIONAL,   -- Need R

...

},

semi-persistent-r18                 SEQUENCE {

periodicityAndOffset-sp-r18         SRS-PeriodicityAndOffset-r16                               OPTIONAL,   -- Need R

periodicityAndOffset-sp-Ext-r18     SRS-PeriodicityAndOffsetExt-r16                            OPTIONAL,   -- Need R

startPosition-r18                   INTEGER (0..13)                                            OPTIONAL,   -- Need R

...

},

periodic-r18                        SEQUENCE {

periodicityAndOffset-p-r18          SRS-PeriodicityAndOffset-r16                               OPTIONAL,   -- Need R

periodicityAndOffset-p-Ext-r18      SRS-PeriodicityAndOffsetExt-r16                            OPTIONAL,   -- Need R

startPosition-r18                   INTEGER (0..13)                                            OPTIONAL,   -- Need S

...

},

...

}

}

-- TAG-SRS-CONFIG-STOP

-- ASN1STOP

SRS-Config field descriptions

dci-TriggeringPosResourceSetLink Indicates whether the single DCI-triggering SRS positioning resource sets across the linked carriers is enabled or not for bandwidth aggregation.

tpc-Accumulation If the field is absent, UE applies TPC commands via accumulation. If disabled, UE applies the TPC command without accumulation (this applies to SRS when a separate closed loop is configured for SRS) (see TS 38.213 [13], clause 7.3).

SRS-Resource, SRS-PosResource field descriptions

cyclicShift-n2 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

cyclicShift-n4 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

cyclicShift-n8 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

combOffsetHopping Configures UE with comb offset hopping. The hoppingId is used to initialize pseudo random comb offset hopping. If UE is configured with both comb offset and cyclic shift hopping, only one hoppingId is configured. The hoppingWithRepetition configures time-domain hopping behavior for repetition factor R>1. The hoppingSubset indicates a set of comb offset by a bit string (see clause 6.4.1.4.3 of TS 38.211 [16]). The i-th bit in the bit string is set to 1 to indicate the scohSRSt=i-1, where t is determined by its ordinary position among the positive bits in bit string, i.e., if the i-th bit is a first positive bit, t=0; if the i-th bit is a second positive bit, t=1 , and so on.

cyclicShiftHopping Configures UE with cyclic shift hopping. The hoppingId is used to initialize pseudo random cyclic shift hopping. If UE is configured with both comb offset and cyclic shift hopping, only one hoppingId is configured. The hoppingFinerGranularity enables finer granular hopping, see TS 38.211 [16], clause 6.4.1.4.2. If hoppingSubset is configured, hoppingFinerGranularity is not configured. The hoppingSubset indicates a set of cyclic shift by a bit string (see clause 6.4.1.4.2 of TS 38.211 [16]). The i-th bit in the bit string is set to 1 to indicate the scshSRSt=i-1, where t is determined by its ordinary position among the positive bits in bit string, i.e., if the i-th bit is a first positive bit, t=0; if the i-th bit is a second positive bit, t=1 , and so on.

enableStartRBHopping When this RRC parameter is configured, start RB location hopping is enabled for partial frequency sounding in different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS as described in clause 6.4.1.4 in TS 38.211.

freqHopping Includes parameters capturing SRS frequency hopping (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, the network always configures this field such that b-hop > b-SRS. For SRS for positioning configuration in multiple cells, the value of this field applies to all cells in the validity area. c-SRS Indicates the maximum bandwidth per hop. When TxHoppingConfig is configured the valid values for c-SRS are such that the maximum bandwidth is: 104 PRBs, 48 PRBs, 132 PRBs, 64 PRBs, for 15,30,60,120 KHz respectively. The same value for c-SRS is configured for all the hops when TxHoppingConfig is configured.

groupOrSequenceHopping Parameter(s) for configuring group or sequence hopping (see TS 38.211 [16], clause 6.4.1.4.2). For CLI SRS-RSRP measurement, the network always configures this parameter to 'neither'. For SRS for positioning configuration in multiple cells, the value of this field applies to all cells in the validity area.

nrofSRS-Ports Number of ports. For CLI SRS-RSRP measurement, the network always configures this parameter to 'port1'.

nrofSRS-Ports-n8 Number of ports if the number of antenna ports is 8. The value 'ports8' configures UE with 8 antenna ports and the value 'ports8tdm' configures the UE with 8 antenna ports which are partitioned into 2 subsets with each subset having 4 different ports, and the subsets are mapped to different OFDM symbols, see TS 38.211 [16], clause 6.4.1.4.2. If combOffsetHopping-r18 or cyclicShiftHopping-r18 is configured, this field is not set to ports8tdm. If this field is present UE ignores the field nrofSRS-Ports.

periodicityAndOffset-p, periodicityAndOffset-p-Ext Periodicity and slot offset for this SRS resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, sl1280 and sl2560 cannot be configured. For SRS-PosResource, values sl20480, sl40960 and sl81920 cannot be configured for SCS=15kHz, values sl40960 and sl81920 cannot be configured for SCS=30kHz, and value sl81920 cannot be configured for SCS=60kHz except when periodicity of 20480ms is configured. When periodicityAndOffset-p-Ext is present, periodicityAndOffset-p shall be ignored by the UE.

periodicityAndOffset-sp, periodicityAndOffset-sp-Ext Periodicity and slot offset for this SRS resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots (see TS 38.214 [19], clause 6.2.1). For SRS-PosResource, values sl20480, sl40960 and sl81920 cannot be configured for SCS=15kHz, values sl40960 and sl81920 cannot be configured for SCS=30kHz, and value sl81920 cannot be configured for SCS=60kHz. When periodicityAndOffset-sp-Ext is present, periodicityAndOffset-sp shall be ignored by the UE.

ptrs-PortIndex The PTRS port index for this SRS resource for non-codebook based UL MIMO. This is only applicable when the corresponding PTRS-UplinkConfig is set to CP-OFDM. The ptrs-PortIndex configured here must be smaller than the maxNrofPorts configured in the PTRS-UplinkConfig (see TS 38.214 [19], clause 6.2.3.1). This parameter is not applicable to CLI SRS-RSRP measurement.

resourceMapping OFDM symbol location of the SRS resource within a slot including nrofSymbols (number of OFDM symbols), startPosition (value 0 refers to the last symbol, value 1 refers to the second last symbol, and so on) and repetitionFactor (see TS 38.214 [19], clause 6.2.1 and TS 38.211 [16], clause 6.4.1.4). The configured SRS resource does not exceed the slot boundary. If resourceMapping-r16 is signalled, UE shall ignore the resourceMapping (without suffix). If resourceMapping-r17 is signalled, resourceMapping-r16 is not signalled and the UE shall ignore the resourceMapping (without suffix) and only the values of nrofSymbols which are integer multiples of the configured repetitionFactor can be configured. The network can only signal repetitionFactor-v1730 if resourceMapping-r17 is signalled. When repetitionFactor-v1730 is signalled, the UE shall ignore repetitionFactor-r17. For CLI SRS-RSRP measurement, the network always configures nrofSymbols and repetitionFactor to 'n1'. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area. nrofSymbols is same for all the hops when TxHoppingConfig is configured.

resourceType Periodicity and offset for semi-persistent and periodic SRS resource, or slot offset for aperiodic SRS resource for positioning (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, only 'periodic' is applicable for resourceType. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

sequenceId Sequence ID used to initialize pseudo random group and sequence hopping (see TS 38.214 [19], clause 6.2.1). If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

slotOffset An offset in number of slots between the triggering DCI and the actual transmission of this SRS-PosResource. If the field is absent the UE applies no offset (value 0).

spatialRelationInfo Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS (see TS 38.214 [19], clause 6.2.1). This parameter is not applicable to CLI SRS-RSRP measurement. This field is not configured if unifiedTCI-StateType is configured for the serving cell.

spatialRelationInfo-PDC Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS/DL-PRS-PDC (see TS 38.214 [19], clause 6.2.1). The field is present in case of resourceType=periodic and usagePDC-r17=true in the SRS-ResourceSet, otherwise the field is absent.

spatialRelationInfoPos Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS/DL-PRS (see TS 38.214 [19], clause 6.2.1). If the IE srs-ResourceId-Ext is present, the IE srs-ResourceId in spatialRelationInfoPos represents the index from 0 to 63. Otherwise the IE srs-ResourceId in spatialRelationInfoPos represents the index from 0 to 31. If the SRS for positioning is configured for RRC_INACTIVE state, the srs-ResourceId and csi-RS-IndexServing are not included in this field.

srs-PosPeriodicConfigHyperSFN-Index Indicates even or odd hyper SFN in which the SRS for positioning is transmitted for the periodicity value of 20480m. If this field is not configured, the UE assumes that SRS for positioning periodictity longer than one Hyper SFN is not configured.

srs-RequestDCI-0-2 Indicate the number of bits for "SRS request" in DCI format 0_2. When the field is absent, then the value of 0 bit for "SRS request" in DCI format 0_2 is applied. If the parameter srs-RequestDCI-0-2 is configured to value 1, 1 bit is used to indicate one of the first two rows of Table 7.3.1.1.2-24 in TS 38.212 [17] for triggered aperiodic SRS resource set. If the value 2 is configured, 2 bits are used to indicate one of the rows of Table 7.3.1.1.2-24 in TS 38.212 [17]. When UE is configured with supplementaryUplink, an extra bit (the first bit of the SRS request field) is used for the non-SUL/SUL indication.

srs-RequestDCI-1-2 Indicate the number of bits for "SRS request" in DCI format 1_2. When the field is absent, then the value of 0 bit for "SRS request" in DCI format 1_2 is applied. When the UE is configured with supplementaryUplink, an extra bit (the first bit of the SRS request field) is used for the non-SUL/SUL indication (see TS 38.214 [19], clause 6.1.1.2).

srs-ResourceSetToAddModListDCI-0-2 List of SRS resource set to be added or modified for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

srs-ResourceSetToReleaseListDCI-0-2 List of SRS resource set to be released for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

srs-TCI-State Configuration of either a UL TCI state or a joint TCI state for the SRS resource. In case of UL TCI-State, refers to the TCI state defined in ul-TCI-StateList in the BWP-UplinkDedicated where the SRS-Config is configured. In case of joint TCI state, refers to a TCI state defined in dl-OrJointTCI-StateList in pdsch-Config of the BWP-DownlinkDedicated and serving cell indicated by cellAndBWP. This field is absent when the SRS resource is in an SRS-ResourceSet configured with followUnifiedTCI-StateSRS-r17 or applyIndicatedTCI-State, or when the field unifiedTCI-StateType is not configured to the serving cell which the SRS resource is located in.

startRBIndexAndFreqScalingFactor Configures the UE with the startRBIndex and freqScalingFactor for partial frequency sounding as described in Clause 6.4.1.4 in TS 38.211. The startRBIndexForFScaling2 gives the startRBIndex when freqScalingFactor is 2 and the startRBIndexForFScaling4 gives the startRBIndex when FreqScalingFactor is 4

transmissionComb, transmissionComb-n2, transmissionComb-n4, transmissionComb-n8 Comb value (2 or 4 or 8) and comb offset (0..combValue-1) (see TS 38.214 [19], clause 6.2.1). If network configures field transmissionComb-n8, the UE ignores transmissionComb. If srs-PosRRC-InactiveValidityAreaPreConfig or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

SRS-ResourceSet, SRS-PosResourceSet field descriptions

alpha alpha value for SRS power control (see TS 38.213 [13], clause 7.3). When the field is absent the UE applies the value 1. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

aperiodicSRS-ResourceTriggerList An additional list of DCI "code points" upon which the UE shall transmit SRS according to this SRS resource set configuration (see TS 38.214 [19], clause 6). When the field is not included during a reconfiguration of SRS-ResourceSet of resourceType set to aperiodic, UE maintains this value based on the Need M; that is, this list is not considered as an extension of aperiodicSRS-ResourceTrigger for purpose of applying the general rule for extended list in clause 6.1.3.

aperiodicSRS-ResourceTrigger The DCI "code point" upon which the UE shall transmit SRS according to this SRS resource set configuration (see TS 38.214 [19], clause 6).

applyIndicatedTCI-State This field indicates, for an SRS-ResourceSet, if UE applies the first or the second "indicated" UL only TCI or joint TCI as specified in TS 38.214 [19], clause 6.2.1.

associatedCSI-RS ID of CSI-RS resource associated with this SRS resource set in non-codebook based operation (see TS 38.214 [19], clause 6.1.1.2).

availableSlotOffsetList Indicates a list of up to four different available slot offset values from slot n+k to the slot where the aperiodic SRS resource set is transmitted, where slot n is the slot with the triggering DCI, and k is the slotOffset (without suffix) as described in clause 6.2.1 of TS 38.214 [19].

csi-RS ID of CSI-RS resource associated with this SRS resource set (see TS 38.214 [19], clause 6.1.1.2).

dl-PRS This field indicates a PRS configuration.

followUnifiedTCI-StateSRS When set to enabled, for SRS resource Set, the UE applies the "indicated" UL only TCI or joint TCI as specified in TS 38.214 [19], clause 5.1.5. This parameter may be configured for aperiodic SRS for BM or SRS of any time-domain behavior for codebook, non-codebook, and antenna switching.

p0 P0 value for SRS power control. The value is in dBm. Only even values (step size 2) are allowed (see TS 38.213 [13], clause 7.3). If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

pathlossReferenceRS A reference signal (e.g. a CSI-RS config or a SS block) to be used for SRS path loss estimation (see TS 38.213 [13], clause 7.3).

pathlossReferenceRS-Pos A reference signal (e.g. a SS block or a DL-PRS config) to be used for SRS path loss estimation (see TS 38.213 [13], clause 7.3).

pathlossReferenceRSList Multiple candidate pathloss reference RS(s) for SRS power control, where one candidate RS can be mapped to SRS Resource Set via MAC CE (clause 6.1.3.27 in TS 38.321 [3]). The network can only configure this field if pathlossReferenceRS is not configured in the same SRS-ResourceSet.

resourceType Time domain behavior of SRS resource configuration, see TS 38.214 [19], clause 6.2.1. The network configures SRS resources in the same resource set with the same time domain behavior on periodic, aperiodic and semi-persistent SRS. The aperiodic SRS is not applicable for the UE in RRC_INACTIVE. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

slotOffset An offset in number of slots between the triggering DCI and the actual transmission of this SRS-ResourceSet. If the field is absent the UE applies no offset (value 0).

srs-PowerControlAdjustmentStates Indicates whether hsrs,c(i) = fc(i,1) or hsrs,c(i) = fc(i,2) (if twoPUSCH-PC-AdjustmentStates are configured) or separate close loop is configured for SRS. This parameter is applicable only for Uls on which UE also transmits PUSCH. If absent or release, the UE applies the value sameAs-Fci1 (see TS 38.213 [13], clause 7.3).

srs-ResourceIdList, srs-PosResourceIdList The IDs of the SRS-Resources/SRS-PosResource used in this SRS-ResourceSet/SRS-PosResourceSet. If this SRS-ResourceSet is configured with usage set to codebook, the srs-ResourceIdList contains at most 2 entries. If this SRS-ResourceSet is configured with usage set to nonCodebook, the srs-ResourceIdList contains at most 4 entries. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, srs-PosResourceIdList is commonly configured across cells within the validity area.

srs-ResourceSetId, srs-PosResourceSetId The ID of this resource set. It is unique in the context of the BWP in which the parent SRS-Config is defined. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, srs-PosResourceSetId is commonly configured across cells within the validity area.

ssb-IndexServing Indicates SSB index belonging to a serving cell where the SRS is configured.

ssb-Ncell This field indicates a SSB configuration from neighboring cell.

usage Indicates if the SRS resource set is used for beam management, codebook based or non-codebook based transmission or antenna switching. See TS 38.214 [19], clause 6.2.1. Reconfiguration between codebook based and non-codebook based transmission is not supported.

usagePDC If configured, it indicates that this SRS resource set is used for propagation delay compensation. The field can be present in only one SRS-ResourceSet.

SRS-SpatialRelationInfoPos field descriptions

csi-RS-IndexServing Indicates CSI-RS index belonging to a serving cell.

dl-PRS This field indicates a PRS configuration.

resourceSelection Indicates whether the configured SRS spatial relation resource is a SRS-Resource or SRS-PosResource.

servingCellId The serving Cell ID of the source SSB, CSI-RS, or SRS for the spatial relation of the target SRS resource. If this field is absent the SSB, the CSI-RS, or the SRS is from the same serving cell where the SRS is configured.

ssb-IndexServing Indicates SSB index belonging to a serving cell.

ssb-Ncell This field indicates a SSB configuration from neighboring cell.

SSB-InfoNCell field descriptions

physicalCellId This field specifies the physical cell ID of the neighbour cell or NCD-SSB of the serving cell for which SSB configuration is provided.

ssb-IndexNcell This field specifies the index of the SSB for a neighbour cell or NCD-SSB of the serving cell. See TS 38.213 [13]. If this field is absent, the UE determines the ssb-IndexNcell of the physicalCellId based on its SSB measurement from the cell.

ssb-Configuration This field specifies the full configuration of the SSB. If this field is absent, the UE obtains the configuration for the SSB from nr-SSB-Config received as part of DL-PRS assistance data in LPP, see TS 37.355 [49], by looking up the corresponding SSB configuration using the field physicalCellId.

DL-PRS-Info field descriptions

dl-PRS-ID This field specifies the UE specific TRP ID (see TS 37.355 [49]) for which PRS configuration is provided.

dl-PRS-ResourceSetId This field specifies the PRS-ResourceSet ID of a PRS resourceSet.

dl-PRS-ResourceId This field specifies the PRS-Resource ID of a PRS resource. If this field is absent, the UE determines the dl-PRS-ResourceID based on its PRS measurement from the TRP (see TS 37.355 [49]) and DL-PRS Resource Set.

SSB-Configuration field descriptions

halfFrameIndex Indicates whether SSB is in the first half or the second half of the frame. Value zero indicates the first half and value 1 indicates the second half.

integerSubframeOffset Indicates the subframe boundary offset of the cell in which SSB is transmited.

sfn0-Offset Indiactes the time offset of the SFN0 slot 0 for the cell with respect to SFN0 slot 0 of serving cell.

sfn-Offset Specifies the SFN offset between the cell in which SSB is transmited and serving cell. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of serving cell to the beginning of the closest subsequent radio frame #0 of the cell in which SSB is transmitted.

sfn-SSB-Offset Indicates the SFN offset of the transmitted SSB relative to the start of the SSB period. Value 0 indicates that the SSB is transmitted in the first system frame, value 1 indicates that SSB is transmitted in the second system frame and so on. The network configures this field according to the field ssb-Periodicity such that the indicated system frame does not exceed the configured SSB periodicity.

ssb-Freq Indicates the frequency of the SSB.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

ssb-Periodicity Indicates the periodicity of the SSB. If the field is absent, the UE applies the value ms5. (see TS 38.213 [13], clause 4.1)

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

ncd-SSB This field indicates that the UE uses NCD-SSB for cell provided in field ssb-Ncell.

TxHoppingConfig field descriptions

numberOfHops This field specifies the number of hops. Value 1 indicates one hop, value 2 indicates two hops and so on.

overlapValue This field specifies the overlap during SRS transmission in terms of number of resource blocks. Value zeroRB implies 0 RB, value oneRB corresponds to 1 RB, value twoRB corresponds to 2 RBs and so on. The same value for the overlapValue is configured to all the hops.

slotOffsetForRemainingHopsList This field specifies the starting slot offset and starting symbol for the SRS resource with tx hopping for different resource types (aperiodic, semi-persistent or periodic SRS transmission). Each hop is configured with the same periodicity.

| Conditional Presence | Explanation |
| --- | --- |
| DLorJointTCI-SRS | The field is mandatory present if srs-DLorJointTCI-State is configured, otherwise it is absent Need R. |
| FollowUTCI | The field is optionally present, Need R, if the field followUnifiedTCI-StateSRS is present. Otherwise, it is absent. |
| NonCodebook | This field is optionally present, Need M, in case of non-codebook based transmission, otherwise the field is absent. |
| Pathloss | The field is mandatory present if the IE SSB-InfoNcell is included in pathlossReferenceRS-Pos; otherwise it is optionally present, Need R |
| Setup | This field is mandatory present upon configuration of SRS-ResourceSet or SRS-Resource and optionally present, Need M, otherwise. |
| NCD-SSB | The field is optionally present, Need S, if nonCellDefiningSSB is configured in this DL BWP. It is absent otherwise. |

End of Changes