3GPP TSG-RAN WG2 Meeting #132	R2-2508996

Dallas, USA, Nov. 17th - 21st

Agenda item:	8.5.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on remaining issues in MAC for R19 NES

Document for:	Discussion and Decision

1. Introduction

In this contribution, we further discuss some remaining issues in MAC specs for NES with respect to OD-SIB1.

2. Discussion

In the current TS 38.331 specs, the following three parameters are introduced for SIB1 request resources (SIB1-RequestResources):

38331-j00 – SIB26 SIB26 contains information to assist OD-SIB1 acquisition from serving cell or neighbour cells. SIB26 information element SIB1-RequestResources-r19 ::= SEQUENCE { sib1-RA-PreambleStartIndex-r19 INTEGER (0..63), sib1-RA-AssociationPeriodIndex-r19 INTEGER (0..15) OPTIONAL, -- Need R sib1-RA-SSB-OccasionMaskIndex-r19 INTEGER (0..15) OPTIONAL -- Need R } SIB1-RequestResources field descriptions. sib1-RA-AssociationPeriodIndex Index of the association period in the sib1-RequestPeriod in which the UE can send the SIB1 request for SIB using the preambles indicated by sib1-RA-PreambleStartIndex and rach occasions indicated by sib1-RA-SSB-OccasionMaskIndex.. sib1-RA-PreambleStartIndex If N SSBs are associated with a RACH occasion, where N > = 1, for the i-th SSB (i=0, …, N-1) the preamble with preamble index = sib1-RA-PreambleStartIndex + i is used for SIB1 request; For N < 1, the preamble with preamble index = sib1-RA-PreambleStartIndex is used for SIB1 request.. sib1-RA-SSB-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3]..

Based on the description of sib1-RA-AssociationPeriodIndex, we can see that this parameter is optional and indicates the index of the association period within the sib1-RequestPeriod. However, in clause 5.1.2 of TS 38.321, the index of association period is erroneously taken from sib1-RA-SSB-OccasionMaskIndex (if present), as shown below:

38321-j00 5.1.2 Random Access Resource selection If the selected RA_TYPE is set to 4-stepRA, the MAC entity shall: ... 1> if ra-AssociationPeriodIndex and si-RequestPeriod are configured: 2> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB in the association period given by ra-AssociationPeriodIndex in the si-RequestPeriod permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] corresponding to the selected SSB). 1> if the Random Access procedure was initiated for SIB1 request (as specified in TS 38.331 [5]); and 1> if sib1-RA-SSB-OccasionMaskIndex and sib1-RequestPeriod are configured: 2> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB in the association period given by sib1-RA-SSB-OccasionMaskIndex in the sib1-RequestPeriod permitted by the restrictions given by the ra-ssb-OccasionMaskIndexSib1 if configured (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] corresponding to the selected SSB).

Therefore, we suggest that the yellow-highlighted text, i.e. sib1-RA-SSB-OccasionMaskIndex to be changed to sib1-RA-AssociationPeriodIndex.

In addition, the green-highlighted IE, i.e. ra-ssb-OccasionMaskIndexSib1 does not match the one defined in TS 38.331; i.e., ra-ssb-OccasionMaskIndexSib1 should be replaced with sib1-RA-SSB-OccasionMaskIndex.

Proposal 1: Clarify in the NR MAC specs that when both sib1-RA-AssociationPeriodIndex and sib1-RequestPeriod are configured, the index of association period shall be determined by sib1-RA-AssociationPeriodIndex.

Proposal 2: Align the NR MAC specs with the NR RRC specs by replacing ra-ssb-OccasionMaskIndexSib1 with sib1-RA-SSB-OccasionMaskIndex.

A TP has been provided in Annex 1 for P1 and P2.

3. Conclusion

In this contribution, we analyzed the remaining issue in MAC. The following proposals were made:

Proposal 1: Clarify in the NR MAC specs that when both sib1-RA-AssociationPeriodIndex and sib1-RequestPeriod are configured, the index of association period shall be determined by sib1-RA-AssociationPeriodIndex.

Proposal 2: Align the NR MAC specs with the NR RRC specs by replacing ra-ssb-OccasionMaskIndexSib1 with sib1-RA-SSB-OccasionMaskIndex.

4. Reference

[1]. 3GPP TS 38.321-j00: "NR; Medium Access Control (MAC) protocol specification".

[2]. 3GPP TS 38.331-j00: "NR; Radio Resource Control (RRC); Protocol specification"..

Annex - TP for P1 and P2

Start of changes

5.1.2	Random Access Resource selection

If the selected RA_TYPE is set to 4-stepRA, the MAC entity shall:

1>	if the Random Access procedure was initiated for SpCell beam failure recovery (as specified in clause 5.17); and

1>	if the beamFailureRecoveryTimer (in clause 5.17) is either running or not configured; and

1>	if the contention-free Random Access Resources for beam failure recovery request associated with any of the SSBs and/or CSI-RSs have been explicitly provided by RRC; and

1>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB amongst the SSBs in candidateBeamRSList or the CSI-RSs with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the CSI-RSs in candidateBeamRSList is available:

2>	select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the SSBs in candidateBeamRSList or a CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the CSI-RSs in candidateBeamRSList;

2>	if CSI-RS is selected, and there is no ra-PreambleIndex associated with the selected CSI-RS:

3>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the SSB in candidateBeamRSList which is quasi-colocated with the selected CSI-RS as specified in TS 38.214 [7].

2>	else:

3>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB or CSI-RS from the set of Random Access Preambles for beam failure recovery request.

1>	else if the ra-PreambleIndex has been explicitly provided by PDCCH; and

1>	if the ra-PreambleIndex is not 0b000000:

2>	set the PREAMBLE_INDEX to the signalled ra-PreambleIndex;

2>	select the SSB signalled by PDCCH.

1>	else if contention-free Random Access Resources have been explicitly provided by an(Enhanced) LTM Cell Switch Command MAC CE and the SS-RSRP of the SSB signalled by the (Enhanced) LTM Cell Switch Command MAC CE is above rsrp-ThresholdSSB:

2>	set the PREAMBLE_INDEX to the Random Access Preamble index signalled by the (Enhanced) LTM Cell Switch Command MAC CE;

2>	select the SSB signalled by the (Enhanced) LTM Cell Switch Command MAC CE.

1>	else if contention-free Random Access Resources have not been explicitly provided by an (Enhanced) LTM Cell Switch Command MAC CE, the Random Access procedure was not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3, contention-free Random Access Resources associated with SSBs have been explicitly provided in rach-ConfigDedicated and at least one SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs is available:

2>	select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB.

1>	else if contention-free Random Access Resources have not been explicitly provided by an (Enhanced) LTM Cell Switch Command MAC CE, the Random Access procedure was not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3, contention-free Random Access Resources associated with CSI-RSs have been explicitly provided in rach-ConfigDedicated and at least one CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the associated CSI-RSs is available:

2>	select a CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the associated CSI-RSs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected CSI-RS.

1>	else if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and

1>	if the Random Access Resources for SI request have been explicitly provided by RRC:

2>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	select a Random Access Preamble corresponding to the selected SSB, from the Random Access Preamble(s) determined according to ra-PreambleStartIndex as specified in TS 38.331 [5];

2>	set the PREAMBLE_INDEX to selected Random Access Preamble.

1>	if the Random Access procedure was initiated for SIB1 request (as specified in TS 38.331 [5]); and

1>	if the Random Access Resources for SIB1 request have been provided by RRC:

2>	if at least one of the SSBs with SS-RSRP above sib1-rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above sib1-rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	select a Random Access Preamble corresponding to the selected SSB, from the Random Access Preamble(s) determined according to sib1-RA-PreambleStartIndex as specified in TS 38.331 [5];

2>	set the PREAMBLE_INDEX to selected Random Access Preamble.

1>	else (i.e. for the contention-based Random Access preamble selection):

2>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	if the RA_TYPE is switched from 2-stepRA to 4-stepRA:

3>	if a Random Access Preambles group was selected during the current Random Access procedure:

4>	select the same group of Random Access Preambles as was selected for the 2-step RA type.

3>	else:

4>	if Random Access Preambles group B is configured; and

4>	if the transport block size of the MSGA payload configured in the rach-ConfigDedicated corresponds to the transport block size of the MSGA payload associated with Random Access Preambles group B:

5>	select the Random Access Preambles group B.

4>	else:

5>	select the Random Access Preambles group A.

2>	else if Msg3 buffer is empty:

3>	if Random Access Preambles group B is configured:

4>	if the RO_TYPE is set to 2nd-RO and sbfd-RACH-SingleConfig-preambleReceivedTargetPower is configured for the Random Access procedure, and the potential Msg3 size (UL data available for transmission plus MAC subheader(s) and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – sbfd-RACH-SingleConfig-preambleReceivedTargetPower – msg3-DeltaPreamble – messagePowerOffsetGroupB; or

4>	if the RO_TYPE is set to 2nd-RO and sbfd-RACH-DualConfig is configured for the Random Access procedure, and the potential Msg3 size (UL data available for transmission plus MAC subheader(s) and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – preambleReceivedTargetPower (included in the sbfd-RACH-DualConfig) – msg3-DeltaPreamble – messagePowerOffsetGroupB; or

4>	if the RO_TYPE is set to 2nd-RO and sbfd-RACH-SingleConfig is configured and sbfd-RACH-SingleConfig-preambleReceivedTargetPower is not configured for the Random Access procedure, and the potential Msg3 size (UL data available for transmission plus MAC subheader(s) and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – preambleReceivedTargetPower – msg3-DeltaPreamble – messagePowerOffsetGroupB; or

4>	if the RO_TYPE is set to 1st-RO, and the potential Msg3 size (UL data available for transmission plus MAC subheader(s) and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – preambleReceivedTargetPower – msg3-DeltaPreamble – messagePowerOffsetGroupB; or

4>	if the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC subheader is greater than ra-Msg3SizeGroupA:

5>	select the Random Access Preambles group B.

4>	else:

5>	select the Random Access Preambles group A.

3>	else:

4>	select the Random Access Preambles group A.

2>	else (i.e. Msg3 is being retransmitted):

3>	select the same group of Random Access Preambles as was used for the Random Access Preamble transmission attempt corresponding to the first transmission of Msg3.

2>	select a Random Access Preamble randomly with equal probability from the Random Access Preambles associated with the selected SSB and the selected Random Access Preambles group;

2>	set the PREAMBLE_INDEX to the selected Random Access Preamble.

1>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and

1>	if ra-AssociationPeriodIndex and si-RequestPeriod are configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB in the association period given by ra-AssociationPeriodIndex in the si-RequestPeriod permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] corresponding to the selected SSB).

1>	if the Random Access procedure was initiated for SIB1 request (as specified in TS 38.331 [5]); and

1>	if sib1-RA-AssociationPeriodIndex and sib1-RequestPeriod are configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB in the association period given by sib1-RA-AssociationPeriodIndex in the sib1-RequestPeriod permitted by the restrictions given by the sib1-RA-SSB-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] corresponding to the selected SSB).

1>	else if an SSB is selected above:

2>	if the set of Random Access resources associated with Msg1 repetition is selected for this Random Access procedure:

3>	determine the next available set of PRACH occasions of the selected RO type (as specified in TS 38.213 [6]) for the Msg1 repetition number applicable for this Random Access procedure corresponding to the selected SSB (the MAC entity shall select a set of PRACH occasions randomly with equal probability amongst sets of PRACH occasions of the selected RO type according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB and selected Msg1 repetition number for this Random Access procedure; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available set of PRACH occasions of the selected RO type corresponding to the selected SSB).

2>	else:

3>	determine the next available PRACH occasion from the PRACH occasions of the selected RO type corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, or ssb-SharedRO-MaskIndex if configured, or indicated by PDCCH, or indicated by the (Enhanced) LTM Cell Switch Command MAC CE (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions of the selected RO type according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available PRACH occasion of the selected RO type corresponding to the selected SSB).

End of changes