3GPP TSG RAN WG2 Meeting #132	 R2-2508997

Dallas, USA, Oct. 17th-21st, 2025

Agenda Item:	10.3.3.2

Source: 	ITL

Title: 	Discussion on energy efficiency for 6GR

Document for:	Discussion and decision

Introduction

In RAN#108, a new Study Item (SI) on 6G Radio (6GR) was approved [1]. This contribution discusses energy efficiency aspects with the intention of facilitating further study in RAN2.

Discussion

Energy efficiency for RRC sub-state

In RAN2#131bis, the following agreement was reached regarding the RRC INACTIVE or sub-state.

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

Energy efficiency is expected to be one of the key design objectives for the 6G RRC state model. While 5G mainly focused on UE-side power saving through mechanisms such as C-DRX and I-DRX, the 6G framework is expected to target joint power optimization for both UE and network (RAN).

The 6G sub-state is envisioned as a lightweight state that minimizes control-plane overhead while allowing both UE and RAN nodes to enter low-power conditions in a coordinated and adaptive manner.

In NR RRC INACTIVE, the DRX operation is semi-static, configured via RRC signaling by the network. Once configured, the UE follows a fixed paging cycle and DRX pattern during the inactive period without dynamic adaptation based on traffic activity or network conditions.

This semi-static configuration limits the flexibility of power saving and responsiveness, since neither the UE nor the network can adjust their sleep timing. Therefore, it is considered beneficial to study adaptive DRX operation in the sub-state that enables both UE and network to dynamically coordinate their low-power operation according to traffic and system behavior.

To enable adaptive and coordinated power saving in the sub-state, two operation approaches can be considered for adjusting the DRX operation:

- Option 1: Traffic Activity-based Adaptation: In this mode, the UE autonomously adjusts its DRX cycle and on-duration timing based on its own traffic pattern or service activity.

  - When the UE detects bursty or frequent small data transmissions, it can shorten its DRX cycle to improve responsiveness.

  - Conversely, during long idle periods or predictable low-activity intervals, the UE can extend its DRX sleep duration to maximize power saving.

- Option 2: Network configuration/indication-based Adaptation: In this mode, the RAN dynamically provides DRX configuration and/or adjustment indications to the UE, considering the network-side load, scheduling window, and power-saving objectives of RAN nodes.

  - For example, the gNB may broadcast DRX pattern indications aligned with network sleep cycles, or send dynamic DRX update commands to UEs during low-traffic periods.

  - This approach allows coordinated sleep scheduling between the UE and the network, enabling both sides to enter low-power states simultaneously.

In this approach, the UE can dynamically adjust its DRX parameters (such as sleep duration or on-duration timing) based on traffic activity and/or network configuration/indication, while the network can align its resource activation and scheduling windows accordingly.

Proposal 1: Study adaptive DRX operation in sub-state

BWP (Bandwidth part) in multi-carrier serving cell

In 5G NR, the concept of Bandwidth Part was introduced to address the challenge of supporting wide system bandwidths (up to 400 MHz) while maintaining device energy efficiency. Instead of requiring the UE to continuously monitor and process the full carrier bandwidth, NR allows the operation on a smaller portion of spectrum through BWP configuration.

A BWP is defined as a contiguous set of physical resource blocks within the overall carrier bandwidth.

UE Operation:
Each UE can be configured with multiple BWPs, but at a given time, only one downlink BWP and one uplink BWP are active.

- Initial BWP: Used at RRC connection setup or cell access.

- Active BWP: The currently used BWP for data transmission/reception.

- Switching: The network may instruct the UE to switch to another BWP depending on traffic demand, service requirements, or energy saving considerations.

Benefits:

- Enables UE power saving by operating on a narrower BWP under low traffic.

- Provides flexibility to switch to wider BWPs for higher throughput services (e.g., eMBB).

- Supports QoS differentiation by associating different BWPs with different service needs.

In 6G, the Multi-Carrier Serving Cell (MCSC) concept is proposed to support flexible spectrum aggregation by managing multiple carriers under a serving cell framework.
In this environment, one or more carriers may dynamically enter energy-saving states, such as partial deactivation, sleep mode, or temporary shutdown, depending on network load or service demand.

In the 6G MCSC environment, BWP operation should be studied in RAN2, since it directly impacts UE power saving, signaling, protocol procedures, and overall system efficiency. Compared to 5G NR, the following issues can be identified:

1. Unified vs. Per-Carrier BWP Control

- In 5G: each carrier has independent BWP operation and switching.

- In 6G MCSC: multiple carriers are grouped as one serving cell → question arises whether BWP should be managed per-carrier or jointly across carriers.

- Issue: Increased signaling complexity if per-carrier management remains; loss of flexibility if only unified management is enforced.

2. HARQ / DRX Alignment

- In 5G: BWP switching is linked to cell-specific DRX/HARQ timers.

- In MCSC: unified HARQ/DRX operation may conflict with carrier-specific BWP configurations.

- Issue: Potential mismatch between per-carrier scheduling needs and unified timers

3. Cross-Carrier Scheduling and Control Monitoring

- In 5G: PDCCH monitoring is tied to a specific BWP per serving cell.

- In MCSC: one control channel may need to schedule multiple carriers with potentially different active BWPs.

- Issue: UE complexity and increasd energy consumption when monitoring control channels across multiple carriers.

4. Energy Saving vs. Performance Trade-off

- In 5G: narrow BWPs help reduce UE energy but limit throughput.

- In MCSC: simultaneous wideband operation across multiple carriers may dilute energy saving benefits unless joint BWP scaling is supported.

- Issue: How to design BWP scaling rules that balance performance and energy efficiency in multi-carrier environments.

Observation 1: New protocol support is needed in RAN2 to manage BWP switching and signaling across multiple carriers under a single serving cell

Proposal 2: Study enhancements of BWP operation for 6GR in the MCSC environment, including:

- Signaling and protocol aspects for unified or coordinated BWP management across carriers

- Mechanisms to ensure efficient HARQ/DRX alignment in multi-carrier serving cell operation

- Procedures for control channel monitoring and cross-carrier scheduling under BWP operation

- Solutions that balance UE energy saving and network performance when operating with multiple carriers

Conclusion

In this contribution, we discussed the energy efficiency aspects. According to the discussion in Section 2, we have the following observation and proposals:

Proposal 1: Study adaptive DRX operation in sub-state

Observation 1: New protocol support is needed in RAN2 to manage BWP switching and signaling across multiple carriers under a single serving cell

Proposal 2: Study enhancements of BWP operation for 6GR in the MCSC environment, including:

- Signaling and protocol aspects for unified or coordinated BWP management across carriers

- Mechanisms to ensure efficient HARQ/DRX alignment in multi-carrier serving cell operation

- Procedures for control channel monitoring and cross-carrier scheduling under BWP operation

- Solutions that balance UE energy saving and network performance when operating with multiple carriers

Reference

RP-252912 revised SID: Study on 6G radio