3GPP TSG-RAN WG2 Meeting #132	                                                              R2-2509027

Dallas, USA, 17th – 21st Nov. 2025

Agenda item:	10.3.1.2

Source:	CEWiT

Title:	Considerations for enhanced service awareness in 6GR

Document for:	Discussion and Decision

# Introduction

In RAN#109 meeting, revised SID on 6G Radio (6GR) was approved [1]. Under the list of items considered for 6GR, the following bullet defines the need to discuss the design of UP for 6GR

- User Plane architecture and protocol design

In RAN2#131bis following up on the UP design the following agreement was made.

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB).

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

The document mainly focuses on the considerations for enhanced service awareness in 6GR based on bullet 4 in the agreement.

# Discussion

In 5G NR, a MAC TB contain several higher layer packets with different QoS requirements but all of them will get same treatment at MAC and PHY layer. For example, even if reliability required by two MAC sub PDUs are different, MAC and PHY layer processing offer same reliability to them. Also in receiver, even if one portion of the MAC TB, for e.g. a CB is failed all the other success CBs cannot be passed to the higher layer, this causes head of line blocking issue and as a result it affects the packet latency. In these senses, service awareness at lower layers in limited in 5G NR.

Observation 1: In 5G NR, service awareness at MAC layer is limited

Observation 2: In 5G NR, MAC layer multiplexing causes head of line blocking issue

It will be beneficial to study for bringing more service awareness in MAC, for example enabling MAC layer to do different processing according to the service requirements of latency and reliability of the higher layer packets.

Proposal 1: Study protocol design to have improved service awareness at MAC layer for satisfying service specific requirements of latency and reliability

For some applications demanding higher reliability along with small latency, ARQ based retransmission may not be able to satisfy latency requirement and small number of HARQ retransmissions may not be sufficient to satisfy required reliability requirement. In the 5G NR HARQ scheme, even though HARQ failure to decode a MAC TB is known to the transmitter L2, ARQ will start retransmission based on the NACK reception at the transmitter RLC.

Observation 3. In 5G NR, ARQ retransmission mechanism that ensures reliability requirement for a service introduces delay in retransmission. For every ARQ retransmission, sequence number related header processing and its overhead is attached to it.

Observation 4. In 5G NR, HARQ retransmission mechanism does not differentiate between packets to achieve the QoS Flow’s demand of reliability.

Observation 5: In 5G NR, if a PDU or PDU set is not delivered in time, it can only be discarded at PDCP layer or RLC layer.

Based on the above observations, it seems a more service aware HARQ retransmissions design could be beneficial in 6GR especially for applications demanding higher reliability along with small latency.

Proposal 2: Study to enhance HARQ retransmissions with better service awareness for improving overall latency and efficiency

# Conclusion

In this contribution, we discuss the considerations for improving service awareness in 6GR compared to 5GNR. Following are the proposals:

Proposal 1: Study protocol design to have improved service awareness at MAC layer for satisfying service specific requirements of latency and reliability

Proposal 2: Study to enhance HARQ retransmissions with better service awareness for improving overall latency and efficiency

# References

- RP-252912		Revised 	SID: Study on 6G Radio