3GPP TSG-RAN WG2 Meeting #132  						   				      	R2-2509030

Dallas, USA, 17th November 2025 - 21st November 2025

Agenda Item:	10.3.3.2

Source: 	CEWiT

Title:  	Discussion on energy efficiency in 6G

Document for:	Discussion and decision

# Introduction

The 6GR study item was established in RAN#108 meeting [1] and RAN2 started a 6G discussion in RAN2#131bis meeting [2]. Sustainability is the main requirement in 6G, and OPEX reduction is a critical issue in the network. Energy efficient operations should be considered in 6G for both the network and the UE. The design target and direction to use for the proposals for network and UE energy saving was provided in RAN2#131bis [2] as given below

## Design goal 1. Strive to develop solutions that consider network energy efficiency and UE power saving Alignment of power saving features - For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

Further the agenda of RAN2#132	provides the guideline for the contribution as given below

#### 10.3.3.2 Energy efficiency Contributions on common aspects of network and UE energy efficiency, including alignment of energy and power saving features. NOTE: aspects related to system information and paging should be discussed in CP AI.

In this paper, we will discuss energy efficiency in 6G to achieve a native green network.

# Discussion

## 2.1 On demand SSB

In 5G NR, multiple solutions for network energy saving were introduced in the network. These techniques can provide significant network energy saving but are not fully explored and are restricted to certain scenarios only due to backward compatibility. Some of these features when exploited to the full extent can benefit both the BS and the UE and hence can be a baseline for green network design for 6G. Some of the NES features from Rel 18 and Rel 19 5G NR is given below.

- Cell-DTX/DRX and Cell switch off

- Spatial and Power domain adaptation

- On-demand SSB for SCell

- SSB adaptation in time domain

Observation 1: NES features from Rel 18 5G NR are restricted and are not fully exploited due to backward compatibility; these can be leveraged for 6GR ES design for better performance.

In Rel 19 NR, on-demand SSB was introduced for SCells, which is applied only for RRC_CONNECTED UE and is network triggered. There were 2 cases agreed for OD-SSB operation in an SCell, case 1 is only on demand SSB present in the cell and case 2 is always on SSB + On demand SSB. In either case, the on-demand SSB is not applicable to PCell or standalone scenarios, therefore its benefits cannot be fully exploited. Further neither of the cases is applicable to RRC idle/inactive UEs. When the OD-SSB is applied on Pcell or in standalone scenario, it may impact the UE in RRC idle mode since the UE has no knowledge whether there is any cell present on a raster, which is an OD-SSB enabled cell. Further enhancements can be investigated such as a discovery RS (DRS) can be introduced for the UE to determine the presence of on-demand SSB cell and perform on-demand SSB procedures. This will reduce unnecessary monitoring at the UE thereby reducing power consumption at UE while providing energy savings at the BS. Furthermore, a UE triggered OD-SSB will benefit both the BS and UE by reducing unnecessary transmissions at BS and monitoring at UE. The WUS configuration can be assumed predefined with respect to the DRS for simplicity for the UEs.

Proposal 1: Support UE triggered On-demand SSB with following considerations

- Applicability to both single cell and multi cell scenarios in 6G.

- Applicability to all RRC states.

- Introduction of DRS for detection of a standalone OD-SSB cell in a raster.

## 2.2. Enhanced Cell DTX/DRX

Cell Discontinuous Transmission (DTX) and Cell Discontinuous Reception (DRX) are key energy saving mechanisms that reduce power consumption at both the BS and UE by allowing transmissions and receptions to be skipped during predefined "off" periods. Release 18 introduced support for configurable DTX/DRX patterns comprising a start time, on-duration, and periodicity, activated via RRC signalling or DCI. However, it is primarily limited to UE-specific semi-periodic signals and a few control and reference signals. Always-on transmissions such as the SSB, SIB1 and some DCIs remain excluded, causing gNBs to remain active during inactive DTX periods when short-period SSBs/SIB1s overlap, diminishing Network Energy Savings. Further, the 5G NR UE-DRX mechanism used for UE power saving is completely independent of Cell DTX/DRX. This may cause ambiguity at UE, increasing unnecessary monitoring and power consumptions if the On-duration of UE-DRX overlaps with off duration of cell DTX, when both the mechanisms are enabled simultaneously.

In 6G, an enhanced Cell DTX/DRX mechanisms can be considered as a joint power-saving mechanism for both base stations (BS) and user equipment (UE). Enhancement should expand applicability to always-on signals like SSB, SIB, and DCIs, enabling synchronized deep sleep for both BS and UE in all states (idle, inactive, connected). This avoids the need for separate mechanisms for power savings in RRC connected and RRC idle/inactive modes of UE. Flexibly adaptable Cell DTX/DRX patterns, guided by User Assistance Information (UAI), tailor energy savings to QoS and traffic needs, while AI/ML-driven predictive models enable intelligent, context-aware network-UE coordination for optimal power usage. Further, enhancing cell DTX/DRX can work as UE DRX thereby avoiding overlapping ambiguities and redundant monitoring. This coordinated design turns rigid “Always-ON” operation into dynamic “Smart ON” management, maximizing energy savings without service compromise, and aims for reduction in network power consumption compared to 5G. The approach leverages AI-native management, multi-type SSB adaptation (normal, energy-saving, on-demand), and low-power wake-up signals for efficient, scalable energy efficiency across 6G networks and devices.

Observation 2: Enhancements of Cell DTX/DRX to all RRC states reduces redundant mechanisms and provides more opportunities of energy saving both at the BS and UE.

Proposal 2: Study enhanced Cell DTX/DRX with 5G Cell DTX/DRX as baseline. Following enhancement can be considered.

- Applicability to IDLE/INACTIVE resp. CONNECTED state

- Common signals and channels are turned off during non-active duration, including both idle/inactive and connected UE modes

- Flexible DTX/DRX pattern including adaptation based on User Assistance Information (UAI)

- Enhancement as joint design of Cell DTX/DRX and UE DRX

- Utilizing AI/ML for DTX/DRX adaptations.

## 2.3. Enhanced BW adaptation for single carrier

5G defined the Bandwidth part (BWP) as a contiguous set of frequency resources within a carrier that can be configured for a UE within a single cell. The 5G BWP framework requires frequent BWP switching under low load/bursty traffic, which can increase latency. This will further increase configuration overhead, complexity, and switching costs (signalling, verification, and mis-decode risk) for both the BS and UE. 6GR should support UE bandwidth adaptations for various bandwidths requirements among different device types and additional use cases. 6G design should aim for a simpler design with reduced RRC overhead. One solution is support of bandwidth adaptation within BWP as a key measure for joint energy-efficiency, the BW of both the BS and UE can be adapted in low load/traffic scenarios, while focussing on just the adaptation of essential bandwidth-dependent parameters without needing complete BWP switching and maintaining consistency across different configurations. Simplifying BWP design to focus parameter changes on bandwidth and frequency location, aligning numerology, and adopting relaxed RRC with efficient DCI and/or group-based indications reduces complexity and enables scalable, reliable, and faster BW adaptation within a BWP.

Proposal 3: Study enhanced BWP adaptation mechanism for 6G EE improvement, considering the following aspects:

- Adaptation of bandwidth-dependent parameters

- Faster BW adaptation without BWP switching/shorter data interruption by restricting parameter set

- L1 based adaptation with Improved reliability

- Cell/group-wise indication for network energy saving use case

2.2.1 Blind Decoding reduction in 6G

A BS in energy saving mode may or may not transmit a control information in DL. For example, a BS can skip transmissions of DCI 2_2 while it can still transmit DCI 1_0 or DCI 2_9. Blind decoding (BD) and DCI monitoring are needed for UEs to receive this control information in 6G systems. UEs perform BD by checking multiple control channel candidates within pre-configured search spaces (SS), which span specific time-frequency regions. Variations in DCI formats and RNTIs affect payload size and increase the number of required BD attempts. While there are limits to the number of BDs a UE can perform per slot to manage complexity and power consumption, current configurations may still require up to 44 BDs per slot per serving cell.

To address this, 6G should introduce mechanisms to reduce BD attempts without compromising scheduling flexibility. An indication can be provided to the UE for an information about upcoming DCI(s), such as dynamic indication of PDCCH occasion count or blind decoding info (search space set, CORESET, DCI format). This enables the UE to narrow its candidate list and avoid unnecessary decoding. Further, mechanisms to provide blind decoding information before PDCCH monitoring occasions to minimize blind decoding attempts can be considered. For example, a BS can indicate in the current transmitted DCI, an information about the dynamic adaptation or updating of SS configurations of subsequent DCIs. This allows the base station to guide the UE’s monitoring more efficiently, reducing overhead while maintaining control reliability. These approaches streamline the DCI detection process, lowering UE computational load and power consumption, while preserving flexible and effective scheduling.

Proposal 4: Study mechanism for reducing blind decoding attempts at UE including mechanisms/scenarios to provide blind decoding information before PDCCH monitoring occasions.

- Indication of blind decoding info includes search space set, CORESET, DCI format, PDCCH occasion count, etc.

# Conclusions

In this contribution, we discussed the enhancements required for energy efficiency at gNB and UE and following proposals are made,

Observation 1: NES features from Rel 18 5G NR are restricted and are not fully exploited due to backward compatibility; these can be leveraged for 6GR ES design for better performance.

Proposal 1: Support UE triggered On-demand SSB with following considerations

- Applicability to both single cell and multi cell scenarios in 6G.

- Applicability to all RRC states.

- Introduction of DRS for detection of a standalone OD-SSB cell in a raster.

Observation 2: Enhancements of Cell DTX/DRX to all RRC states reduces redundant mechanisms and provides more opportunities of energy saving both at the BS and UE.

Proposal 2: Study enhanced Cell DTX/DRX with 5G Cell DTX/DRX as baseline. Following enhancement can be considered.

- Applicability to IDLE/INACTIVE resp. CONNECTED state

- Common signals and channels are turned off during non-active duration, including both idle/inactive and connected UE modes

- Flexible DTX/DRX pattern including adaptation based on User Assistance Information (UAI)

- Enhancement as joint design of Cell DTX/DRX and UE DRX

- Utilizing AI/ML for DTX/DRX adaptations.

Proposal 3: Study enhanced BWP adaptation mechanism for 6G EE improvement, considering the following aspects:

- Adaptation of bandwidth-dependent parameters

- Faster BW adaptation without BWP switching/shorter data interruption by restricting parameter set

- L1 based adaptation with Improved reliability

- Cell/group-wise indication for network energy saving use case

Proposal 4: Study mechanism for reducing blind decoding attempts at UE including mechanisms/scenarios to provide blind decoding information before PDCCH monitoring occasions.

- Indication of blind decoding info includes search space set, CORESET, DCI format, PDCCH occasion count, etc.

# Reference

[1] RP-251881

[2] RAN2 Chairman notes, RAN2#131bis, October 2025