3GPP TSG RAN WG2 #131			R2-2509068

Dallas, USA, November 15th – 19th, 2025

Agenda Item:	10.3.3.1

Source:	BT Plc, Orange, T-Mobile USA, Deutsche Telekom, Turkcell, Verizon, KDDI, Vodafone, TIM, Nokia, Ericsson

Title:	Guidelines for 6G AI_ML for the air interface model delivery options

Document for:	Discussion, Decision

# Introduction

In the Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 [1], the following is captured where RAN2 is the leading working group

- Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

Due to time constraints over the AI/ML for the air interface WID, RAN2 has had no time to properly study the model transfer.

In RAN2, the first discussion related to model transfer was done during RAN2#130 where the following TDocs were discussed.

R2-2504379	Discussion on AIML model transfer delivery	CMCC,China Telecom,CATT,ZTE,Apple,Samsung,Xiaomi	discussion	Rel-19	NR_AIML_air-Core

R2-2504641	Operators views for the AI_ML model delivery options	BT Plc, Turkcell, T-Mobile, Deutsche Telekom, Orange	discussion	Rel-19	R2-2504635 [2]

Both documents werewhere noted

R2-2504379 Discussion on AIML model transfer delivery CMCC,China Telecom,CATT,ZTE,Apple,Samsung,Xiaomi discussion Rel-19 NR_AIML_air-Core Proposal 1: There is no requirements on controllability or visibility for UE-sided model transfer/delivery case y, since the model trained in OTT server can be sent to the UE directly - Tmobile disagrees and thinks its important to have visibility. How would you manage something for which you have no knowledge of. CMCC is not sure how to have visibility as this is going over OTT. Qualcomm thinks that the whole monitoring procedure is sufficient to check the performance of UE. - Mediatek thinks that we shouldn’t decide whether we will have same of different tunnel. Proposal 2: It is proposed to de-prioritize case z1 for one-sided model transfer/delivery, since there is no strong requirement on visibility and controllability for UE-sided model transfer/delivery from the perspective of operator. Proposal 3: It is proposed to follow case y for UE-sided model transfer/delivery in R19, i.e. the UE-sided model is delivered over the top, which is transparent to 3GPP. Noted R2-2504635 Operators views for the AI_ML model delivery options BT Plc, Turkcell, T-Mobile, Deutsche Telekom, Orange discussion Rel-19 => Revised in R2-504641 R2-2504641 Operators views for the AI_ML model delivery options BT Plc, Turkcell, T-Mobile, Deutsche Telekom, Orange discussion Rel-19 R2-2504635 Proposal 1: Solution 4a, OTT without 3GPP impact, captured on [1] is no further discussed in RAN2 Proposal 2: One-sided dataset/model transfer solution shall follow below principles: • A1 - Size: from RAN2 point of view, aim to support various sizes of model parameter transfer (FFS on model size); • A2 - Continuity: service continuity of model transfer/delivery during UE mobility needs to be supported; • A3 - Controllability: NW decides on if and when to transfer/delivery over the air interface; • A4 - Latency: relaxed latency requirement and infrequent update; • A5 - Visibility: dataset and model parameters to be visible and understandable by MNO; - Huawei asks which case you are referring to. BT explains is the model transfer to the UE. Qualcomm asks how do you control the equivalent of 1a over the top. BT explains that this beyond 1a. - Qualcomm asks how does it work, if the network configures you with inference config and you don’t have model you have to download. - Samsung, Apple, Xiaomi, Oppo, thinks that the discussion should focus on 2sided model and doesn’t see how the network knows whether the model is trusted. Apple explains that RAN1 already concluded to focus on 2sided. - Nokia, BT and Ericsson think controllability is important as the UE shouldn’t download a model if it is going to idle. Also it is strange how RAN1 can decide on data transfer that is not in their expertise in scope. Proposal 3: Adopt additional list of requirements for one-sided and two-sided model transfer/delivery. • Activation: once the AI/ML one-sided or two-sided model is transferred to NW and/or to UE, MNO should be capable to activate and deactivate them at any point in time • Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic. • Differentiability: model transfer/delivery traffic should be differentiated from other user traffic. • Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded. • Addressability: Models need to be addressable such that the UE can request the transfer/delivery of a specific one • Controllability: the NW is in control of if and when to transfer/deliver a model Noted

Due to the absence of any prior study in RAN2, a paper [3] co-signed by 11 operators, BT, T-Mobile USA, Deutsche Telekom, Turkcell, Orange, TIM, AT&T, Verizon, Docomo, KPN, and KDDI, and one RAN vendor, Nokia, was submitted for discussion at RAN2#131. Unfortunately, despite the high number of co-signers, no online time was allocated to discuss it.

In RAN2#131bis, the following agreement was made

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

Given the number of MNOs that consider this topic highly relevant for their networks, it is requested that the 6G 3GPP AI for air interface study includes model transfer.

# Discussion

## AI model transfer requirements

For operators, it is essential to have a standardized solution for UE-side radio AI model transfer. Managing the mobile network is part of the operators’ core business. Leaving the model transfer unspecified (OTA) may negatively impact network KPIs for several reasons. For example, the AI/ML model could be transferred during peak hours, introducing unnecessary congestion into the network. UEs at the cell edge or with low bandwidth configurations should skip the model transfer, as it would severely affect their power consumption. This operational knowledge is specific to the MNO, and an OTA solution is not aware of such conditions..

- The MNO needs control over when to initiate or allow UE-side model transfer considering network load and radio conditions, which are unknown to an OTT server, meaning that a NW-controlled solution is required to protect network performance.

There may be emergency situations in a given area, and it is well-known that such events lead to network congestion. If first responders are using the 3GPP network, the congestion becomes even more severe, leaving very limited resources available for other users. The knowledge of the emergency situation is restricted to the MNO. Therefore, attempting to transfer the AI model to UEs within the affected area would further exacerbate the congestion issue.

- Leaving transfer/delivery of the AI models unspecified by 3GPP will have harmful risks, including negative impacts on MNOs network KPIs.

These are just a few examples that highlight the importance of operator control over the decision of when the model should be transferred. Currently, both the model size and the periodicity of transfers are unknown. Given this uncertainty, MNOs will be required to adopt a cautious approach and assume model sizes could reach hundreds of megabytes.

The periodicity of model transfers to UEs is also undefined, meaning models could be transferred daily, weekly, monthly, or at other intervals. Moreover, the criteria used to determine when a model should be transferred remain outside the scope of 3GPP.

In summary, the MNO cannot control the model size, the transfer periodicity, or the criteria triggering the transfer. At the same time, the model owner cannot be aware of network KPIs, congestion levels, UE radio configurations, or the radio conditions at the time of transfer. For all these reasons, it is essential that the MNO retains control over the AI model transfers.

- Without any 3GPP study of the model transfer patterns, it is difficult to evaluate impact on the system KPIs.

The 6G Study item [4] should ensure this topic is studied in deep, as it was agreed during RAN2#131bis [5].

- RAN2#131bis captures the following agreement, “Study model transfer/delivery requirements/functions”.

To address MNO concerns, the 6G study should consider key guidelines for transitioning from UE measurements to AI predictions: AI model transfer over the air interface should be non-chargeable to end customers, treated as best-effort traffic with lower priority than any ongoing customer data, it shall not introduce any impact to user data transmission, and should support transfer continuity after a handover avoiding to re-send data already confirmed over the air interface.

- The 6G study needs to avoid any impact to end customers data, precluding extra-charges, and ensuring AI model transfers continuity during the handover.

The principles identified by operators during the 5G specification needs to be addressed during the 6G study. Figure 1 graphically shows the points to be studied during the 6G study item, and captured in detail in Proposal 1.

Figure 1 Summary of the requirements and principles to be studied for 6G AI for the air interface

- The AI model transfer solution shall consider at least the below requirements:

- Size: from RAN2 point of view, aim to support various sizes of the model parameter transfer (FFS on model size);

- Continuity: service continuity of model transfer during UE mobility needs to be supported;

- Controllability: NW decides on if and when to transfer over the air interface;

- Latency: relaxed latency requirement and infrequent update;

## Principles to transfer/delivery the AI model

AI/ML functionality has the potential to optimize procedures by improving key aspects such as energy consumption and signaling overhead. To achieve this, well-established and tested procedures may be replaced. For example, traditional measurements could be avoided, along with the associated work already covered by RAN4 and RAN5 to ensure expected behavior. Instead, decisions will be made based on predictions. As with any prediction, there are inherent error margins. Well-trained models can reduce these margins to a level where expected performance is maintained. However, it is important to note that this represents the best-case scenario.

- The NW should have the capability of validating the performance of an AI/ML-enabled configuration and take actions based on that analysis, e.g., AI model transfer functionality and configuration may be deactivated if the end-to-end performance is impacted.

Model traffic should be prioritized lower than user traffic to avoid any impact on the service quality, Figure 2. During peak hours or congestion, transferring large models could degrade user experience. Assigning lower priority ensures network stability and preserves resources for critical user and control-plane traffic.

Figure 2 AI model transfer to be set at a lower priority than any user data

Model transfer should be distinguishable from regular user traffic to enable optimized handling. Differentiation allows operators to apply specific policies, monitor performance, and manage resources effectively, ensuring that model delivery does not interfere with latency-sensitive or high-priority services. Figure 3 shows the idea, but it does not intent to make any reference to concepts like 5G PDU.

Figure 3 AI model transfer traffic should be distinguishable form user data

Secure, network-aware model transfer is essential to prevent unauthorized or malicious models from being downloaded, Figure 4. Only trusted models should be delivered, ensuring integrity and protecting both the UE and network from potential threats or performance degradation caused by unverified AI behavior.

Figure 4 AI models should always be transfered from a trusted server

Models should be uniquely addressable so that UEs can request specific models and/or versions of models and so that the NW can push specific models and/or versions of models to the UE, Figure 5. This enables targeted updates, supports model lifecycle management, and avoids unnecessary transfers. Addressability ensures that only relevant models are delivered to the UE, reducing overhead and improving operational efficiency.

Figure 5 AI models should be addressable to avoid models being transfered unnecessarily

- RAN2 needs to perform the study based on at least the following principles for the AI model transfer:

- Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic.

- Differentiability: model transfer/delivery traffic should be differentiated from other user traffic.

- Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models or models from untrested AI model servers cannot be downloaded.

- Addressability: Models need to be addressable such that the UE can request the transfer of a specific one and/or versions of models

# Conclusions

In this TDoc, we have made the following observations:

Observation 1	The MNO needs control over when to initiate or allow UE-side model transfer considering network load and radio conditions, which are unknown to an OTT server, meaning that a NW-controlled solution is required to protect network performance.

Observation 2	Leaving transfer/delivery of the AI models unspecified by 3GPP will have harmful risks, including negative impacts on MNOs network KPIs.

Observation 3	Without any 3GPP study of the model transfer patterns, it is difficult to evaluate impact on the system KPIs.

Observation 4	RAN2#131bis captures the following agreement, “Study model transfer/delivery requirements/functions”.

Observation 5	The 6G study needs to avoid any impact to end customers data, precluding extra-charges, and ensuring AI model transfers continuity during the handover.

Observation 6	The NW should have the capability of validating the performance of an AI/ML-enabled configuration and take actions based on that analysis, e.g., AI model transfer functionality and configuration may be deactivated if the end-to-end performance is impacted.

For those reasons, we propose the following:

Proposal 1	The AI model transfer solution shall consider at least the below requirements:

	Size: from RAN2 point of view, aim to support various sizes of the model parameter transfer (FFS on model size);

	Continuity: service continuity of model transfer during UE mobility needs to be supported;

	Controllability: NW decides on if and when to transfer over the air interface;

	Latency: relaxed latency requirement and infrequent update;

Proposal 2	RAN2 needs to perform the study based on at least the following principles for the AI model transfer:

	Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic.

	Differentiability: model transfer/delivery traffic should be differentiated from other user traffic.

	Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models or models from untrested AI model servers cannot be downloaded.

	Addressability: Models need to be addressable such that the UE can request the transfer of a specific one and/or versions of models

# References

- RP-243245 - New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

- R2-2504635 – RAN2#130 – Operators views for the AI_ML model delivery options

- R2-2506169 – RAN2#131 – Operators views for the AI_ML model delivery options

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, 3GPP TSG RAN#108, Prague, Czech Republic, June 9-13, 2025

- RAN2#131-bis chair notes