3GPP TSG RAN WG2 #132	R2-2509072

Dallas, US, Nov 11th – 21st, 2025

Agenda item:		10.3.3.2

Source:	Nordic Semiconductor ASA

Title:					6G and UE power saving simplification considerations

Document for:		Discussion

# Introduction

6GR Rel-20 has a great opportunity to simplify and streamline the implementation and deployment of UE targeting power saving features. Looking back to both 4G and 5G standardization and the greater ecosystem there are few obvious lessons learned:

- The key power saving features were optional, and not all operators deployed them. And if they were deployed, the configurations on the network side had such variety that developing global end products was greatly challenged. Product rather opted to use brute force approach rather than relying on 3GPP specified features.

- Deploying services that require mobile terminated traffic are hard to get right. Getting UE active time, core network and RAN synchronized, and the service knowing UE active time window has turned out to be a great challenge in practise. Especially, synchronizing RAN and core network has been surprisingly hard.

- Over time standardization introduced competing features with partial or complete overlap, which slowed down the adoption of the features and occasionally preventing any of those being globally deployed.

- WUS/WUR/LR had great promise but

  - a) was introduced too late,

  - b) had multiple options (e.g. LP-WUS with OOK and OFDM based solutions with different coverage),

  - c) was not deployable on all frequency bands operators wanted it to be deployed,

  - d) to make the LR solution on the UE side power efficient required too many simplifications and minimal band support.

Going forward with 6GR there is an opportunity to learn from the past shortcomings. This paper discusses how to ease the global deployment from the day one. Also, in this paper we elaborate some ideas how to streamline and simplify both connected mode and especially IDLE/INACTIVE (like) mode power saving features.

# UE power saving considerations

In this chapter we discuss few key points we have identified.

## LP-WUS considerations

Based on our analysis there were several reasons why WUS/LP-WUS failed. First, the feature was a late addition to specifications and the ecosystem by that time had already chosen the winning power saving features. Especially, the major chipset implementations were already out and had little incentive to implement LR as part of the chipset or as an external component. And due to lack of initial device side support network side also never materialized. Furthermore, network operators struggled to find use cases to justify the extra spectrum needed by WUS.

For WUS/LP-WUS to be successful it must be available in the first release of the specification and deployments.

WUS/LP-WUS must also be available and deployed at the same time on the UE and the network side.

WUS/LP-WUS should be part of the initial 6G deployment.

Second, the band planning for WUS/LP-WUS turned out non-trivial. For the LR implementation to be cheap, simple and power efficient the LR design could not support too many bands. Due to lack of global “WUS” band a discrete LR that would meet all the set expectations was not possible. Another approach would have been implementing an LR as a (special) low power mode of the MR, with most of the functions turned off. The power saving would reduce from envisioned ~95% to ~60%, however, that would have been justifiable saving for most of the use cases.

WUS/LP-WUS should be possible on the UE side practically on any band the UE supports on its MR with possible power saving cost.

WUS/LP-WUS should be usable and deployable on the same bands the MR uses.

Third, expected use cases in standardization did not meet the actual customer use case expectations. From our experience the most asked use case was for hardly moving always sleeping devices that wanted to wake up with few second delay independent of the used sleep mode or eDRX cycle. Also, it was quickly realized that the WUS/LP-WUS must have the same cell coverage as the MR, otherwise the use of LR was not interesting from service point of view.

If the delay to next wake-up opportunity triggered by WUS/LP-WUS is tied to other power saving feature cycles such as what eDRX has, it unnecessarily limits possible use cases for WUS/LP-WUS.

Use of WUS/LP-WUS should not unnecessarily be tied to other power saving features’ cyclic behaviour.

## IDLE and INACTIVE-like modes

DRX and eDRX are (finally) widely deployed and successful UE power saving features. However, they suffer from one major shortcoming i.e. how to make mobile terminated traffic use cases work properly in practical deployments. Especially eDRX turned out to be troublesome due RAN and core having synchronization issues and lack of buffering of traffic that missed the UE active time window. Secondly, the longer the sleep time the better the power saving, but at the same time the latency to reach of the UE increased.

WUS/LP-WUS was hoped to solve some of the issues on reachability and reducing unnecessary monitoring. However, as discussed in the previous section e.g. WUS never really happened. And even if WUS/LP-WUS were deployed, it would not solve the reachability latency issue as they are defined today.

eDRX with long sleep cycles are hard to get right and suffer from long UE reachability latencies.

A radical way forward would be dropping the entire eDRX concept and replace it with WUS/LP-WUS. Basically, a UE entering “eDRX” would only monitor WUS/LP-WUS when to wake up instead of periodic (long) eDRX cycle. The goal would be twofold:

- Reduce the latency to deliver mobile terminated traffic to the UE

- Avoid the need to synchronize all the UE active time, core and RAN understanding when the UE can be paged, and when the service can reach the UE.

Obviously, a lot is expected from the WUR/LR implementation for the approach to be power efficient. The simpler the WUR/LR design and feature set is the more probable it is for the WUS/LP-WUS based UE sleep concept to work. Full (IDLE/INACTIVE) mobility or elaborate measurement framework cannot be expected from such a solution.

WUS/LP-WUS replacing e.g., eDRX could reduce the latency to reach a sleeping device from the network side.

WUS/LP-WUS replacing e.g., eDRX could help services to reach sleeping devices without a need to track UE activity time windows.

eDRX power saving mode is replaced with a sleep mode / power saving mode where the network initiated wake up is only triggered by WUS/LP-WUS.

## Simplification considerations

3GPP has successfully managed to standardize partly or completely overlapping optional power saving features. This creates deployment fragmentation and for chipsets/networks incremental improvements with minimal additional gain are hard to justify. RAI and DCP with/without LP-WUS are good examples of such features.

Fragmentation implies that end products cannot rely on the features be globally available, thus solutions tend to fall back to lowest nominator or non-3GPP brute force approaches and generally slow down the adoption of features because everybody is waiting for the next incremental improvement to materialize. From chipsets point of view small incremental features that eventually do not get globally adopted by network operator/ecosystem are just potential dead code with increased verification burden.

3GPP should avoid standardizing overlapping and competing optional features to the same problem.

3GPP should avoid standardizing small incremental and optional improvements to existing features.

# Conclusions

In this paper we discussed how to ease the global deployment of UE benefitting power saving features. Also, in this paper we elaborated some ideas how to streamline and simplify both connected mode and especially IDLE/INACTIVE (like) mode power saving features:

- For WUS/LP-WUS to be successful it must be available in the first release of the specification and deployments.

WUS/LP-WUS must also be available and deployed at the same time on the UE and the network side.

WUS/LP-WUS should be possible on the UE side practically on any band the UE supports on its MR with possible power saving cost.

If the delay to next wake-up opportunity triggered by WUS/LP-WUS is tied to other power saving feature cycles such as what eDRX has, it unnecessarily limits possible use cases for WUS/LP-WUS.

eDRX with long sleep cycles are hard to get right and suffer from long UE reachability latencies.

WUS/LP-WUS replacing e.g., eDRX could reduce the latency to reach a sleeping device from the network side.

WUS/LP-WUS replacing e.g., eDRX could help services to reach sleeping devices without a need to track UE activity time windows.

- WUS/LP-WUS should be part of the initial 6G deployment.

Use of WUS/LP-WUS should not unnecessarily be tied to other power saving features’ cyclic behaviour.

eDRX power saving mode is replaced with a sleep mode / power saving mode where the network initiated wake up is only triggered by WUS/LP-WUS.

3GPP shall avoid standardizing overlapping and competing features to the same problem.

3GPP shall avoid standardizing small incremental and optional improvements to existing features.