3GPP TSG-RAN WG2 Meeting #132	R2-2509080

Dallas, USA, Nov. 17th - 21th, 2025

Agenda Item:	8.11.2

Source:	OPPO

Title:	Remaining MAC issues of SBFD operation

Document for:	Discussion, Decision

# Introduction

According to the post email discussion of RAN2#131bis meeting for drafting MAC running CR of SBFD, the following two remaining issues were discussed, and need to be resolved in the subsequent RAN2 meeting.

- MAC-1: Whether/how to address the issue in case RO type switching and Msg1 repetition number fallback are triggered simultaneously. - MAC-2: Content to be captured in the LS for requesting RAN1 to update the definition of the first PRACH occasions in TS 38.213, to let it inclusively cover the ROs for cases where tdd-UL-DL-ConfigurationCommon is not applied.

In this contribution, we provide our understanding s for the above two issues.

# Discussion

## MAC-1: simultaneous RO type switching and Msg1 repetition number fallback

TS 38.321: 2> if the Random Access procedure is not completed: 3> if preambleTransMaxRO-Type is applied, and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure, and PREAMBLE_TRANSMISSION_COUNTER = preambleTransMaxRO-Type + 1: 4> if the RO_TYPE is set to 2nd-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the first PRACH occasions as defined in TS 38.213 [6]: 5> set the RO_TYPE to 1st-RO; 5> select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure; 5> if sbfd-RACH-DualConfig is configured for the Random Access procedure (see TS 38.331 [5]): 6> set PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP; 6> (re-)initialize the parameters specified in clause 5.1.1 for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources; 6> re-initialize PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI as specified in clause 5.1.1a; 6> set POWER_OFFSET_RO_TYPE to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP). 4> else if the RO_TYPE is set to 1st-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the second PRACH occasions as defined in TS 38.213 [6]: 5> set the RO_TYPE to 2nd-RO; 5> select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure; 5> if sbfd-RACH-DualConfig is configured for the Random Access procedure (see TS 38.331 [5]): 6> set PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP; 6> (re-)initialize the parameters specified in clause 5.1.1 for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources; 6> re-initialize PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI as specified in clause 5.1.1a; 6> set POWER_OFFSET_RO_TYPE to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP). 3> if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure: 4> if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or 4> if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1: 5> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available: 6> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure; 6> initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

According to the MAC-1 issue, when “preambleTransMaxRO-Type” and “preambleTransMax-Msg1-Repetition” is configured with the same value, the above two conditions (highlighted in yellow) are met. Then the UE will perform “Msg1 repetition number fallback” twice (i.e. the two texts highlighted in blue are performed). From our understanding, if the MAC specification is not changed, the consequence is that the UE will use higher Msg1 repetition number, which does not cause any critical issue for the UE and the network, as for the contention-based RACH procedure, there are anyway some UEs selecting higher Msg1 repetition number at “Msg1 repetition number fallback”.

Proposal 1: No need to resolve “[MAC-1] Simultaneous Fallbacks of RO Type and Msg1 Repetition Number”, i.e. no MAC specification change.

## MAC-2: update the definition of the first PRACH occasions in TS 38.213

TS 38.321: ### 5.1.1 Random Access procedure initialization … 1> perform the BWP operation as specified in clause 5.15, except when the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell; 1> if the Random Access procedure is initiated by PDCCH order and if the ra-PreambleIndex explicitly provided by PDCCH is not 0b000000 and if the RACH occasion indicator is set to 1 (as specified in TS 38.212 [9]); or 1> if the Random Access procedure was initiated for SpCell beam failure recovery (as specified in clause 5.17) and if the contention-free Random Access Resources for beam failure recovery request for 4-step RA type have been explicitly provided by RRC for the BWP selected for Random Access procedure and if the ra-OccasionType is set to sbfd for the Random Access procedure (as specified in TS 38.331 [5]); or 1> if the Random Access procedure was initiated for reconfiguration with sync not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3 and if the contention-free Random Access Resources for 4-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure and if the ra-OccasionType is set to sbfd for the Random Access procedure (as specified in TS 38.331 [5]): 2> set the RO_TYPE to 2nd-RO. 1> else if neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure and either sbfd-RACH-SingleConfig or sbfd-RACH-DualConfig is configured by RRC for the Random Access procedure (as specified in TS 38.331 [5]): … 1> else: 2> set the RO_TYPE to 1st-RO.

TS 38.213: The modification parts are expected change of the RAN1 specification if RAN1 confirms to update the definition of first PRACH occasion. # 8 Random access procedure … In the remaining of this clause, when a symbol is not stated as an SBFD symbol, the symbol is a non-SBFD symbol. Prior to initiation of the physical random access procedure, Layer 1 may receive from higher layers an indication to perform a random access procedure using: - first PRACH occasions each including only symbols that are indicated as uplink or flexible and considered as uplink for the random access procedure, or - second PRACH occasions, that are in RBs that are both in the active UL BWP and in the UL sub-band, and associated either only with SBFD symbols that include at least one SBFD symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon when the UE is provided either sbfd-RACHSingleConfig or sbfd-RACHDualConfig, or start from an SBFD symbol and end in a non-SBFD symbols when the UE is provided sbfd-RACHDualConfig and sbfd-RACHDualConfig-ValidROAcrossSymbolTypes The procedures in clause 8.1 apply separately for the first PRACH occasions and the second PRACH occasions. Only a Type-1 random access procedure is applicable for the second PRACH occasions.

According to TS 38.321, when SBFD RO is not configured (e.g. for FDD), the SBFD-aware UE “set the RO_TYPE to 1st-RO”, i.e. “first PRACH occasions” in TS 38.213. However, according TS 38.213, the definition of “first PRACH occasions” is for non-SBFD symbols which “are indicated as uplink or flexible by tdd-UL-DL-ConfigurationCommon” (which is not configured for FDD and SUL). According to the email discussion, most companies consider that we can send an LS to RAN1 to update the texts of TS 38.213 to update the definition of “first PRACH occasions” to cover the non-SBFD symbols for the carrier where tdd-UL-DL-ConfigurationCommon is not configured. From our understanding, the expected change of the RAN1 specification would be to simply remove the configuration restriction of “tdd-UL-DL-ConfigurationCommon” as highlighted above.

Proposal 2: For [MAC-2], send an LS (as given in the annex) to RAN1, including:

- Explaining the MAC specification issue that the UE can set RO_type to 1st-RO, i.e. “first PRACH occasions” in TS 38.213, when tdd-UL-DL-ConfigurationCommon is not configured (e.g. FDD, SUL, and TDD without tdd-UL-DL-ConfigurationCommon).

- Requesting the update of the definition of “first PRACH occasions” in TS 38.213 to cover the non-SBFD RO(s) regardless of whether tdd-UL-DL-ConfigurationCommon is configured.

- Asking if RAN1 has any concern on changing the definition of “first PRACH occasions”.

# Conclusion

According to the above analysis, we have the following proposals.

Proposal 1: No need to resolve “[MAC-1] Simultaneous Fallbacks of RO Type and Msg1 Repetition Number”, i.e. no MAC specification change.

Proposal 2: For [MAC-2], send an LS (as given in the annex) to RAN1, including:

- Explaining the MAC specification issue that the UE can set RO_type to 1st-RO, i.e. “first PRACH occasions” in TS 38.213, when tdd-UL-DL-ConfigurationCommon is not configured (e.g. FDD, SUL, and TDD without tdd-UL-DL-ConfigurationCommon).

- Requesting the update of the definition of “first PRACH occasions” in TS 38.213 to cover the non-SBFD RO(s) regardless of whether tdd-UL-DL-ConfigurationCommon is configured.

- Asking if RAN1 has any concern on changing the definition of “first PRACH occasions”.

# Reference

[1] [Post131bis][212][SBFD] CR for TS 38.321 (Samsung)

# Annex for draft LS to RAN1

3GPP TSG-RAN WG2 Meeting #132	R2-250xxxx

Dallas, USA, Nov. 17th - 21th, 2025

Title:		         DRAFT LS on the definition of first PRACH occasion

Response to:

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	RAN2

To:	RAN1

Cc:

Contact Person:

- Name:	xxxx

- E-mail Address: 	xxxxxx

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

According to the TS 38.321, the SBFD-aware UE can set the RO-type to 1st-RO, i.e. “first PRACH occasions” in TS 38.213, when tdd-UL-DL-ConfigurationCommon is not configured (e.g. FDD, SUL, and TDD without tdd-UL-DL-ConfigurationCommon). However, the “first PRACH occasions” in TS 38.213 only refers to the non-SBFD RO when tdd-UL-DL-ConfigurationCommon is configured. This will cause the failure of the RACH procedure. To resolve the RACH procedure issue and align the UE behaviours between TS 38.321 and TS 38.213, RAN2 considers that the definition of the first PRACH occasion in TS 38.213 can be updated to include the non-SBFD RO(s) regardless of whether tdd-UL-DL-ConfigurationCommon is configured.

2. Actions:

To RAN1:

ACTION:

RAN2 respectfully requests RAN1 to update the definition of first PRACH occasion in TS38.213 to include the non-SBFD RO(s) regardless of whether tdd-UL-DL-ConfigurationCommon is configured. RAN2 would also like to know whether RAN1 has any concern on changing the definition of first PRACH occasion.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2  Meeting #133	   09-13 February   2025	Stor-Göteborg, Sweden

TSG-RAN2  Meeting #133bis	 13-17 April       2025	Malta, Malta