3GPP TSG-RAN WG2 #132	R2-2509086

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	8.4.2

Source:	InterDigital, Inc., Nokia

Title:	LP-WUS monitoring in CONNECTED mode

WID/SID:	NR_LPWUS-Core – Release 19

Document for:	Discussion

# Introduction

In the previous RAN2#131-Bis meeting, it was discussed whether any condition to exit LP-WUS monitoring needs to be specified for the UE in RRC_CONNECTED mode related to RILs E043 and H050.

This TDoc discusses those conditions.

# Discussion

Currently, the NW has no knowledge on whether the UE is able to monitor LP-WUS in CONNECTED mode and the UE may rely solely on NW configuration, ie., whether the LP-WUS is configured or not. Furthermore, regardless of the UE is not required to monitor LP-WUS while it is configured, it would intend to do so to save on energy – the UE does not really have the knowledge of the LP-WUS coverage. In addition, it is challenging for the network to accurately determine, based on regular MR measurement reporting, whether the UE can monitor LP-WUS or not. This is because the coverage of LP-WUS may differ from MR coverage and LR coverage, and UEs may be equipped with varying types of LRs, each offering distinct performance characteristics. Hence, it would be reasonable to have a condition to exit the LP-WUS monitoring at some point. Also, it is beneficial to configure an explicit exit LP-WUS monitoring condition for the UE in CONNECTED mode to support service continuity when LP-WUS reception failure happens.

Proposal 1: Specify an exit condition for the UE stop monitoring LP-WUS in RRC_CONNECTED mode.

RAN2 has agreed entry/exit conditions for LP-WUS in RRC_IDLE/INACTIVE modes where based on cell quality (MR and LR (optional))the UE can either entry to the LP-WUS monitoring or exit it. This baseline seems to work as a reasonable foundation for at least an exit condition also for RRC_CONNECTED mode where the UE monitors PDCCH solely based on C-DRX (with LP-WUS Option 1-1) or based on starting autonomously lpwus-PDCCH-MonitoringTimer (with LP-WUS Option 1-2) when the serving cell quality does not satisfy the condition to monitor LP-WUS. Given the dedicated configuration of the LP-WUS monitoring for RRC_CONNECTED mode UE agreed, it seems reasonable to allow the configuration of these thresholds separately for RRC_CONNECTED UE from RRC_IDLE/INACTIVE conditions, or the same can be applied when no dedicated configuration is provided.

Proposal 2: Support the same entry/exit condition criteria based on serving cell quality at least to deactivate LP-WUS monitoring in RRC_CONNECTED mode as agreed for RRC_IDLE/INACTIVE mode.

It is worth discussing whether only the serving cell quality-based entry/exit condition is enough to maintain synchronization between UE and the gNB and avoid any service interruptions if the UE cannot receive the LP-WUS signal regardless of entry/exit condition being satisfied. Similarly to data inactivity timer (which protects the system from RRC mismatch), a timer could be exploited to avoid an issue where the entry/exit condition is satisfied but the UE cannot receive the LP-WUS signal while gNB tries to wake up the UE. While the timer is running (and entry/exit condition satisfied), the UE can activate LP-WUS monitoring whereas when the timer expires, the UE deactivates LP-WUS monitoring and monitors PDCCH based on the other rules in the specification (i.e., C-DRX or lpwus-PDCCH-MonitoringTimer). The timer can be re-started whenever the UE identifies a LP-WUS signal. Alternatively, a configuration could be supported for the number of LP-WUS occasions where the UE did not decode any LP-WUS signal to deactivate the monitoring.

Proposal 3: Support a timer as condition to at least deactivate LP-WUS monitoring where the timer counts the number of LP-WUS occasions where the UE did not decode any LP-WUS signal.

Proposal 4: Alternatively, support a configuration for number of LP-WUS occasions where the UE did not decode any LP-WUS signal to deactivate the LP-WUS monitoring.

While the UE activating the LP-WUS monitoring based on the entry/exit condition being satisfied doesn’t need to be known to the gNB, since gNB knows based on the timer if the UE monitors LP-WUS or not, the UE deactivating the LP-WUS monitoring based on the events as above would seem reasonable to be made known to the gNB, especially, if it happens through the timer while the entry/exit condition was still satisfied. This way, the gNB can reconfigure the UE, e.g., remove the LP-WUS configuration when it does not serve the purpose for the given UE.

Proposal 5: UE reports via UAI the events of autonomous deactivation of LP-WUS monitoring to the gNB.

# Conclusion

This document has made the following observations and proposals:

Proposal 1: Specify an exit condition for the UE stop monitoring LP-WUS in RRC_CONNECTED mode.

Proposal 2: Support the same entry/exit condition criteria based on serving cell quality at least to deactivate LP-WUS monitoring in RRC_CONNECTED mode as agreed for RRC_IDLE/INACTIVE mode.

Proposal 3: Support a timer as condition to at least deactivate LP-WUS monitoring where the timer counts the number of LP-WUS occasions where the UE did not decode any LP-WUS signal.

Proposal 4: Alternatively, support a configuration for number of LP-WUS occasions where the UE did not decode any LP-WUS signal to deactivate the LP-WUS monitoring.

Proposal 5: UE reports via UAI the events of autonomous deactivation of LP-WUS monitoring to the gNB.