3GPP TSG-RAN WG2 #132	R2-2509139

# Dallas, TX, USA, 17th – 21st November 2025

Agenda Item:	 8.1.2

Source:	Ericsson

Title:	Discussion on RILs for LCM (E065, S050, H015, N079)

Document for:	Discussion

# 1	Introduction

In this paper, we discuss and address some RILs identified for LCM for UE-side models.

# 2 	Discussion

## 2.1 (E065) Handling inapplicable periodic CSI report configurations

RAN2 asked RAN1 what is the best option for handling the inapplicable/inactive periodic CSI report configurations and RAN1 provided the reply LS [R1-2508122] with the following content:

“RAN1 thanks RAN2 for the LS on when RRC layer submits periodic CSI inference configuration to lower layer. RAN 1 discussed the two options identified by RAN 2: • Option 1: Upon reception of RRC Reconfiguration message, UE’s RRC layer immediately submits inference configuration of periodic CSI to lower layer, regardless of whether the configuration is applicable/inapplicable. • Option 2: Upon reception of RRC Reconfiguration message, UE’s RRC layer submits inference configuration of periodic CSI to lower layer only if it is reported as applicable in RRCReconfigurationComplete. From RAN1 perspective, Option 2 can resolve the issue. No additional RAN 1 impact was identified for Option 2.”

RAN2 needs to discuss what specification changes are needed based on RAN1’s reply. We note that how to handle inapplicable/inactive periodic CSI report configurations for inference have been causing many issues during Rel-19 and RAN2 has already spent many meetings in trying to find solutions to support this case. In our view, in practice it would be much better to use semi-persistent CSI reporting instead, to achieve the wanted UE behavior (i.e. periodic reporting), while at the same time make use of the legacy framework which allows activation/deactivation of semi-persistent CSI report configurations. Thus, using periodic CSI report configurations for inference is a corner case and RAN2 should not make optimizations/exceptions for this case that change much the legacy behavior for periodic CSI configurations. Furthermore, handling the periodic CSI report configurations for inference should not impact negatively the procedures and performance for semi-persistent or aperiodic CSI report configurations, which are more relevant in practice than periodic CSI configurations.

To achieve this, we think there are two possible ways forward:

1) Periodic CSI report configurations for inference are not supported. In our view this is the cleanest solution, given all the issues that RAN2 has been spending a lot of time on. However, RAN1 would have to be informed about this solution, since RAN1 specifications could be impacted.

2) When a UE receives a periodic CSI configuration, it reports it to the lower layers, if it is applicable. If it is inapplicable, the periodic configuration is not reported to the lower layers (compliant with RAN1’s reply) and the UE releases it (compliant with the legacy principle that a periodic CSI report configuration cannot be “inactive”). The NW finds out about it by receiving the status “inapplicable” in the applicability report. We see no point in the UE keeping the inapplicable periodic CSI configuration, which is anyway inactive and can be activated only via re-configuring it via RRCReconfiguration. This behavior would just mimic semi-persistent activation with an RRCReconfiguration instead of a MAC CE. We do not see any benefit of it over semi-persistent configurations; on the contrary, it comes with additional problems.

We also note that the inference configurations can be configured at the RRC resume procedure and that the UE then reports applicability in the RRCResumeComplete message. Similar updates to the handling of periodic CSI report configurations, which are introduced for the RRC reconfiguration procedure should therefore be considered also for the RRC resume procedure.

(E065) RAN2 to select one of the following two options for handling periodic CSI report configurations:
a) Periodic CSI report configurations for inference are not supported;
b) When a UE receives a periodic CSI configuration, it reports it to the lower layers, if it is applicable. If it is inapplicable, the periodic configuration is not reported to the lower layers and the UE releases it.

(E065) RAN2 also to take the RRC resume procedure into account in the changes for handling of inapplicable periodic CSI report configurations

## 2.2 (S050) UAI retransmission after HO/CHO/LTM cell switch

In RIL S050 an issue regarding the retransmission of UEAssistanceInformation (UAI) message with applicability reporting, in the target cell after a handover/cell switch, was raised.

In 5.3.5.3 it is specified that the UE, after a HO, CHO or LTM cell switch, initiates a UAI transmission to the target cell if one was sent to the source cell during the last 1 second (in case of HO execution) or since it was configured with the CHO or LTM (in case of CHO or LTM execution). The reporting of applicability status of configurations in UAI consists of reporting changes in applicability compared to the last transmission of a message containing an applicability report (in an RRCReconfigurationComplete, UEAssistanceInformation or RRCResumeComplete).

The proposal in S050 is to add an exception in 5.3.5.3 so that the UAI is not transmitted in the above case if it was initiated for applicability reporting in the source cell. As can be seen below, the corresponding procedure in 5.3.5.3 consists of that the transmission of UAI is initiated in accordance with clause 5.7.4.3:

[…]

3>	if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution or an LTM cell switch procedure, and the UE is configured to provide UE assistance information for the corresponding cell group, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.7.4.2:

4>	initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information;

4>	start or restart the prohibit timer (if exists) associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration;

4>	start or restart the leave without response timer (if exists) with the timer value set to the value in the musim-LeaveAssistanceConfig or the wait timer (if exists) with the timer value set to the value in musim-CapabilityRestrictionConfig;

[…]

According to the procedure in clause 5.7.4.3, the applicability information will however only be included in the UEAssistanceInformation message if the procedure was triggered from 5.7.4.2 (i.e. not from 5.3.5.3):

[…]

1>	if transmission of the UEAssistanceInformation message is initiated to report assistance information about the applicability of configurations subject to applicability determination procedure according to 5.7.4.2:

2>	include applicabilityReportList in this UEAssistanceInformation message;

[…]

There will thus not be any applicability reporting information in the UAI to the target cell, triggered by a late transmission in the source cell. The proposed change in S050 is thus not needed and we therefore propose that it is rejected.

(S050) The trigger of UAI transmission in 5.3.5.3 does not lead to any inclusion of applicability reporting in that UAI.

(S050) RAN2 to not make any changes for trigger of UAI transmission in 5.3.5.3 (i.e. solution in S050 rejected).

## 2.3 (H015) Remove unnecessary configuration in Report configuration type in ApplicabilitySetConfig-r19

RAN2#131bis agreed to reject RIL H010 and RIL H015 proposes an alternative solution to that in H010. H010 was proposing to reduce the content of reportConfigType within ApplicabilitySetConfig-r19 (in otherConfig) for applicability reporting with Option B (i.e. sets of inference related parameters). As an alternative, H015 proposes to make some of the content of reportConfigType optional.

We note that RAN1 provided the list of parameters to be configured for Option B for beam management, which included reportConfigType, without mentioning that some of its content should be removed, and without mentioning any other problem/overhead regarding this parameter. Thus, since this list of parameters is in the scope of RAN1, RAN2 should not directly modify its content. If RAN2 thinks that a change may be needed, RAN2 should instead check with RAN1 whether some content of reportConfigType should be removed or changed (e.g. turned into optional) for Option B.

(H015) If RAN2 identifies that a change may be needed for the content of reportConfigType within otherConfig (i.e. for applicability reporting based on sets of inference related parameters - Option B), RAN2 to ask RAN1 whether the change can be made.

## 2.4 (N079) UE side candidate data collection request

RAN2# 131bis agreed the following:

1 [X003]: Change the name of the “start” indication to a “request” indication. The condition of sending this “request” indication (if it is configured by NW to do so) is the UE has no candidate list. Update the procedural text in RRC to reflect this condition.

RIL N079 discusses the following related issue. If the NW does not configure a list of candidate UE-side data collection configurations, the NW needs to configure an empty IE DataCollectionPreferenceConfig with SetupRelease, in order to enable the UE to send a simple “request” indication (i.e. the flag dataCollectionRequest-r19) to the NW:

dataCollectionPreferenceConfig-r19 SetupRelease {DataCollectionPreferenceConfig-r19} OPTIONAL, -- Need M ... DataCollectionPreferenceConfig-r19 ::= SEQUENCE { dataCollectionCandidateConfigToAddModList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateConfig-r19 OPTIONAL, -- Need N dataCollectionCandidateConfigToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex OPTIONAL, -- Need N ... }

RIL N079 claims that in this case (if DataCollectionPreferenceConfig is empty) the setup procedure is unclear and proposes to introduce a new flag within DataCollectionPreferenceConfig, to enable the UE to send a UE data collection request flag dataCollectionRequest, when the UE is not provided with any candidate UE-side data collection configuration by the network. In other words, the DataCollectionPreferenceConfig would not be empty anymore

In our view, the proposed solution in N079 is not needed. The solution to allow the DataCollectionPreferenceConfig to be empty works, as long as the procedural text clarifies what the UE behaviour is in this case, which was already captured in the CR:

#### 5.7.4.3 Actions related to transmission of UEAssistanceInformation message ... 1> if transmission of the UEAssistanceInformation message is initiated to report the UE preference to be configured or stop being configured with radio resources to perform UE-side data collection according to 5.7.4.2: 2> include dataCollectionPreference in this UEAssistanceInformation message; 2> if the UE prefers to be configured with radio resources to perform data collection: 3> if the UE is configured with dataCollectionPreferenceConfig, but has no stored candidate configurations for UE-side data collection: 4> set dataCollectionRequest to true;

Thus, we do not think that any change is needed.

(N079) No extra flag is needed in DataCollectionPreferenceConfig to allow the UE to provide the dataCollectionRequest when the list of candidate UE-side data collection configurations is not configured. N079 is rejected.

# Conclusion

In the previous sections we made the following observations:

Observation 1	(S050) The trigger of UAI transmission in 5.3.5.3 does not lead to any inclusion of applicability reporting in that UAI.

Based on the discussion in the previous sections we propose the following:

Proposal 1	(E065) RAN2 to select one of the following two options for handling periodic CSI report configurations: a) Periodic CSI report configurations for inference are not supported; b) When a UE receives a periodic CSI configuration, it reports it to the lower layers, if it is applicable. If it is inapplicable, the periodic configuration is not reported to the lower layers and the UE releases it.

Proposal 2	(E065) RAN2 also to take the RRC resume procedure into account in the changes for handling of inapplicable periodic CSI report configurations

Proposal 3	(S050) RAN2 to not make any changes for trigger of UAI transmission in 5.3.5.3 (i.e. solution in S050 rejected).

Proposal 4	(H015) If RAN2 identifies that a change may be needed for the content of reportConfigType within otherConfig (i.e. for applicability reporting based on sets of inference related parameters - Option B), RAN2 to ask RAN1 whether the change can be made.

Proposal 5	(N079) No extra flag is needed in DataCollectionPreferenceConfig to allow the UE to provide the dataCollectionRequest when the list of candidate UE-side data collection configurations is not configured. N079 is rejected.