3GPP TSG-RAN WG2 Meeting #132											    R2-2509168

Dallas, the US, Nov.17th-21th

Agenda Item:	8.5.2

Source:	vivo

Title:	Remaining issues of NES

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the remaining open issues of R19 NES.

Discussion

[H130] whether a UE receives DCI2-9 from PCell for SSB adaptation status for deactivated Scells

[H130] raised the issue that according to [1], the MAC entity shall not monitor the PDCCH for the SCell when an SCell is deactivated, which is intended to restrict the presence condition of adap-SSB-Config-r19:

Ts 38.321 V19.0.0 5.9 Activation/Deactivation of SCells The MAC entity shall for each configured SCell: 1> if the SCell is deactivated: 2> not transmit SRS on the SCell; 2> not report CSI for the SCell; 2> not transmit on UL-SCH on the SCell; 2> not transmit on RACH on the SCell; 2> not monitor the PDCCH on the SCell; 2> not monitor the PDCCH for the SCell;

[Proposed Change of H130] od-ssb-r19 SetupRelease {OD-SSB-r19} OPTIONAL, -- Need M adap-SSB-Config-r19 SetupRelease {Adap-SSB-Config-r19} OPTIONAL -- Cond SSB-AdaptM Conditional Presence; Explanation. SCG; The field is mandatory present in an SpCellConfig for the PSCell. It is absent otherwise.. SCellSIB20-Opt; This field is optionally present, Need M, if the field sCellSIB20 is configured. It is absent otherwise.. SCG-Opt; The field is optionally present, Need M, in an SpCellConfig for the PSCell. It is absent otherwise.. SSB-Adapt; The field is optionally present, Need M, for an activated SCell. It is absent otherwise..

During last RAN2 meeting, companies had diverse opinions on whether a UE receives DCI2-9 from PCell for SSB adaptation status for deactivated Scells. It was agreed to check the understanding in this meeting.

To better understand the issue, we suggest to discuss it in two steps:

- Whether to support SSB adaptation on SCell when the SCell is deactivated?

- How does the UE monitor DCI 2_9 from PCell, and whether it is applied for a deactivated SCell?

Whether to support SSB adaptation on SCell when the SCell is deactivated?

Up to now, there is no consensus on whether SSB adaptation is supported for a deactivated SCell according to the RAN1/2/4 progress. However, RAN4 previously agreed that not to define requirement for DCI based SSB adaption for deactivated SCell measurement [2]:

RAN4 #116 Agreement (202508) 1.1 Sub-topic 1-1 SSB adaptation Agreement (on Thursday): RAN4 not to define requirement for DCI based SSB adaption for deactivated SCell measurement and SCell activation. - Note: this RAN4 agreement does not intend to impact the discussion and decision in other WGs.

From our point of view, we suggest to confirm not to support SSB adaptation for deactivated SCell from RAN2 point of view. Firstly, RAN4 has not yet defined any requirement for supporting this use case. If RAN2 suggests supporting this feature, further RAN4 work is needed for defining the requirement. We do not prefer further work to involve RAN4 at this late Rel-19 stage. Moreover, RAN1 needs to discuss whether the timeline is the same as the case for activated SCell. Secondly, we observe there may be some further RAN2 issues regarding how the SSB adaptation for deactivated SCell poses impact on the SSB measurement. Finally, we understand OD-SSB mechanism for SCell can cover the use case for deactivated SCell, and thus there is no strong motivation to pursue supporting SSB adaptation for deactivated SCell.

Observation 1: RAN4 agreed not to define requirement for DCI based SSB adaptation for deactivated SCell measurement and SCell activation.

Observation 2: OD-SSB activation/deactivation on deactivated SCell has been supported to cover the need for further supporting SSB adaptation on deactivated SCell.

Observation 3: Further RAN1/2/4 specification work is needed for supporting SSB adaptation on deactivated SCell.

In sum, we propose RAN2 to confirm not to support DCI based SSB adaptation for deactivated SCell from RAN2 point of view.

Proposal 1: RAN2 to confirm not to support DCI based SSB adaptation for deactivated SCell from RAN2 point of view.

How does the UE monitor DCI 2_9 from PCell, and whether it is applied for a deactivated SCell?

If proposal 1 is agreed, RAN2 needs to further discuss whether the UE monitors DCI 2_9 from PCell for SSB adaptation on SCell and whether SSB adaptation indication can be applied for a deactivated SCell. Our understanding is

- When there is at least one activated SCell configured with adap-SSB-Config-r19, the UE should monitor DCI 2_9 on PCell or on the activated SCell even if the other SCell(s) configured with adap-SSB-Config-r19 are deactivated. For example, if the UE is configured with two SCells, and both of them are configured with adap-SSB-Config-r19. When SCell X is active and SCell Y is deactivated, the UE may monitor DCI 2_9 on PCell for the activated SCell X;

- When all SCells configured with adap-SSB-Config-r19 are deactivated, whether the UE monitors DCI 2_9 can be left to UE implementation. If the UE monitors and decodes DCI 2_9, the SSB adaptation for deactivated SCells should be ignored by the UE, if indicated.

Therefore, we propose two alternatives to address the concern that the UE may falsely apply the SSB adaptation for a deactivated SCell, if RAN2 agrees that SSB-adaptation is not supported for deactivated SCell:

- Alt1: It is up to NW implementation whether to configure adap-SSB-Config-r19 for deactivated SCell. RAN2 confirms the understanding that the legacy UE behavior in MAC ‘not monitor the PDCCH for the deactivated SCell’  means if the UE receives DCI 2_9 for SSB adaptation on a deactivated SCell, the UE ignores it;

- Alt2: Clarify in stage-2 spec that SSB-adaptation is not supported for deactivated SCell.

We think whether to configure adap-SSB-Config-r19 for deactivated SCell is up to the NW implementation. There’s no strong motivation to restrict that the NW can only configure it for activated SCell.

Proposal 2: If RAN2 agrees that SSB-adaptation is not supported for deactivated SCell, discuss two alternatives to address the concern that the UE may falsely apply the SSB adaptation for a deactivated SCell:

- Alt1: It is up to NW implementation whether to configure adap-SSB-Config-r19 for deactivated SCell. RAN2 confirms the understanding that the legacy UE behavior in MAC ‘not monitor the PDCCH for the deactivated SCell’  means if the UE receives DCI 2_9 for SSB adaptation on a deactivated SCell, the UE ignores it;

- Alt2: Clarify in stage-2 spec that SSB-adaptation is not supported for deactivated SCell;

[C184] The subset relationship between ssb-ToMeasure and od-ssb-PositionsInBurst

During the ASN.1 review procedure, companies discussed the impact of od-ssb-PositionsInBurst on ssb-ToMeasure and the key controversies about the proposal comes from several alternatives:

- Alt1: It is recommended to clarify that OD-SSB-PositionsInBurst is configured as a subset of ssb-ToMeasure within the associated MeasObjectNR should be added to the field description of the OD-SSB-PositionsInBurst [3].

[Proposed Change of C184] od-ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks for OD-SSB in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. If absent, UE applies the value ssb-PositionsInBurst provided in ServingCellConfigCommon. It is a subset of ssb-ToMeasure within the associated MeasObjectNR.

- Alt2: OD-SSB specific MO includes values for ssb-ToMeasure to match the OD-SSB activated by the MAC CE [4].

Regarding Alt2, the proponent clarified that the intention here is ssb-ToMeasure shall be configured per od-ssb-r19 candidate configuration.

From our understanding, when the NW configures the ssb-ToMeasure for a certain frequency, it has taken both AO-SSB and OD-SSB of the serving cell into consideration. This is similar to the legacy case where the cell may only be configured with AO-SSB, and the NW guarantees that ssb-ToMeasure covers ssb-PositionsInBurst of AO-SSB.

Considering that the legacy ssb-ToMeasure field description does not contain any specific description to restrict its relation with ssb-PositionsInBurst of AO-SSB, neither do we observe a strong need to clarify it for OD-SSB. Therefore, we propose RAN2 to confirm the understanding that it is up to the NW to guarantee ssb-ToMeasure within the associated MeasObjectNR covers all the candidate od-ssb-PositionsInBurst in od-ssb-r19 in the same way as legacy ssb-PositionsInBurst of AO-SSB.

Observation 4: For legacy AO-SSB on SCell, it is up to the NW to guarantee ssb-ToMeasure within the associated MeasObjectNR covers ssb-PositionsInBurst of AO-SSB. No spec change is defined to restrict the relation between ssb-ToMeasure and ssb-PositionsInBurst of AO-SSB.

Proposal 3: It is up to the NW to guarantee ssb-ToMeasure within the associated MeasObjectNR covers all the candidate od-ssb-PositionsInBurst in od-ssb-r19 in the same way as legacy ssb-PositionsInBurst of AO-SSB.

[V503][J002][E023] UE capability for the UE taking other cells for timing reference in OD-SSB case#1

Background

During the RAN2 #131bis meeting [5], it was agreed that for OD-SSB case 1, if od-ssb-r19 is absent in SCellConfig, or if absoluteFrequencySSB is absent and od-ssb-r19 is present in SCellConfig but OD-SSB is not activated, the UE can take other cells for timing reference, if applicable:

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent and od-ssb-r19 is absent in SCellConfig, or if the field is absent and od-ssb-r19 is present in SCellConfig but OD-SSB is not activated, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For PCell, this field corresponds to the CD-SSB.

Ambiguity issue

The prerequisite for a UE to obtain timing reference from an SSB-less referenceCell or an OD-SSB is that, RAN2 assumes the UE can simultaneously support SSB-less operation and OD-SSB operation. However, SSB-less operation in Rel-15/18 and OD-SSB operation in Rel-19 are separate UE capabilities, so the above assumption maybe not guaranteed. As such, there is ambiguity issue here, and hence it is necessary to introduce separate new UE capabilities to support the expected UE behavior.

To clarify the issue here, we show a Table in Annex 5.3 for further discussion. The current UE capabilities related to this issue is listed as below [6]:

Corresponding UE capability of SSB-less SCell

scellWithoutSSB Defines whether the UE supports configuration of SCell that does not transmit SS/PBCH block for intra-band CA. This is conditionally mandatory with capability signalling for intra-band CA.

scellWithoutSSB-InterBandCA-r18 Indicates whether the UE supports SCell without SS/PBCH block for inter-band CA. <Omit some text for demonstration simplicity>

Corresponding UE capability of OD-SSB case#1

od-SSB-NoAlwaysOn-MAC-CE-r19 Indicates whether the UE supports MAC CE based signalling to indicate activation, adaptation, and deactivation of on-demand SSB transmission on the SCell in Case #1, i.e., no always-on SSB on the cell. <Omit some text for demonstration simplicity>

od-SSB-NoAlwaysOn-RRC-r19 Indicates whether the UE supports RRC based signalling to indicate activation and deactivation of on-demand SSB transmission on the SCell in Case #1, i.e., no always-on SSB on the cell.

od-SSB-NoAlwaysOn-RRC-MAC-CE-r19 Indicates whether the UE supports RRC based signalling to indicate activation and MAC CE based signalling to indicate adaptation and deactivation of on-demand SSB transmission on the SCell in Case #1, i.e., no always-on SSB on the cell. A UE supporting this feature shall also indicate support of od-SSB-NoAlwaysOn-RRC-r19 and od-SSB-NoAlwaysOn-MAC-CE-r19.

Since the support of the SSB-less operation in Rel-15/18 and OD-SSB operation in Rel-19 are in separate UE capabilities, the UE support Rel-15/18 intra/inter-band SSB-less SCell operation applies the legacy UE behavior to obtain timing reference. In other words, the OD-SSB operation capable UE which indicates the support of legacy intra/inter-band SSB-less SCell operation capabilities does not apply the behavior described in the current field description of absoluteFrequencySSB as what we expect, to check whether od-ssb-r19 is absent in SCellConfig, or whether absoluteFrequencySSB is absent and od-ssb-r19 is present in SCellConfig but OD-SSB is not activated.

Observation 5: Since the support of the SSB-less SCell and OD-SSB on SCell are in separate UE capabilities, the OD-SSB capable UE which also indicates supporting SSB-less SCell operation, e.g. scellWithoutSSB-InterBandCA-r18, does not check whether od-ssb-r19 is absent in SCellConfig, or whether absoluteFrequencySSB is absent and od-ssb-r19 is present in SCellConfig but OD-SSB is not activated when it needs other cells for timing reference.

Besides, the OD-SSB operation capable UE that does not indicate the support of intra/inter-band SSB-less SCell operation, can not take other cells for timing reference when OD-SSB is not configured, or configured but not activated. Therefore, we cannot rely on only changing the field description of the OD-SSB UE capability to say that a UE supporting OD-SSB case#1 also naturally supports SSB-less SCell operation.

Observation 6: The OD-SSB capable UE that does not indicate the support of intra/inter-band SSB-less SCell operation, can not take other cells for timing reference when OD-SSB is not configured, or configured but not activated.

In sum, we propose RAN2 to discuss introducing two new UE capabilities to indicate the support of: 1) intra-band SSB-less SCell&OD-SSB on SCell; 2) inter-band SSB-less SCell&OD-SSB on SCell, respectively. We provide the corresponding TPs for TS 38.306/38.331 in Annex 5.1/5.2.

Proposal 4: Introduce two new UE capabilities to indicate the support of: 1) intra-band SSB-less SCell&OD-SSB on SCell; 2) inter-band SSB-less SCell&OD-SSB on SCell; respectively. TPs for TS 38.306/38.331 are provided in Annex 5.1/5.2.

[V504] Correction of IE ul-SubCarrierSpacing-r19

During the ASN.1 review procedure, the conditional presence of IE ul-SubCarrierSpacing is captured as ‘OPTIONAL, -- Need R’ in SIB26 information element.

SIB26 information element

SIB1-RequestConfig-r19 ::=              SEQUENCE {

ul-FrequencyBandList-r19                 MultiFrequencyBandListNR-SIB                                          OPTIONAL, -- Need R

ul-SubCarrierSpacing-r19                 ENUMERATED {kHz15, kHz30, kHz60, kHz120,kHz240, kHz480, kHz960, spare1}       OPTIONAL, -- Need R

p-Max-r19                                P-Max                                                                 OPTIONAL, -- Need R

sib1-restrictedSetConfig-r19             ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB, spare1}

}

The field description of IE ul-SubCarrierSpacing indicates as below. We understand that the IE ul-SubCarrierSpacing-r19 is necessary for determining carrier and BWP.

SIB1-RequestConfig field descriptions

ul-SubCarrierSpacing Indicates the reference SCS for msg1-FrequencyStart-r19, offsetToCarrier-r19, carrierBandwidth-r19, and locationAndBandwidth-r19.

Therefore, we suggest that ul-SubCarrierSpacing-r19 should be a mandatory IE.

Observation 7: The IE ul-SubCarrierSpacing-r19 should be a mandatory configuration, as it is necessary for other parameters msg1-FrequencyStart-r19, offsetToCarrier-r19, carrierBandwidth-r19, and locationAndBandwidth-r19.

Proposal 5: Adopt the TP3 in annex 5.4 to remove ‘OPTIONAL’ for the IE ul-SubCarrierSpacing-r19.

[V505] Correction of IE ul-FrequencyBandList-r19

During the ASN.1 review procedure, the conditional presence of IE ul-FrequencyBandList-r19 is captured as ‘Need R’ currently in SIB26 information element.

SIB26 information element

SIB1-RequestConfig-r19 ::=              SEQUENCE {

ul-FrequencyBandList-r19                 MultiFrequencyBandListNR-SIB                                          OPTIONAL, -- Need R

ul-SubCarrierSpacing-r19                 ENUMERATED {kHz15, kHz30, kHz60, kHz120,kHz240, kHz480, kHz960, spare1}       OPTIONAL, -- Need R

p-Max-r19                                P-Max                                                                 OPTIONAL, -- Need R

sib1-restrictedSetConfig-r19             ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB, spare1}

}

According to the agreements from RAN1 as below,  frequencyBandList refers to IE FrequencyInfoUL-SIB for FDD. Therefore, the need code for ul-FrequencyBandList-r19 should be conditionally present for FDD, i.e. ‘Cond FDD’ rather than ‘Need R’, to accurately reflect the RAN1 agreements. We propose to adopt the TP4 in annex 5.5 for this issue.

Agreement@RAN1 120-bis From RAN1 perspective, for agreed UL WUS parameters, regarding their mandatory or optional presence and applicability to TDD and/or FDD, adopt the followings: - PhysCellId and ARFCN-ValueNR are mandatory - frequencyBandList and absoluteFrequencyPointA are present in IE FrequencyInfoUL for FDD (as in the legacy specification) - K_SSB is mandatory - searchSpaceZero and controlResourceSetZero are mandatory - ra-PreambleStartIndex, od-sib1-duration, offsetToTimeWindow are mandatory Agreement@RAN1 120-bis RAN1 clarifies frequencyBandList in UL WUS configuration refers to the IE within FrequencyInfoUL-SIB.

Observation 8: According to RAN1 agreement, the IE ul-FrequencyBandList-r19 refers to IE FrequencyInfoUL-SIB for FDD, similar to that of absoluteFrequencyPointA-r19. The need code for ul-FrequencyBandList-r19 should be ‘Cond FDD’ rather than ‘Need R’.

Proposal 6: Adopt the TP4 in annex 5.5 to change the need code for ul-FrequencyBandList-r19 to ‘Cond FDD’.

Conclusion

Based on the discussion, we have the following observation and proposals:

[H130] whether a UE can receive DCI2-9 from Pcell for SSB adaptation status for deactivated Scells

Observation 1: RAN4 agreed not to define requirement for DCI based SSB adaptation for deactivated SCell measurement and SCell activation.

Observation 2: OD-SSB activation/deactivation on deactivated SCell has been supported to cover the need for further supporting SSB adaptation on deactivated SCell.

Observation 3: Further RAN1/2/4 specification work is needed for supporting SSB adaptation on deactivated SCell.

Proposal 1: RAN2 to confirm not to support DCI based SSB adaptation for deactivated SCell from RAN2 point of view.

Proposal 2: If RAN2 agrees that SSB-adaptation is not supported for deactivated SCell, discuss two alternatives to address the concern that the UE may falsely apply the SSB adaptation for a deactivated SCell:

- Alt1: It is up to NW implementation whether to configure adap-SSB-Config-r19 for deactivated SCell. RAN2 confirms the understanding that the legacy UE behavior in MAC ‘not monitor the PDCCH for the deactivated SCell’  means if the UE receives DCI 2_9 for SSB adaptation on a deactivated SCell, the UE ignores it;

- Alt2: Clarify in stage-2 spec that SSB-adaptation is not supported for deactivated SCell;

[C184] The subset relationship between ssb-ToMeasure and od-ssb-PositionsInBurst

Observation 4: For legacy AO-SSB on SCell, it is up to the NW to guarantee ssb-ToMeasure within the associated MeasObjectNR covers ssb-PositionsInBurst of AO-SSB. No spec change is defined to restrict the relation between ssb-ToMeasure and ssb-PositionsInBurst of AO-SSB.

Proposal 3: It is up to the NW to guarantee ssb-ToMeasure within the associated MeasObjectNR covers all the candidate od-ssb-PositionsInBurst in od-ssb-r19 in the same way as legacy ssb-PositionsInBurst of AO-SSB.

[V503][J002][E023] Relation between OD-SSB case 1 & SSB-less SCell

Observation 5: Since the support of the SSB-less SCell and OD-SSB on SCell are in separate UE capabilities, the OD-SSB capable UE which also indicates supporting SSB-less SCell operation, e.g. scellWithoutSSB-InterBandCA-r18, does not check whether od-ssb-r19 is absent in SCellConfig, or whether absoluteFrequencySSB is absent and od-ssb-r19 is present in SCellConfig but OD-SSB is not activated when it needs other cells for timing reference.

Observation 6: The OD-SSB capable UE that does not indicate the support of intra/inter-band SSB-less SCell operation, can not take other cells for timing reference when OD-SSB is not configured, or configured but not activated.

Proposal 4: Introduce two new UE capabilities to indicate the support of: 1) intra-band SSB-less SCell&OD-SSB on SCell; 2) inter-band SSB-less SCell&OD-SSB on SCell; respectively. TPs for TS 38.306/38.331 are provided in Annex 5.1/5.2.

[V504] Correction of IE ul-SubCarrierSpacing-r19

Observation 7: The IE ul-SubCarrierSpacing-r19 should be a mandatory configuration, as it is necessary for other parameters msg1-FrequencyStart-r19, offsetToCarrier-r19, carrierBandwidth-r19, and locationAndBandwidth-r19.

Proposal 5: Adopt the TP3 in annex 5.4 to remove ‘OPTIONAL’ for the IE ul-SubCarrierSpacing-r19.

[V505] Correction of IE ul-FrequencyBandList-r19

Observation 8: According to RAN1 agreement, the IE ul-FrequencyBandList-r19 refers to IE FrequencyInfoUL-SIB for FDD, similar to that of absoluteFrequencyPointA-r19. The need code for ul-FrequencyBandList-r19 should be ‘Cond FDD’ rather than ‘Need R’.

Proposal 6: Adopt the TP4 in annex 5.5 to change the need code for ul-FrequencyBandList-r19 to ‘Cond FDD’.

References

- Ts 38.321, V19.0.0

- R4-2512137	WF on RRM requirements for Netw_Energy_NR_enh_Part2 v1

- R2-2506817	[C184] Impact of od-ssb-PositionsInBurst on ssb-ToMeasure

- R2-2507334	Discussion on RILS E023, E024, E025, X201, O006, J002, H128, H129, J005, Z101, Z102, V503

- R2-2507703	Report from Break-out session on NES, NR-NTN and IoT-NTN

- Ts 38.306, V19.0.0

Annex

TP1 to TS 38.306 for proposal 4

4.2.7.2	BandNR parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| od-SSB-ScellWithout-AO-SSB-IntraBandCA-r19 Defines whether the UE supports obtaining timing reference from the intra-band SpCell or intra-band SCell for an SCell in on-demand SSB transmission Case #1, i.e., no always-on SSB on the cell, for intra-band CA. A UE supporting this feature shall also indicate support of scellWithoutSSB and at least one of od-SSB-NoAlwaysOn-RRC-r19, od-SSB-NoAlwaysOn-MAC-CE-r19, or od-SSB-NoAlwaysOn-RRC-MAC-CE-r19. | FS | No | N/A | N/A |
| od-SSB-ScellWithout-AO-SSB-InterBandCA-r19 Defines whether the UE supports obtaining timing reference from the inter-band SpCell or inter-band SCell for an SCell in on-demand SSB transmission Case #1, i.e., no always-on SSB on the cell, for inter-band CA. A UE supporting this feature shall also indicate support of scellWithoutSSB-InterBandCA-r18 and at least one of od-SSB-NoAlwaysOn-RRC-r19, od-SSB-NoAlwaysOn-MAC-CE-r19, or od-SSB-NoAlwaysOn-RRC-MAC-CE-r19. | FS | No | N/A | FR1 only |
| scellWithoutSSB Defines whether the UE supports configuration of SCell that does not transmit SS/PBCH block for intra-band CA. This is conditionally mandatory with capability signalling for intra-band CA. | FS | CY | N/A | N/A |
| scellWithoutSSB-InterBandCA-r18 Indicates whether the UE supports SCell without SS/PBCH block for inter-band CA. For each band within the band combination, UE indicates if it supports the inter-band SSB-less SCell operation with supportOfSingleGroup or supportOfMultipleGroups: - For supportOfSingleGroup, the band indicated as 'referenceBand' can be configured as the reference band for all other band(s) indicated as 'scellWithoutSSB'. The band indicated as 'both' can be configured as either a reference band or an SSB-less band. If the UE indicates "both" for any band, the UE shall not indicate 'referenceBand' or 'scellWithoutSSB' in any other band in the band combination. - For supportOfMultipleGroups, the band indicated as 'referenceBand1' can be configured as the reference band for all other band(s) indicated as 'scellWithoutSSB1', and the band indicated as 'referenceBand2' can be configured as the reference band for all other band(s) indicated as 'scellWithoutSSB2'. If the field scellWithoutSSB-InterBandCA-r18 is absent for a band, this band is not involved in the inter-band SSB-less SCell operation. If the inter-band SSB-less SCell operation is supported between two bands, it is understood that there is no direction between the two bands, which means that the network can configure either band as the reference band and the other band as the SSB-less band. | FS | No | N/A | FR1 only |
| od-SSB-NoAlwaysOn-MAC-CE-r19 Indicates whether the UE supports MAC CE based signalling to indicate activation, adaptation, and deactivation of on-demand SSB transmission on the SCell in Case #1, i.e., no always-on SSB on the cell. Value ‘explicit’ indicates UE supports explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication. Value ‘both’ indicates UE supports both explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication and implicit deactivation via od-ssb-nrofBurst of on-demand SSB bursts to be transmitted after on-demand SSB is indicated. | Band | No | N/A | N/A |
| od-SSB-NoAlwaysOn-RRC-r19 Indicates whether the UE supports RRC based signalling to indicate activation and deactivation of on-demand SSB transmission on the SCell in Case #1, i.e., no always-on SSB on the cell. | Band | No | N/A | N/A |
| od-SSB-NoAlwaysOn-RRC-MAC-CE-r19 Indicates whether the UE supports RRC based signalling to indicate activation and MAC CE based signalling to indicate adaptation and deactivation of on-demand SSB transmission on the SCell in Case #1, i.e., no always-on SSB on the cell. A UE supporting this feature shall also indicate support of od-SSB-NoAlwaysOn-RRC-r19 and od-SSB-NoAlwaysOn-MAC-CE-r19. | Band | No | N/A | N/A |

TP2 to TS 38.331 for proposal 4

6.3.3 UE capability information elements

–	FeatureSetDownlink

The IE FeatureSetDownlink indicates a set of features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetDownlink information element

-- ASN1START

-- TAG-FEATURESETDOWNLINK-START

FeatureSetDownlink ::=                  SEQUENCE {

featureSetListPerDownlinkCC             SEQUENCE (SIZE (1..maxNrofServingCells)) OF FeatureSetDownlinkPerCC-Id,

intraBandFreqSeparationDL               FreqSeparationClass                                                     OPTIONAL,

scalingFactor                           ENUMERATED {f0p4, f0p75, f0p8}                                          OPTIONAL,

dummy8                                  ENUMERATED {supported}                                                  OPTIONAL,

scellWithoutSSB                         ENUMERATED {supported}                                                  OPTIONAL,

csi-RS-MeasSCellWithoutSSB              ENUMERATED {supported}                                                  OPTIONAL,

dummy1                                  ENUMERATED {supported}                                                  OPTIONAL,

type1-3-CSS                             ENUMERATED {supported}                                                  OPTIONAL,

pdcch-MonitoringAnyOccasions            ENUMERATED {withoutDCI-Gap, withDCI-Gap}                                OPTIONAL,

dummy2                                  ENUMERATED {supported}                                                  OPTIONAL,

ue-SpecificUL-DL-Assignment             ENUMERATED {supported}                                                  OPTIONAL,

searchSpaceSharingCA-DL                 ENUMERATED {supported}                                                  OPTIONAL,

timeDurationForQCL                      SEQUENCE {

scs-60kHz                           ENUMERATED {s7, s14, s28}                                               OPTIONAL,

scs-120kHz                          ENUMERATED {s14, s28}                                                   OPTIONAL

}                                                                                                           OPTIONAL,

<Omitted text>

FeatureSetDownlink-v1900 ::=      SEQUENCE {

-- R1 59-4-4a: Support two TAs enhancement for intra-cell beam management

twoTA-IntraCellBM-r19             ENUMERATED {supported}                                                                OPTIONAL,

-- R1 59-4-4b: Support two TAs enhancement for inter-cell beam management operation

twoTA-InterCellBM-r19             ENUMERATED {supported}                                                                OPTIONAL,

-- R1 59-4-4d: PDCCH ordered sent by one TRP triggers RACH procedure towards a different TRP based on CRFA for inter-cell

-- without CORESETPoolIndex

interCellCRFA-r19                 ENUMERATED {supported}                                                                OPTIONAL,

-- R1 59-3-3-1: Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching

srs-AntennaSwitching3T6R2SP-1Periodic-r19                  ENUMERATED {supported}                                       OPTIONAL,

-- R1 59-3-3a-1: Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching

srs-AntennaSwitching3T3R2SP-1Periodic-r19                  ENUMERATED {supported}                                       OPTIONAL

}

FeatureSetDownlink-v19x0 ::=      SEQUENCE {

-- R2 xx-x: Support of obtaining timing reference from the intra-band SpCell or intra-band SCell for an SCell

-- in on-demand SSB transmission Case #1, i.e., no always-on SSB on the cell, for intra-band CA

od-SSB-ScellWithout-AO-SSB-IntraBandCA-r19                 ENUMERATED {supported}                                       OPTIONAL,

-- R2 xx-x: Support of obtaining timing reference from the inter-band SpCell or inter-band SCell for an SCell

-- in on-demand SSB transmission Case #1, i.e., no always-on SSB on the cell, for inter-band CA

od-SSB-ScellWithout-AO-SSB-InterBandCA-r19                 ENUMERATED {supported}                                       OPTIONAL

}

<Omitted text>

-- TAG-FEATURESETDOWNLINK-STOP

-- ASN1STOP

–	FeatureSets

The IE FeatureSets is used to provide pools of downlink and uplink features sets. A FeatureSetCombination refers to the IDs of the feature set(s) that the UE supports in that FeatureSetCombination. The BandCombination entries in the BandCombinationList then indicate the ID of the FeatureSetCombination that the UE supports for that band combination.

The entries in the lists in this IE are identified by their index position. For example, the FeatureSetUplinkPerCC-Id = 4 identifies the 4th element in the featureSetsUplinkPerCC list.

NOTE:	When feature sets (per CC) IEs require extension in future versions of the specification, new versions of the FeatureSetDownlink, FeatureSetUplink, FeatureSets, FeatureSetDownlinkPerCC and/or FeatureSetUplinkPerCC will be created and instantiated in corresponding new lists in the FeatureSets IE. For example, if new capability bits are to be added to the FeatureSetDownlink, they will instead be defined in a new FeatureSetDownlink-rxy which will be instantiated in a new featureSetDownlinkList-rxy list. If a UE indicates in a FeatureSetCombination that it supports the FeatureSetDownlink with ID #5, it implies that it supports both the features in FeatureSetDownlink #5 and FeatureSetDownlink-rxy #5 (if present). The number of entries in the new list(s) shall be the same as in the original list(s).

FeatureSets information element

-- ASN1START

-- TAG-FEATURESETS-START

FeatureSets ::=    SEQUENCE {

featureSetsDownlink                 SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink               OPTIONAL,

featureSetsDownlinkPerCC            SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC            OPTIONAL,

featureSetsUplink                   SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink                   OPTIONAL,

featureSetsUplinkPerCC              SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC              OPTIONAL,

...,

<Omitted text>

[[

featureSetsDownlink-v1860           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1860         OPTIONAL

]],

[[

featureSetsDownlinkPerCC-v1900      SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1900      OPTIONAL,

featureSetsUplinkPerCC-v1900        SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1900        OPTIONAL,

featureSetsDownlink-v1900           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1900         OPTIONAL,

featureSetsUplink-v1900             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1900             OPTIONAL

]],

[[

featureSetsDownlink-v19x0           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v19x0         OPTIONAL

]]

}

<Omitted text>

-- TAG-FEATURESETS-STOP

-- ASN1STOP

UE capabilities related to obtaining timing reference in OD-SSB case#1

| Scenario | UE indicates support of OD-SSB Case#1 by indicating at least one of od-SSB-NoAlwaysOn-MAC-CE-r19, od-SSB-NoAlwaysOn-RRC-r19, od-SSB-NoAlwaysOn-RRC-MAC-CE-r19? | UE indicates support of SSB-less SCell by indicating at least one of scellWithoutSSB, scellWithoutSSB-InterBandCA-r18? | TBD new UE capabilities | Expected UE behavior | The actual UE behavior which inherits from the support of legacy SSB-less SCell |
| --- | --- | --- | --- | --- | --- |
| #1 in Current V19.0.0 | √ | × | N/A for now | UE obtain timing reference only from OD-SSB on this SCell | UE obtain timing reference only from OD-SSB on this SCell |
| #2 in Current V19.0.0 | × | √ | N/A for now | UE follows legacy absoluteFrequencySSB field description behavior ‘If the absoluteFrequencySSB is absent, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable’ (Note: The NW never configures od-ssb-r19 in this case, and the UE does not check the presence of od-ssb-r19, either. Hence there is no inter-operability issue here) | UE follows legacy absoluteFrequencySSB field description behavior ‘If the absoluteFrequencySSB is absent, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable’ |
| #3 in Current V19.0.0 | √ | √ | N/A for now | UE follows the Rel-19 absoluteFrequencySSB field description behavior ‘If the absoluteFrequencySSB is absent and od-ssb-r19 is absent in SCellConfig, or if absoluteFrequencySSB is absent and od-ssb-r19 is present in SCellConfig but OD-SSB is not activated, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable’ | UE follows legacy absoluteFrequencySSB field description behavior ‘If the absoluteFrequencySSB is absent, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable’ |

TP3 to TS 38.331 for proposal 5

6.3.1	System information blocks

–	SIB26

SIB26 contains information to assist OD-SIB1 acquisition from serving cell or neighbour cells.

SIB26 information element

-- ASN1START

-- TAG-SIB26-START

<Omitted text>

SIB1-RequestConfig-r19 ::=              SEQUENCE {

totalNumberOfRA-Preambles-r19           INTEGER (1..63),

rach-OccasionsSIB1-r19                  SEQUENCE {

ssb-perRACH-Occasion-r19                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

},

sib1-RequestPeriod-r19                  ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}          OPTIONAL, -- Need R

sib1-RequestResources-r19               SIB1-RequestResources-r19,

sib1-PDCCH-RestrictionToPRACH-r19       ENUMERATED {true}                                                      OPTIONAL, -- Need R

rach-ConfigSIB1-r19                     RACH-ConfigGeneric,

ra-SearchSpace-r19                      SearchSpace                                                            OPTIONAL, -- Need S

n-TimingAdvanceOffset-r19               ENUMERATED { n0, n25600, n39936, spare1 }                              OPTIONAL, -- Need R

msg1-SubcarrierSpacing-r19              ENUMERATED {kHz1dot25, kHz5, kHz15, kHz30, kHz60, kHz120, spare2, spare1} OPTIONAL, -- Need R

prach-RootSequenceIndex-r19             CHOICE {

l839                                    INTEGER (0..837),

l139                                    INTEGER (0..137)

},

sib1-rsrp-ThresholdSSB-r19               RSRP-Range                                                            OPTIONAL, -- Need R

rsrp-ThresholdSSB-SUL                    RSRP-Range                                                            OPTIONAL, -- Cond SUL

locationAndBandwidth-r19                 INTEGER (0..37949),

absoluteFrequencyPointA-r19              ARFCN-ValueNR                                                         OPTIONAL, -- Cond FDD

ul-FrequencyBandList-r19                 MultiFrequencyBandListNR-SIB                                          OPTIONAL, -- Need R

ul-SubCarrierSpacing-r19                 ENUMERATED {kHz15, kHz30, kHz60, kHz120,kHz240, kHz480, kHz960, spare1}

p-Max-r19                                P-Max                                                                 OPTIONAL, -- Need R

sib1-restrictedSetConfig-r19             ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB, spare1}

}

<Omitted text>

-- TAG-SIB26-STOP

-- ASN1STOP

TP4 to TS 38.331 for proposal 6

6.3.1	System information blocks

–	SIB26

SIB26 contains information to assist OD-SIB1 acquisition from serving cell or neighbour cells.

SIB26 information element

-- ASN1START

-- TAG-SIB26-START

<Omitted text>

SIB1-RequestConfig-r19 ::=              SEQUENCE {

totalNumberOfRA-Preambles-r19           INTEGER (1..63),

rach-OccasionsSIB1-r19                  SEQUENCE {

ssb-perRACH-Occasion-r19                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

},

sib1-RequestPeriod-r19                  ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}          OPTIONAL, -- Need R

sib1-RequestResources-r19               SIB1-RequestResources-r19,

sib1-PDCCH-RestrictionToPRACH-r19       ENUMERATED {true}                                                      OPTIONAL, -- Need R

rach-ConfigSIB1-r19                     RACH-ConfigGeneric,

ra-SearchSpace-r19                      SearchSpace                                                            OPTIONAL, -- Need S

n-TimingAdvanceOffset-r19               ENUMERATED { n0, n25600, n39936, spare1 }                              OPTIONAL, -- Need R

msg1-SubcarrierSpacing-r19              ENUMERATED {kHz1dot25, kHz5, kHz15, kHz30, kHz60, kHz120, spare2, spare1} OPTIONAL, -- Need R

prach-RootSequenceIndex-r19             CHOICE {

l839                                    INTEGER (0..837),

l139                                    INTEGER (0..137)

},

sib1-rsrp-ThresholdSSB-r19               RSRP-Range                                                            OPTIONAL, -- Need R

rsrp-ThresholdSSB-SUL                    RSRP-Range                                                            OPTIONAL, -- Cond SUL

locationAndBandwidth-r19                 INTEGER (0..37949),

absoluteFrequencyPointA-r19              ARFCN-ValueNR                                                         OPTIONAL, -- Cond FDD

ul-FrequencyBandList-r19                 MultiFrequencyBandListNR-SIB                                          OPTIONAL, -- Cond FDD

ul-SubCarrierSpacing-r19                 ENUMERATED {kHz15, kHz30, kHz60, kHz120,kHz240, kHz480, kHz960, spare1}       OPTIONAL, -- Need R

p-Max-r19                                P-Max                                                                 OPTIONAL, -- Need R

sib1-restrictedSetConfig-r19             ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB, spare1}

}

<Omitted text>

-- TAG-SIB26-STOP

-- ASN1STOP