# 3GPP TSG RAN WG2 Meeting #131b      	                           R2-2507762
Prague, Czech Republic, Oct. 13 – 17, 2025

Agenda item:	8.5.2

Source:	Apple

Title:	Summary report of [AT131bis][302][NES] SSB-less vs OD-SSB case 1 (Apple)

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

This is summary report of the following offline:]

[AT131bis][302][NES] SSB-less vs OD-SSB case 1 (Apple)

Scope: Continue the discussion on RRC open issues 1

Intended outcome: summary of the offline discussion (in R2-2507762)

F2F time: FFS

Deadline for offline discussion summary: Tuesday 2025-10-14 14:00

# 2 Discussion

## 2.1 Online conclusion in Monday session

- RRC issue 1: [E023/E024/E025/H128/J002/O006/V503] The relationship between SSB-less and OD-SSB case 1

R2-2507162	[J001][J002][J005] Discussion on OD-SSB and SSB Adaption	Sharp	discussion	Rel-19	Late

Proposal 3: An OD-SSB case#1 SCell is not an SSB-less SCell.

R2-2507334	Discussion on RILS E023, E024, E025, X201, O006, J002, H128, H129, J005, Z101, Z102, V503 	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

Proposal 1: RAN2 to conclude that Case 1 does not require always on SSB, even in reference cells.

-	HW thinks this is restrictive from NW perspective

Continue in offline 302

R2-2507115	Control plane open issues on Rel-19 NES (including RIL E204/E205/A103/X200/O005)	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Observation 1: Specifications changes to TS 38.331 are needed to differentiate the following two scenarios:

•	Scenario 1 (SSB-less SCell): absoluteFrequencySSB is absent in the concerned SCell, and the UE obtains its timing reference from the SpCell or a SCell in the same or different frequency band.

•	Scenario 2 (OD-SSB case 1): absoluteFrequencySSB is absent in the concerned SCell, but od-ssb-absoluteFrequency-r19 is configured in SCellConfig.

Observation 2: As agreed in RAN2#130, NW may configure ServingCellMO for SSB-less SCell if the UE reports it supports capability intraF-NeighMeasForSCellWithoutSSB. Otherwise, NW doesn’t configure ServingCellMO.

Proposal 1 (E024/E025): RAN2 confirm Table.1 on how to configure different scenarios with SSB-less SCell and OD-SSB SCells.

RAN2 confirms the understanding in Table.1 (with an FFS for Scenario 2) with the understanding that some field description might have to be updated

-	Ericsson wonders if also the field description plays a role (and if we need to change the field description for absoluteFrequencySSB). Apple thinks we need to update the field description

|  | absoluteFrequencySSB in ServingCellConfigCommon | OD-SSB-r19 in SCellConfig | ssbFrequency in ServingCellMO | ServingCellMO-OD |
| --- | --- | --- | --- | --- |
| Scenario 0: SCell associated with AO-SSB only | Present | Absent | Present | Absent |
| Scenario 1-a: SSB-less SCell with ServingCellMO configured | Absent | Absent | Present | Absent |
| Scenario 1-b: SSB-less SCell with ServingCellMO not configured | Absent | Absent | Absent | Absent |
| Scenario 2: OD-SSB case 1 (i.e. only associated with OD-SSB) | Absent | Present, and its od-ssb-absoluteFrequency-r19 is present | Present (same as od-ssb-absoluteFrequency-r19) | Absent |
| Scenario 3: AO-SSB and OD-SSB in same frequency | Present | Present, but its od-ssb-absoluteFrequency-r19 is absent. | Present (same as absoluteFrequencySSB) | Absent |
| Scenario 4: AO-SSB and OD-SSB in different frequency | Present | Present, and its od-ssb-absoluteFrequency-r19 is present and different from absoluteFrequencySSB. | Present (same as absoluteFrequencySSB) | Present, and its ssbFrequency is same as od-ssb-absoluteFrequency-r19 |

Proposal 2 (E024/E025): To differentiate SSB-less SCell and OD-SSB case 1, adopt the following spec changes:

- Update the field description of absoluteFrequencySSB to preclude OD-SSB 1 case1 from SSB-less SCell.

- In field description of ServingCellMO, clarify that ServingCellMO is configured for OD-SSB case 1.

- In presence condition of ssbFrequency (i.e. SSBorAssociatedSSB2) under MeasObjectNR, clarify that ssbFrequency is mandatory present in OD-SSB case 1.

-	HW and vivo don’t agree with the first change. vivo thinks it should refer to the OD-SSB configuration not just the frequency

-	HW and ZTE think we don’t need to discuss the third change based on the decision in the previous meeting

Continue in offline 302

Observation 3: RAN1 is discussing whether the UE can perform SSB-less operation (i.e. obtain timing from SpCell or anther SCell) in OD-SSB case 1, when OD-SSB is not activated. However, in case it is agreed in RAN1, RAN2 just need to add some text in field description of absoluteFrequencySSB (i.e. no need to impact ASN.1).

Proposal 3 (E024/E025): RAN2 wait RAN1 conclusion whether the UE can perform SSB-less operation (i.e. obtain timing from PCell or anther SCell) in OD-SSB case 1, when OD-SSB is not activated.

Continue in offline 302

## 2.2 Proposed way-forward

The different possible cases are listed in Table.1, We need to further confirm Scenario-4 and Scenario-5.

|  | reference cell is present | OD-SSB | How the UE obtains timing |
| --- | --- | --- | --- |
| Scenairo-1 | No | Not configured | UE doesn’t perform serving cell measurement |
| Scenairo-2 | No | deactivated | UE doesn’t perform serving cell measurement |
| Scenario-3 | No | activated | Obtain from OD-SSB |
| Scenario 4 | Yes | Not configured | From reference cell (legacy SSB-less) |
| Scenario-5 | Yes | deactivated | The only feasible option is to obtain timing from SpCell or anther SCell (i.e. SSB-less operation). RAN1 discussed whether to support it in August meeting, but no conclusion is made yet. |
| Scenario-6 | Yes | activated | Obtain from OD-SSB |

It seems common understanding that scenario-4 needs to be supported. Thus, Rapporteur suggest:

Proposal 1: When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is not configured, the UE can obtain timing from SpCell or anther SCell (i.e. SSB-less operation).

RAN1 discussed scenario-5 in August meeting but no conclusion was made. But from RAN2 perspective, it seems to work without further specification. Thus, Rapporteur suggest:

Proposal 2: When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is deactivated, RAN2 assume that the UE can obtain timing from SpCell or anther SCell (i.e. SSB-less operation as legacy Rel-18 behavior). And when OD-SSB is activated, the UE obtains timing reference based on OD-SSB. RAN2 updates CR based on this assumption.

Proposal 3: RAN2 determine whether to send LS to RAN1 to check if any issue in Thursday CB.

If Proposal 1 and Proposal 2 can be agreed, the spec change is to just revise field description of absoluteFrequencySSB. to reflect scenario-4 and scenario-5. Thus, Rapporteur suggest:

Proposal 4: To reflect RAN2 assumption of Proposal 2, RAN2 agree the following CR on field description of absoluteFrequencySSB

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent and od-ssb-r19 is absent in SCellConfig, or if the field is absent and od-ssb-r19 is present in SCellConfig but OD-SSB is not activated, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. if the field is absent and OD-SSB is activated, the UE obtains timing reference based on OD-SSB. For PCell, this field corresponds to the CD-SSB.

Discussion

- On Scenario 5, vivo have different understanding whether to support it and suggest postponing one meeting for RAN1 progress. Add FFS whether support this scenario.

- Huawei disagree with vivo. Not sure whether to combine the case. We should allow the combination of SSB-less and OD-SSB case 1.

- LG prefer to conclude this issue in RAN2. Huawei supported LG to conclude in RAN2.

- Vivo questioned what is the use case to support obtaining timing before OD-SSB is activated? Apple explained that UE may need timing to perform measurement for CSI-RS and/or TRS.

- Vivo further proposed to wait 2 more days for CB.

- Samsung think RAN2 assumption is fine to make progress.

- Ericsson think maybe we can determine whether to send LS to RAN1 at the end of this meeting.

# 3 Conclusion

Based on F2F offline discussion, the following compromised proposals are agreed to confirm in Tuesday online session:

Proposal 1: When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is not configured, the UE can obtain timing from SpCell or anther SCell (i.e. SSB-less operation).

Proposal 2: When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is deactivated, RAN2 assume that the UE can obtain timing from SpCell or anther SCell (i.e. SSB-less operation as legacy Rel-18 behavior). And when OD-SSB is activated, the UE obtains timing reference based on OD-SSB. RAN2 updates CR based on this assumption.

Proposal 3: RAN2 determine whether to send LS to RAN1 to check if any issue in Thursday CB.

Proposal 4: To reflect RAN2 assumption, RAN2 agree the following CR on field description of absoluteFrequencySSB:

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent and od-ssb-r19 is absent in SCellConfig, or if the field is absent and od-ssb-r19 is present in SCellConfig but OD-SSB is not activated, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. if the field is absent and OD-SSB is activated, the UE obtains timing reference based on OD-SSB. For PCell, this field corresponds to the CD-SSB.

# 4 References

[1] RAN2#131, Chair Notes.

# Appendix 1 (E024/E025)

RAN1#121 had discussed Scenario-5, but no conclusion was made.

### Proposal #2-2 (SSB-less SCell): · SSB-less SCell supporting on-demand SSB SCell operation is considered as Case #1. o If UE reports the capability that both SSB-less SCell operation and on-demand SSB SCell operation are supported, gNB can configure it with an SSB-less SCell on which OD-SSB can be indicated and the relative behaviours follow previous agreements in Case#1.