3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507779

# Prague, CZ, 13–17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 1047 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous Stage 2 corrections for NR NTN phase 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | THALES (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Miscellaneous stage 2 corrections for NR NTN phase 3 after RAN2#131 post-email discussion [301] and during RAN2#131-bis meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The corrections to the issues listed in R2-2507648 adressed by this CR are: - The description of the cell re-selection frequency prioritization for the UE capable of MBS Broadcast with ISA(s) - The description of the related enhancements for measurements in Downlink Coverage enhancements (i.e. 2 SMTC periodicities, 7 SMTC offsets and N closest location report) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR NTN phase 3 enhancements corrections not included in Rel-19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.14.3.3, 16.14.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2507649 submitted to RAN2#131-bis |  |  |  |  |  |  |  |  |

First Modified Subclause

16.14.3.3	Measurements

The same principle as described in 9.2.4 applies to measurements in NTN unless hereunder specified.

The network can configure:

-	multiple SMTCs in parallel per carrier and for a given set of cells depending on UE capabilities;

-	measurement gaps based on multiple SMTCs;

-	assistance information (e.g., ephemeris, Common TA parameters, kmac) provided in SIB19 for UE to perform measurement on neighbour cells in RRC_IDLE/RRC_INACTIVE/RRC_CONNECTED;

-	two different SMTC periodicities for SMTCs per frequency layer for UE in RRC_IDLE/RRC_INACTIVE/RRC_CONNECTED;

-	up to 7 SMTCs for UE in RRC_IDLE/RRC_INACTIVE, including the SMTC of the serving cell.

NOTE: If the network configures the UE with more than 4 SMTCs in RRC_IDLE/RRC_INACTIVE, the UE selects up to 4 SMTCs based on location and only considers them for measurements.

NW-controlled adjustment of SMTCs can be based on UE assistance information reported in RRC_CONNECTED. For the UE reporting the closest reference location(s) capability, the UE can report the up to 4 closest reference locations via UE assistance information or RRC reconfiguration complete message to the network to (re)configure the most relevant SMTCs based on the UE location. A UE in RRC_IDLE/RRC_INACTIVE can adjust SMTCs based on its location and assistance information in SIB19.

UE assistance information consists of the service link propagation delay difference(s) between serving the cell and neighbour cell(s).

For a UE in idle/inactive mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on a cell indicated in SIB3/SIB4 but not included in SIB19.

For a UE in connected mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on a cell included in the measurement configuration, but without corresponding satellite information in measurement configuration or in SIB19.

UE can perform time-based and location-based measurements on neighbour cells in RRC_IDLE/RRC_INACTIVE:

-	The timing and location information associated to the serving cell is provided in SIB19;

-	Timing information refers to the UTC time when the serving cell stops serving the current geographical area;

-	Location information refers:

-	In the (quasi-)Earth fixed cell scenario, to the reference location of the serving cell and a distance threshold to the reference location.

-	In the Earth moving cell scenario, to the reference location of the serving cell at the epoch time and a distance threshold to the reference location.

The time-based measurement initiation may be applicable for the feeder link switchover case for cell re-selection.

Measurement rules for cell re-selection based on timing information and location information are specified in clause 5.2.4.2 in TS 38.304 [10].

Next Modified Subclause

16.14.12	Support of ISA for MBS Broadcast in NTN

An intended service area (ISA) is a geographical area that can be signaled for MBS broadcast services. The ISA(s) is broadcasted in SIB27 and associated to MBS broadcast service(s) by indicating the ISA identifier(s) in MCCH. It may be included in USD within target service area [46].

The ISA(s) associated with MBS broadcast service(s) can be used for frequency prioritization in cell reselection in NTN (see clause 5.2.4.1 of TS 38.304 [10]).

The NTN-capable UE supporting the MBS broadcast may initiate the broadcast MRB establishment procedure when it is inside an ISA. The UE may initiate the broadcast MRB release procedure when the UE leaves an ISA (see clause 5.9.3 of TS 38.331 [12]).

NOTE:	When using ISA(s) for MBS broadcast service reception or MBS service continuity, it is up to the UE implementation how to determine if it is in the ISA(s) of MBS broadcast service or not.

End of Changes