3GPP TSG-RAN2 Meeting #132	R2-2509226

# Dallas, USA, Nov 17th - Nov 21th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0563 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE capability for UTW in positioning SRS frequency hopping in 37355-r18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN1 has sent LS R1-2508188, asking RAN2 to implement the updated Rel-18 UE feature 41-5-3 into Rel-18 specification. The 41-5-3 is that RedCap UE reports to gNB whether the UE supports UL time window and transmission of SRS for positioning with Tx Frequency hopping within the window. There is no description of ‘need for location server to know’ so the capability should only be sent to gNB, not to LMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Dummy the Rel-18 UE capability of supporting UL time window for RedCap UE positioning SRS frequency hopping in LPP specification Impact analysis Impacted 5G architecture options: SA Impacted functionality: RedCap UE positioning SRS frequency hopping Inter-operability: - If the UE is implemented according to this CR while the NW is not, there is no inter-operability issue; - If the NW is implemented according to this CR while the UE is not, LMF may receive the UE capability that it does not understand/support. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may wrongly report a UE capability that LMF does not need to know, and RAN1 agreed updated Rel-18 UE capability cannot be correctly captured in the Rel-18 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS38.331 CR 5567, CR 5568 TS38.306 CR 1372, CR 1373 |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The CR is the revision of R2-2508166 |  |  |  |  |  |  |  |  |

# START OF CHANGE

6.4.3	Common NR Positioning Information Elements

<omitted>

–	NR-UL-SRS-Capability

The IE NR-UL-SRS-Capability defines the UE uplink SRS capability.

-- ASN1START

NR-UL-SRS-Capability-r16 ::= SEQUENCE {

srs-CapabilityBandList-r16					SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

SRS-CapabilityPerBand-r16,

srs-PosResourceConfigCA-BandList-r16		SEQUENCE (SIZE (1..nrMaxConfiguredBands-r16)) OF

SRS-PosResourcesPerBand-r16			OPTIONAL,

maxNumberSRS-PosPathLossEstimateAllServingCells-r16

ENUMERATED {n1, n4, n8, n16}			OPTIONAL,

maxNumberSRS-PosSpatialRelationsAllServingCells-r16

ENUMERATED {n0, n1, n2, n4, n8, n16}	OPTIONAL,

...

}

SRS-CapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16			FreqBandIndicatorNR-r16,

olpc-SRS-Pos-r16				OLPC-SRS-Pos-r16									OPTIONAL,

spatialRelationsSRS-Pos-r16		SpatialRelationsSRS-Pos-r16							OPTIONAL,

...,

[[

posSRS-RRC-Inactive-InInitialUL-BWP-r17		PosSRS-RRC-Inactive-InInitialUL-BWP-r17	OPTIONAL,

posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17

PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17

OPTIONAL,

olpc-SRS-PosRRC-Inactive-r17				OLPC-SRS-Pos-r16						OPTIONAL,

spatialRelationsSRS-PosRRC-Inactive-r17		SpatialRelationsSRS-Pos-r16				OPTIONAL

]],

[[

posSRS-SP-RRC-Inactive-InInitialUL-BWP-r17	PosSRS-SP-RRC-Inactive-InInitialUL-BWP-r17

OPTIONAL

]],

[[

posSRS-Preconfigured-RRC-InactiveInitialUL-BWP-r18			ENUMERATED {supported}	OPTIONAL,

posSRS-Preconfigured-RRC-InactiveOutsideInitialUL-BWP-r18	ENUMERATED {supported}	OPTIONAL,

posSRS-ValidityAreaRRC-InactiveInitialUL-BWP-r18			ENUMERATED {supported}	OPTIONAL,

posSRS-ValidityAreaRRC-InactiveOutsideInitialUL-BWP-r18		ENUMERATED {supported}	OPTIONAL,

posSRS-TxFH-RRC-Connected-r18	PosSRS-TxFrequencyHoppingRRC-Connected-r18			OPTIONAL,

posSRS-TxFH-RRC-Inactive-r18	PosSRS-TxFrequencyHoppingRRC-Inactive-r18			OPTIONAL,

dummy								ENUMERATED {supported}	OPTIONAL,

posSRS-BWA-RRC-Inactive-r18						PosSRS-BWA-RRC-Inactive-r18			OPTIONAL

]]

}

OLPC-SRS-Pos-r16 ::= SEQUENCE {

olpc-SRS-PosBasedOnPRS-Serving-r16		ENUMERATED {supported}						OPTIONAL,

olpc-SRS-PosBasedOnSSB-Neigh-r16		ENUMERATED {supported}						OPTIONAL,

olpc-SRS-PosBasedOnPRS-Neigh-r16		ENUMERATED {supported}						OPTIONAL,

maxNumberPathLossEstimatePerServing-r16	ENUMERATED {n1, n4, n8, n16}				OPTIONAL,

...

}

SpatialRelationsSRS-Pos-r16 ::=	SEQUENCE {

spatialRelation-SRS-PosBasedOnSSB-Serving-r16		ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnCSI-RS-Serving-r16	ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Serving-r16		ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnSRS-r16				ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnSSB-Neigh-r16			ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Neigh-r16			ENUMERATED {supported}			OPTIONAL,

...

}

SRS-PosResourcesPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16							FreqBandIndicatorNR-r16,

maxNumberSRS-PosResourceSetsPerBWP-r16			ENUMERATED {n1, n2, n4, n8, n12, n16},

maxNumberSRS-PosResourcesPerBWP-r16				ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberPeriodicSRS-PosResourcesPerBWP-r16		ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberAP-SRS-PosResourcesPerBWP-r16			ENUMERATED {n1, n2, n4, n8, n16, n32, n64}

OPTIONAL,

maxNumberSP-SRS-PosResourcesPerBWP-r16			ENUMERATED {n1, n2, n4, n8, n16, n32, n64}

OPTIONAL,

...,

[[

posSRS-BWA-RRC-Connected-r18					PosSRS-BWA-RRC-Connected-r18		OPTIONAL,

posSRS-BWA-IndependentCA-RRC-Connected-r18		PosSRS-BWA-IndependentCA-RRC-Connected-r18

OPTIONAL

]]

}

PosSRS-RRC-Inactive-InInitialUL-BWP-r17 ::= SEQUENCE {

maxNumOfSRSposResourceSets-r17			ENUMERATED {n1, n2, n4, n8, n12, n16 }		OPTIONAL,

maxNumOfPeriodicAndSemiPersistentSRSposResources-r17

ENUMERATED {n1, n2, n4, n8, n16, n32, n64 }

OPTIONAL,

maxNumOfPeriodicAndSemiPersistentSRSposResourcesPerSlot-r17

ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

OPTIONAL,

maxNumOfPeriodicSRSposResources-r17

ENUMERATED {n1, n2, n4, n8, n16, n32, n64 }

OPTIONAL,

maxNumOfPeriodicSRSposResourcesPerSlot-r17

ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

OPTIONAL,

dummy1									ENUMERATED {n1, n2, n4, n8, n16, n32, n64}	OPTIONAL,

dummy2									ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n10, n12, n14 }

OPTIONAL,

...

}

PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 ::= SEQUENCE {

maxSRSposBandwidthForEachSCS-withinCC-FR1-r17

ENUMERATED { mhz5, mhz10, mhz15, mhz20, mhz25, mhz30,
														mhz35, mhz40, mhz45, mhz50, mhz60, mhz70,

mhz80, mhz90, mhz100 }			OPTIONAL,

maxSRSposBandwidthForEachSCS-withinCC-FR2-r17

ENUMERATED {mhz50, mhz100, mhz200, mhz400}	OPTIONAL,

maxNumOfSRSposResourceSets-r17			ENUMERATED { n1, n2, n4, n8, n12, n16 }		OPTIONAL,

maxNumOfPeriodicSRSposResources-r17		ENUMERATED { n1, n2, n4, n8, n16, n32, n64 }

OPTIONAL,

maxNumOfPeriodicSRSposResourcesPerSlot-r17

ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n10, n12, n14 }

OPTIONAL,

differentNumerologyBetweenSRSposAndInitialBWP-r17

ENUMERATED { supported }					OPTIONAL,

srsPosWithoutRestrictionOnBWP-r17

ENUMERATED { supported }					OPTIONAL,

maxNumOfPeriodicAndSemiPersistentSRSposResources-r17

ENUMERATED {n1, n2, n4, n8, n16, n32, n64}	OPTIONAL,

maxNumOfPeriodicAndSemiPersistentSRSposResourcesPerSlot-r17

ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n10,

n12, n14 }						OPTIONAL,

differentCenterFreqBetweenSRSposAndInitialBWP-r17

ENUMERATED { supported }					OPTIONAL,

maxNumOfSemiPersistentSRSposResources-r17

ENUMERATED { n1, n2, n4, n8, n16, n32, n64 }

OPTIONAL,

maxNumOfSemiPersistentSRSposResourcesPerSlot-r17

ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n10,

n12, n14 }						OPTIONAL,

switchingTimeSRS-TX-OtherTX-r17			ENUMERATED { us100, us140, us200, us300, us500 }

OPTIONAL,

...

}

PosSRS-SP-RRC-Inactive-InInitialUL-BWP-r17 ::= SEQUENCE {

maxNumOfSemiPersistentSRSposResources-r17

ENUMERATED {n1, n2, n4, n8, n16, n32, n64}		OPTIONAL,

maxNumOfSemiPersistentSRSposResourcesPerSlot-r17

ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

OPTIONAL,

...

}

PosSRS-TxFrequencyHoppingRRC-Connected-r18 ::= SEQUENCE {

maximumSRS-BandwidthAcrossAllHopsFR1-r18	ENUMERATED {mhz40, mhz50, mhz80, mhz100}

OPTIONAL,

maximumSRS-BandwidthAcrossAllHopsFR2-r18	ENUMERATED {mhz100, mhz200, mhz400}		OPTIONAL,

maximumTxFH-Hops-r18						ENUMERATED {n2, n3, n4, n5, n6}			OPTIONAL,

rf-TxRetuneTimeFR1-r18						ENUMERATED {n70, n140, n210}			OPTIONAL,

rf-TxRetuneTimeFR2-r18						ENUMERATED {n35, n70, n140}				OPTIONAL,

switchTimeBetweenActiveBWP-FrequencyHop-r18	ENUMERATED {n100, n140,n200,n300,n500}	OPTIONAL,

numOfOverlappingPRB-r18						ENUMERATED {n0, n1, n2, n4}				OPTIONAL,

maximumSRS-ResourcePeriodic-r18				ENUMERATED {n1, n2, n4, n8, n16, n32, n64}

OPTIONAL,

maximumSRS-ResourceAperiodic-r18			ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64}

OPTIONAL,

maximumSRS-ResourceSemipersistent-r18		ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64}

OPTIONAL,

...

}

PosSRS-TxFrequencyHoppingRRC-Inactive-r18 ::=SEQUENCE {

maximumSRS-BandwidthAcrossAllHopsFR1-r18	ENUMERATED {mhz40, mhz50, mhz80, mhz100}

OPTIONAL,

maximumSRS-BandwidthAcrossAllHopsFR2-r18	ENUMERATED {mhz100, mhz200, mhz400}		OPTIONAL,

maximumTxFH-Hops-r18						ENUMERATED {n2, n3, n4, n5, n6}			OPTIONAL,

rf-TxRetuneTimeFR1-r18						ENUMERATED {n70, n140, n210}			OPTIONAL,

rf-TxRetuneTimeFR2-r18						ENUMERATED {n35, n70, n140}				OPTIONAL,

switchTimeBetweenActiveBWP-FrequencyHop-r18	ENUMERATED {n100, n140,n200,n300,n500}	OPTIONAL,

numOfOverlappingPRB-r18						ENUMERATED {n0, n1, n2, n4}				OPTIONAL,

maximumSRS-ResourcePeriodic-r18				ENUMERATED {n1, n2, n4, n8, n16, n32, n64}

OPTIONAL,

maximumSRS-ResourceSemipersistent-r18		ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64}

OPTIONAL,

...

}

PosSRS-BWA-RRC-Connected-r18 ::=SEQUENCE {

numOfCarriersIntraBandContiguous-r18		ENUMERATED {two, three, twoandthree},

maximumAggregatedBW-TwoCarriersFR1-r18		ENUMERATED {mhz20, mhz40, mhz50, mhz80, mhz100,

mhz160, mhz180, mhz190, mhz200}

OPTIONAL,

maximumAggregatedBW-TwoCarriersFR2-r18		ENUMERATED {mhz50, mhz100, mhz200, mhz400, mhz600,

mhz800}						OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR1-r18	ENUMERATED {mhz80, mhz100, mhz160, mhz200, mhz240,

mhz300}						OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR2-r18	ENUMERATED {mhz50, mhz100, mhz200, mhz300, mhz400,

mhz600, mhz800, mhz1000, mhz1200}

OPTIONAL,

maximumAggregatedResourceSet-r18			ENUMERATED {n1, n2, n4, n8, n12, n16},

maximumAggregatedResourcePeriodic-r18		ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maximumAggregatedResourceAperiodic-r18		ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64},

maximumAggregatedResourceSemi-r18			ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64},

maximumAggregatedResourcePeriodicPerSlot-r18

ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10,

n12, n14},

maximumAggregatedResourceAperiodicPerSlot-r18

ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8, n10,

n12, n14},

maximumAggregatedResourceSemiPerSlot-r18	ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8, n10,

n12, n14},

...

}

PosSRS-BWA-IndependentCA-RRC-Connected-r18 ::=SEQUENCE {

numOfCarriersIntraBandContiguous-r18		ENUMERATED {two, three, twoandthree},

maximumAggregatedBW-TwoCarriersFR1-r18		ENUMERATED {mhz20, mhz40, mhz50, mhz80, mhz100,

mhz160, mhz180, mhz190, mhz200}

OPTIONAL,

maximumAggregatedBW-TwoCarriersFR2-r18		ENUMERATED {mhz50, mhz100, mhz200, mhz400, mhz600,

mhz800}						OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR1-r18	ENUMERATED {mhz80, mhz100, mhz160, mhz200, mhz240,

mhz300}						OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR2-r18	ENUMERATED {mhz50, mhz100, mhz200, mhz300, mhz400,

mhz600, mhz800, mhz1000, mhz1200}

OPTIONAL,

maximumAggregatedResourceSet-r18			ENUMERATED {n1, n2, n4, n8, n12, n16},

maximumAggregatedResourcePeriodic-r18		ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maximumAggregatedResourceAperiodic-r18		ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64},

maximumAggregatedResourceSemi-r18			ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64},

maximumAggregatedResourcePeriodicPerSlot-r18

ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10,

n12, n14},

maximumAggregatedResourceAperiodicPerSlot-r18

ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8, n10,

n12, n14},

maximumAggregatedResourceSemiPerSlot-r18	ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8, n10,

n12, n14},

guardPeriod-r18								ENUMERATED {n0, n30, n100, n140, n200},

powerClassForTwoAggregatedCarriers-r18		ENUMERATED {pc2, pc3}					OPTIONAL,

powerClassForThreeAggregatedCarriers-r18	ENUMERATED {pc2, pc3}					OPTIONAL,

...

}

PosSRS-BWA-RRC-Inactive-r18 ::=SEQUENCE {

numOfCarriersIntraBandContiguous-r18		ENUMERATED {two, three, twoandthree},

maximumAggregatedBW-TwoCarriersFR1-r18		ENUMERATED {mhz20, mhz40, mhz50, mhz80, mhz100,

mhz160, mhz180, mhz190, mhz200}

OPTIONAL,

maximumAggregatedBW-TwoCarriersFR2-r18		ENUMERATED {mhz50, mhz100, mhz200, mhz400, mhz600,

mhz800}						OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR1-r18	ENUMERATED {mhz80, mhz100, mhz160, mhz200, mhz240,

mhz300}						OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR2-r18	ENUMERATED {mhz50, mhz100, mhz200, mhz300, mhz400,

mhz600, mhz800, mhz1000, mhz1200}

OPTIONAL,

maximumAggregatedResourceSet-r18			ENUMERATED {n1, n2, n4, n8, n12, n16},

maximumAggregatedResourcePeriodic-r18		ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maximumAggregatedResourceSemi-r18			ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64},

maximumAggregatedResourcePeriodicPerSlot-r18

ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10,

n12, n14},

maximumAggregatedResourceSemiPerSlot-r18	ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8, n10,

n12, n14},

guardPeriod-r18								ENUMERATED {n0, n30, n100, n140, n200},

powerClassForTwoAggregatedCarriers-r18		ENUMERATED {pc2, pc3}					OPTIONAL,

powerClassForThreeAggregatedCarriers-r18	ENUMERATED {pc2, pc3}					OPTIONAL,

...

}

-- ASN1STOP

NR-UL-SRS-Capability field descriptions

srs-PosResourceConfigCA-BandList This field indicates the number of SRS for positioning resources supported by the target device. The target device includes this field for each band which belongs to the srs-CapabilityBandList for the current configured CA band combination. The capability signalling comprises the following parameters: - freqBandIndicatorNR indicates the current configured NR band of the target device. - maxNumberSRS-PosResourceSetsPerBWP indicates the maximum number of SRS Resource Sets for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n12, n16 correspond to 1, 2, 4, 8, 12, 16 SRS Resource Sets for positioning, respectively. - maxNumberSRS-PosResourcesPerBWP indicates the maximum number of periodic, semi-persistent, and aperiodic SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 SRS Resources for positioning, respectively. - maxNumberPeriodicSRS-PosResourcesPerBWP indicates the maximum number of periodic SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 periodic SRS Resources for positioning, respectively. - maxNumberAP-SRS-PosResourcesPerBWP indicates the maximum number of aperiodic SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 aperiodic SRS Resources for positioning, respectively. - maxNumberSP-SRS-PosResourcesPerBWP indicates the maximum number of semi-persistent SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 semi-persistent SRS Resources for positioning, respectively.

maxNumberSRS-PosPathLossEstimateAllServingCells Indicates the maximum number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning across all cells in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions. The UE shall include this field if the UE supports any of olpc-SRS-PosBasedOnPRS-Serving, olpc-SRS-PosBasedOnSSB-Neigh and olpc-SRS-PosBasedOnPRS-Neigh. Otherwise, the UE does not include this field.

maxNumberSRS-PosSpatialRelationsAllServingCells indicates the maximum number of maintained spatial relations for all the SRS resource sets for positioning across all serving cells in addition to the spatial relations maintained spatial relations per serving cell for the PUSCH/PUCCH/SRS transmissions. It is only applied for FR2. The UE can include this field only if the UE supports any of spatialRelation-SRS-PosBasedOnSSB-Serving, spatialRelation-SRS-PosBasedOnCSI-RS-Serving, spatialRelation-SRS-PosBasedOnPRS-Serving, spatialRelation-SRS-PosBasedOnSSB-Neigh or spatialRelation-SRS-PosBasedOnPRS-Neigh. Otherwise, the UE does not include this field.

olpc-SRS-Pos Indicates whether the UE supports open-loop power control for SRS for positioning. The capability signalling comprises the following parameters: - olpc-SRS-PosBasedOnPRS-Serving indicates whether the UE supports OLPC for SRS for positioning based on DL-PRS from the serving cell in the same band. The UE can include this field only if the UE supports NR-DL-PRS-ProcessingCapability and srs-PosResources TS38.331 [35] Otherwise, the UE does not include this field. - olpc-SRS-PosBasedOnSSB-Neigh indicates whether the UE supports OLPC for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - olpc-SRS-PosBasedOnPRS-Neigh indicates whether the UE supports OLPC for SRS for positioning based on DL-PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports olpc-SRS-PosBasedOnPRS-Serving. Otherwise, the UE does not include this field. Note: A DL-PRS from a PRS-only TP is treated as DL-PRS from a non-serving cell. - maxNumberPathLossEstimatePerServing indicates the maximum number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning per serving cell in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions. The UE shall include this field if the UE supports any of olpc-SRS-PosBasedOnPRS-Serving, olpc-SRS-PosBasedOnSSB-Neigh and olpc-SRS-PosBasedOnPRS-Neigh. Otherwise, the UE does not include this field.

spatialRelationsSRS-Pos Indicates whether the UE supports spatial relations for SRS for positioning. It is only applicable for FR2. The capability signalling comprises the following parameters: - spatialRelation-SRS-PosBasedOnSSB-Serving indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the serving cell in the same band. The UE can include this field only if the UE supports srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnCSI-RS-Serving indicates whether the UE supports spatial relation for SRS for positioning based on CSI-RS from the serving cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnPRS-Serving indicates whether the UE supports spatial relation for SRS for positioning based on DL-PRS from the serving cell in the same band. The UE can include this field only if the UE supports any of DL-PRS Resources for DL-AoD, DL-PRS Resources for DL-TDOA or DL-PRS Resources for Multi-RTT, or srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnSRS indicates whether the UE supports spatial relation for SRS for positioning based on SRS in the same band. The UE can include this field only if the UE supports srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnSSB-Neigh indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnPRS-Neigh indicates whether the UE supports spatial relation for SRS for positioning based on DL-PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnPRS-Serving. Otherwise, the UE does not include this field. Note: A DL-PRS from a PRS-only TP is treated as DL-PRS from a non-serving cell.

posSRS-RRC-Inactive-InInitialUL-BWP Indicates whether the UE supports positioning SRS transmission in RRC_INACTIVE state for initial UL BWP. The capability signalling comprises the following parameters: - maxNumOfSRSposResourceSets indicates the maximum number of SRS Resource Sets for positioning supported by the UE. - maxNumOfPeriodicAndSemiPersistentSRSposResources indicates the maximum number of periodic and semi-persistent SRS Resources for positioning supported by the UE. - maxNumOfPeriodicAndSemiPersistentSRSposResourcesPerSlot indicates the maximum number of periodic and semi-persistent SRS Resources for positioning per slot supported by the UE. - maxNumOfPeriodicSRSposResources indicates the maximum number of periodic SRS Resources for positioning supported by the UE. - maxNumOfPeriodicSRSposResourcesPerSlot indicates the maximum number of periodic SRS Resources for positioning per slot supported by the UE. - dummy1, dummy2 are not used in the specification. If received they shall be ignored by the receiver.

posSRS-RRC-Inactive-OutsideInitialUL-BWP Indicates whether the UE supports positioning SRS transmission in RRC_INACTIVE state outside initial UL BWP. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maxSRSposBandwidthForEachSCS-withinCC-FR1 indicates the maximum SRS bandwidth in MHz supported for each SCS that UE supports within a single CC for FR1. - maxSRSposBandwidthForEachSCS-withinCC-FR2 indicates the maximum SRS bandwidth in MHz supported for each SCS that UE supports within a single CC for FR2. - maxNumOfSRSposResourceSets indicates the maximum number of SRS Resource Sets for positioning supported by the UE. - maxNumOfPeriodicSRSposResources indicates the maximum number of periodic SRS Resources for positioning supported by the UE. - maxNumOfPeriodicSRSposResourcesPerSlot indicates the maximum number of periodic SRS Resources for positioning per slot supported by the UE. - differentNumerologyBetweenSRSposAndInitialBWP indicates whether different numerology between the SRS and the initial UL BWP is supported by the UE. If the field is absent, the UE only supports same numerology between the SRS and the initial UL BWP. - srsPosWithoutRestrictionOnBWP indicates whether SRS operation without restriction on the BW is supported by the UE; BW of the SRS may not include BW of the CORESET#0 and SSB. If the field is absent, the UE supports only SRS BW that includes the BW of the CORESET #0 and SSB. - maxNumOfPeriodicAndSemiPersistentSRSposResources indicates the maximum number of periodic and semi-persistent SRS Resources for positioning supported by the UE. - maxNumOfPeriodicAndSemiPersistentSRSposResourcesPerSlot indicates the maximum number of periodic and semi-persistent SRS Resources for positioning per slot supported by the UE. - differentCenterFreqBetweenSRSposAndInitialBWP indicates whether different center frequency between the SRS for positioning and the initial UL BWP is supported by the UE. If the field is absent, the UE only supports same center frequency between the SRS for positioning and initial UL BWP. - maxNumOfSemiPersistentSRSposResources indicates the maximum number of semi-persistent SRS Resources for positioning supported by the UE. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field. - maxNumOfSemiPersistentSRSposResourcesPerSlot indicates the maximum number of semi-persistent SRS Resources for positioning per slot supported by the UE. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field. - switchingTimeSRS-TX-OtherTX indicates the switching time between SRS Tx and other Tx in initial UL BWP or Rx in initial DL BWP.

olpc-SRS-PosRRC-Inactive Indicates whether the UE supports open-loop power control for SRS for positioning in RRC_INACTIVE state. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field.

spatialRelationsSRS-PosRRC-Inactive Indicates whether the UE supports spatial relations for SRS for positioning in RRC_INACTIVE state on FR2. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field.

posSRS-SP-RRC-Inactive-InInitialUL-BWP Indicates whether the UE supports positioning SRS transmission in RRC_INACTIVE state for initial UL BWP with semi-persistent SRS. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maxNumOfSemiPersistentSRSposResources indicates the maximum number of semi-persistent SRS Resources for positioning supported by the UE. - maxNumOfSemiPersistentSRSposResourcesPerSlot indicates the maximum number of semi-persistent SRS Resources for positioning per slot supported by the UE.

posSRS-Preconfigured-RRC-InactiveInitialUL-BWP Indicates whether the UE supports pre-configured SRS with validity area in RRC_INACTIVE for initial UL BWP. The UE can include this field only if the UE supports posSRS-ValidityAreaRRC-InactiveInitialUL-BWP. Otherwise, the UE does not include this field.

posSRS-Preconfigured-RRC-InactiveOutsideInitialUL-BWP Indicates whether the UE supports pre-configured SRS with validity area in RRC_INACTIVE outside initial UL BWP. The UE can include this field only if the UE supports posSRS-ValidityAreaRRC-InactiveOutsideInitialUL-BWP. Otherwise, the UE does not include this field.

posSRS-ValidityAreaRRC-InactiveInitialUL-BWP Indicates whether the UE supports SRS for positioning configuration in multi cells in RRC_INACTIVE for initial UL BWP. The UE can include this field only if the UE support posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field.

posSRS-ValidityAreaRRC-InactiveOutsideInitialUL-BWP Indicates whether the UE supports SRS for positioning configuration in multi cells in RRC_INACTIVE outside initial UL BWP. The UE can include this field only if the UE supports posSRS-RRC-Inactive-OutsideInitialUL-BWP and posSRS-ValidityAreaRRC-InactiveInitialUL-BWP. Otherwise, the UE does not include this field.

posSRS-TxFH-RRC-Connected Indicates the UE capability for support of positioning SRS with Tx frequency hopping in RRC_CONNECTED for RedCap UEs. The UE can include this field only if the UE supports SRS-AllPosResources and one of supportOfRedCap and supportOfERedCap defined in TS 38.331 [35]. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maximumSRS-BandwidthAcrossAllHopsFR1: Indicates the maximum positioning SRS bandwidth across all hops in MHz for FR1, which is supported and reported by UE. - maximumSRS-BandwidthAcrossAllHopsFR2: Indicates the maximum positioning SRS bandwidth across all hops in MHz for FR2, which is supported and reported by UE. - maximumTxFH-Hops: Indicates the maximum number of transmission hops, which is supported and reported by UE. - rf-TxRetuneTimeFR1: Indicates the RF Tx retune times between consecutive hops for FR1. Enumerated values indicate 70, 140, 210µs. - rf-TxRetuneTimeFR2: Indicates the RF Tx retune times between consecutive hops for FR2. Enumerated values indicate 35, 70, 140µs. - switchTimeBetweenActiveBWP-FrequencyHop: Indicates the switching time between active BWP and frequency hop. Enumerated values indicate 100, 140, 200, 300, 500µs. - numOverlappingPRB: Indicates the overlapping PRB(s) between adjacent hops. Enumerated values indicate 0,1,2,4 PRBs. - maximumSRS-ResourcePeriodic: Indicates the maximum number of periodic positioning SRS resources with Tx frequency hopping. - maximumSRS-ResourceAperiodic: Indicates the maximum number of aperiodic positioning SRS resources with Tx frequency hopping. - maximumSRS-ResourceSemipersistent: Indicates the maximum number of Semi-persistent positioning SRS resources with Tx frequency hopping. NOTE 1: No additional UE requirements shall be specified for the case of Tx hopping with non-overlapping hops compared to the case of Tx hopping with overlapping hops, e.g., a UE is not responsible for keeping phase continuity across the hops in either case of overlapping or non-overlapping hops.

posSRS-TxFH-RRC-Inactive Indicates the UE capability for support of positioning SRS with Tx frequency hopping in RRC_INACTIVE for RedCap UEs. The UE can include this field only if the UE supports posSRS-RRC-Inactive-OutsideInitialUL-BWP and one of supportOfRedCap and supportOfERedCap defined in TS 38.331 [35]. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maximumSRS-BandwidthAcrossAllHopsFR1: Indicates the maximum positioning SRS bandwidth across all hops in MHz for FR1, which is supported and reported by UE. - maximumSRS-BandwidthAcrossAllHopsFR2: Indicates the maximum positioning SRS bandwidth across all hops in MHz for FR2, which is supported and reported by UE. - maximumTxFH-Hops: Indicates the maximum number of transmission hops, which is supported and reported by UE. - rf-TxRetuneTimeFR1: Indicates the RF Tx retune times between consecutive hops for FR1. Enumerated values indicate 70, 140, 210µs. - rf-TxRetuneTimeFR2: Indicates the RF Tx retune times between consecutive hops for FR2. Enumerated values indicate 35, 70, 140µs. - switchTimeBetweenActiveBWP-FrequencyHop: Indicates the switching time between active BWP and frequency hop. Enumerated values indicate 100, 140, 200, 300, 500µs. - numOfOverlappingPRB: Indicates the overlapping PRB(s) between adjacent hops. Enumerated values indicate 0,1,2,4 PRBs. - maximumSRS-ResourcePeriodic indicates the maximum number of periodic positioning SRS resources with Tx frequency hopping. - maximumSRS-ResourceSemipersistent indicates the maximum number of Semi-persistent positioning SRS resources with Tx frequency hopping. NOTE 2: No additional UE requirements shall be specified for the case of Tx hopping with non-overlapping hops compared to the case of Tx hopping with overlapping hops, e.g., a UE is not responsible for keeping phase continuity across the hops in either case of overlapping or non-overlapping hops.

dummy This field is not used in the specification. If received, it shall be ignored by the receiver.

posSRS-BWA-RRC-Connected Indicates the UE capability for support of positioning SRS bandwidth aggregation in RRC_CONNECTED and comprises the support of the same SRS power reduction across aggregated carriers. The UE can include this field only if the UE supports SRS-AllPosResources and supportedBandCombinationList defined in TS 38.331 [35]. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - numOfCarriersIntraBandContiguous: Indicates the number of supported aggregated carriers in intra band contiguous carriers, which is supported and reported by UE. - maximumAggregatedBW-TwoCarriersFR1: Indicates the maximum aggregated SRS bandwidth in MHz for two aggregated carriers for FR1, which is supported and reported by UE. - maximumAggregatedBW-TwoCarriersFR2: Indicates the maximum aggregated SRS bandwidth in MHz for two aggregated carriers for FR2, which is supported and reported by UE. - maximumAggregatedBW-ThreeCarriersFR1: Indicates the maximum aggregated SRS bandwidth in MHz for three aggregated carriers for FR1, which is supported and reported by UE. - maximumAggregatedBW-ThreeCarriersFR2: Indicates the maximum aggregated SRS bandwidth in MHz for three aggregated carriers for FR2, which is supported and reported by UE. - maximumAggregatedResourceSet: Indicates the max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourcePeriodic: Indicates the maximum number of aggregated periodic SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourceAperiodic: Indicates the maximum number of aggregated aperiodic SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourceSemi: Indicates the maximum number of aggregated semi-persistent SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourcePeriodicPerSlot: Indicates the maximum number of aggregated periodic SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - maximumAggregatedResourceAperiodicPerSlot: Indicates the maximum number of aggregated aperiodic SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - maximumAggregatedResourceSemiPerSlot: Indicates the maximum number of aggregated semi-persistent SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. NOTE 3: The UE supports the simultaneous transmission in a coherent manner of 2 or 3 SRS resources in 2 or 3 intra-band contiguous CCs. NOTE 4: Each two or three linked SRS resources are counted as 1 resource NOTE 5: A UE that support SRS-PosResourceAP defined in TS 38.331 [35] must signal a non-zero value for maximumAggregatedResourceAperiodic and maximumAggregatedResourceAperiodicPerSlot; NOTE 6: UE only reports the number on bands for the current configured CA band combination. NOTE 6a: For numOfCarriersIntraBandContiguous, it shall be less than or equal to the maximum number of the component carrier associated with ca-BandwidthClassUL-NR in TS 38.331 [35]. NOTE 6b: For maximum aggregated UL SRS bandwidth, it shall be less than or equal to the maximum aggregated transmission bandwidth associated with ca-BandwidthClassUL-NR in TS 38.331 [35]. Additionally, it shall be less than or equal to the maximum aggregated bandwidth for the supported CA configuration in Table 5.5A.1-1 in TS 38.101-1 [37] for FR1 bands or Table 5.5A.1-1 in TS 38.101-2 [34] for FR2 bands for the band where aggregated SRS CCs is configured.

posSRS-BWA-IndependentCA-RRC-Connected Indicates the UE capability for support of positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED and comprises the support of the same SRS power reduction across aggregated carriers. The UE can include this field only if the UE supports SRS-AllPosResources defined in TS 38.331 [35]. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - numOfCarriersIntraBandContiguous: Indicates the number of supported aggregated carriers in intra band contiguous carriers, which is supported and reported by UE. - maximumAggregatedBW-TwoCarriersFR1: Indicates the maximum aggregated SRS bandwidth in MHz for two aggregated carriers for FR1, which is supported and reported by UE. - maximumAggregatedBW-TwoCarriersFR2: Indicates the maximum aggregated SRS bandwidth in MHz for two aggregated carriers for FR2, which is supported and reported by UE. - maximumAggregatedBW-ThreeCarriersFR1: Indicates the maximum aggregated SRS bandwidth in MHz for three aggregated carriers for FR1, which is supported and reported by UE. - maximumAggregatedBW-ThreeCarriersFR2: Indicates the maximum aggregated SRS bandwidth in MHz for three aggregated carriers for FR2, which is supported and reported by UE. - maximumAggregatedResourceSet: Indicates the max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourcePeriodic: Indicates the maximum number of aggregated periodic SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourceAperiodic: Indicates the maximum number of aggregated aperiodic SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourceSemi: Indicates the maximum number of aggregated semi-persistent SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourcePeriodicPerSlot: Indicates the maximum number of aggregated periodic SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - maximumAggregatedResourceAperiodicPerSlot: Indicates the maximum number of aggregated aperiodic SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - maximumAggregatedResourceSemiPerSlot: Indicates the maximum number of aggregated semi-persistent SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - guardPeriod: Indicates the guard period in microseconds before and after aggregated SRS transmission. - powerClassForTwoAggregatedCarriers: Indicates the power class of supported two aggregated carriers in intra band contiguous carriers. - powerClassForThreeAggregatedCarriers: Indicates the power class of supported three aggregated carriers in intra band contiguous carriers. NOTE 7: The UE supports the simultaneous transmission in a coherent manner of 2 or 3 SRS resources in 2 or 3 intra-band contiguous CCs. NOTE 8: Each two or three linked SRS resources are counted as 1 resource NOTE 9: UE only reports the number on bands for the current configured CA band combination. NOTE 10: Guard period is needed before and after the aggregated SRS transmissions when SRS resource is configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC. NOTE 11: For a given band, independent of the band combination, the UE must signal the same guard period. NOTE 12: The power class is only applicable for FR1 bands.

posSRS-BWA-RRC-Inactive Indicates the UE capability for support of positioning SRS bandwidth aggregation in RRC_INACTIVE and comprises the support of the same SRS power reduction across aggregated carriers. The UE can include this field only if the UE supports posSRS-RRC-Inactive-OutsideInitialUL-BWP. Otherwise, the UE does not include this field. If the UE includes this field, the fields srsPosWithoutRestrictionOnBWP and differentCenterFreqBetweenSRSposAndInitialBWP in posSRS-RRC-Inactive-OutsideInitialUL-BWP shall be set to supported. The capability signalling comprises the following parameters: - numOfCarriersIntraBandContiguous: Indicates the number of supported aggregated carriers in intra band contiguous carriers, which is supported and reported by UE. - maximumAggregatedBW-TwoCarriersFR1: Indicates the maximum aggregated SRS bandwidth in MHz for two aggregated carriers for FR1, which is supported and reported by UE. - maximumAggregatedBW-TwoCarriersFR2: Indicates the maximum aggregated SRS bandwidth in MHz for two aggregated carriers for FR2, which is supported and reported by UE. - maximumAggregatedBW-ThreeCarriersFR1: Indicates the maximum aggregated SRS bandwidth in MHz for three aggregated carriers for FR1, which is supported and reported by UE. - maximumAggregatedBW-ThreeCarriersFR2: Indicates the maximum aggregated SRS bandwidth in MHz for three aggregated carriers for FR2, which is supported and reported by UE. - maximumAggregatedResourceSet: Indicates the max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourcePeriodic: Indicates the maximum number of aggregated periodic SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourceSemi: Indicates the maximum number of aggregated semi-persistent SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourcePeriodicPerSlot: Indicates the maximum number of aggregated periodic SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - maximumAggregatedResourceSemiPerSlot: Indicates the maximum number of aggregated semi-persistent SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - guardPeriod: Indicates the guard period in microseconds before and after aggregated SRS transmission. - powerClassForTwoAggregatedCarriers: Indicates the power class of supported two aggregated carriers in intra band contiguous carriers. - powerClassForThreeAggregatedCarriers: Indicates the power class of supported three aggregated carriers in intra band contiguous carriers. NOTE 13: The power class is only applicable for FR1 bands.

# END OF CHANGE