3GPP TSG-RAN2 Meeting #132	R2-2509341

Dallas, USA, 17 - 21 November 2025 	 Revision of R2-2508435

Agenda Item:	8.4.2 RRC issues

Source: 	Ericsson

Title:  	LP-WUS RRC issues (E057, E059, E060, E043, E066, E008)

Document for:	Discussion and Decision

# Introduction

The following RRC issues for LP-WUS are discussed:

- E057: Periodicity in lpwus-Mo1-2-r19 should be in msec instead of slots.

- E059: Clarification for UE supporting OFDM and OOK with different minimum time gaps

- E060: Clarification for the time offsets configured with lpwus-LoFrameOffsetList.

- E043: Exit condition for LP-WUS in connected with UAI signalling

- E066: Timing requirements for transitions between MR and LR

- E008: Offloading thresholds above the legacy measurements thresholds

# Discussion

## E057: LP-WUS periodicity in msec instead of slots

The value range for the LP-WUS periodicity with Option 1-2 is:

lpwus-Mo1-2-r19                          CHOICE {

sl1                                NULL,

sl2                                INTEGER (0..1),

sl4                                INTEGER (0..3),

sl5                                INTEGER (0..4),

sl8                                INTEGER (0..7),

sl10                               INTEGER (0..9),

sl16                               INTEGER (0..15),

sl20                               INTEGER (0..19),

sl40                               INTEGER (0..39),

sl80                               INTEGER (0..79),

sl160                              INTEGER (0..159),

sl320                              INTEGER (0..319),

sl640                              INTEGER (0..639),

sl1280                             INTEGER (0..1279),

sl2560                             INTEGER (0..2559),

sl5120                             INTEGER (0..5119),

sl10240                            INTEGER (0..10239)

},                                                           OPTIONAL,    -- Cond Option1-2

The periodicity is in slots and dependent on the SCS. For LP-WUS 15, 30 and 120 kHz SCS are supported, which means that the value range is:

- {1, 2, ..., msec           	up to 2.56, 5.12, 10.24 sec} 	(15 kHz)

- {0.5, 1, ..., msec        	up to 1,28, 2.56, 5.12 sec} 	(30 kHz)

- {0.125, 0.25, ..., msec 	up to 0.32, 0.64, 1.28 sec} 	(120 kHz)

In our understanding the periodicity should be in time units and independent from the SCS, similar as the C-DRX parameters:

drx-LongCycle {ms10, ms20, ms32, ms40, ms60, …, ms5120, ms10240}

shortDRX      {ms2, ms3, ms4, ms5, ms6, ms7, …, ms512, ms640}

We do not think there is a need to configure a fraction of a msec. And to take the long/short DRX cycle length as a reference it is proposed:

RAN2 to discuss whether the LP-WUS periodicity in connected mode should be in time units and independent from the SCS for example {1, 2, 4, 6, 8, 10, 12, 16, 20, 24, 32, 40, 48, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240} mseconds.

## E059: Minimum time gaps supported by the UE

The UE can indicate a minimum time gap for OFDM and OOK based WUR:

And the NW can configure a long and a short time offset:

It should be clarified that when the configured time offsets are larger than the minimum time gap that the UE signals for both OOK and OFDM, that the UE monitors the LO that is the closest to the PO, i.e. the NW is not required to transmit LP-WUS twice to wake up the UE:

In case the UE is able to monitor the LO associated with longer and shorter wake up delay then the UE monitors the LO associated with the shorter wake up delay.

In case multiple time offsets are configured, it needs to be clarified which LO the UE monitors. For example for the configuration below:

Clarify that when the UE monitors the i-th PO in the Paging Frame, that the UE uses the i-th offset in the lpwus-LoFrameOffsetList when Ns is equal or larger than the number of entries in offsetFlorLongerWakeUpDelay and offsetFlorShorfterWake UpDelay.

In case the LO is associated with POs in different Paging Frames (PF):

It first should be clarified that longer and shorter wakeup delay are realized with offsetFlorLongerWake UpDelay and offsetFlorShorfterWakeUpDelay. Longer and shorter wakeup delay is not realized when the LO is associated with POs in different PFs:

Clarify that the longer and shorter wakeup delay is realized with offsetFlorLongerWakeUpDelay and offsetFlorShorfterWakeUpDelay. Longer and shorter wakeup delay is not realized when the LO is associated with POs in different PFs.

Clarify that when the LO is associated with POs in different PFs, that the UE uses the i-th entry when the UE monitors the PO in the i-th PF.

Clarify that the NW configures the entries in the offsetFlorLongerWakeUpDelay larger than the entries in the offsetFlorShorfterWakeUpDelay.

lpwus-LoFrameOffsetList Indicates the frame-level offset between an LO and a reference PO/PF for LP-WUS in the cell for RRC IDLE/INACTIVE (see TS 38.213 [13], clause 10.4C). The number of entries of offsetForLongerWakeUpDelay or offsetForShorterWakeUpDelay is equal to CEIL (Ns / lpwus-PoNumPerLo), where Ns is the number of paging occasions per paging frame configured in SIB1. Network configures at least one of offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay.

lpwus-LoFrameOffsetListForPagingAdapt Indicates the frame-level offset between an LO and a reference PO/PF for LP-WUS in the cell for RRC IDLE/INACTIVE (see TS 38.213 [13], clause 10.4C) for paging adaptation. If the UE supports paging adaptation for LP-WUS, it ignores lpwus-LoFrameOffsetList, if configured. The number of entries of offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay is equal to CEIL (Ns / lpwus-PoNumPerLo), where Ns is the number of paging occasions per paging frame configured in SIB1 for paging adaptation. Network configures at least one of offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay.

## E060: About the possible time offset configurations

The NW can configure different values for:

- Ns: 			number of POs per PF {1, 2, 4, 8 (paging adaptation)}

- Long time offset(s):	frame offset between LO and PO {1, 2, 4, 8 (paging adaptation)}

- Short time offset(s):	frame offset between LO and PO {1, 2, 4, 8 (paging adaptation)}

- LO-to-PO mapping:	the number of POs per LO {1, 2, 4}

PCCH-Config ::=             SEQUENCE {

…

ns                                  ENUMERATED {four, two, one},

…

pagingAdaptation-r19                  SEQUENCE {

pagingAdapt-NS-r19                    ENUMERATED {eight,four, two, one},

…

LowPowerConfig-r19 ::=                    SEQUENCE {

…

lpwus-LoFrameOffsetList-r19               SEQUENCE {

offsetForLongerWakeUpDelay-r19            SEQUENCE (SIZE (1..4)) OF INTEGER (8..200)

offsetForShorterWakeUpDelay-r19           SEQUENCE (SIZE (1..4)) OF INTEGER (8..200)

},

lpwus-LoFrameOffsetListForPagingAdapt-r19                 SEQUENCE {

offsetForLongerWakeUpDelay-r19               SEQUENCE (SIZE (1..8)) OF INTEGER (8..200)

offsetForShorterWakeUpDelay-r19              SEQUENCE (SIZE (1..8)) OF INTEGER (8..200)

},

lpwus-MoNumPerLo-r19                      ENUMERATED {n1, n2, n3, n4},

lpwus-PoNumPerLo-r19                      ENUMERATED {po1, po2, po4}

The NW does not configure 3, 5, 6 or 7 time offsets.

For each PF one or more time offsets are configured.

The number of long (and short) time offsets configured per PF depends on the #POs per PF (Ns) and LO-to-PO mapping.

The following configuration combinations are supported:

- 1 long and/or short Time offset:

  - Ns=1 and PoNumPerLO=4: one LO with 4 POs in 4 consecutive PFs

  - Ns=1 and PoNumPerLO=2: one LO with 2 POs in 2 consecutive PFs

  - Ns=1 and PoNumPerLO=1: one LO with 1 PO in next PF

  - Ns=2 and PoNumPerLO=2: one LO with 2 POs in next PF

  - Ns=4 and PoNumPerLO=4: one LO with 4 POs in next PF

- 2 long and/or short Time offsets:

  - Ns=2 and PoNumPerLO=1: two LOs with each LO with 1 PO in next PF

  - Ns=4 and PoNumPerLO=2: two LOs with each LO with 2 POs in next PF

  - Ns=8 and PoNumPerLO=4: two LOs with each LO with 4 POs in next PF

- 4 long and/or short Time offsets:

  - Ns=4 and PoNumPerLO=1: four LOs with each LO with 1 PO in next PF

  - Ns=8 and PoNumPerLO=4: four LOs with each LO with 2 POs in next PF

- 8 long and/or short Time offsets:

  - Ns=8 and PoNumPerLO=1: eigth LOs with each LO with 1 PO in next PF

RAN2 confirms the possible configuration options above.

## E043: Exit condition LP-WUS in connected with UAI signalling

RAN2 has discussed that the MR measurements can be good while LR reception is bad. The gNB configures LP-WUS in connected based on the MR measurement reporting of the UE, e.g. RRM and CSI reporting. Thus it may happen that LP-WUS is configured while the LP-WUS reception is bad.

RAN2 has discussed to configure an exit condition for LP-WUS monitoring in connected mode. However RAN4 has not defined measurement requirements on LR in connected mode and the UE is not required to measure on LP-SS/SSB in connected. Furthermore it is possible that gNB does not support OOK-based WUR and LP-SS is not transmitted.

First it should be clarified what is the cause of the MR being good and the LR being bad:

- Because MR and LR are receiving different physical channels.

- Because the WUR receiver is less accurate compared to the MR receiver.

In case the LR is capable to measure SSB, can it then happen that the LR SSB measurements are good, while the LP-WUS reception is bad? And in case the LR uses LP-SS for measurements, can the LP-SS reception be good, while the LP-WUS reception is bad?:

RAN2 to clarify that in case the LR measures on SSB or LP-SS whether the LR measurements can be good while the LP-WUS reception is bad?

It has been discussed whether the UE can detect when the LR is bad based on decoding error when receiving LP-WUS. However the UE either has a match, i.e. LP-WUS is detected or LP-WUS is not detected. Furthermore LP-WUS is send on demand and not periodically. Thus when the UE does not detect LP-WUS it can be because the gNB did not send LP-WUS or because the LP-WUS reception was bad.

The UE would be able to evaluate the LP-WUS reception quality, when “dummy” LP-WUS’s are transmitted periodically. A “dummy” LP-WUS does not wake the UE up but it enables the UE to evaluate the LP-WUS reception quality based on detecting LP-WUS on “dummy” LP-WUS occasions. The gNB can configure the “dummy” LP-WUS transmission pattern when LP-WUS is configured in the UE:

The LP-WUS exit condition is based on periodic “dummy” LP-WUS transmissions. The UE sends UAI message that the LP-WUS link is bad when the UE fails to detect X consecutive “dummy” LP-WUS transmission. The UE sends UAI message that the LP-WUS link is good when the UE has detected Y consecutive “dummy” LP-WUS transmission (Y is larger than X).

## E066: Timing requirements for transitions between MR and LR

When the UE is in LP-WUS monitoring state, then the UE monitors LP-WUS using LR and the MR is not required to monitor the PO directly.

To ensure that the UE does not miss any paging when it goes into and out of LP-WUS monitoring state there are timing requirements when the MR and LR should be activated and deactivated.

In TS 38.133 section it says:

The requirements in this clause 4.8.2 apply when the LP-WUR is in ON state.

Before entering LP-WUS monitoring, RRM offloading or RRM relaxation mode and after exiting LP-WUS monitoring, RRM offloading and RRM relaxation mode, it is up to UE implementation when and how to turn the LP-WUR to ON state for serving cell measurement.

When LP-WUR is in the ON state, the UE may perform serving cell measurements based on LP-SS or PSS/SSS, where the requirements for LP-SS based measurements and evaluations in the clause 4.8.2 apply only if the LP-SS is configured and transmitted on the same carrier frequency as the serving cell.

To ensure that the UE does not miss paging during the transitions between MR and LR:

When the entry condition for LP-WUS monitoring condition is fulfilled and the UE decides to monitor LP-WUS, the LR shall be in ON state to monitor the next LP-WUS occasion when the MR stops monitoring PO directly.

When the exit condition for LP-WUS monitoring condition is fulfilled the UE shall activate the MR to monitor PO directly. The UE shall continue LP-WUS monitoring using LR until the MR is in ON state and the MR can monitor the next PO directly.

These requirements can be specified in the field description of the entry/exit condition for LP-WUS monitoring:

entryCondition The entry condition for LP-WUS monitoring related configuration. When the entry condition for LP-WUS monitoring condition is fulfilled and the UE decides to monitor LP-WUS, the LR shall be in ON state to monitor the next LP-WUS occasion when the MR stops monitoring PO directly.

exitCondition The exit condition for LP-WUS monitoring related configuration. When the exit condition for LP-WUS monitoring condition is fulfilled the UE shall activate the MR to monitor PO directly. The UE shall continue LP-WUS monitoring using LR until the MR is in ON state and can monitor the next PO directly

## E008:

RAN2 agreed that full cell coverage is not excluded when LP-WUS is configured:

- Full coverage for LP-WUS is not precluded, e.g., if there needs to be a threshold value so that the condition is always fulfilled for all LPWUS UEs.

Full cell coverage means that the UE can use LP-WUS directly after cell reselection, provided that the cell is suitable to camp and the entry condition is met. Thus it is possible that the UE reselects to a cell where the measured RSRP is below SIntraSearchP and SnonIntraSearchP.

Further the NW may deploy offloading without relaxation, and in such case it should be possible to configure the offloading threshold below the legacy measurement thresholds.

s-SearchThresholdP5 and s-SearchThresholdP6 to be larger than or equal to s-IntraSearchP and s-NonIntraSearchP only

The network restriction that the offloading thresholds have to be configured above the legacy measurement thresholds only applies when both RRM offloading and relaxation are configured.

The proposed change to 38.331 is given below:

s-SearchThresholdP, s-SearchThresholdP2, s-SearchThresholdP3, s-SearchThresholdP4, s-SearchThresholdP5, s-SearchThresholdP6 Parameters "SSearchThresholdP", "SSearchThresholdP2", "SSearchThresholdP3", "SSearchThresholdP4", "SSearchThresholdP5", and "SSearchThresholdP6" in TS 38.304 [20]. The network configures s-SearchThresholdP and s-SearchThresholdP2 to be less than or equal to s-IntraSearchP and s-NonIntraSearchP. If the network configures both RRM serving cell relaxation (relaxedMeasurementForServingAndNeighboringCell) and RRM serving cell offloading (offloadMeasurementForServingCell) then the network configures both s-SearchThresholdP5 and s-SearchThresholdP6 to be larger than or equal to s-IntraSearchP and s-NonIntraSearchP, if there is such configuration(s). The network configures s-SearchThresholdP5 and s-SearchThresholdP6 to be larger than or equal to s-SearchThresholdP3 and s-SearchThresholdP4, respectively, if there is such configuration(s).

# Summary

RAN2 is kindly asked to discuss the following LP-WUS RRC issues:

Observation 1	The NW does not configure 3, 5, 6 or 7 time offsets.

Observation 2	For each PF one or more time offsets are configured.

Observation 3	The number of long (and short) time offsets configured per PF depends on the #POs per PF (Ns) and LO-to-PO mapping.

Proposal 1	RAN2 to discuss whether the LP-WUS periodicity in connected mode should be in time units and independent from the SCS for example {1, 2, 4, 6, 8, 10, 12, 16, 20, 24, 32, 40, 48, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240} mseconds.

Proposal 2	In case the UE is able to monitor the LO associated with longer and shorter wake up delay then the UE monitors the LO associated with the shorter wake up delay.

Proposal 3	Clarify that when the UE monitors the i-th PO in the Paging Frame, that the UE uses the i-th offset in the lpwus-LoFrameOffsetList when Ns is equal or larger than the number of entries in offsetFlorLongerWakeUpDelay and offsetFlorShorfterWake UpDelay.

Proposal 4	Clarify that the longer and shorter wakeup delay is realized with offsetFlorLongerWakeUpDelay and offsetFlorShorfterWakeUpDelay. Longer and shorter wakeup delay is not realized when the LO is associated with POs in different PFs.

Proposal 5	Clarify that when the LO is associated with POs in different PFs, that the UE uses the i-th entry when the UE monitors the PO in the i-th PF.

Proposal 6	Clarify that the NW configures the entries in the offsetFlorLongerWakeUpDelay larger than the entries in the offsetFlorShorfterWakeUpDelay.

Proposal 7	RAN2 confirms the possible configuration options above.

Proposal 8	RAN2 to clarify that in case the LR measures on SSB or LP-SS whether the LR measurements can be good while the LP-WUS reception is bad?

Proposal 9	The LP-WUS exit condition is based on periodic “dummy” LP-WUS transmissions. The UE sends UAI message that the LP-WUS link is bad when the UE fails to detect X consecutive “dummy” LP-WUS transmission. The UE sends UAI message that the LP-WUS link is good when the UE has detected Y consecutive “dummy” LP-WUS transmission (Y is larger than X).

Proposal 10	When the entry condition for LP-WUS monitoring condition is fulfilled and the UE decides to monitor LP-WUS, the LR shall be in ON state to monitor the next LP-WUS occasion when the MR stops monitoring PO directly.

Proposal 11	When the exit condition for LP-WUS monitoring condition is fulfilled the UE shall activate the MR to monitor PO directly. The UE shall continue LP-WUS monitoring using LR until the MR is in ON state and the MR can monitor the next PO directly.

Proposal 12	The network restriction that the offloading thresholds have to be configured above the legacy measurement thresholds only applies when both RRM offloading and relaxation are configured.