3GPP TSG-RAN WG3 Meeting #129	R3-255164

Bangalore, India, 25 – 29 August, 2025

Agenda item:	10.2

Source:	Nokia

Title:	(TP for SON BL CR for TS 38.401) MRO Enhancements for LTM

Document for:	Discussion

# 1	Introduction

This is a contribution related to MRO for LTM in Rel-19.

# 2	Discussion

## 2.1 MRO for LTM

### 2.1.1 Agreements achieved in RAN3 and RAN2

The agreements achieved in RAN3 #128 [1]:

BFR shortly after successful LTM cell switch to the wrong beam (Case 1)

Use DU-CU ACCESS AND MOBILITY INDICATION message for target DU send recovery beam information to the CU. Only one recovery beam information included.

Use ACCESS AND MOBILITY INDICATION message for CU forward recovery beam information to the source DU. Only one recovery beam information included.

No new failure type for beam failure recovery is needed for TS38.300.

The Timer configured in DU (e.g., Tstore_UE_cntxt) used for the DU to detect BFR shortly after successful LTM cell switch to the wrong beam is up to implementation.

UHI and ping-pong issue

CU informs relevant information to the target DU via CU-DU Cell Switch Notification.

No stage2, work in stage3 with 1a.

If the above 1a information is included in F1AP message from CU to DU, the DU may consider the potential mobility issue detected including ping-pong.

FFS on the extra indication for ping-pong case.

RACH less LTM cell switch failure due to wrong beam (Case 2)

Network-based solution for the LTM cell switch failure due to wrong beam using the same signaling flow as for Case1.

In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization.

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization.

Target DU needs to send the reconnect/re-established/recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

Reply LS to RAN2 on the network based solution for case1 and case2.

Work on stage3 semantic description for C-RNTI IE in ACCESS AND MOBILITY INDICATION message.

The agreements achieved in RAN3 #127bis [2]:

BFR shortly after successful LTM cell switch to the wrong beam

RAN3 supports network-based solution for the case of BFR shortly after a successful LTM cell switch.

The target DU identifies the BFR happened in UE shortly after successful LTM cell switch caused due to wrong beam.

In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization

Target DU needs to send the recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

FFS if/how the solution for case 1 could also be used for the case of LTM cell switch failure due to wrong beam (case 2)?

LTM failure due to outdated TA

MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM.

RAN3 will not consider the case of LTM cell switch failure due to outdated TA calculated by UE.

For PDCCH order triggered early TA acquisition, RAN3 consider the case of RACH-less LTM fails due to outdate TA and UE performs RACH-based LTM failure recovery or RRC re-establishment to the same cell.

Whether UE based solution or network based solution is used for LTM failure due to outdated TA?

Whether LTM failure due to outdated TA is defined as a new failure case or it can covered by existing failure case?

Network based or UE based solution for TA acquisition type

For inter-DU case, CU aware TA acquisition type without any signaling impact in RAN3.

FFS for intra-DU case.

The agreements achieved in RAN2 #130 [3]:

We will not introduce UE reporting regarding TAs that were “unnecessarily” acquisitioned.

We will not add TA difference between TA provided by network in Cell Switch Command for which the RACH-less LTM failed, and TA received in RAR for recovery/re-establishment.

Introduce new values of raPurpose for RACH-based LTM-related RA reports.

UE logs available L1 measurement results in the SHR for the serving, target and and neighbouring cells when the UE performs L3  handover and has an LTM configuration.

We will not enhance MHI to include a “HO-type” (i.e., LTM or L3 HO).

The agreements achieved in RAN2 #129bis [4]:

We will not add support for including the unnecessary LTM configuration, e.g. LTM candidate cell(s) unused, in SON reports, for both failure and near failure cases.

Do not specify support for reporting difference between the TA value received from the LTM cell switch command and TA value measured by the UE.

Do not specify support for reporting of elapsed time between UE obtains the target cell TA and UE receives the cell switch command, or an indication to indicate whether the UE obtains TA before UE receives LTM cell switch command

For LTM-related RA reports, the UE sets the cellId to the CGI if available, otherwise sets it based on ltm-CandidatePCI and ssb-Frequency in corresponding LTM-Candidate.

### 2.1.2 Near Failure Monitoring for MCG LTM

In the context of RAN3 # 128 meeting some companies proposed to focus also on near-failure optimization in terms of almost wrong TA value. According to those companies the near-failure case occurs when the first transmission from the UE was received almost outside the cyclic prefix. In other words, the access succeeded, but the timing was off so much that a better TA value could have been used. In that case the network will need to correct the TA with the UE. However, from the contribution the company provided it is not entirely clear the definition and how the identification of near failure for TA value shall be done.

Proactive way of monitoring is based on discovering an issue with validity of TA value in advance when the PCell change was successful but there is symptom which may show invalid TA scenario may happen soon may be useful for operators. The symptom is that despite the PCell change was successful the time interval between successful transmission of the RRCReconfigurationComplete message and first TA command sent from network (indicating change in TA value) to UE evaluated as T1-T0 is relatively short (let’s say it is less than a configured threshold). In other words, despite the PCell change has been successful in the given observation some small change in point of time of sending of the Cell switch command to UE or radio conditions may slightly delay the transmission of the first message to point in time when TA sent to UE in the Cell Switch Command becomes invalid. Based on the proactive monitoring the operator can identify the invalid TA issue in advance and take preventive actions which in combination with reactive way may help to completely fix the issue.

Fig. 1

Fig. 1 shows an example where UE is moving in direction to target DU. In this example, there are shown some areas representing validity of the TA value to ensure UL synchronization between the UE and the DU. If UE moves from example from Area 3 to Area 2 the target DU shall send TA command as MAC CE to UE with new TA2 value to be used by UE to send the data and control information in UL.  Let’s consider the UE received Cell switch command with TA3 value shortly before UE leaves the Area 3. Because UE is in area 3 the TA3 obtained via early TA acquisition procedure and communicated to UE via Cell Switch Command is still valid and UE will execute the LTM cell switch successfully (i.e. successfully transmits the RRCReconfigurationComplete message to target cell handled by target DU). However shortly after this successful execution regular TA command form target DU is sent to UE to instruct the UE to use new TA2 valid in Area 2.

Observation 1-1: Identification of the near failure for TA value may be based on the time interval between successful transmission of RRCReconfigurationComplete message and consequent sending the TA command to UE signalling the TA value which differs from the one communicated to UE via Cell Switch Command message.

The companies also argued the preference for network-based solution for near failure for TA value. The network based on first propagation time measurement shall be able to determine the TA value UE used for sending the RRCReconfigurationComplete message.

Observation 1-2: The network solution is applicable for near failure TA value monitoring.

Due to fact that the near failure for TA value first discussed in RAN3 # 128 meeting it is proposed to move the discussion to Rel. 20

- Proposal 1-1: RAN3 agrees not to decide on any solution for near failure for TA in Rel. 19 and move the discussion to Rel. 20.

### 2.1.3 Connection Failure Monitoring for MCG LTM

- In RAN3 # 128 the following was agreed for LTM failure due to outdated TA:

- RAN3 supports network-based solution for outdated TA.

- Reusing the same signalling flow as case1 and case2.

- The TS 38.401 was enhanced with the following part:

- The target gNB-DU identifies that the UE successfully performed a RACH-based access while reconnecting/re-establishing/recovering to the same target cell and same target beam but with TA value different from the one which was included in CU-DU CELL SWITCH NOTIFICATION message received from gNB-CU at LTM Cell Switch execution. The target gNB-DU sends the TA value used at successful RACH-based access to the source gNB-DU via the gNB-CU.

- The current proposal is overly restrictive. Connection failures due to outdated TA values are not necessarily limited to the same beam. Furthermore, the CU-DU CELL SWITCH NOTIFICATION message may not always contain the TA value from the Cell Switch Command.

- Further modification of TS 38.401, as detailed in Annex 1, is proposed.

- Proposal 1-2: RAN3 agrees TP for BL CR 38.401 in Annex 1.

# 3	Conclusions

We have the following observations and proposals to enhance MRO for LTM:

Observation 1-1: Identification of the near failure for TA value may be based on the time interval between successful transmission of RRCReconfigurationComplete message and consequent sending the TA command to UE signalling the TA value which differs from the one communicated to UE via Cell Switch Command message.

Observation 1-2: The network solution is applicable for near failure TA value monitoring.

- Proposal 1-1: RAN3 agrees not to decide on any solution for near failure for TA in Rel. 19 and move the discussion to Rel. 20.

- Proposal 1-2: RAN3 agrees TP for BL CR 38.401 in Annex 1.

# References

- Report of 3GPP TSG RAN3 meeting #128

- Report of 3GPP TSG RAN3 meeting #127bis

- Report of 3GPP TSG RAN2 meeting #130

- Report of 3GPP TSG RAN2 meeting #129bis

# Annex 1: Text proposal to TS 38.401 (MRO for LTM)

First change, ommited text not changed

## 7.X	MRO support for LTM

The target gNB-DU identifies that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch by detecting a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt).The target gNB-DU performs initial analysis and may send the recovery beam information to the source gNB-DU via the gNB-CU.

The target gNB-DU identifies that the UE successfully performs a RACH-based access while reconnecting/re-establishing/recovering to the same target cell.  The target gNB-DU may send used TA value and beam information (in case the beam is different from the beam used during the failed LTM cell switch execution) at the successful RACH based reconnect/re-establish/recovery to the source gNB-DU via the gNB-CU. The source gNB-DU performs root cause analysis to detect RACH-less LTM failure caused by wrong beam selection and outdated TA.

Remaining text not changed