3GPP TSG-RAN WG3 Meeting #129	R3-255288

Bangalore, India, 25-29 August 2025

Agenda item:	20.2

Source:	CEWiT, IITM, Tejas Networks

Title:	Remaining Issues for AI/ML based positioning case 3a and TP to NRPPa

WID/SID:	NR_AIML_air-Core

Document for:	Decision & Approval

# Introduction

The Work Item on artificial intelligence/machine learning for NR air interface has the following objectives on AI/ML positioning [1]:

Provide specification support for the following aspects: - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

The remaining issues on positioning case 3a and case 3b with potential RAN3 specification impact is discussed in this document. The agreements so far achieved in RAN 3 and discussed solution 1(on going positioning based) and solution 2 (NGAP based) are provided in Anex I and Anex II respectively for detailed information.

# Remaining issues on AI/ML positioning case 3a:

In RAN 3 meeting #128, (TP for TS 38.455) Support of data collection procedure (case 3a) (Ericsson) [2] is agreed. There are some open issues that remained to be address in [2]. The are discussed in the below sections

## Reporting Positioning Data Unavailable cause:

For positioning Information collection for UEs under positioning (opportunistic method), a new NRPPa message Positioning Data Collection Report is introduced which provide the positioning information related to data collection (Part B as RAN 1 agreements). But there is possibility that due to some reason LMF may decide not to provide the information to the NG-RAN. This reason could be positioning of the UE is failed (due to internal error or incorrect positioning measurements) or may be UE does not provide the consent for data sharing.

In agreed TP case 3a [2], the Positioning Data Unavailable IE is defined to indicate that the LMF could not provide any positioning information. Encoding of this IE is just Enumerated as “true”, but no cause of unavailability defined. It will be helpful for NG-RAN if the cause is provided by the LMF. gNB can avoid Such UE(s) at least when the reporting is periodic. The Possible causes than can be defined are “positioning data error”, and “UE consent not granted”.

Further the current TP mentions encoding as Enumerated value “True” to indicate the positioning data unavailable but more readable string will be “Unavailable”. Therefore, we propose to update the Enumeration to “Unavailable”.

Proposal 1: The Positioning Data Unavailable IE should provide the cause of the non-availability data. The possible causes could be “positioning data error” and “UE consent not granted”. LMF can select the appropriate cause.

Proposal 2: The encoding of Positioning Data Unavailable IE should be as “ENUMERATED (unavailable, ...)”.

## 2.2 Solution 2-NGAP based method:

For AIML for positioning accuracy improvement, both solutions solution 1 and solution 2 are desirable. Thus, it is expected to standardize both solutions to provide better control at NG-RAN for model training and monitoring in case 3a. Solution 2 is standalone whereas solution 1 being exploring ongoing positioning if available.

SA2 sent the following reply in S2-2505943 to the RAN 3 R3-252485.

For solution 1 it may be desirable to document related LMF procedures in TS 23.273. There are SA2 impacts to support the solution but SA2 could address those impacts as alignment with RAN in Rel-19. For solution 2, SA2 concludes that this solution is not feasible in current frozen Rel-19. SA2 is currently discussing for data collection for training models to be used by the LMF for positioning (RAN case 3b) whether a user consent check and/or a LCS privacy profile check is required. SA2 is anticipating that similar checks could also be required for case 3a (both for solution 1 and solution 2).

SA2 clearly mentioned that solution 2 is not possible to complete in the Rel 19 and need to be completed in Rel 20 to make AIML positioning case 3a to completely specify as SA2 has already frozen the Rel 19 work and has very minimum time for the cross TSG work.

Observation 1: Solution 1 being opportunistic method does not provide control to NG-RAN for the model training and monitoring. Therefore, defining solution 2 is important to complete case 3a related work.

Observation 2: Based on the reply-LS from SA2, it is evident that though SA2 can specify the solution 2, it is not possible to complete in rel. 19 as SA2 Rel 19 specification are frozen. Therefore, it should be carried forward to Rel 20 5G-A.

Proposal 3: Left over items of AIML for positioning accuracy detection case 3a one of the two options should be considered,

Option 1: The NGAP based solution (solution 2 proactive method) should be considered in Rel-20 as leftover of the Rel-19 AI/ML for PHY WID (as a RAN3 only objective).

Option 2: The NGAP based solution (solution 2 proactive method) should be considered as RAN3-led TEI20.

Though the option 1 requires change in WID at RAN and require TU allocation, it will a desirable approach as there is cross TSG impact mainly on SA2 specification. Proper work item objective in the WID will help to coordinate this with SA2 better.

## LoS/NLoS inferred reporting:

In the last meeting, it is agreed to report by the gNB to indicate the result whether inferred using AIML model. To indicate this IE Inferred measurement is introduced in the TRP Measurement Result IE. But whether LoS/NLoS Information to be informed as inferred is still FSS.

The following are the RAN 1 agreements in this regard:

Agreement (RAN1#118bis) From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. - (Optional) LOS/NLOS indicator with legacy format Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. Note: It is up to RAN3 to decide whether the field of LOS/NLOS indicator should be removed or kept. Conclusion (RAN1#119) For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator, - LOS/NLOS indicator can’t be reported independently from other measurements

Based on these agreements it is clear that LoS/NLoS information can be inferred using AIML model but once inferred should be reported along with one or more positioning measurements and not to be reported standalone. This should be made clear in the IE descriptions.

Therefore RAN 3 should allow LoS/NLoS information to be indicated as inferred measurement.

Proposal 4: RAN 3 should allow LoS/NLoS information to be inferred measurement. Accordingly, the BL TP TS 38.455 should be updated as below.

## LoS/NLoS information reporting for data collection:

Further it is clear from RAN 1 agreement that LoS/NLoS information is provided along with other positioning measurements and not standalone. This agreement is from inference perspective but for training purpose gNB can ask the LoS/NLoS information as label alone without the any associated other measurements e.g. UL RTOA or gNB Rx-Tx Time Difference. In the BL TP on TS 38.455 a new IE Positioning Data Information is introduced which includes the LoS/NLoS information but semantics description limits this reporting only if other measurement IE is present e.g. UL RTOA or gNB Rx-Tx Time Difference. This means if gNB indicated only LoS/NLoS information in Positioning Data Collection Needed IE, LMF will discard this request as no other measurement is asked. But according to us RAN 1 agreement does not limit this for data collection for model training purpose. Therefore, we propose to remove this limitation from the agreed TP.

Proposal 5: For LoS/NLoS information reporting from LMF to gNB in POSITIONING DATA COLLECTION REPORT message, standalone LoS/NLoS information reporting for data collection purpose should be supported. The semantic description from BL TP on TS 38.455 (R3-253888) should be corrected accordingly.

# Conclusion

In this contribution, the following observations and proposals are drawn. Also, the TP to TS 38.455 (NRPPa) is provided in section 4.

Reporting Positioning Data Unavailable cause

Proposal 1: The Positioning Data Unavailable IE should provide the cause of the non-availability data. The possible causes could be “positioning data error” and “UE consent not granted”. LMF can select the appropriate cause.

Proposal 2: The encoding of Positioning Data Unavailable IE should be as “ENUMERATED (unavailable, ...)”.

On NGAP based method

Observation 1: Solution 1 being opportunistic method does not provide control to NG-RAN for the model training and monitoring. Therefore, defining solution 2 is important to complete case 3a related work.

Observation 2: Based on the reply-LS from SA2, it is evident that though SA2 can specify the solution 2, it is not possible to complete in rel. 19 as SA2 Rel 19 specification are frozen. Therefore, it should be carried forward to Rel 20 5G-A.

Proposal 3: Left over items of AIML for positioning accuracy detection case 3a one of the two options should be considered,

Option 1: The NGAP based solution (solution 2 proactive method) should be considered in Rel-20 as leftover of the Rel-19 AI/ML for PHY WID (as a RAN3 only objective).

Option 2: The NGAP based solution (solution 2 proactive method) should be considered as RAN3-led TEI20.

LoS/NLoS inferred reporting

Proposal 4: RAN 3 should allow LoS/NLoS information to be inferred measurement. Accordingly, the BL TP TS 38.455 should be updated as below

LoS/NLoS information reporting for data collection

Proposal 5: For LoS/NLoS information reporting from LMF to gNB in POSITIONING DATA COLLECTION REPORT message, standalone LoS/NLoS information reporting for data collection purpose should be supported. The semantic description from BL TP on TS 38.455 (R3-253888) should be corrected accordingly.

# TP to NRPPa BL CR based on agreed TP R3-253888

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< change start>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.1.X	Messages for Positioning Data Collection Transfer Procedures

#### 9.1.X.1	POSITIONING DATA COLLECTION REPORT

This message is sent to report positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Positioning Data Information | O |  | 9.2.Z2 |  | YES | ignore |
| Positioning Data Unavailable | O |  | ENUMERATED(Unavailable, …) |  | YES | ignore |
| Positioning Data Unavailable cause | O |  | ENUMERATED(PosDataError, UEConsent NotGranted, …) | This IE is included only if Postioning Data Unavaible is set to Unavailable. | Yes | ignored |

Next change

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |
| >Inferred measurement | O |  | ENUMERATED(true, …) | This IE is only valid for UL-RTOA, gNB Rx-Tx Time Difference. | YES | ignore |
| >Inferred LoS/NLoS Information | C-ifInferred measurementtrue |  | ENUMERATED(true, …) | LoS/NLoS Information is included only if UL RTOA or gNB Rx-Tx Time Difference is provided | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoaggregatedPosSRS-Resources | Maximum no of aggregated Positioning SRS resources per UL BWP. Value is 3. |

Next change

### 9.2.Z1	Positioning Data Collection Needed

This information element contains the requested positioning information.

| IE/Group Name |  | Presence |  | Range |  | IE Type and Reference |  | Semantics Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Requested Positioning Data Collection |  | M |  |  |  | BIT STRING (SIZE(8)) |  | Each position in the bitmap represents a requested positioning information: first bit: UL-RTOA Second bit: gNB Rx-Tx Time Difference Third bit: LoS/NLoS information Value ‘1’ indicates ‘needed’, Value ‘0’ indicates ‘not needed’. Other bits reserved for future use. |

### 9.2.Z2	Positioning Data Information

This information element contains positioning information for data collection.

| IE/Group Name |  | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| TRP Measurement List |  |  | 0..1 |  | Indicates the area of interest. |
| >TRP Measurement Item |  |  | 1..<maxnoofMeasTRPs> |  |  |
| >>TRP ID |  | M |  | 9.2.24 |  |
| >>UL RTOA |  | O |  | 9.2.39 |  |
| >>gNB Rx-Tx Time Difference |  | O |  | 9.2.40 |  |
| >>LoS/NLoS Information |  | O |  | 9.2.77 |  |
| >>Time Stamp |  | M |  | 9.2.42 |  |
| >>Measurement Quality |  | O |  | 9.2.43 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< change end>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

# Reference

[1]: RP-251186, WID update of AIML for PHY, RAN#108, Prague, Czech Republic, June 9-13, 2025

[2]: R3-253888, (TP for TS 38.455) Support of data collection procedure (case 3a) (Ericsson)

[3] S2-2505943 Reply LS on AI/ML Positioning Case 3a

[4] RAN3 #127-bis Meeting Report Wuhan, China, 07/04/2025 - 11/04/2025

# Annex I

### RAN 3 Agreements:

RAN3#125bis The existing LCS framework is re-used for AI/ML positioning. Case 3b: Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1. Case 3a: The AI/ML model (inference and training functions) is at the gNB. LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1. WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. RAN3#126 Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed. FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1). FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified. Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used. FFS if LMF should be aware that the reported measurements are AI/ML generated. RAN3#127 Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement. For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner. For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs. WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB. RAN3#127bis Case 3a data collection: An NRPPA based solution could be used to collect UE location information in an opportunistic way, i.e. from UEs that are being positioned. FFS on Whether to introduce an NGAP based procedure to collect UE location information in a proactive way, i.e. from UEs that are being served by the NG-RAN. Pending feedback from SA2. RAN3#128 Define the Positioning Data Collection Needed IE as a new bitmap. The requested information in the bitmap is timing measurement (UL-TDOA, gNB Rx-Tx Time Difference) together with optional LoS/NLoS indicator. FFS if UE location can be requested. The Positioning Data Collection Needed IE is introduced in the MEASUREMENT REPORT message. Introduce POSITIONING DATA COLLECTION REPORT message as class 2 procedure including Positioning Data Information IE and RAN/LMF Measurement IDs Introduce an optional Positioning Data Unavailable IE (FFS on encoding) when the Positioning Data Information IE is absent in the POSITIONING DATA COLLECTION REPORT message. The codepoints indicating the reason. RAN3 confirms that Performance monitoring is done in the RAN in case 3a. RAN3 confirms the WA: For case 3a data collection, Part A is known internally by the gNB and is not signalled over any network interface. Introduce the new flag “Inferred measurement” in the TRP Measurement Result to align with RAN1 conclusion

# Annex 2:

### 2.1 Proposed solution 1 in RAN 3 LS:

Solution 1: Positioning Information collection for UEs under positioning

- The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to positioning procedures. - The gNB(s) determines that data collection is needed for the UE being positioned. - The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned. Note: Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [35]. - The LMF sends the (new) NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated that positioning data collection is needed for the UE being positioned. The message includes information related to UE location and correlation information that enables the gNB to correlate the information related to UE location with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID).

### 2.1 Proposed solution 2 in RAN 3 LS:

Solution 2: Positioning Information collection for UEs served by the NG-RAN which are not being positioned.

- The gNB wishes to receive positioning information, consisting of labels (e.g. UE location information) to form a data set for training. Based on gNB implementation, the gNB decides when/if to trigger a (new) NG: POSITIONING INFORMATION REQUEST message to the serving AMF of the served UE(s).

- The POSITIONING INFORMATION REQUEST message is an NGAP non-UE associated message containing e.g. the NGAP UE IDs of the UEs for which positioning information is required or other criteria.

- The AMF receives the request message from the NG-RAN node. If the requested information is already available at AMF as part of UE positioning context, the AMF responds directly with the message in step 6. Otherwise, it selects an LMF to request positioning of each UE for Positioning Information collection purposes. 
The AMF triggers a Positioning Information Request towards the LMF (it could consist of the (legacy) Nlmf_Location_DetermineLocation service operation) to request the current location of the UE.

- The LMF performs one or more of the positioning procedures described in clause 6.11.1, 6.11.2 and 6.11.3 of TS 23.273.

- The LMF returns the requested information towards the AMF. The message Nlmf_Location_DetermineLocation Response might be reused.

- The AMF returns the requested label(s) to the NG-RAN node in a POSITIONING INFORMATION RESPONSE message. The AMF may return instead a failure message with appropriate cause value if the procedure does not succeed.

- The gNB combines the received label with the TRP measurements to form the data needed.